MATHEMATICS AND MEASUREMENT

for Electrical Engineering

Book 2 of the Electrical Engineering Foundations Series

The units, algebra, trigonometry, complex numbers, and calculus that the rest of the series assumes you have.

# Preface — How to Use This Book

This is the second book in a series that walks through electrical engineering from the ground up. The first book covered the physics — charge, fields, potential, current, magnetism, induction. This one covers the mathematical language in which all of that physics, and everything built on top of it, is actually written down.

That framing matters, because it explains what this book is not. It is not a mathematics course. A mathematics course would prove things; this book explains things, and proves only when the proof is the explanation. It is not comprehensive, either — there is a great deal of beautiful mathematics that electrical engineers never use, and none of it is here. What is here is the specific, and surprisingly small, body of mathematics that shows up again and again once you start analysing circuits and signals: unit algebra, logarithms and decibels, sinusoids, complex numbers, phasors, and just enough calculus to know what a derivative and an integral are doing in an equation when you meet one.

The book assumes you can do ordinary secondary-school algebra — rearrange an equation, factor a quadratic, handle a fraction without panic. It does not assume trigonometry beyond the definitions, it does not assume you have met complex numbers, and it does not assume any calculus at all. Chapters 12 builds calculus from scratch, at the level of "what is this symbol telling me to do," which is the level at which practising engineers actually use it.

Each chapter follows the same shape as the rest of the series. It opens with the question the chapter answers, builds the idea in words first, then introduces the notation needed to make the idea precise, and closes with worked examples and practice problems. Three kinds of boxed panel appear throughout. "Key Idea" panels flag the handful of facts in each chapter worth memorising outright. "Worked Example" panels take a representative problem all the way to a number, showing the reasoning at each step rather than just the arithmetic. "Common Misconception" panels address specific wrong turns that show up reliably among people meeting this material for the first time — the aim being to catch them before they calcify into habits that are much harder to break later.

A note on notation. Scalar quantities are italic (V, I, R). Units are upright and never italic (V, A, Ω) — the distinction between the quantity V and the unit V is carried entirely by the typeface, which is a genuine nuisance and the reason context matters so much when reading equations. Complex quantities are introduced in Chapter 9 and are written in bold in later books of this series, but in this book they are written plainly, because the whole point of Chapters 9 through 11 is to make you comfortable with them as ordinary numbers rather than as special objects requiring special decoration.

A note on the practice problems. Every chapter ends with four, and full solutions to all of them are in Appendix B. The solutions are worth more to you than the answers, so resist looking until you have made a real attempt — the value of a worked solution lies almost entirely in comparing it against your own reasoning and noticing where the two diverged. A solution read passively teaches very little; the same solution, attempted first and then checked, teaches a great deal.

# About This Series

This book is the second of a sequence covering electrical engineering from first principles through to the specialised fields where practising engineers eventually settle. The sequence is arranged so each stage rests on the ones before it. It is worth knowing the overall shape before starting, if only so an unfamiliar term can be filed away as "covered later" rather than mistaken for a gap in this particular book.

## Foundations

Three books: the physics of electricity and magnetism (Book 1), the mathematics and measurement conventions in this volume (Book 2), and DC and AC circuit theory (Book 3), which brings the first two together into Kirchhoff's laws, Thevenin and Norton equivalents, and the systematic analysis of networks built from resistors, capacitors, and inductors.

## Semiconductor Physics and Devices

With circuit theory in hand, the series turns to the devices themselves: crystal structure and band theory, the PN junction, the family of diodes, bipolar transistors and MOSFETs, then the compound-semiconductor and advanced-device technologies used where silicon alone falls short, and the fabrication and layout techniques used to manufacture them.

## RF, Microwave, and mmWave Engineering

A substantial part of the series covers high-frequency work: transmission lines, the Smith chart, S-parameters, the RF building blocks assembled from those devices, noise as a subject in its own right, modulation and communication theory, oscillators and frequency synthesis, and dedicated treatments of microwave and millimetre-wave engineering, where physical wavelength becomes comparable to circuit dimensions and lumped-element intuition stops working.

## Antennas, EMC, and Wireless Systems

From transmission lines the series proceeds to antennas and radiation, then electromagnetic compatibility and the regulatory standards every real product must meet, then the wireless systems and protocols — Wi-Fi, Bluetooth, cellular, radar — that put all of it to use.

## Signal Processing, Power, and Control

A parallel track covers digital signal processing and software-defined radio, electrical power systems, and control and automation theory, ending with robotics.

## Tools, Practice, and Emerging Topics

The series closes with volumes on simulation tools and methods, debugging and validation practice, PCB layout and packaging, and a survey of emerging technologies.

Key Idea

This book is the one you come back to. Books 1 and 3 are read once, in order. This one is a reference you will re-open — for a unit conversion, a decibel calculation, a trigonometric identity, or a reminder of what j actually means. Appendices A and C are built for that purpose: Appendix A collects every formula in the book on a few pages, and Appendix C defines every term.

# Table of Contents

Preface — How to Use This Book

About This Series

Chapter 1 — Quantities, Units, and the SI System

Chapter 2 — Prefixes, Scientific Notation, and Engineering Notation

Chapter 3 — Precision, Significant Figures, and Uncertainty

Chapter 4 — Algebra for Circuit Analysis

Chapter 5 — Systems of Linear Equations

Chapter 6 — Functions, Graphs, and Limiting Behaviour

Chapter 7 — Exponentials, Logarithms, and the Decibel

Chapter 8 — Trigonometry and the Sinusoid

Chapter 9 — Complex Numbers

Chapter 10 — Euler's Formula and the Complex Exponential

Chapter 11 — Phasors and Impedance

Chapter 12 — Calculus for Electrical Engineering

Chapter 13 — Where This Book Leads Next

Appendix A — Formula Reference

Appendix B — Practice Problem Solutions

Appendix C — Glossary

Appendix D — Chapter Summaries

# Chapter 1 — Quantities, Units, and the SI System

Every number in engineering is a number of something. This sounds too obvious to state, and yet the single most common source of catastrophic error in engineering calculation is a number that lost track of what it was a number of. This chapter is about keeping that attachment intact.

## 1.1 Quantity, Numerical Value, and Unit

A physical quantity is written as a numerical value multiplied by a unit. When we write R = 4.7 kΩ, we are making a multiplicative statement: the resistance equals 4.7 times the quantity "one kilohm." The multiplication is real, not notational, and this single realisation is what makes unit algebra work.

Because the multiplication is real, units obey ordinary algebraic rules. They multiply, they divide, they cancel. If a current of 2 A flows through a resistance of 3 Ω, then

V = IR = (2 A)(3 Ω) = 6 A·Ω = 6 V

and the last step is not a convention but an identity: the volt is defined so that one ampere times one ohm is one volt. Every derived SI unit works this way. Learning to carry units through a calculation, rather than dropping them and reattaching them at the end, converts a whole class of silent errors into loud ones.

## 1.2 The Seven Base Units

The SI system defines seven base quantities and gives each a base unit. Everything else is built from them by multiplication and division.

| Quantity | Unit | Symbol |
| --- | --- | --- |
| Length | metre | m |
| Mass | kilogram | kg |
| Time | second | s |
| Electric current | ampere | A |
| Thermodynamic temperature | kelvin | K |
| Amount of substance | mole | mol |
| Luminous intensity | candela | cd |

Electrical engineering uses four of these constantly — metre, kilogram, second, ampere — and the older name "MKSA system" for that subset still appears in older texts. Note that the base unit of mass is the kilogram, not the gram; the prefix is built into the base unit, which is a historical wrinkle with no deeper meaning and occasionally trips up dimensional bookkeeping.

Since 2019 all seven are defined by fixing the numerical values of seven physical constants — the speed of light, the Planck constant, the elementary charge, and so on — rather than by reference to physical artefacts. The ampere, for instance, is now defined by fixing the elementary charge at exactly 1.602176634 × 10⁻¹⁹ C. This matters philosophically and to national measurement institutes; it changes nothing about how you calculate.

## 1.3 Derived Units in Electrical Engineering

The units you will actually use every day are derived: each is a specific combination of base units that has been given its own name because the combination occurs so often.

| Quantity | Unit | Symbol | In base units |
| --- | --- | --- | --- |
| Charge | coulomb | C | A·s |
| Voltage | volt | V | W/A = kg·m²/(A·s³) |
| Resistance | ohm | Ω | V/A |
| Conductance | siemens | S | A/V |
| Capacitance | farad | F | C/V |
| Inductance | henry | H | Wb/A |
| Magnetic flux | weber | Wb | V·s |
| Frequency | hertz | Hz | 1/s |
| Power | watt | W | J/s |
| Energy | joule | J | W·s |

Two entries deserve comment. The hertz is defined as one per second, and so is the becquerel, and so — dimensionally — is the radian per second; they are dimensionally identical and semantically distinct, which is why you cannot simply cancel Hz against rad/s without thinking. And the siemens is the reciprocal ohm; you will occasionally still meet its charming obsolete name, the mho, written with an upside-down omega.

## 1.4 Dimensional Analysis as an Error Check

Because units multiply and divide algebraically, any equation you write must balance dimensionally: the units on the left must reduce to the same combination of base units as the units on the right. If they do not, the equation is wrong, and you have found the error without knowing anything about the physics.

This is the cheapest debugging tool in engineering and it is badly underused. Consider the time constant of an RC circuit, τ = RC. Is that dimensionally sensible? Resistance is V/A and capacitance is C/V, so

RC = (V/A)(C/V) = C/A = (A·s)/A = s

which is a time. Good. Had you misremembered the formula as R/C, the same check would have produced units of V²/(A·C) = V²/(A²·s), which is not a time, and the error would have surfaced immediately.

Worked Example 1.1

Verify that the resonant frequency formula f = 1/(2π√(LC)) has units of hertz.

The constant 2π is dimensionless, so we need only the units of 1/√(LC).

Inductance: H = Wb/A = (V·s)/A. Capacitance: F = C/V = (A·s)/V.

Their product: LC = [(V·s)/A]·[(A·s)/V] = s². The volts cancel, the amperes cancel, and we are left with seconds squared.

Therefore √(LC) has units of seconds, and 1/√(LC) has units of 1/s, which is the hertz. The formula is dimensionally sound.

Notice what this check does and does not tell you. It confirms the structure of the formula. It cannot confirm the factor of 2π, because 2π is dimensionless — dimensional analysis is blind to every dimensionless constant. A formula can be dimensionally perfect and still numerically wrong by a factor of 2π, which is in fact one of the more common errors in resonance calculations.

Key Idea

Carry units through every calculation. Do not strip them at the start and reattach them at the end. Units that fail to cancel correctly are telling you the equation is wrong, and they are telling you before you have wasted an hour on the arithmetic.

## 1.5 Units That Are Not Really Units

Some quantities are ratios of two quantities with the same units, and therefore dimensionless. Gain, efficiency, power factor, reflection coefficient, and the Q factor of a resonant circuit are all pure numbers. They are often given a pseudo-unit anyway — "dB", "%", "dimensionless" — as a reminder of what kind of ratio they are.

The radian is the most important of these. An angle in radians is defined as an arc length divided by a radius: metres over metres, dimensionless. This is precisely why angular frequency ω, measured in radians per second, has base units of 1/s, identical to frequency f in hertz — and precisely why ω = 2πf is not a unit conversion but a genuine numerical relationship between two different quantities that happen to share a dimension.

Common Misconception

It is tempting to treat radians per second and hertz as interchangeable because they share the same base units, and to convert between them by "cancelling" the radian. They are not interchangeable. A signal at 1 kHz has ω = 6283 rad/s, not 1000 rad/s. The factor of 2π between them is real and physical: it is the number of radians in one full cycle. The radian being dimensionless means the units cannot catch this error for you — one of the few places where dimensional analysis offers no protection, and correspondingly one of the places where the mistake is most common.

### Practice Problems

- A capacitor of 100 nF is charged to 12 V. Using C = Q/V, find the stored charge, and express your answer in coulombs and in nanocoulombs.

- Show by dimensional analysis that the quantity L/R has units of time.

- The energy stored in an inductor is given by W = ½LI². Verify that this expression has units of joules.

- A student writes the impedance of a capacitor as Z = ωC. Use dimensional analysis to show this cannot be correct, and determine what the correct expression must look like dimensionally.

# Chapter 2 — Prefixes, Scientific Notation, and Engineering Notation

Electrical engineering spans an absurd range of magnitudes. The gate capacitance of a modern transistor is measured in femtofarads; the capacitance of a power-factor correction bank is measured in millifarads. That is twelve orders of magnitude, in the same quantity, in the same field. This chapter is about writing such numbers without drowning in zeros.

## 2.1 The SI Prefixes

A prefix multiplies the unit it is attached to by a power of ten. The prefixes used routinely in electrical engineering are these.

| Prefix | Symbol | Factor |
| --- | --- | --- |
| tera | T | 10¹² |
| giga | G | 10⁹ |
| mega | M | 10⁶ |
| kilo | k | 10³ |
| (none) |  | 10⁰ |
| milli | m | 10⁻³ |
| micro | µ | 10⁻⁶ |
| nano | n | 10⁻⁹ |
| pico | p | 10⁻¹² |
| femto | f | 10⁻¹⁵ |

Case is not decorative. A capital M is mega, a lower-case m is milli, and they differ by a factor of a billion. A capital K is not a valid prefix at all — kilo is always lower-case k — while a capital K is the symbol for the kelvin. Writing 5 KΩ instead of 5 kΩ is a small sin; writing 5 mA when you meant 5 MA is a career-defining one.

Note also that the prefix binds tighter than the exponent. The expression mm² means (mm)², that is, 10⁻⁶ m², not 10⁻³ m². This catches people constantly in current-density and sheet-resistance calculations.

## 2.2 Scientific Notation

Scientific notation writes every number as a value between 1 and 10, multiplied by a power of ten: 47000 becomes 4.7 × 10⁴, and 0.00033 becomes 3.3 × 10⁻⁴. The mantissa always has exactly one digit before the decimal point.

Its virtue is unambiguity. The number 4.70 × 10⁴ states plainly that three digits are meaningful, where the plain form 47000 leaves you guessing whether the trailing zeros are measured or merely placeholders — a question Chapter 3 takes up in earnest.

## 2.3 Engineering Notation

Engineering notation is a variant that restricts the exponent to multiples of three, so that the exponent always corresponds to an SI prefix. Instead of 4.7 × 10⁴ Ω you write 47 × 10³ Ω, and read it directly as 47 kΩ.

This is what practising engineers actually use, and it is what calculators mean by their "ENG" mode. The reason is entirely practical: component values, datasheet entries, and instrument displays are all expressed with prefixes, so a notation whose exponents map one-to-one onto those prefixes eliminates a translation step, and with it a class of error.

Worked Example 2.1

A current of 0.0000047 A flows through a resistance of 2200000 Ω. Express both in engineering notation, then find the voltage.

The current: 0.0000047 A = 4.7 × 10⁻⁶ A = 4.7 µA.

The resistance: 2200000 Ω = 2.2 × 10⁶ Ω = 2.2 MΩ.

Now the voltage, keeping the powers of ten explicit:

V = IR = (4.7 × 10⁻⁶)(2.2 × 10⁶) = (4.7 × 2.2) × 10⁻⁶⁺⁶ = 10.34 × 10⁰ = 10.34 V.

The exponents cancelled exactly, which is the useful thing to notice: micro times mega is unity. Similar pairings are worth committing to memory — milli times kilo is unity, nano times giga is unity, pico times tera is unity. Once you internalise these, a great many circuit calculations can be done in your head, because the prefixes annihilate and you are left with a one-digit multiplication.

Key Idea

Reciprocal prefix pairs cancel. µ×M = 1, m×k = 1, n×G = 1, p×T = 1. A resistance in kilohms times a current in milliamps gives a voltage in volts. A capacitance in microfarads times a resistance in megohms gives a time constant in seconds. Recognising these pairings is the single biggest speed-up available in routine circuit arithmetic.

## 2.4 Arithmetic With Powers of Ten

Multiplication adds exponents; division subtracts them. Raising to a power multiplies the exponent. Taking a square root halves it — which is why square roots in engineering notation are awkward, since halving an exponent that is a multiple of three rarely lands on another multiple of three.

Addition and subtraction are the operations that catch people. You cannot add mantissas unless the exponents match. To compute 4.7 × 10³ + 2.2 × 10², first rewrite the second term as 0.22 × 10³, then add mantissas: 4.92 × 10³. Attempting to add 4.7 and 2.2 directly gives an answer wrong by nearly a factor of two.

Common Misconception

A frequent error is to assume that because prefixes are convenient, they can be mixed freely inside a single expression. They cannot. Adding 4.7 kΩ to 220 Ω requires converting one to match the other — either 4.7 kΩ + 0.22 kΩ = 4.92 kΩ, or 4700 Ω + 220 Ω = 4920 Ω. Writing "4.7 k + 220" and reading off 4.92 works only because you performed the conversion silently in your head; write it out, because the day you do it silently and get it wrong is the day it costs you.

### Practice Problems

- Express the following in engineering notation with an appropriate prefix: 0.000000015 F, 68000 Ω, 0.0025 A, 1500000000 Hz.

- A 4.7 kΩ resistor carries 3.2 mA. Find the voltage across it and the power dissipated in it, using prefix cancellation rather than long decimals.

- Add the following capacitances, giving the result in nanofarads: 470 pF, 1.2 nF, and 0.01 µF.

- A signal has a period of 20 ns. Find its frequency, and express the answer with an appropriate prefix. Then find the angular frequency ω in rad/s.

# Chapter 3 — Precision, Significant Figures, and Uncertainty

A measurement is not a number; it is a number together with a statement of how well it is known. An oscilloscope that reads 5.02 V is not telling you the voltage is 5.02 V. It is telling you the voltage is near 5.02 V, with a spread that depends on the instrument, the probe, the temperature, and the day. This chapter is about carrying that spread through a calculation honestly.

## 3.1 Significant Figures

The significant figures of a number are the digits that carry real information. The rules are mechanical: all non-zero digits are significant; zeros between non-zero digits are significant; leading zeros are never significant; trailing zeros after a decimal point are significant.

So 0.00470 has three significant figures — the leading zeros are placeholders, the trailing zero is a claim of precision. The genuinely ambiguous case is a whole number ending in zeros: 4700 might have two, three, or four significant figures, and the plain notation cannot tell you which. Scientific notation resolves this completely, which is one of its main practical virtues: 4.7 × 10³ has two, 4.70 × 10³ has three, 4.700 × 10³ has four.

## 3.2 Propagating Precision Through a Calculation

Two rules cover most cases. For multiplication and division, the result carries as many significant figures as the least precise input. For addition and subtraction, the result carries as many decimal places as the input with the fewest decimal places.

The rules differ because the operations degrade precision differently, and it is worth seeing why rather than memorising both. Multiplication scales relative error; addition scales absolute error. If you multiply a quantity known to 1% by one known to 10%, the answer is known to roughly 10%, regardless of the units involved. If you add a quantity known to ±0.5 V to one known to ±0.001 V, the sum is known to ±0.5 V — the precise measurement is swamped, and no amount of decimal places in the second number rescues it.

Common Misconception

The most common error with significant figures is rounding at every intermediate step. Do not. Carry full precision through the calculation and round only the final answer. Rounding intermediate values injects error at each step, and those errors accumulate — in a long chain of calculations, aggressive intermediate rounding can easily shift a final answer by more than the measurement uncertainty you were trying to represent. The significant-figure rules describe how to report a result, not how to compute one.

## 3.3 Absolute and Relative Uncertainty

Absolute uncertainty is expressed in the units of the quantity: 5.02 V ± 0.05 V. Relative uncertainty is the same thing as a fraction or percentage of the value: 5.02 V ± 1%. Converting between them is straightforward, and which one you want depends on the operation you are about to perform.

For products and quotients, relative uncertainties add. If R is known to ±2% and I is known to ±3%, then V = IR is known to roughly ±5%. For sums and differences, absolute uncertainties add. This is the formal version of the observation in the previous section, and it has a sharp practical consequence.

Worked Example 3.1

Two voltages are measured as V₁ = 10.0 V ± 0.1 V and V₂ = 9.8 V ± 0.1 V. Find the difference and its uncertainty, as an absolute figure and as a percentage.

The difference: V₁ − V₂ = 10.0 − 9.8 = 0.2 V.

The absolute uncertainties add: ±0.1 + ±0.1 = ±0.2 V.

So the difference is 0.2 V ± 0.2 V — a relative uncertainty of 100%.

Each individual measurement was good to 1%. Their difference is good to 100%. Nothing went wrong in the measurement; this is what subtracting two nearly equal numbers does. The signal is the small residue between two large quantities, and the uncertainty of each large quantity survives into that small residue undiminished.

This phenomenon has a name — catastrophic cancellation — and it is a permanent hazard. It is why four-wire (Kelvin) resistance measurement exists, why instrumentation amplifiers are built the way they are, and why a bridge circuit that measures a small imbalance directly will always beat a scheme that measures two large quantities and subtracts them.

Key Idea

Subtracting nearly equal numbers destroys precision. The absolute uncertainty is preserved while the value shrinks, so the relative uncertainty explodes. Whenever a design calls for the small difference between two large measured quantities, look for a way to measure the difference directly instead.

## 3.4 Accuracy, Precision, and Resolution

These three words are not synonyms, and instrument datasheets depend on the distinction.

- Accuracy is how close a measurement is to the true value. It is limited by calibration, and it drifts with time and temperature.

- Precision (or repeatability) is how close repeated measurements are to each other. An instrument can be precise and inaccurate: reading 4.512 V every time when the true value is 5.000 V.

- Resolution is the smallest change the instrument can display. A six-digit meter has fine resolution, which says nothing whatever about whether those six digits are accurate. Resolution is free; accuracy is expensive.

A typical bench multimeter specification reads something like "±(0.5% of reading + 3 counts)". The first term is a relative error that scales with the measurement; the second is an absolute error, in units of the last displayed digit, that dominates at the low end of a range. This is why measuring 0.1 V on a 1000 V range is a poor idea even though the meter will happily display a number.

## 3.5 Component Tolerance

Real components differ from their marked values. A resistor marked 4.7 kΩ with a 5% tolerance is guaranteed only to lie between 4.465 kΩ and 4.935 kΩ. This is not a defect; it is the specification, and designs must work across the whole range.

The standard E-series values — E12, E24, E96 — are chosen as geometric progressions so that consecutive values are spaced by roughly the tolerance. E24 steps by about 10%, matching 5% parts, so that the tolerance bands of adjacent values just meet without large gaps or heavy overlap. This is why the odd-looking sequence 10, 12, 15, 18, 22, 27, 33, 39, 47, 56, 68, 82 exists: it is a logarithmic ladder, not an arbitrary list, and once you see that, the values stop looking random.

Worst-case analysis assumes every component sits at the tolerance extreme that hurts most. It is pessimistic — the probability of every part being simultaneously worst-case is negligible — but it is the honest way to establish that a design cannot fail, as opposed to establishing that it usually works.

### Practice Problems

- State the number of significant figures in each: 0.00250, 1200, 1.200 × 10³, 100.0, 0.0001.

- A resistance of 4.7 kΩ ±5% carries a current of 2.0 mA ±2%. Find the voltage across it and its percentage uncertainty.

- Two lengths are measured as 100.0 mm ± 0.2 mm and 99.5 mm ± 0.2 mm. Find their difference with its absolute and percentage uncertainty, and comment on the result.

- A 4½-digit multimeter with a specification of ±(0.05% of reading + 2 counts) is used on its 20 V range, where the last digit represents 1 mV. Find the total uncertainty when measuring 10.000 V, and again when measuring 0.100 V. Express each as a percentage of the reading.

# Chapter 4 — Algebra for Circuit Analysis

Circuit analysis produces equations, and equations must be rearranged. The algebra involved is not difficult, but a few specific manipulations recur so often in electrical engineering that they are worth practising until they are automatic — because an engineer who has to stop and think about them loses the thread of the circuit while doing so.

## 4.1 Rearranging for Any Variable

Ohm's law, V = IR, is three formulas in one. Solving for each in turn gives I = V/R and R = V/I. Every constitutive relation in this series works the same way, and you should be able to move between forms without hesitation.

The general procedure is to isolate the target variable by applying inverse operations to both sides, working outward from the target. What makes engineering formulas occasionally awkward is that the target often appears inside a reciprocal, inside a square root, or in more than one place at once.

Worked Example 4.1

The parallel resistance of two resistors is 1/R = 1/R₁ + 1/R₂. Given R and R₁, solve for R₂.

Start by isolating the term containing R₂:

1/R₂ = 1/R − 1/R₁

Combine the right-hand side over a common denominator:

1/R₂ = (R₁ − R)/(R·R₁)

Now invert both sides — and this is the step people rush:

R₂ = (R·R₁)/(R₁ − R)

Inverting both sides of an equation is legitimate provided neither side is zero, but note that you must invert each side as a whole. A frequent error is to invert term by term, turning 1/R − 1/R₁ into R − R₁, which is wrong. The reciprocal of a difference is not the difference of the reciprocals.

Sanity check the result: if R₁ = R, the denominator vanishes and R₂ is infinite, which is correct — if the parallel combination already equals one of the resistors, the other must be an open circuit.

Common Misconception

The reciprocal of a sum is not the sum of the reciprocals. 1/(a + b) ≠ 1/a + 1/b. This is the single most common algebraic error in parallel-impedance work, and it is worth testing yourself on numbers once: 1/(2 + 2) = 0.25, while 1/2 + 1/2 = 1. The two are not close. The parallel-resistance formula is stated as 1/R = 1/R₁ + 1/R₂ precisely because it is the reciprocals that add, not the resistances.

## 4.2 The Product-Over-Sum Shortcut

For exactly two resistors in parallel, the formula above rearranges to a form worth memorising:

R = R₁R₂ / (R₁ + R₂)

This is faster than the reciprocal form for hand calculation, and it generalises to any two impedances in parallel, which is why it recurs constantly in later books. Its limitation is that it applies to exactly two elements. For three or more you must either use the reciprocal form or apply product-over-sum repeatedly, combining two at a time.

Two special cases are worth having at your fingertips. Two equal resistors in parallel give half the value. A resistor in parallel with one ten times larger gives approximately 91% of the smaller value — near enough to the smaller value that in a first-pass estimate the larger one can often be ignored entirely.

## 4.3 Ratios, Dividers, and Fractions

The voltage divider is the most-used formula in electronics, and it is pure algebra:

V_out = V_in · R₂/(R₁ + R₂)

What makes it worth dwelling on is the structure: the output is the input scaled by a dimensionless ratio between zero and one. Recognising this shape lets you read a divider off a schematic without writing anything down. If the two resistors are equal, the output is half the input. If R₂ is nine times R₁, the output is nine-tenths of the input.

The current divider has the complementary structure, and the thing to notice is that the resistances swap:

I₁ = I_total · R₂/(R₁ + R₂)

Current through R₁ is proportional to the other resistance. This is not a typographical accident — current preferentially takes the path of lower resistance, so the branch current is larger when its own resistance is smaller, which requires the opposite resistance in the numerator.

Key Idea

Voltage divides in proportion to resistance; current divides in inverse proportion. The voltage-divider numerator is the resistance you are measuring across. The current-divider numerator is the other resistance. Mixing these up is the most common error in first-year circuit analysis, and the check is always the limiting case: as one resistance goes to zero, does the formula give what physics demands?

## 4.4 Quadratics and Where They Come From

Quadratic equations appear in circuit work whenever power is involved, since power is proportional to the square of voltage or current, and whenever a second-order system's characteristic equation is written down. The solution is the familiar formula:

x = [−b ± √(b² − 4ac)] / 2a

The discriminant b² − 4ac carries the physical meaning. Positive discriminant gives two distinct real roots — in a second-order circuit, the overdamped case. Zero gives one repeated root, the critically damped case. Negative gives two complex conjugate roots, the underdamped case, which oscillates. Chapters 9 and 10 make the last of these tractable; for now, note only that a negative discriminant is not an error but a signal that the system rings.

## 4.5 Approximations Worth Knowing

Engineers approximate constantly, and two approximations do most of the work. The first is the binomial approximation: for small x,

(1 + x)ⁿ ≈ 1 + nx

So √(1 + x) ≈ 1 + x/2 and 1/(1 + x) ≈ 1 − x when x is small. "Small" here means roughly below 0.1, where the error stays under about 1%.

The second is the observation that when two quantities differ greatly in size, the smaller can often be dropped. In R₁ + R₂ with R₂ a hundred times R₁, dropping R₁ costs 1%. Whether 1% matters depends on the tolerance of the components, which is exactly the kind of judgement Chapter 3 was preparing you to make: there is no point computing to 0.01% with 5% resistors.

### Practice Problems

- Solve V = IR for R, and evaluate for V = 3.3 V and I = 150 µA.

- Three resistors of 1 kΩ, 2 kΩ, and 3 kΩ are connected in parallel. Find the equivalent resistance, first with the reciprocal formula and then by applying product-over-sum twice, and confirm the two agree.

- A voltage divider must produce 2.5 V from a 9 V supply. If R₂ = 10 kΩ, find R₁.

- Use the binomial approximation to estimate √(1.06) and 1/0.98, then compare each with the exact value and state the percentage error.

# Chapter 5 — Systems of Linear Equations

Nodal and mesh analysis, introduced in Book 3, turn any resistive circuit into a system of simultaneous linear equations. The circuit theory is the interesting part; solving the system is mechanical. This chapter supplies the mechanics so that Book 3 can concentrate on the circuits.

## 5.1 What a Linear System Is

A linear equation in several unknowns contains those unknowns only to the first power, multiplied by constants and added. No squares, no products of unknowns, no unknowns inside functions. Two equations in two unknowns look like this:

a₁x + b₁y = c₁     and     a₂x + b₂y = c₂

Circuit equations from Kirchhoff's laws are always of this form when the components are resistors and the sources are independent, which is why the systematic methods work at all. Nonlinear elements — diodes, transistors — break linearity and require entirely different techniques, taken up later in the series.

## 5.2 Substitution and Elimination

For two equations, substitution is usually quickest: solve one equation for one unknown, put that expression into the other, solve the resulting single-variable equation, then back-substitute.

Elimination scales better. Multiply the equations by whatever constants make the coefficients of one unknown equal in magnitude, then add or subtract to eliminate it. For three or more equations, elimination performed systematically becomes Gaussian elimination, which is what every circuit simulator does internally.

Worked Example 5.1

Solve the system: 3x + 2y = 16 and 5x − y = 9.

Elimination is convenient here because y has a coefficient of −1 in the second equation. Multiply the second equation by 2:

10x − 2y = 18

Add this to the first equation, and the y terms cancel:

(3x + 10x) + (2y − 2y) = 16 + 18, so 13x = 34, giving x = 2.615.

Back-substitute into the second original equation:

5(2.615) − y = 9, so 13.077 − y = 9, giving y = 4.077.

Always verify in the equation you did not use for back-substitution. First equation: 3(2.615) + 2(4.077) = 7.846 + 8.154 = 16.000. Correct.

That verification step costs ten seconds and catches arithmetic slips reliably. In circuit work, where a sign error in one Kirchhoff equation propagates into every node voltage, it is time exceptionally well spent.

## 5.3 Determinants and Cramer's Rule

For a two-by-two system, the determinant is defined as

D = a₁b₂ − a₂b₁

and Cramer's rule gives the solution directly: x = Dₓ/D and y = D_y/D, where Dₓ is the determinant formed by replacing the x-coefficient column with the constants, and D_y likewise for y.

Cramer's rule is not the fastest method for large systems — its cost grows factorially, which makes it useless beyond about three equations — but for two or three unknowns it is quick, mechanical, and hard to get lost in. Its real value is diagnostic. If D = 0, the system has no unique solution, and in a circuit context that almost always means a modelling error: two ideal voltage sources placed in parallel with different values, an ideal current source left in series with an open circuit, or a floating section with no reference to ground.

Key Idea

A zero determinant is a physics error, not an arithmetic one. When the determinant of a circuit's node equations vanishes, stop and re-examine the circuit rather than the algebra. You have almost certainly written down something physically impossible — most often a node with no DC path to the reference node.

## 5.4 Matrix Form and What Comes Later

Any linear system can be written compactly as a matrix equation. For circuit work the standard arrangement is

G·V = I

where G is the conductance matrix, V is the vector of unknown node voltages, and I is the vector of source currents. This is exactly the form SPICE assembles and solves at every operating point, and recognising it now will make the later book on simulation considerably less mysterious.

The conductance matrix has a pleasant structure that makes it easy to write down by inspection: each diagonal entry is the sum of all conductances connected to that node, and each off-diagonal entry is the negative of the conductance directly between the two nodes it indexes. That structure is not a coincidence — it is Kirchhoff's current law, written in matrix form.

### Practice Problems

- Solve by substitution: 2x + 3y = 12 and x − y = 1.

- Solve by elimination: 4a + 5b = 22 and 3a − 2b = 5.

- Evaluate the determinant of the system 6x + 2y = 10, 3x + y = 5, and explain what your result implies about the solution.

- Two mesh equations for a circuit are 100I₁ − 40I₂ = 5 and −40I₁ + 90I₂ = 0. Solve for both currents using Cramer's rule, and state the physical meaning of the sign of each result.

# Chapter 6 — Functions, Graphs, and Limiting Behaviour

Engineers read graphs more often than they read equations. A frequency response, a transfer characteristic, a load line — each is a function drawn rather than written, and the skill being exercised is the ability to extract behaviour from shape. This chapter is about that skill.

## 6.1 Functions as Input-Output Relations

A function assigns exactly one output to each input. In circuit terms, the input is often a frequency or an input voltage, and the output is a gain or an output voltage. The notation H(f) means "the value of H at frequency f" — it is not H multiplied by f, a confusion that causes real difficulty when the same letters are used for both quantities and functions.

## 6.2 Linear Functions and Straight Lines

A linear function has the form y = mx + c, with m the slope and c the intercept. Ohm's law is linear through the origin, which is why an ohmic resistor plots as a straight line through (0,0) on a current-versus-voltage graph, with slope 1/R.

A load line is the same idea used in reverse. For a source of voltage V_s with internal resistance R_s driving an unknown element, Kirchhoff gives V = V_s − I·R_s: a straight line on the same axes as the device characteristic, with negative slope. Where the two curves cross is the operating point. This graphical technique is how transistor bias points were found before circuit simulators existed, and it remains the clearest way to understand what biasing actually means.

## 6.3 Asymptotes and Limiting Behaviour

The fastest way to understand an unfamiliar transfer function is to ask what it does at the extremes. Consider the response of a simple low-pass filter:

H(f) = 1 / √(1 + (f/f_c)²)

At very low frequency, f/f_c is nearly zero, so the square root is nearly 1 and H ≈ 1: the filter passes the signal untouched. At very high frequency, the 1 becomes negligible next to (f/f_c)², so H ≈ f_c/f: the response falls in inverse proportion to frequency. At f = f_c exactly, H = 1/√2 ≈ 0.707, which is the definition of the corner frequency.

Three numbers — the low-frequency asymptote, the high-frequency asymptote, and the value at the corner — sketch the entire curve to engineering accuracy, without evaluating the function anywhere else. This is the habit worth building: probe the extremes first, fill in the middle only if you need to.

Key Idea

Read a transfer function at its limits before you plot it. What happens as the variable goes to zero, and as it goes to infinity? Those two answers plus the behaviour at the corner give you the shape. Everything else is detail.

## 6.4 Logarithmic Axes

Electrical engineering plots almost everything on logarithmic axes, for a reason that follows directly from Chapter 2: the quantities span many orders of magnitude, and a linear axis cannot show a microvolt and a volt on the same picture.

On log axes, a power-law relationship becomes a straight line, and the exponent becomes the slope. The high-frequency asymptote H ≈ f_c/f above is a power law with exponent −1, so on a log-log plot it is a straight line falling at a constant rate — the familiar 20 dB per decade rolloff. A second-order filter falls at exponent −2, giving 40 dB per decade. Reading the slope off a log-log plot tells you the order of the system directly, which is a genuinely useful diagnostic when looking at measured data.

Common Misconception

A logarithmic axis has no zero. It cannot: the logarithm of zero is undefined, tending to negative infinity. This surprises people the first time a plotting program refuses to display their data, or silently drops the DC point. It also means you cannot plot a signal that changes sign on a log axis without taking the magnitude first — which is exactly why frequency responses are conventionally plotted as magnitude and phase separately rather than as a single signed quantity.

### Practice Problems

- A resistor obeys Ohm's law. Sketch its current-versus-voltage characteristic for R = 2 kΩ, and state the slope in siemens.

- For the low-pass response H(f) = 1/√(1 + (f/f_c)²) with f_c = 1 kHz, evaluate H at 100 Hz, 1 kHz, and 10 kHz.

- A 12 V source with 50 Ω internal resistance drives a load. Write the load-line equation and find the current when the load voltage is 8 V.

- A measured response falls by a factor of 100 in magnitude when the frequency increases by a factor of 10. What is the slope on a log-log plot, what is the rolloff in dB per decade, and what order of filter does this suggest?

# Chapter 7 — Exponentials, Logarithms, and the Decibel

The exponential function describes every natural charging and discharging process in electronics, and the logarithm is its inverse. Between them they underpin the decibel, which is the unit electrical engineers use for almost every ratio they care about. This chapter builds all three.

## 7.1 The Exponential Function

The function eˣ has the defining property that its rate of change equals its own value. Nothing else does this, and that single property is why it appears in every first-order circuit: a capacitor discharging through a resistor loses charge at a rate proportional to the charge remaining, which is precisely the condition that produces an exponential.

The standard forms are decay and growth toward a limit:

v(t) = V₀·e^(−t/τ)     and     v(t) = V_f·(1 − e^(−t/τ))

where τ is the time constant, equal to RC for a resistor-capacitor pair and L/R for a resistor-inductor pair — both of which you verified dimensionally in Chapter 1.

The numbers worth memorising are these: after one time constant, a decaying quantity has fallen to 37% of its initial value, and a rising one has reached 63% of its final value. After three time constants, 95%. After five, 99.3%, which is why "five time constants" is the conventional engineering definition of settled.

Worked Example 7.1

A 10 µF capacitor charges through a 47 kΩ resistor toward 5 V. How long until it reaches 4 V?

First the time constant: τ = RC = (47 × 10³)(10 × 10⁻⁶) = 0.47 s. Note the prefix cancellation from Chapter 2 — kilo times micro is milli, so 47 × 10 = 470 ms directly.

The charging equation: v(t) = 5(1 − e^(−t/τ)).

Set v = 4 and solve. Divide by 5: 0.8 = 1 − e^(−t/τ), so e^(−t/τ) = 0.2.

Take natural logarithms of both sides: −t/τ = ln(0.2) = −1.609.

So t = 1.609τ = 1.609 × 0.47 = 0.756 s.

Sanity check: 4 V is 80% of the final value, which should fall between one time constant (63%) and three (95%), and closer to the lower end. It does — 1.6 time constants. That kind of rough check catches sign errors in the logarithm, which are easy to make and otherwise invisible.

## 7.2 Logarithms

The logarithm answers the question "to what power?" If 10³ = 1000, then log₁₀(1000) = 3. Two bases matter in engineering: base 10, written log, and base e ≈ 2.71828, written ln. Exponential circuit behaviour requires ln; decibels and log-frequency axes use log.

The three properties that do all the work convert multiplication into addition, division into subtraction, and powers into multiplication:

log(ab) = log a + log b;   log(a/b) = log a − log b;   log(aⁿ) = n·log a

The first of these is the entire reason decibels exist. Cascaded stages multiply their gains; expressed logarithmically, they add. A chain of amplifiers and losses becomes a sum you can do in your head instead of a product you cannot.

## 7.3 The Decibel

The decibel expresses a power ratio logarithmically:

G(dB) = 10·log₁₀(P_out/P_in)

For voltage or current ratios, the factor is 20 rather than 10, because power is proportional to the square of voltage and the square becomes a factor of two in front of the logarithm:

G(dB) = 20·log₁₀(V_out/V_in)

The values worth memorising are few. 3 dB is a factor of 2 in power. 6 dB is a factor of 2 in voltage. 10 dB is a factor of 10 in power. 20 dB is a factor of 10 in voltage. Negative values are the reciprocals: −3 dB is half the power, which is why the corner frequency of a filter — where the response is 1/√2 in voltage, and therefore half in power — is called the 3 dB point.

Key Idea

Use 10·log for power ratios and 20·log for voltage ratios. The factor of two comes from P ∝ V², not from a different definition of the decibel. Applying 20·log to a power ratio, or 10·log to a voltage ratio, produces an answer wrong by a factor of two in dB — which is a factor of two in the exponent, and therefore an enormous error in the underlying quantity.

The decibel is a ratio and therefore dimensionless. To express an absolute quantity, a reference must be specified, and it is written into the unit: dBm is decibels relative to one milliwatt, dBV relative to one volt, dBµV relative to one microvolt. A power of 1 mW is 0 dBm; 1 W is +30 dBm; 1 µW is −30 dBm. These absolute forms are used constantly in RF work, and a later book in this series depends heavily on fluency with them.

Common Misconception

Decibel values add only when they are ratios. You may add gains in dB along a signal chain because the underlying gains multiply. You may not add two dBm values: adding +10 dBm to +10 dBm does not give +20 dBm, because that would be adding absolute powers, not ratios. Two 10 mW signals combine to 20 mW, which is +13 dBm — a 3 dB increase, because doubling power is always 3 dB regardless of the starting level.

### Practice Problems

- A 100 nF capacitor discharges through 22 kΩ from an initial 9 V. Find the time constant, and the voltage after 5 ms.

- Express the following power ratios in dB: 2, 10, 100, 0.5, 1000.

- An amplifier has a voltage gain of 250. Express this in dB. If it is followed by a 6 dB attenuator, what is the overall gain in dB, and as a plain voltage ratio?

- A receiver input is −95 dBm. Convert this to watts. If an antenna and cable together provide a gain of +12 dB, what is the input power in dBm and in watts at the far end?

# Chapter 8 — Trigonometry and the Sinusoid

Alternating current is sinusoidal, and so is every signal that can be built from alternating currents — which, as a later book on Fourier analysis shows, is every signal whatsoever. The sinusoid is therefore not one waveform among many; it is the waveform, and the trigonometry describing it is worth genuine fluency.

## 8.1 Angles, Degrees, and Radians

An angle in radians is the arc length subtended divided by the radius. A full circle is 2π radians, so 180° = π rad, and the conversion is a simple proportion. Radians are used throughout analysis because the calculus of trigonometric functions is clean only in radians — the derivative of sin x is cos x only when x is in radians, picking up an awkward factor of π/180 otherwise.

Degrees survive in practice because they are convenient for phase, where quarter and half cycles matter and 90° reads more naturally than π/2. Both appear constantly, sometimes in the same equation, and the single most common cause of an inexplicably wrong answer in phase calculations is a calculator left in the wrong angular mode.

## 8.2 The Sinusoid and Its Parameters

A general sinusoid is written

v(t) = V_m·sin(ωt + φ)

with three independent parameters. V_m is the amplitude, the peak departure from zero. ω is the angular frequency in radians per second, related to ordinary frequency by ω = 2πf and to the period by ω = 2π/T. φ is the phase, which shifts the waveform along the time axis; a positive φ advances the waveform, moving it earlier.

Only these three numbers are needed. Two sinusoids of the same frequency can differ only in amplitude and phase, and that observation is the seed of the entire phasor method developed in Chapter 11.

## 8.3 Sine, Cosine, and the Relationship Between Them

Cosine is sine shifted by a quarter cycle: cos θ = sin(θ + 90°). Which one you use as the reference is arbitrary, and different textbooks choose differently, but you must be consistent within a single problem. This series follows the common convention of taking cosine as the reference for phasor work, for reasons that Chapter 10 makes clear.

The identities that earn their keep in circuit analysis are these:

- Pythagorean: sin²θ + cos²θ = 1. This is what turns a resistance and a reactance into a magnitude, and it is why impedance magnitudes involve a square root of a sum of squares.

- Sum of angles: sin(A + B) = sin A cos B + cos A sin B. This is the identity behind mixing and modulation — multiply two sinusoids and you get sum and difference frequencies, which is the foundation of every radio receiver.

- Double angle: sin 2θ = 2 sin θ cos θ, and cos 2θ = 1 − 2sin²θ. The second form explains why the instantaneous power in an AC circuit oscillates at twice the signal frequency.

Worked Example 8.1

A voltage is described by v(t) = 15·sin(377t + 30°). Find the amplitude, angular frequency, ordinary frequency, period, phase, and the instantaneous value at t = 5 ms.

Amplitude: V_m = 15 V. Angular frequency: ω = 377 rad/s.

Frequency: f = ω/2π = 377/6.283 = 60.0 Hz. This is the North American mains frequency, and 377 is a number you will meet often enough to recognise on sight.

Period: T = 1/f = 16.67 ms. Phase: φ = 30°, meaning the waveform leads a pure sine by one-twelfth of a cycle.

At t = 5 ms, the argument is ωt + φ = (377)(0.005) + 30° = 1.885 rad + 30°.

The mixed units must be reconciled. Converting 1.885 rad to degrees: 1.885 × 180/π = 108.0°. So the total argument is 108.0° + 30° = 138.0°.

Therefore v = 15·sin(138.0°) = 15 × 0.669 = 10.0 V.

That reconciliation step is where errors happen. The ωt term is inherently in radians because ω is in radians per second; the phase was given in degrees. One must be converted before they can be added, and no calculator will warn you if you skip it.

## 8.4 Average and RMS Values

The average value of a sinusoid over a whole number of cycles is zero, since it spends as much time positive as negative. That makes the plain average useless for describing AC magnitude, and something better is needed.

The root-mean-square value is defined as the square root of the mean of the square. Squaring makes everything positive, so nothing cancels, and the definition is chosen so that the RMS value of an AC waveform is exactly the DC value that would dissipate the same power in a resistor. For a sinusoid the result is

V_rms = V_m/√2 ≈ 0.707·V_m

When a supply is described as 230 V AC, that is an RMS figure; its peak is 325 V, and the peak-to-peak swing is 650 V. Insulation must survive the peak while power calculations use the RMS, and confusing the two has consequences at both ends — under-specified insulation on one side, power estimates wrong by a factor of two on the other.

Key Idea

RMS is defined by equal heating, not by averaging. V_rms = V_m/√2 holds for sinusoids only. A square wave has V_rms = V_m; a triangle wave has V_m/√3. Applying the 0.707 factor to a non-sinusoidal waveform is simply wrong, and the error grows with how far the waveform departs from a sinusoid.

### Practice Problems

- Convert to radians: 45°, 90°, 120°, 270°. Convert to degrees: π/6, 2π/3, 3π/2 rad.

- A current is i(t) = 2.5·sin(2π·1000·t − 45°) A. State its amplitude, frequency, period, and phase, and find its value at t = 0.

- A sinusoidal voltage has a peak-to-peak value of 10 V. Find its amplitude and its RMS value.

- Two sinusoids of the same frequency have amplitudes 3 V and 4 V and are 90° apart in phase. Use the Pythagorean identity to find the amplitude of their sum, and explain why you may not simply add 3 and 4.

# Chapter 9 — Complex Numbers

Complex numbers have an unfortunate name. There is nothing complicated about them, and nothing imaginary about the imaginary part — both terms are historical accidents from a period when mathematicians distrusted the objects they had just invented. What a complex number actually is, for our purposes, is a way of carrying two related pieces of information as a single quantity, and then doing arithmetic on the pair at once.

For electrical engineering the two pieces of information are magnitude and phase. Chapter 8 established that two sinusoids of the same frequency can differ only in those two respects. Complex numbers are the tool that lets us manipulate both at once, which is why they took over AC analysis completely and never gave it back.

## 9.1 Why j and Not i

The imaginary unit is defined by j² = −1. Mathematicians write it i; electrical engineers write j, because i was already taken by current and reusing it would be intolerable. Every textbook in this series uses j. No other difference is implied.

The defining property is worth stating plainly rather than treating as a mystery. There is no real number whose square is negative, so j is not a real number. That is the entire content of the word "imaginary" — it means "not on the real number line," not "fictitious." The number j is exactly as real, in the ordinary sense, as the number 3; it simply lives in a different direction.

## 9.2 Rectangular Form

A complex number in rectangular form is written z = a + jb, where a is the real part and b the imaginary part. Both a and b are ordinary real numbers. The whole object can be pictured as a point in a plane, with a measured along the horizontal axis and b along the vertical — the complex plane, or Argand diagram.

That geometric picture is the useful one. A complex number is a point, or equivalently an arrow from the origin to that point. Its length is the magnitude and the angle it makes with the positive real axis is the phase, which connects directly to the sinusoid parameters of Chapter 8.

Addition and subtraction in rectangular form are componentwise, exactly like adding vectors:

(a + jb) + (c + jd) = (a + c) + j(b + d)

Multiplication follows the ordinary distributive law, with the single extra rule that j² = −1:

(a + jb)(c + jd) = (ac − bd) + j(ad + bc)

The cross terms produce the imaginary part; the j² in the product of the two imaginary parts flips a sign and lands in the real part. There is nothing to memorise here beyond j² = −1 — the rest is expansion of brackets.

## 9.3 The Complex Conjugate and Division

The conjugate of z = a + jb, written z*, is a − jb: the same number with the sign of the imaginary part flipped, or geometrically, the point reflected across the real axis.

Its usefulness comes from one fact: a number times its conjugate is always real.

z·z* = (a + jb)(a − jb) = a² + b²

This is what makes division possible. To divide by a complex number, multiply top and bottom by the conjugate of the denominator; the denominator becomes real, and the division reduces to dividing by an ordinary number.

Worked Example 9.1

Evaluate (3 + j4)/(1 − j2).

Multiply numerator and denominator by the conjugate of the denominator, which is (1 + j2):

Numerator: (3 + j4)(1 + j2) = 3 + j6 + j4 + j²8 = 3 + j10 − 8 = −5 + j10.

Denominator: (1 − j2)(1 + j2) = 1 + 4 = 5.

So the quotient is (−5 + j10)/5 = −1 + j2.

Check by multiplying back: (−1 + j2)(1 − j2) = −1 + j2 + j2 − j²4 = −1 + j4 + 4 = 3 + j4. Correct.

Note that the denominator became 1² + 2² = 5 without any cross terms — that is the conjugate doing its work, and it always happens. If your denominator still has a j in it after multiplying by the conjugate, you have made an arithmetic error.

## 9.4 Polar Form

The same number can be described by its magnitude and angle instead of its two components. The magnitude is

|z| = √(a² + b²)

which is the Pythagorean identity from Chapter 8 doing exactly the job advertised, and the angle is

θ = arctan(b/a)

with a crucial caveat: the arctangent function returns values only between −90° and +90°, so it cannot distinguish the second quadrant from the fourth, or the third from the first. You must look at the signs of a and b to place the number in the correct quadrant and adjust by 180° if the real part is negative. Nearly every programming language provides an atan2 function that handles this; hand calculation does not, and this is a reliable source of answers that are exactly 180° wrong.

Conversion back to rectangular form is straightforward trigonometry: a = |z|·cos θ and b = |z|·sin θ.

Common Misconception

Rectangular form is convenient for addition and subtraction; polar form is convenient for multiplication and division. Trying to multiply in rectangular form is possible but tedious, and trying to add in polar form is a genuine mistake — the magnitudes of a sum are not the sum of the magnitudes unless the two numbers happen to have the same angle. Convert to whichever form suits the operation, then convert back. The conversion is cheap; the error from adding polar magnitudes is not.

## 9.5 Multiplication and Division in Polar Form

In polar form the two operations become remarkably simple. To multiply, multiply the magnitudes and add the angles. To divide, divide the magnitudes and subtract the angles.

|z₁z₂| = |z₁|·|z₂|,   ∠(z₁z₂) = ∠z₁ + ∠z₂

This is the property that makes complex numbers indispensable in AC analysis, and it deserves a moment of attention. Multiplying by a complex number scales and rotates. Multiplying by a number of magnitude 1 and angle 90° — which is to say, multiplying by j — rotates by a quarter turn without changing size at all.

Key Idea

Multiplying by j is a 90° rotation. Multiplying by j twice is a 180° rotation, which is multiplication by −1 — and that is exactly the statement j² = −1, seen geometrically. The defining property of j is not an arbitrary rule; it is what a quarter-turn does when you apply it twice.

Once you see multiplication as rotation, the behaviour of capacitors and inductors in AC circuits stops being a set of rules to memorise and becomes a single geometric statement, which is where Chapter 11 goes.

### Practice Problems

- Given z₁ = 4 + j3 and z₂ = 2 − j5, compute z₁ + z₂, z₁ − z₂, and z₁z₂ in rectangular form.

- Convert 5 + j12 to polar form, and convert 10∠−60° to rectangular form.

- Evaluate (2 + j3)/(4 − j) by the conjugate method, and verify your answer by multiplying back.

- Compute j², j³, j⁴, and 1/j. Explain each result as a rotation in the complex plane, and state the general rule for jⁿ.

# Chapter 10 — Euler's Formula and the Complex Exponential

This chapter contains one equation. It is arguably the most useful single equation in electrical engineering, and it is the hinge on which the whole of AC analysis turns.

## 10.1 The Formula

Euler's formula states that

e^(jθ) = cos θ + j·sin θ

The left-hand side is an exponential — the function from Chapter 7 whose rate of change equals its own value. The right-hand side is a pair of sinusoids — the functions from Chapter 8. The formula asserts they are the same thing, provided the exponent is imaginary.

Geometrically the statement is transparent. The right-hand side is a complex number with real part cos θ and imaginary part sin θ, so by the Pythagorean identity its magnitude is √(cos²θ + sin²θ) = 1, and its angle is θ. So e^(jθ) is simply the point on the unit circle at angle θ. As θ increases steadily, the point travels around the circle at a constant rate.

That last observation is the whole reason the formula matters to engineers. Steady rotation around a circle, projected onto an axis, is a sinusoid. Euler's formula is the bridge between rotation and oscillation, and AC circuit theory lives on that bridge.

Key Idea

e^(jθ) is a unit-length arrow at angle θ. Nothing more mysterious than that. Multiplying any complex number by e^(jθ) rotates it by θ without changing its length, which is the generalisation of "multiplying by j rotates by 90°" from the previous chapter — since j itself is e^(j90°).

## 10.2 Polar Form, Properly Written

With Euler's formula in hand, the polar form of Chapter 9 can be written as a genuine product rather than as the shorthand angle notation:

z = |z|·e^(jθ)

The engineering shorthand |z|∠θ means exactly this and nothing more. Writing it as an exponential is not mere notation, though — it means the ordinary rules of exponents now apply, and those rules are why multiplication in polar form works the way it does:

|z₁|e^(jθ₁) · |z₂|e^(jθ₂) = |z₁||z₂| · e^(j(θ₁+θ₂))

Magnitudes multiply and angles add, not as a special rule for complex numbers but as the ordinary law that exponents add when you multiply powers of the same base. The polar multiplication rule of Chapter 9 was a consequence of Euler's formula all along.

## 10.3 Extracting Sine and Cosine

Applying the formula at −θ and using cos(−θ) = cos θ, sin(−θ) = −sin θ gives a companion equation, and adding or subtracting the two isolates the trigonometric functions:

cos θ = (e^(jθ) + e^(−jθ))/2,     sin θ = (e^(jθ) − e^(−jθ))/2j

These are worth knowing because they let you replace trigonometric functions with exponentials, and exponentials are far easier to differentiate, integrate, and multiply. A trigonometric identity that takes half a page to prove geometrically often takes two lines in exponential form. The sum-of-angles identity from Chapter 8, for instance, is just the statement that exponents add.

## 10.4 The Rotating Vector

Now let the angle grow with time at a steady rate, θ = ωt, and attach an amplitude:

v(t) = V_m·e^(j(ωt + φ))

This describes an arrow of length V_m rotating counterclockwise at ω radians per second, starting at angle φ. Its projection onto the real axis is V_m·cos(ωt + φ), which is exactly the sinusoid of Chapter 8.

So a real sinusoidal voltage can be regarded as the real part of a rotating complex quantity. Nothing has been lost — the real signal is recoverable at any moment by taking the real part — but a great deal has been gained, because the rotating exponential is far easier to do algebra with than the cosine is.

Common Misconception

Students often ask what the imaginary part of the rotating vector physically is, expecting it to correspond to some measurable quantity. It does not. The imaginary part is mathematical scaffolding: we deliberately added it because the resulting object is easier to compute with, and we discard it at the end by taking the real part. The physical voltage is, and always was, the real part alone. This is not a defect of the method; it is the method.

## 10.5 Why This Solves AC Circuits

Here is the payoff, stated now and demonstrated in Chapter 11. Differentiating the rotating exponential with respect to time gives

d/dt [V_m·e^(j(ωt+φ))] = jω · V_m·e^(j(ωt+φ))

The function reproduces itself, multiplied by jω. Differentiation — a calculus operation — has become multiplication by a constant. Integration likewise becomes division by jω.

The consequence is enormous. The equations governing capacitors and inductors are differential equations, and differential equations are hard. Written in terms of rotating exponentials, those same equations become algebraic equations in which the derivative has been replaced by the factor jω. An entire class of calculus problems collapses into complex arithmetic of the kind you practised in Chapter 9.

Worked Example 10.1

Use Euler's formula to prove the identity cos(A + B) = cos A cos B − sin A sin B.

Start from the exponential of the sum, and use the fact that exponents add:

e^(j(A+B)) = e^(jA) · e^(jB)

Expand each factor by Euler's formula:

e^(jA)·e^(jB) = (cos A + j sin A)(cos B + j sin B)

Multiply out, remembering j² = −1:

= cos A cos B + j cos A sin B + j sin A cos B − sin A sin B

= (cos A cos B − sin A sin B) + j(cos A sin B + sin A cos B)

Now apply Euler's formula to the left-hand side: e^(j(A+B)) = cos(A + B) + j sin(A + B).

Two complex numbers are equal only if their real parts match and their imaginary parts match. Equating real parts gives the identity we wanted; equating imaginary parts gives the sine sum identity for free.

Compare this with the geometric proof, which requires a carefully constructed diagram and several similar triangles. The exponential proof is four lines and produces two identities at once. This is representative of what the complex exponential does throughout the subject.

### Practice Problems

- Evaluate e^(jπ), e^(jπ/2), and e^(j2π), expressing each as an ordinary complex number.

- Write 3 + j4 in the form |z|e^(jθ), giving θ in both radians and degrees.

- Use the exponential definitions of sine and cosine to show that cos²θ + sin²θ = 1.

- A signal is v(t) = Re{10·e^(j(500t + π/4))}. Write v(t) explicitly as a cosine, state its frequency in hertz, and find its value at t = 2 ms.

# Chapter 11 — Phasors and Impedance

This chapter assembles everything the book has built. Units gave us dimensional discipline, algebra gave us rearrangement, trigonometry gave us the sinusoid, complex numbers gave us two-quantity arithmetic, and Euler's formula connected rotation to oscillation. The phasor is what they combine into, and it is the single most powerful labour-saving device in circuit analysis.

## 11.1 The Central Observation

In a linear circuit driven by a sinusoid at a single frequency, every voltage and every current in the circuit is a sinusoid at that same frequency. The frequency never changes. Only amplitude and phase differ from point to point.

If the frequency is the same everywhere, carrying it through every equation is wasted effort. The phasor method simply stops carrying it. We agree on the frequency once, set it aside, and track only the amplitude and phase — which, as Chapter 9 established, is exactly what a complex number is good for.

## 11.2 Defining the Phasor

Given a sinusoid v(t) = V_m·cos(ωt + φ), the corresponding phasor is the complex number

V = V_m∠φ = V_m·e^(jφ)

The time dependence has been dropped entirely. To recover the real signal, multiply by e^(jωt) and take the real part.

Two conventions must be fixed and then adhered to. First, this series uses cosine as the reference — a phasor angle of zero means a cosine, not a sine. A sine must be converted first, using sin θ = cos(θ − 90°). Second, phasor magnitudes in this series are RMS values rather than peak values, since that makes power calculations come out without stray factors of two. Other texts use peak amplitudes; neither is wrong, but mixing them within one problem produces answers off by √2.

Common Misconception

A phasor is not a function of time, and writing V(t) for a phasor is a category error. The whole point of the phasor is that time has been factored out. Nor is a phasor a vector in physical space — the "arrow" of the complex plane points in a mathematical direction, not a spatial one. A phasor diagram showing a current 90° behind a voltage is saying something about timing, not geometry.

## 11.3 Impedance

With signals represented as phasors, the relationship between voltage and current across any passive component becomes a simple ratio, and that ratio is called impedance:

Z = V/I

Impedance is a complex number measured in ohms. Its real part is resistance, which dissipates power; its imaginary part is reactance, which stores and returns energy without dissipating it. Deriving the three basic impedances is now a short exercise, using the result from Chapter 10 that differentiation becomes multiplication by jω.

For a resistor, v = iR with no derivative involved, so

Z_R = R

For an inductor, v = L·di/dt. Replacing the derivative with jω gives V = jωL·I, so

Z_L = jωL

For a capacitor, i = C·dv/dt, so I = jωC·V, and inverting,

Z_C = 1/(jωC) = −j/(ωC)

Three differential relationships have become three algebraic ones. Read the signs: inductive reactance is positive imaginary, capacitive reactance is negative imaginary, and the opposite signs are why they cancel at resonance.

Key Idea

Impedance turns AC circuit analysis into DC circuit analysis with complex numbers. Series impedances add. Parallel impedances combine by product-over-sum. Voltage dividers, current dividers, Thevenin equivalents — every technique from resistive circuit analysis carries over unchanged, with R replaced by Z and real arithmetic replaced by complex arithmetic. Nothing new needs to be learned; the old methods simply work.

## 11.4 Reading Phase From Impedance

The angle of the impedance is the phase by which voltage leads current. For a pure resistor the impedance is real, its angle is zero, and voltage and current are in phase. For a pure inductor, Z = jωL has angle +90°, so voltage leads current by a quarter cycle. For a pure capacitor, angle −90°, so voltage lags current.

The traditional mnemonic is "ELI the ICE man": in an inductor (L), voltage E leads current I; in a capacitor (C), current I leads voltage E. It is a silly device and it works.

Real components are never pure, and the impedance angle tells you how far from ideal a component is. A capacitor whose measured impedance angle is −88° rather than −90° has a small resistive component, and that two-degree discrepancy is precisely the loss mechanism quantified later in the series as equivalent series resistance.

Worked Example 11.1

A 1 kΩ resistor is in series with a 0.1 µF capacitor across a 10 V RMS, 1 kHz supply. Find the total impedance, the current, and the voltage across each component.

First the angular frequency: ω = 2πf = 2π(1000) = 6283 rad/s.

Capacitive impedance: Z_C = −j/(ωC) = −j/(6283 × 0.1 × 10⁻⁶) = −j/(6.283 × 10⁻⁴) = −j1592 Ω.

Total impedance in series is the sum: Z = 1000 − j1592 Ω.

In polar form: |Z| = √(1000² + 1592²) = √(10⁶ + 2.534 × 10⁶) = √(3.534 × 10⁶) = 1880 Ω.

The angle: θ = arctan(−1592/1000) = −57.9°. The real part is positive, so we are in the fourth quadrant and no 180° adjustment is needed.

Current: I = V/Z = 10∠0° / 1880∠−57.9° = 5.32∠+57.9° mA. Dividing means dividing magnitudes and subtracting angles, so the current leads the supply voltage by 57.9° — as expected for a circuit that is predominantly capacitive.

Voltage across the resistor: V_R = I·R = (5.32 mA)(1000 Ω) = 5.32 V, in phase with the current, so at +57.9°.

Voltage across the capacitor: V_C = I·Z_C = (5.32 mA)(1592 Ω) = 8.47 V, at an angle of 57.9° − 90° = −32.1°.

Now the check that catches nearly every error in this kind of problem. The two component voltages are 5.32 V and 8.47 V, which sum arithmetically to 13.79 V — considerably more than the 10 V supply. That is not a violation of Kirchhoff's voltage law. The voltages are 90° apart in phase, so they add as phasors, not as numbers: √(5.32² + 8.47²) = √(28.3 + 71.7) = √100 = 10.0 V. Exactly the supply voltage.

Common Misconception

It looks impossible for the voltages across two series components to exceed the supply voltage, and in a DC circuit it would be. In AC circuits it is routine, because component voltages add as phasors rather than as plain numbers. In a resonant circuit the effect becomes dramatic: the voltage across the inductor and the capacitor individually can exceed the supply by a factor equal to the Q of the circuit, sometimes by hundreds of times. Those voltages are entirely real and will destroy components rated only for the supply voltage. They cancel each other in the sum precisely because they are 180° apart.

## 11.5 Admittance, and When to Use It

The reciprocal of impedance is admittance, Y = 1/Z, measured in siemens. Its real part is conductance and its imaginary part is susceptance.

Admittance exists for the same reason conductance does: for parallel combinations, admittances simply add, whereas impedances require the reciprocal formula. In a circuit with many parallel branches, working in admittance turns a nested reciprocal expression into a plain sum. Nodal analysis, which is formulated around parallel connections at each node, is therefore naturally written in admittances — the conductance matrix of Chapter 5 becomes an admittance matrix, and every technique carries straight over.

### Practice Problems

- Convert to phasor form, using cosine reference and RMS magnitudes: v(t) = 170·cos(377t + 30°) V, and i(t) = 4.24·sin(377t) A.

- Find the impedance of a 22 mH inductor and a 470 nF capacitor, each at 2 kHz. State the magnitude and angle of each.

- A 470 Ω resistor is in series with a 100 mH inductor across a 24 V RMS, 400 Hz supply. Find the total impedance in polar form, the current, and the phase angle between supply voltage and current.

- For the series RC circuit of Worked Example 11.1, find the frequency at which the resistive and capacitive impedance magnitudes are equal. What is the phase angle of the total impedance at that frequency, and what is the ratio of output to input voltage if the output is taken across the resistor?

# Chapter 12 — Calculus for Electrical Engineering

Calculus has a reputation it does not deserve. Stripped of the machinery, it consists of two ideas: a derivative measures how fast something is changing, and an integral measures how much of something has accumulated. Everything else is technique for computing them. This chapter builds both ideas from the circuit behaviour that motivates them, and covers the specific results the rest of this series relies on.

## 12.1 The Derivative as a Rate

The derivative of a quantity with respect to time is its rate of change: how fast it is going up or down at a particular instant. Written dv/dt, it is the slope of the graph of v against t at that moment.

Circuit theory is full of derivatives because two of the three basic components are defined by them. A capacitor's current is proportional to the rate of change of its voltage, and an inductor's voltage is proportional to the rate of change of its current:

i = C·dv/dt     and     v = L·di/dt

Read these as physical statements rather than formulas. A capacitor passes current only while its voltage is changing — hold the voltage constant and the current stops, which is why a capacitor blocks DC. An inductor develops voltage only while its current is changing — hold the current constant and the voltage vanishes, which is why an inductor is a short circuit to DC.

The derivatives you need are few:

- A constant has derivative zero. Nothing changing means no rate of change.

- A straight line v = kt has derivative k, the constant slope.

- A power tⁿ has derivative n·t^(n−1).

- The exponential e^(kt) has derivative k·e^(kt) — it reproduces itself, scaled. This is the defining property from Chapter 7.

- Sine and cosine swap: the derivative of sin(ωt) is ω·cos(ωt), and the derivative of cos(ωt) is −ω·sin(ωt). Note that each differentiation brings down a factor of ω and advances the phase by 90°, which is exactly the jω of Chapter 11 seen without complex notation.

Common Misconception

The derivative of sin(ωt) is ω·cos(ωt), not cos(ωt). The factor of ω is dropped constantly and the consequences are severe, because ω is often a large number — at mains frequency it is 377, and in RF work it is in the billions. Omitting it does not make an answer slightly wrong; it makes it wrong by orders of magnitude. The factor appears because of the chain rule: differentiating the inside function ωt gives ω, which multiplies the result.

## 12.2 The Integral as an Accumulation

Integration is the reverse of differentiation: given a rate, it recovers the total accumulated. Geometrically it is the area under a curve.

Rearranging the capacitor relation shows the accumulation directly. If i = C·dv/dt, then the voltage is the accumulated current divided by capacitance:

v(t) = (1/C)·∫ i dt

This says a capacitor's voltage is proportional to the total charge that has flowed into it — the integral of current is charge, which is exactly the statement that a coulomb is an ampere-second. The capacitor is a device that remembers how much current has passed, which is why it is a memory element and why its state must be specified as an initial condition when solving a circuit.

The integrals worth knowing are the derivatives read backwards: the integral of tⁿ is t^(n+1)/(n+1), the integral of e^(kt) is e^(kt)/k, the integral of cos(ωt) is sin(ωt)/ω, and the integral of sin(ωt) is −cos(ωt)/ω. Each integration divides by ω and retards the phase by 90°, which is the 1/jω of capacitive impedance.

## 12.3 Definite Integrals, Averages, and RMS

A definite integral evaluates the accumulation between two limits, and dividing by the width of the interval gives the average value:

average of f over [a, b] = (1/(b−a))·∫ f dt

This is where the RMS definition of Chapter 8 comes from, and it is worth seeing the calculation once. The mean square of a sinusoid over one period is

mean of V_m²sin²(ωt) = V_m²·(mean of sin²)

and the double-angle identity from Chapter 8 gives sin²θ = (1 − cos 2θ)/2. The cosine term averages to zero over a whole number of cycles, leaving a mean of exactly one half. So the mean square is V_m²/2, and the root mean square is V_m/√2.

That is the entire derivation of the 0.707 factor, and it makes plain why the factor is specific to sinusoids: it came from the average of sin², and a different waveform has a different average of its square.

Worked Example 12.1

A capacitor of 4.7 µF carries a current that rises linearly from 0 to 10 mA over 5 ms, then stops. Find the voltage across it at the end of that interval, assuming it started at 0 V.

The current is i(t) = kt with k chosen so that i = 10 mA at t = 5 ms: k = 0.010/0.005 = 2 A/s.

The voltage is the integral of current divided by capacitance:

v(t) = (1/C)·∫₀^t kt dt = (1/C)·k·t²/2

Evaluating at t = 5 ms:

v = (1/4.7 × 10⁻⁶)(2)(0.005²/2) = (1/4.7 × 10⁻⁶)(2)(1.25 × 10⁻⁵) = (2.5 × 10⁻⁵)/(4.7 × 10⁻⁶) = 5.32 V

A useful cross-check: the average current over the ramp is half the peak, or 5 mA, sustained for 5 ms, giving a total charge of 25 µC. Then v = Q/C = 25 × 10⁻⁶ / 4.7 × 10⁻⁶ = 5.32 V. The two routes agree, and the second is often quicker — for a linear ramp, the area under the curve is just a triangle, and you do not need the integral at all.

Key Idea

In sinusoidal steady state, d/dt becomes ×jω and ∫dt becomes ÷jω. This is the whole reason Chapter 11 works. Once you accept it, no differential equation ever needs to be solved to analyse a linear circuit at a single frequency — the calculus has been done in advance, once, and packaged into the factor jω.

## 12.4 Differential Equations, Briefly

A differential equation relates a quantity to its own derivatives. The RC circuit produces the simplest interesting example. Kirchhoff's voltage law around a charging loop gives

RC·dv/dt + v = V_s

and the solution is the exponential of Chapter 7, v(t) = V_s(1 − e^(−t/RC)). You are not expected to derive that here. What matters is recognising the pattern: a first-order differential equation produces an exponential response with a single time constant, and a second-order one produces either two exponentials or a damped oscillation, depending on the sign of the discriminant met in Chapter 4.

This is why the phasor method is such a relief. Differential equations must be solved for the complete response, including the transient. But if all you need is the steady state — the behaviour after the transients have died away, which is what most AC analysis concerns — the phasor method delivers it algebraically. Transient analysis, where the differential equation genuinely must be solved, is taken up properly in Book 3.

### Practice Problems

- A capacitor of 1 µF has a voltage rising at a steady 2000 V/s. Find the current through it.

- An inductor of 50 mH carries a current i(t) = 3·sin(1000t) A. Find the voltage across it, including its amplitude and phase relative to the current.

- Find the derivative of v(t) = 10·cos(2000t + 30°) with respect to time, and state the amplitude of the result.

- A current of 5 mA flows into a 10 µF capacitor, initially uncharged, for 200 ms and then stops. Find the final voltage, first by integration and then by the charge method, and confirm the two agree.

# Chapter 13 — Where This Book Leads Next

The mathematics is now in place. It is worth a short chapter to say explicitly where each piece gets used, both so the effort feels justified and so you know where to return when a later book assumes something you have half-forgotten.

## 13.1 Immediately: Book 3, Circuit Theory

Book 3 opens with Kirchhoff's laws and resistive networks, which need only Chapters 4 and 5 — rearrangement and simultaneous equations. It then introduces capacitors and inductors, at which point Chapter 12's derivatives become necessary to state what those components do. First-order transients use the exponentials of Chapter 7 directly, including the 63% and five-time-constant rules.

The second half of Book 3 is sinusoidal steady-state analysis, and it uses Chapters 8 through 11 continuously and without further explanation. The phasor and impedance material of Chapter 11 is assumed knowledge from the moment AC appears. If one chapter of this book is worth revisiting before starting Book 3, it is that one.

## 13.2 Soon: Filters, Frequency Response, and Bode Plots

Frequency response is the impedance of Chapter 11 evaluated across a range of frequencies rather than at one. The asymptotic reading of transfer functions from Chapter 6, combined with the logarithmic axes and decibels of Chapter 7, is precisely the skill set the Bode plot demands. An engineer fluent in those two chapters can sketch the response of a filter from its component values in a few seconds, without computing a single point.

## 13.3 Later: RF, Noise, and Communications

The RF books lean on this material harder than any others. The Smith chart is a conformal map of the complex plane, and it is unusable without genuine comfort with complex arithmetic. S-parameters are complex ratios, quoted in magnitude and phase. Link budgets are sums of decibel quantities, and the dBm conventions of Chapter 7 are used on every page. Noise figure is a ratio of ratios, expressed logarithmically, and the uncertainty reasoning of Chapter 3 governs how precisely any of it can be measured.

Modulation theory rests on the sum-of-angles identity from Chapter 8 — multiplying two sinusoids produces sum and difference frequencies, which is what a mixer does and what every superheterodyne receiver is built around.

## 13.4 Eventually: Transforms

The Fourier and Laplace transforms, which arrive later in the series, are direct generalisations of what you have already done. The phasor represents a signal at one frequency; the Fourier transform represents a signal as a continuous superposition of all frequencies at once. The jω of Chapter 11 becomes the more general complex variable s = σ + jω of the Laplace transform, where the real part σ describes growth or decay and the imaginary part describes oscillation.

Both transforms turn differential equations into algebraic ones, exactly as the phasor method did — they are the same trick, applied more broadly. Having understood why jω works, you will find the transforms considerably less alien than students who meet them without this grounding.

Key Idea

Nothing later in this series introduces mathematics harder than what is in this book. It introduces more of it, and applies it in less obvious ways, but the underlying operations remain complex arithmetic, logarithms, sinusoids, and first-order calculus. If this book is solid, the rest of the series is about engineering rather than about mathematics — which is the point.

# Appendix A — Formula Reference

## Units and Measurement

Prefix pairs that cancel: µ×M = m×k = n×G = p×T = 1

Relative uncertainty of a product: add the relative uncertainties

Absolute uncertainty of a sum or difference: add the absolute uncertainties

## Circuit Algebra

Series resistance: R = R₁ + R₂ + …

Parallel resistance: 1/R = 1/R₁ + 1/R₂ + …, and for two, R = R₁R₂/(R₁ + R₂)

Voltage divider: V_out = V_in·R₂/(R₁ + R₂)

Current divider: I₁ = I_total·R₂/(R₁ + R₂)

Quadratic roots: x = [−b ± √(b² − 4ac)]/2a

Binomial approximation, small x: (1 + x)ⁿ ≈ 1 + nx

Two-by-two determinant: D = a₁b₂ − a₂b₁

## Exponentials, Logarithms, Decibels

Decay: v(t) = V₀·e^(−t/τ);  Rise: v(t) = V_f(1 − e^(−t/τ))

Time constants: τ = RC and τ = L/R

Settling: 63% at 1τ, 86% at 2τ, 95% at 3τ, 99.3% at 5τ

Log laws: log(ab) = log a + log b; log(a/b) = log a − log b; log(aⁿ) = n log a

Power ratio: G(dB) = 10·log₁₀(P₂/P₁);  Voltage ratio: G(dB) = 20·log₁₀(V₂/V₁)

Key values: 3 dB = ×2 power; 6 dB = ×2 voltage; 10 dB = ×10 power; 20 dB = ×10 voltage

Absolute: 0 dBm = 1 mW; +30 dBm = 1 W; −30 dBm = 1 µW

## Trigonometry and Sinusoids

General sinusoid: v(t) = V_m·sin(ωt + φ), with ω = 2πf = 2π/T

Pythagorean: sin²θ + cos²θ = 1

Sum of angles: sin(A+B) = sin A cos B + cos A sin B; cos(A+B) = cos A cos B − sin A sin B

Double angle: sin 2θ = 2 sin θ cos θ; cos 2θ = 1 − 2sin²θ

RMS of a sinusoid: V_rms = V_m/√2 ≈ 0.707 V_m (square wave: V_m; triangle: V_m/√3)

## Complex Numbers

Rectangular: z = a + jb, with j² = −1

Magnitude: |z| = √(a² + b²);  Angle: θ = arctan(b/a), adjusted for quadrant

Conjugate: z* = a − jb, and z·z* = a² + b²

Polar product: magnitudes multiply, angles add

Euler: e^(jθ) = cos θ + j sin θ;  cos θ = (e^(jθ) + e^(−jθ))/2;  sin θ = (e^(jθ) − e^(−jθ))/2j

## Phasors and Impedance

Phasor of V_m·cos(ωt + φ): V = (V_m/√2)∠φ using RMS convention

Impedances: Z_R = R;  Z_L = jωL;  Z_C = 1/(jωC) = −j/(ωC)

Series impedance adds; parallel impedance by product-over-sum; admittance Y = 1/Z adds in parallel

Impedance angle = phase by which voltage leads current

## Calculus

Component laws: i = C·dv/dt;  v = L·di/dt

d/dt of e^(kt) = k·e^(kt);  of sin(ωt) = ω cos(ωt);  of cos(ωt) = −ω sin(ωt);  of tⁿ = n·t^(n−1)

∫e^(kt) dt = e^(kt)/k;  ∫cos(ωt) dt = sin(ωt)/ω;  ∫tⁿ dt = t^(n+1)/(n+1)

In sinusoidal steady state: d/dt → ×jω, and ∫dt → ÷jω

# Appendix B — Practice Problem Solutions

## Chapter 1

- Q = CV = (100 × 10⁻⁹)(12) = 1.2 × 10⁻⁶ C = 1.2 µC, or 1200 nC.

- L/R has units H/Ω = (V·s/A)/(V/A) = (V·s/A)(A/V) = s. A time, as required.

- W = ½LI² has units H·A² = (V·s/A)(A²) = V·A·s = W·s = J. Joules, as required.

- Z = ωC would have units (1/s)(A·s/V) = A/V = siemens, which is admittance, not impedance. Since impedance must be V/A, the correct expression must have ωC in the denominator: Z = 1/(ωC), which gives V/A as required.

## Chapter 2

- 15 nF; 68 kΩ; 2.5 mA; 1.5 GHz.

- V = (4.7 kΩ)(3.2 mA) = 15.04 V, since kilo × milli = 1. P = VI = (15.04)(3.2 mA) = 48.1 mW.

- Convert all to nanofarads: 470 pF = 0.47 nF, 1.2 nF stays, 0.01 µF = 10 nF. Total = 11.67 nF.

- f = 1/T = 1/(20 × 10⁻⁹) = 50 MHz. ω = 2πf = 3.142 × 10⁸ rad/s.

## Chapter 3

- 0.00250 has three; 1200 is ambiguous (two, three, or four); 1.200 × 10³ has four; 100.0 has four; 0.0001 has one.

- V = (4.7 kΩ)(2.0 mA) = 9.4 V. Relative uncertainties add for a product: 5% + 2% = 7%, so 9.4 V ± 0.66 V.

- Difference = 0.5 mm. Absolute uncertainties add: ±0.4 mm. That is 80% relative uncertainty — the difference of two nearly equal measurements, each good to 0.2%, is known only to 80%.

- At 10.000 V: 0.05% of 10 V = 5 mV, plus 2 counts = 2 mV, total 7 mV, which is 0.07% of reading. At 0.100 V: 0.05% of 0.1 V = 0.05 mV, plus the same 2 mV, total 2.05 mV, which is 2.05% of reading — thirty times worse, because the fixed count term now dominates.

## Chapter 4

- R = V/I = 3.3/(150 × 10⁻⁶) = 22000 Ω = 22 kΩ.

- Reciprocal method: 1/R = 1/1000 + 1/2000 + 1/3000 = 11/6000, so R = 545.5 Ω. Product-over-sum twice: 1k∥2k = 2 × 10⁶/3000 = 666.7 Ω; then 666.7∥3k = 2 × 10⁶/3666.7 = 545.5 Ω. They agree.

- 2.5 = 9 × 10k/(R₁ + 10k), so R₁ + 10k = 90k/2.5 = 36 kΩ, giving R₁ = 26 kΩ.

- √1.06 ≈ 1 + 0.03 = 1.030 against an exact 1.02956, an error of +0.04%. 1/0.98 = (1 − 0.02)⁻¹ ≈ 1 + 0.02 = 1.020 against an exact 1.02041, an error of −0.04%.

## Chapter 5

- From the second equation x = y + 1. Substituting: 2(y + 1) + 3y = 12, so 5y = 10, y = 2 and x = 3.

- Multiply the first by 2 and the second by 5: 8a + 10b = 44 and 15a − 10b = 25. Adding gives 23a = 69, so a = 3, and then b = 2.

- D = (6)(1) − (3)(2) = 0. The determinant vanishes, so there is no unique solution. Inspecting the equations, the second is exactly half the first — they are the same equation written twice, so there are infinitely many solutions.

- D = (100)(90) − (−40)(−40) = 9000 − 1600 = 7400. D₁ = (5)(90) − (0)(−40) = 450, so I₁ = 60.8 mA. D₂ = (100)(0) − (−40)(5) = 200, so I₂ = 27.0 mA. Both are positive, meaning both mesh currents flow in the directions originally assumed.

## Chapter 6

- A straight line through the origin. Slope = 1/R = 1/2000 = 0.5 mS.

- At 100 Hz, H = 1/√(1 + 0.01) = 0.995. At 1 kHz, H = 1/√2 = 0.707. At 10 kHz, H = 1/√101 = 0.0995.

- V = 12 − 50I. Setting V = 8 gives 50I = 4, so I = 80 mA.

- A factor of 100 in magnitude per decade of frequency is a slope of −2 on log-log axes, which is 40 dB per decade. That indicates a second-order response.

## Chapter 7

- τ = RC = (22 × 10³)(100 × 10⁻⁹) = 2.2 ms. At t = 5 ms, v = 9·e^(−5/2.2) = 9·e^(−2.273) = 9(0.1031) = 0.928 V.

- 2 → 3.01 dB; 10 → 10 dB; 100 → 20 dB; 0.5 → −3.01 dB; 1000 → 30 dB.

- 20·log₁₀(250) = 47.96 dB, near enough 48 dB. After a 6 dB attenuator, 41.96 dB, which as a voltage ratio is 250/2 = 125.

- −95 dBm = 10^(−9.5) mW = 3.16 × 10⁻¹⁰ mW = 3.16 × 10⁻¹³ W. Adding 12 dB gives −83 dBm = 5.01 × 10⁻⁹ mW = 5.01 × 10⁻¹² W.

## Chapter 8

- 45° = π/4 = 0.785 rad; 90° = π/2 = 1.571 rad; 120° = 2π/3 = 2.094 rad; 270° = 3π/2 = 4.712 rad. And π/6 = 30°; 2π/3 = 120°; 3π/2 = 270°.

- Amplitude 2.5 A; f = 1000 Hz; T = 1 ms; phase −45°. At t = 0, i = 2.5·sin(−45°) = −1.77 A.

- Amplitude = 10/2 = 5 V. RMS = 5/√2 = 3.54 V.

- Because they are 90° apart, they add in quadrature: √(3² + 4²) = 5 V. Adding 3 and 4 directly would assume they peak at the same instant, which they do not — the quarter-cycle offset means one is at zero when the other is at its maximum.

## Chapter 9

- Sum = 6 − j2. Difference = 2 + j8. Product = (4 + j3)(2 − j5) = 8 − j20 + j6 − j²15 = 8 + 15 − j14 = 23 − j14.

- |5 + j12| = 13, angle arctan(12/5) = 67.4°, so 13∠67.4°. And 10∠−60° = 10cos(−60°) + j10sin(−60°) = 5 − j8.66.

- Multiply above and below by (4 + j): numerator (2 + j3)(4 + j) = 8 + j2 + j12 − 3 = 5 + j14; denominator 16 + 1 = 17. So the answer is 0.294 + j0.824. Checking: (0.294 + j0.824)(4 − j) = 1.176 + 0.824 + j(3.296 − 0.294) = 2.00 + j3.00. Correct.

- j² = −1 (two quarter-turns, a half-turn). j³ = −j (three quarter-turns). j⁴ = 1 (full turn, back to start). 1/j = −j (a quarter-turn backwards). In general jⁿ repeats with period 4, so jⁿ depends only on n modulo 4.

## Chapter 10

- e^(jπ) = cos π + j sin π = −1. e^(jπ/2) = j. e^(j2π) = 1.

- |3 + j4| = 5, angle arctan(4/3) = 0.927 rad = 53.13°. So 5e^(j0.927).

- Squaring and adding the exponential forms: [(e^(jθ) + e^(−jθ))/2]² + [(e^(jθ) − e^(−jθ))/(2j)]². The first expands to (e^(2jθ) + 2 + e^(−2jθ))/4; the second, noting (2j)² = −4, to −(e^(2jθ) − 2 + e^(−2jθ))/4. Adding, the e^(±2jθ) terms cancel and the constants give (2 + 2)/4 = 1.

- v(t) = 10·cos(500t + π/4). Frequency f = 500/2π = 79.6 Hz. At t = 2 ms the argument is (500)(0.002) + π/4 = 1 + 0.785 = 1.785 rad, and cos(1.785) = −0.213, so v = −2.13 V.

## Chapter 11

- For the voltage: RMS = 170/√2 = 120.2 V, already a cosine, so V = 120.2∠30° V. For the current: sin(377t) = cos(377t − 90°), and RMS = 4.24/√2 = 3.00 A, so I = 3.00∠−90° A.

- At 2 kHz, ω = 12566 rad/s. Z_L = jωL = j(12566)(0.022) = j276 Ω, that is 276∠90°. Z_C = −j/(ωC) = −j/(12566 × 470 × 10⁻⁹) = −j169 Ω, that is 169∠−90°.

- ω = 2π(400) = 2513 rad/s, so Z_L = j251 Ω and Z = 470 + j251 Ω. |Z| = √(470² + 251²) = 533 Ω at arctan(251/470) = +28.1°. I = 24/533 = 45.0 mA, lagging the voltage by 28.1°.

- Equal magnitudes require R = 1/(ωC), so ω = 1/RC = 1/(1000 × 10⁻⁷) = 10⁴ rad/s, giving f = 1592 Hz. There Z = 1000 − j1000, whose angle is −45°, and |Z| = 1414 Ω. The output across R is R/|Z| = 1000/1414 = 0.707 of the input — the −3 dB point, as expected.

## Chapter 12

- i = C·dv/dt = (1 × 10⁻⁶)(2000) = 2 × 10⁻³ = 2 mA.

- v = L·di/dt = (0.05)(3)(1000)cos(1000t) = 150·cos(1000t) V. The amplitude is 150 V and, since cosine leads sine by 90°, the voltage leads the current by 90°, as it must for an inductor.

- dv/dt = −(10)(2000)·sin(2000t + 30°) = −20000·sin(2000t + 30°). The amplitude is 20000 V/s, the factor of ω = 2000 having come down from the chain rule.

- By integration: v = (1/C)∫i dt = (1/10⁻⁵)(5 × 10⁻³)(0.2) = (1/10⁻⁵)(10⁻³) = 100 V. By charge: Q = It = (5 mA)(200 ms) = 1 mC, and v = Q/C = 10⁻³/10⁻⁵ = 100 V. The two agree.

# Appendix C — Glossary

Admittance — the reciprocal of impedance, Y = 1/Z, in siemens. Adds for parallel branches.

Accuracy — closeness of a measurement to the true value. Distinct from precision and from resolution.

Amplitude — the peak departure of a sinusoid from zero, written V_m. Not the same as the RMS value.

Argand diagram — the complex plane, with real part plotted horizontally and imaginary part vertically.

Conjugate — the complex number with the sign of its imaginary part reversed, written z*. Used to make denominators real.

Decibel — a logarithmic ratio. 10·log₁₀ for power ratios, 20·log₁₀ for voltage ratios. Dimensionless unless a reference is stated, as in dBm.

Derivative — the instantaneous rate of change of a quantity, written dv/dt.

Determinant — a scalar computed from a square matrix. A zero determinant signals that a system has no unique solution.

Dimensional analysis — checking that both sides of an equation reduce to the same combination of base units. Blind to dimensionless constants.

Engineering notation — scientific notation restricted to exponents that are multiples of three, so each maps onto an SI prefix.

Euler's formula — e^(jθ) = cos θ + j sin θ. The link between exponentials and sinusoids, and the foundation of phasor analysis.

Impedance — the complex ratio of voltage phasor to current phasor, in ohms. Real part resistance, imaginary part reactance.

Integral — the accumulated total of a quantity, equivalently the area under its curve.

j — the imaginary unit, defined by j² = −1. Written i by mathematicians; electrical engineers use j because i denotes current.

Phase — the angular offset of a sinusoid from a reference, in degrees or radians.

Phasor — a complex number encoding the amplitude and phase of a sinusoid, with the time dependence factored out.

Precision — the repeatability of a measurement. An instrument can be precise and inaccurate simultaneously.

Reactance — the imaginary part of impedance. Positive for inductors, negative for capacitors. Stores energy rather than dissipating it.

Resolution — the smallest change an instrument can display. Independent of accuracy.

RMS — root mean square: the DC value that would dissipate the same power. For a sinusoid, V_m/√2; for other waveforms, different.

Significant figures — the digits of a number that carry real information about its precision.

Time constant — τ, the characteristic time of a first-order exponential. Equal to RC or L/R.

Tolerance — the guaranteed range within which a component value lies, usually a percentage of the marked value.

Worst-case analysis — evaluating a design with every component at its least favourable tolerance extreme simultaneously.

# Appendix D — Chapter Summaries

Chapter 1 — A quantity is a number times a unit, and the multiplication is real, so units obey algebra. Seven SI base units; the electrical derived units are built from four of them. Dimensional analysis catches structural errors for free but is blind to dimensionless constants, including the 2π that separates hertz from radians per second.

Chapter 2 — Prefixes multiply by powers of ten and case is significant. Engineering notation restricts exponents to multiples of three so they map onto prefixes. Reciprocal prefix pairs cancel: micro times mega is one, milli times kilo is one. Exponents must match before mantissas can be added.

Chapter 3 — A measurement includes its uncertainty. Significant figures describe how to report a result, not how to compute one, so do not round intermediate steps. Relative uncertainties add for products; absolute uncertainties add for sums. Subtracting nearly equal numbers destroys precision. Accuracy, precision, and resolution are three different things.

Chapter 4 — The reciprocal of a sum is not the sum of the reciprocals. Product-over-sum handles exactly two parallel elements. Voltage divides in proportion to resistance, current in inverse proportion. The discriminant of a quadratic tells you whether a second-order circuit is overdamped, critically damped, or oscillatory.

Chapter 5 — Circuit equations from Kirchhoff's laws are linear, so systematic solution methods apply. Elimination scales better than substitution. A zero determinant indicates a physically impossible circuit, not an arithmetic mistake. The matrix form G·V = I is what simulators actually solve.

Chapter 6 — Read a transfer function at its limits before plotting it: low-frequency asymptote, high-frequency asymptote, and behaviour at the corner give the shape. Log axes turn power laws into straight lines whose slope reveals the order of the system. A logarithmic axis has no zero.

Chapter 7 — The exponential is the function whose rate of change equals itself, which is why it governs every first-order circuit. Logarithms convert multiplication to addition, which is the entire reason decibels exist. Use 10·log for power and 20·log for voltage. dB values add along a chain; dBm values do not.

Chapter 8 — A sinusoid is fixed by three numbers: amplitude, angular frequency, and phase. Radians are required for calculus; degrees are convenient for phase. The Pythagorean identity produces impedance magnitudes and the sum-of-angles identity produces mixing. RMS is defined by equal heating, and the 0.707 factor applies only to sinusoids.

Chapter 9 — A complex number carries two quantities at once, which for engineers means magnitude and phase. Rectangular form suits addition; polar form suits multiplication. The conjugate makes denominators real. Multiplying by j is a 90° rotation, and j² = −1 is that fact applied twice.

Chapter 10 — Euler's formula identifies e^(jθ) with a unit arrow at angle θ, bridging rotation and oscillation. A real sinusoid is the real part of a rotating complex exponential. Differentiating that exponential multiplies it by jω, which converts calculus into arithmetic.

Chapter 11 — In a linear circuit at one frequency, only amplitude and phase vary, so the frequency can be factored out into a phasor. Impedance is the resulting complex voltage-to-current ratio: R, jωL, and 1/(jωC). Every DC circuit technique carries over with R replaced by Z. Component voltages add as phasors, so they may exceed the supply voltage.

Chapter 12 — A derivative is a rate and an integral is an accumulation. Capacitors and inductors are defined by derivatives, which is why a capacitor blocks DC and an inductor passes it. Differentiating a sinusoid brings down a factor of ω and advances the phase by 90°, which is exactly the jω of phasor analysis.

Chapter 13 — Book 3 uses Chapters 4, 5, 7, and 12 for transients and Chapters 8 to 11 continuously for AC. Filters use Chapters 6 and 7. RF work leans on complex arithmetic and decibels throughout. The Fourier and Laplace transforms generalise the phasor idea, and nothing later in the series is mathematically harder than what is here.