Semiconductor Physics and Devices

Book 3 of the Electrical Engineering Series

Parts I–VII  —  From the Crystal to the Chip

Lothar J. Musiol

Copyright © 2026 Lothar J. Musiol. Kindle edition.

# How to Use This Book

Book 1 taught the circuit laws and the passive parts. Book 2 treated a diode, a transistor, and an op-amp as blocks you could drop into a schematic. This book opens those blocks. By the end of Part I you can say, with a number, how many free carriers a piece of doped silicon has, why a pn junction grows a built-in voltage with nobody applying a battery, and why the phrase "the diode turns on at 0.7 volts" is a useful sketch and a bad physical claim.

Two numbers are fixed for the whole book, so that a later chapter cannot quietly adopt a different room temperature. Thermal voltage VT is 26 mV. The intrinsic carrier concentration of silicon, ni, is 1.0×10^10 per cubic centimeter. Real tables scatter a little around both of those, because the bandgap model and the exact temperature move them. Hand work in this book uses the declared pair. When a vendor model quotes a different ni, use the vendor number for that part and keep this pair for the teaching problems.

Electron and hole mobilities depend on doping and temperature. Whenever a problem in this book does not hand you a mobility, use μn = 1000 cm²/V·s and μp = 480 cm²/V·s. Those are round working values for lightly doped silicon near room temperature, not a promise about a particular wafer.

Part II takes up the bipolar transistor and Part III the MOSFET, down to the fin and the nanosheet. Part IV builds the op-amp of Book 2 from those transistors and derives the noise they make. Part V treats CMOS as a chip: gate delay and power, memory cells, and the failures that end a chip's life. Part VI turns the junction into a source and detector of light and into a high-voltage switch. Part VII follows a transistor through the fab, builds the piezoelectric thin films and acoustic resonators behind every phone's filters, and explains the compact models and process design kits that carry all of these devices into a simulator. The circuit check for Parts II to IV is Tietze, Schenk, and Gamm, Halbleiter-Schaltungstechnik, 16th edition (2019), whose op-amp chapter was rewritten for that edition. High-speed boards, radio layout, switching regulators, and software-defined radio are later books in this series, named in Appendix C so this book does not pretend to cover them.

# Part I. Carriers and Junctions

# Chapter 1. A Crystal That Can Be Talked Into Conducting

Window glass and a processor die both contain silicon. One of them sits in a wall and refuses to carry a current you would bother to measure. The other switches billions of times a second because a small voltage was allowed to decide how many charges are free to move. The difference is not a mystery metal. It is how pure the crystal is, and which few atoms you deliberately put in.

## 1.1 Three ways a solid treats its electrons

A useful picture, good enough for every circuit in this series, sorts solids by the energy gap between the last filled band of electron states and the next empty one. In a metal the bands overlap, so a tiny electric field finds electrons already free to drift. In a good insulator the gap is several electron-volts wide, and ordinary voltages do not lift electrons across it. Silicon sits in between. Its gap is about 1.12 eV at room temperature. Germanium is narrower, about 0.66 eV, which is why a germanium junction leaks more when the board gets warm. Gallium arsenide is wider, about 1.42 eV. Silicon carbide, used where the voltage and the heat are both high, is wider still, a few electron-volts depending on the crystal form. Silicon won the ordinary chip not because its gap is magical, but because the industry learned to grow it as a nearly perfect crystal and to put an excellent insulator, silicon dioxide, on top of it.

## 1.2 Intrinsic means almost empty

In a perfect silicon crystal at absolute zero, every bond is full and no charge moves. At room temperature a few electrons have enough thermal energy to break a bond. Each broken bond leaves an electron in the conduction band and a hole, a missing bond, in the valence band. They are created in pairs, so in pure material their concentrations are equal. That common value is ni, and this book takes it as 1.0×10^10 cm⁻³.

A cubic centimeter of silicon holds about 5×10^22 atoms. Ten billion free electrons in that same volume is a very small fraction. Intrinsic silicon is a poor conductor. The whole point of doping, in the next chapter, is to raise one of those two populations by many orders of magnitude while the other falls.

## 1.3 Conductivity is charge times speed per field

Drift current density is conductivity times electric field. For a semiconductor that still has both electrons and holes,

σ = q (n μn + p μp)

where q is the elementary charge, n and p are the electron and hole concentrations, and μn and μp are the mobilities. Resistivity is 1/σ. In doped material one term usually swamps the other, which is why a resistor formula and a doped-silicon formula can look alike: both say "lots of carriers, easy current."

Key idea. Pure silicon at room temperature has very few free charges. Doping, not the word "semiconductor" by itself, is what makes a useful current.

## Worked Example 1.1

A piece of n-type silicon is doped so that n = 1.0×10^16 cm⁻³. The hole population has already fallen to the value required by the mass-action law in Chapter 2, p = ni²/n = 1.0e+04 cm⁻³. Using this book's μn, find the resistivity. The hole term is negligible next to the electron term.

σ = q n μn = (1.602×10⁻¹⁹)(1.0×10^16)(1000) = 1.602 (Ω·cm)⁻¹. ρ = 1/σ = 0.624 Ω·cm. A metal is a million times more conductive. This doped slice is already a practical semiconductor, and it is still nothing like copper.

## Practice

1. Repeat Worked Example 1.1 with n = 1.0×10^15 cm⁻³ and the same mobility. Find p and ρ.

2. Why does a wider bandgap, all else equal, mean a smaller ni and a junction that leaks less when hot?

3. Intrinsic silicon has n = p = ni. Estimate σ with both mobilities. Is the hole term ignorable here, the way it was in the worked example?

# Chapter 2. Doping, and the Partner That Gets Scarce

Soup tastes salty after a pinch of salt, not after you have replaced the water. A semiconductor is the same kind of trick. You leave the crystal almost entirely silicon, and you swap in about one foreign atom per million, or per billion. That pinch decides whether the free charges are electrons or holes, and about how many of them there are.

## 2.1 Donors and acceptors

A column-V atom such as phosphorus has one more valence electron than silicon. Sitting on a silicon site, it completes the four bonds and has one electron left, loosely held. A little thermal energy frees that electron into the conduction band. The phosphorus atom, now positive, stays put. It donated an electron. Call its concentration ND. The material is n-type, and the electrons are the majority carriers.

A column-III atom such as boron has one valence electron too few. It completes three bonds and borrows an electron from a neighboring bond, leaving a hole that can hop. The boron atom, now negative, stays put. Call its concentration NA. The material is p-type, and the holes are the majority carriers.

## 2.2 Mass action

In equilibrium, generation and recombination balance. The product of the two carrier concentrations is fixed by the crystal and the temperature, not by how you doped it:

n p = ni²

Add donors and you raise n. The product cannot rise with it, so p falls. Add acceptors and the mirror happens. This is why a heavily doped n-type region is full of electrons and starved of holes. The starved population is still real, and it will matter as soon as a junction sweeps those minorities across a boundary.

## 2.3 Charge neutrality, the short form

A lump of semiconductor on the bench is not charged up like a capacitor plate. In the bulk, away from surfaces,

n + NA = p + ND

for ionized donors and acceptors, which is the right assumption at room temperature for ordinary silicon doping. If donors dominate and ND is far above ni, then n ≈ ND and p ≈ ni²/ND. For the 10^16 cm⁻³ example, p falls to 1.0e+04 cm⁻³, six decades below the majority population. The same arithmetic with acceptors swaps the names.

Key idea. Doping sets the majority. Mass action sets the minority. You do not choose them independently.

## Worked Example 2.1

Boron is added to silicon at NA = 1.0×10^17 cm⁻³. Donors are negligible. Find the majority and minority concentrations.

Holes are the majority: p ≈ NA = 1.0×10^17 cm⁻³. Electrons are the minority: n = ni²/NA = 1.0e+03 cm⁻³. There is one free electron for every ten million holes.

## Practice

1. Phosphorus at 1.0×10^15 cm⁻³, acceptors negligible. Find n and p.

2. Someone says "n-type means there are no holes." Answer with the mass-action law and one number from this chapter.

3. Both ND = 1.0×10^16 cm⁻³ and NA = 1.0×10^15 cm⁻³ are present. Who wins, and what is the approximate majority concentration? Use n + NA = p + ND and n p = ni², and drop the small term only after you have named it.

# Chapter 3. Two Ways Charge Moves

A crowd leaves a stadium two ways. People walk downhill because the ground slopes, and people also spread from the packed gate into the empty street even if the street is flat. Electrons do both. The slope is the electric field. The packed gate is a concentration gradient. Circuits need both pictures, because a diode uses diffusion at the junction and drift in the bulk.

## 3.1 Drift

An electric field pulls electrons one way and holes the other. The drift speeds are μn E and μp E, in opposite directions, but both movements carry positive current in the direction of the field. Holes moving with the field are a positive current. Electrons moving against the field are also a positive current, because negative charge moving backward is the same as positive charge moving forward. That is why the conductivity formula adds the two terms instead of subtracting them.

## 3.2 Diffusion and the Einstein relation

Where the concentration changes with position, carriers spread from crowded toward empty. The diffusion constants Dn and Dp have units of cm²/s. In equilibrium they are not a second independent property of the crystal. Einstein's relation ties them to the mobilities and to VT:

Dn = μn VT,    Dp = μp VT

with VT in volts. This book's μn and VT give Dn = 26.0 cm²/s. Dp = μp VT = 12.48 cm²/s. The electron diffusion current density along x is q Dn dn/dx, and the sign is fixed by the fact that electrons diffusing toward +x carry negative charge that way, which is a negative current in the +x direction. Holes diffusing toward +x are a positive current. The formulas used below take magnitudes when the question is "how large," and they keep the sign when the question is "which way."

## 3.3 Drift and diffusion together

In the bulk of a uniformly doped resistor, the gradient is about zero and drift does the work. At a pn junction in equilibrium, a large diffusion current wants to pour carriers across the boundary, and a drift current from the built-in field cancels it. Net current is zero. The cancellation is exact in equilibrium and is the reason the next chapter can compute a voltage without ever connecting a battery.

Key idea. Field moves carriers as drift. A crowd moving into an empty region is diffusion. Equilibrium at a junction is those two currents canceling.

## Worked Example 3.1

Electron concentration falls from 1.0×10^16 cm⁻³ to 1.0×10^15 cm⁻³ across 1 μm (1×10⁻⁴ cm). Using this book's Dn, find the magnitude of the diffusion current density.

|dn/dx| = 9.0×10^15 / 1×10⁻⁴ = 9.0×10^19 cm⁻⁴. |Jn| = q Dn |dn/dx| = (1.602×10⁻¹⁹)(26.0)(9.0×10^19) = 374.909 A/cm². That is a serious current density for a gradient only a micron long, which is why junctions, whose gradients are even steeper, care so much about diffusion.

## Practice

1. Compute Dn and Dp from this book's mobilities and VT. Give both in cm²/s.

2. A hole gradient of 1.0×10^20 cm⁻⁴ drives hole diffusion. Find the magnitude of Jp. Which way does conventional current point, relative to the direction the holes are spreading?

3. In a long uniformly doped bar, dn/dx = 0 and E = 10 V/cm, n = 1.0×10^16 cm⁻³. Find the drift current density and compare it with Worked Example 3.1.

# Chapter 4. The Junction Sits There with a Voltage Already On It

Push a crowded room against an empty hall and leave the door open. People spill through until a jam at the doorway, people pressed shoulder to shoulder, makes further spilling as hard as the crowding that caused it. A pn junction does that with electrons and holes. Nobody has to apply a battery. The voltage is the jam.

## 4.1 What crosses, and what is left behind

On the n-side, electrons are many and holes are few. On the p-side, the reverse. Electrons diffuse into the p-side. Holes diffuse into the n-side. Each electron that leaves the n-side leaves a positive donor ion behind. Each hole that leaves the p-side leaves a negative acceptor ion behind. Those ions cannot wander off. They are stuck in the crystal. A sheet of positive charge therefore faces a sheet of negative charge, with a region between them that has been emptied of mobile carriers. That region is the depletion layer, or space-charge region.

## 4.2 Built-in voltage

The field of those two sheets points from the positive donor ions toward the negative acceptor ions, which is from n toward p. It pushes electrons back toward the n-side and holes back toward the p-side, opposing the diffusion that created it. Equilibrium is the balance. The potential difference that belongs to that balance is the built-in voltage,

Vbi = VT ln(NA ND / ni²)

for an abrupt junction with both sides doped well above ni. Notice what raises Vbi: heavier doping on either side, or a smaller ni. Notice what does not appear: the applied voltage. Vbi is a property of the two dopings and the temperature. Applying a battery changes the barrier the carriers actually face, which is Vbi minus the applied forward voltage, but it does not rewrite this formula.

## 4.3 How wide the empty region is

For a one-sided abrupt junction, almost all of the depletion width sits on the lightly doped side. If that side is n-type with doping ND and the applied voltage is V (positive V means forward),

W = sqrt( 2 εs (Vbi − V) / (q ND) )

εs is the permittivity of silicon, 11.7 times the permittivity of free space. Forward bias shrinks W. Reverse bias grows it. The square root is why doubling the reverse voltage does not double the width.

Key idea. A pn junction in a drawer already has a built-in voltage. The battery later adds to that story. It does not start it.

## Worked Example 4.1

Both sides are doped at 1.0×10^16 cm⁻³. Find Vbi. Then treat the junction as one-sided on an n-type side of that same doping, at zero applied volts, and find W. This is a teaching idealization of a symmetric junction, used so the arithmetic stays visible.

NA ND / ni² = 1.0×10^32 / 1.0×10^20 = 1.0×10^12. ln(10^12) = 27.631. Vbi = (0.026)(27.631) = 0.718 V. W = 3.048e-05 cm = 0.305 μm. A few tenths of a micron of emptied crystal is enough to hold about seven-tenths of a volt.

## Practice

1. NA = 1.0×10^18 cm⁻³ and ND = 1.0×10^16 cm⁻³. Find Vbi with this book's constants.

2. Using the W of Worked Example 4.1 as the zero-bias width, does a reverse bias of a few volts make W several times larger, or only somewhat larger? Argue from the square root before you compute.

3. Why does heating the crystal, which raises ni, lower Vbi?

# Chapter 5. The Diode Law Is a Crowd Crossing a Barrier

A turnstile at a stadium lets a few people through when the bar is high, and a great many when someone lowers it by a hand's width. The diode is that turnstile for the minority carriers waiting at the edge of the depletion layer. Lower the barrier a little and the forward current multiplies. Raise it and the forward crowd disappears, leaving only a small reverse trickle.

## 5.1 The equation

For an ideal diode the current is

I = IS ( e^(V / VT) − 1 )

V is the applied voltage, positive when the p-side is positive, which is forward bias. IS is the saturation current, set by the area, the doping, and the lifetime of the minorities. This chapter's worked diode uses IS = 1 fA = 10⁻¹⁵ A, a small-junction teaching value, not a claim about a particular part number. VT is 26 mV. The "− 1" matters near zero and in reverse. A few tenths of a volt forward, the exponential is so large that I ≈ IS e^(V/VT).

## 5.2 Sixty millivolts per decade

Ask how much extra forward voltage multiplies the current by ten, once you are well above VT. The exponential must grow by 10, so the added voltage is VT ln(10).

VT ln(10) = 0.026 × 2.3026 = 59.9 mV.

Call it 60 mV per decade of current at this book's room temperature. A real silicon diode is often closer to 60 mV only in a mid-current window. At very low current, recombination in the depletion layer makes the effective step larger. At high current, series resistance steals voltage. The ideal law is the spine. The part on the bench wears extra clothes, which is Chapter 6.

## 5.3 There is no switch at 0.7 V

A forward drop near 0.7 V is what you see when a small silicon diode is carrying a current of a good fraction of a milliamp, with an IS on the order of a femtoamp. It is an operating point, not a threshold the device crosses. Below that voltage the diode is already conducting. It is conducting less. Circuit sketches that draw an ideal switch plus a 0.7 V battery are welcome in Book 1 problems. They are the wrong picture if you are asking how leakage, a photodiode, or a low-current bias network behaves.

Key idea. Forward current is exponential in voltage. A "turn-on voltage" is a current you chose to care about, read back through that exponential.

## Worked Example 5.1

IS = 1.0×10⁻¹⁵ A and VT = 26 mV. Find I at 0.50 V, 0.60 V, and 0.70 V. Then find the forward voltage that produces 1.0 mA.

At 0.50 V, I = 2.248e-07 A = 0.225 μA. At 0.60 V, I = 1.052e-05 A = 10.52 μA. At 0.70 V, I = 4.927e-04 A = 0.493 mA. From 0.60 V to 0.70 V the current rises by a factor of 46.8, close to the decade-and-a-half that 100 mV predicts (100/60 ≈ 1.7 decades, and 10^1.67 ≈ 47). For 1.0 mA, V = VT ln(I/IS + 1) = 0.718 V. The familiar seven-tenths of a volt showed up because we asked for about a milliamp, not because the silicon changed state.

## Practice

1. Same diode. Find I at 0.65 V.

2. How much must V rise to take this diode from 0.1 mA to 1 mA, using the 60 mV/decade rule? Check it against the exact formula.

3. Reverse bias of 0.2 V. Compute I from the full equation. Why is the result about −IS, and why is a real reverse current often larger than this ideal?

# Chapter 6. The Diode You Can Buy

The part in the drawer has a glass or plastic body, two leads, a junction area someone chose, and a data sheet with a curve that only roughly matches Chapter 5. The law is still in there. Three extras decide whether a circuit works: the junction behaves as a capacitor, a large reverse voltage eventually breaks the crystal down, and a forward current does not stop the instant you reverse the voltage.

## 6.1 Junction capacitance

The depletion layer is an insulator between two conducting regions, which is a capacitor. Its value falls as reverse bias widens the layer. For an abrupt junction a serviceable model is

Cj = Cj0 / sqrt(1 − V/Vbi)

with V negative in reverse, and Cj0 the zero-bias value set by area and doping. Varactor diodes are ordinary junctions used on purpose as voltage-controlled capacitors. Rectifier diodes carry the same capacitance whether you asked for it or not. At radio frequency that capacitance is part of the matching network. At audio it is often ignorable. The frequency, not the part's name, decides.

## 6.2 Breakdown

Push reverse bias high enough and the field in the depletion layer rips bonds apart or gives carriers enough energy between collisions to create more carriers. Current then rises steeply at nearly constant voltage. A Zener diode is a junction designed so that this knee sits at a useful, specified voltage, and so that the part can survive the current you intend to run through that knee. A signal diode's breakdown voltage is a limit, not a feature. Exceed it without a current limit and the junction overheats.

Two microscopic stories share that knee. Zener tunneling dominates at low breakdown voltages. Avalanche, the carrier-multiplication story, dominates at higher ones. Data sheets still say "Zener" for both. The circuit fact is the steep reverse knee. The name on the bag is older than the distinction.

## 6.3 Reverse recovery

Forward current stores minority charge on both sides of the junction. When the circuit reverses the voltage, that stored charge has to come back out, or recombine, before the diode can block. For a interval on the order of the storage delay the diode still conducts backward. Fast-recovery and Schottky diodes exist because that interval is expensive in a switching supply. A Schottky diode is a metal-semiconductor junction. It has a lower forward drop and almost no stored minority charge, and it generally leaks more and breaks down sooner than a pn diode of similar size. Choose it when the switching edge is the problem. Choose a pn diode when the reverse voltage and the leakage are the problem.

Key idea. The exponential law is the junction. Capacitance, breakdown, and stored charge are why the data sheet is thicker than the equation.

## Worked Example 6.1

An abrupt-junction diode has Cj0 = 10 pF and the Vbi of Worked Example 4.1, 0.718 V. Find Cj at a reverse bias of 5 V.

V/Vbi = −5 / 0.718 = -6.960. 1 − V/Vbi = 7.960. The square root is 2.821. Cj = 10 pF / 2.821 = 3.54 pF. Five volts of reverse bias cut the capacitance to about a third. A tuner that expects a wide capacitance range is using this same square root, over whatever bias its varactor is allowed.

## Practice

1. Cj0 = 20 pF, same Vbi, reverse bias 2 V. Find Cj.

2. A rectifier data sheet lists a reverse-recovery time. What stored thing is that time waiting on, and why does a Schottky diode largely skip it?

3. A 5.1 V Zener is used as a reference with a series resistor from a 12 V rail. In one sentence, what does the resistor do that the junction itself does not?

# Chapter 7. GaAs, GaN, and InP Are Not One Transistor

A handset, a radar brick, and a 94 GHz receiver can all be described as "a microwave transistor on a chip." The crystals are not interchangeable. Gallium arsenide is the mature microwave compound semiconductor. Gallium nitride is the wide-bandgap material for watts, and also, on a silicon wafer, for hundreds of volts. Indium phosphide is the III-V you pick when the gate is tens of nanometers wide and the noise figure is the specification. They meet in millimeter-wave power amplifiers. Almost nowhere else.

## 7.1 Two figures of merit, not one winner

Johnson's figure of merit rewards breakdown voltage times frequency. Transit time and mobility reward a fast, light electron. GaN wins the first contest. InP wins the second. That is the whole split, and it is why a chapter that ranks them on a single "which is more advanced" scale is already wrong.

Use these working numbers, the same way Part I used a declared ni. GaAs bandgap about 1.42 eV, breakdown field about 0.4 MV/cm, bulk electron mobility about 8500 cm²/V·s. GaN bandgap about 3.4 eV, breakdown field about 3.3 MV/cm, two-dimensional-electron mobility about 1500 to 2000 cm²/V·s. InP itself is about 1.34 eV. The channel in an InP HEMT is usually InGaAs, about 0.7 to 1.3 eV, with a very high mobility and the highest saturation velocity of the usual RF III-Vs. RF supply voltage is about 3 to 12 V for GaAs, about 1 to 3 V for an InP HBT and about 1 to 5 V for an InP HEMT, and about 20 to 50 V for GaN. Power density is about 0.5 to 2 W per millimeter of gate for GaAs, about 0.3 to 1.5 W/mm for InP, and about 5 to 12 W/mm for GaN. Production GaN reaches roughly 80 to 200 GHz ft. The best published InP HEMTs are past 600 GHz ft, with fmax in the terahertz class.

## 7.2 Say HEMT or HBT before you say the crystal

GaAs comes as a MESFET, which is old, a pHEMT, and an HBT. The pHEMT is the low-noise amplifier, the switch, and some power amplifiers. The HBT is the handset power amplifier and the gain block. GaN, in any part you can buy for RF or for power conversion, is a HEMT. There is no serious GaN HBT in production. InP comes as a HEMT, or a metamorphic HEMT, with an InGaAs channel, and as an HBT. The HEMT is the low-noise amplifier, the mixer, the switch, and the low-power millimeter-wave amplifier. The HBT is high-speed analog, data converters, and optical modulator drivers, plus some power amplifiers.

Most RF HEMTs are depletion-mode. The gate wants a negative supply, and the drain must not come up before that gate is in a safe place. That is a sequencer, not the enhancement-mode MOSFET of a later chapter. Enhancement-mode GaN shows up in power conversion. Do not import that bias story into a radar PA without reading the data sheet.

Key idea. Name the device. "InP versus GaN" is not a comparison until you have said HEMT or HBT.

## 7.3 Why voltage buys watts

For a sine-wave load line, the optimum resistance is the voltage swing divided by the current swing, and the RF power is half the product of those peaks. A higher legal voltage, at the same current, is both more power and an easier match.

## Worked Example 7.1

A cell is allowed 0.20 A peak. Compare a 5 V peak swing with a 28 V peak swing. This is a GaAs-like rail against a GaN-like rail, not a promise about a foundry.

At 5 V, Ropt = 5 / 0.20 = 25 Ω, and P = (5)(0.20)/2 = 0.50 W. At 28 V, Ropt = 28 / 0.20 = 140 Ω, and P = (28)(0.20)/2 = 2.8 W. Same current, 5.6 times the voltage, 5.6 times the power, and the load the matcher has to present moved from 25 Ω toward 140 Ω, closer to a 50 Ω system and kinder to a wideband network. GaN's extra power density, several watts per millimeter rather than about one, is a second gift on top of this arithmetic. Heat still collects the bill. SiC is the reason a GaN PA can pay it. InP's substrate cannot.

## 7.4 Where each one is the right part

Use a GaAs pHEMT when the noise figure of a microwave receiver is the spec. Use a GaAs HBT for a phone or WLAN power amplifier at 3 to 5 V. Use GaAs when you want a dense MMIC whose passives sit on semi-insulating GaAs, and for many sockets above about 40 GHz where that foundry still has more gain and a better noise figure than production GaN.

Use GaN when the radio needs watts: radar, electronic warfare, a satcom uplink, a macro base station, anything past about 5 to 10 W. The rail is 28 to 50 V. The device is rugged into a bad mismatch when it is GaN on SiC. Use GaN-on-silicon for a 100 to 900 V switch that replaces a silicon MOSFET. That market contains no GaAs and no InP.

Use an InP HEMT for the lowest noise figure from about 30 GHz through W-band and into G-band, and whenever ft in the hundreds of gigahertz is the reason the gate exists. Use an InP HBT for an optical driver and for mixed-signal above about 100 Gbaud. InP is also a photonics crystal. GaN is not.

The only fair fight is millimeter-wave power. A 28 to 40 GHz base-station PA is GaN. From about 77 GHz up, at milliwatts to a few watts, InP often still has more gain and needs less combining. GaN shows up where watts and bandwidth are both required. A module may use an InP low-noise amplifier in front of a GaN power amplifier. Neither crystal is the phone PA. Below about 3 GHz at hundreds of watts, look at LDMOS or GaN, not InP. Cheap WLAN is SiGe, CMOS, or GaAs.

Key idea. A 94 GHz low-noise amplifier starts with InP. A 20 W X-band brick starts with GaN. A 650 V buck starts with GaN-on-silicon and never with InP.

## 7.5 What the circuit books still are

Cripps's load line still draws both the GaAs-era example and the GaN PA. GaN adds trapping, knee walkout, and memory, so the linearity fix is backoff or digital predistortion, not a polynomial copied from a handset HBT. InP circuit design looks like high-speed analog plus a foundry HBT or HEMT kit. A GaN PA looks like that load line, a thermal path through SiC, and a 50 V supply whose hot loop was already a subject in the regulator discussion of a later book. There is no popular textbook chapter that replaces the foundry model handbook.

## Practice

1. Repeat Worked Example 7.1 for a 40 V peak and a 0.50 A peak. Find Ropt and P.

2. A receiver at 94 GHz is 0.4 dB short of its noise-figure spec, and the input power is small. Which device family do you open first, and which do you not open?

3. Why can the same word "GaN" mean a 50 V radar transistor and a 650 V switch, while "InP" cannot follow it into the switch?

# Part II. The Bipolar Transistor

Part I stopped at the diode. This part builds the two-junction device that sits on that physics: the bipolar junction transistor, in this book's own words, checked against Tietze/Schenk 16th, Sedra/Smith, and Gray/Meyer. The constants stay the ones Part I declared: VT = 26 mV, and IS = 1 fA unless a new value is named. Book 2 uses the transistor inside circuits. This part explains why it behaves the way Book 2 assumes.

# Chapter 8. Two Junctions, One Current

Put two diodes back to back and you do not get a transistor. You get two diodes. The bipolar transistor works because its two junctions share a region so thin that carriers injected across the first junction reach the second before they have time to recombine. That one geometric fact, a base far thinner than a diffusion length, turns the diode law of Chapter 5 into a controlled current source. This chapter follows the electrons across that base and counts what arrives.

## 8.1 Three regions and four modes

An NPN transistor is an n-type emitter, a p-type base, and an n-type collector. The emitter is doped heavily, the base moderately, and the collector lightly. Each of the two junctions can be forward- or reverse-biased, which gives four modes. Forward-active, the amplifying mode, has the emitter-base junction forward-biased and the collector-base junction reverse-biased. Saturation has both forward-biased. Cutoff has both reverse-biased. Reverse-active swaps the roles of emitter and collector, works poorly because the device was not built symmetrically, and is mostly of interest when a circuit puts a transistor there by accident.

## 8.2 What crosses the base

In forward-active mode the emitter-base junction behaves like the diode of Chapter 5: it injects electrons from the emitter into the base, and the number injected rises exponentially with VBE. In a diode those minority electrons would diffuse into the p-side and recombine within a diffusion length, typically tens of micrometers. In a transistor the base width WB is a fraction of a micrometer. The electrons diffuse straight across it, reach the edge of the collector-base depletion layer, and the field there sweeps them into the collector. The collector junction's reverse bias does not stop them; it collects them, the same way Chapter 4's built-in field moves minority carriers across a junction.

Because the base is thin, the electron concentration falls almost linearly from its injected value at the emitter edge to nearly zero at the collector edge. Chapter 3's diffusion law then gives the collector current directly. The result has the diode's form, with a saturation current set by the base:

IC = IS e^(VBE / VT),    IS = q A Dn ni² / (NA,B WB)

Here A is the emitter area, Dn the electron diffusion constant in the base, and NA,B the base doping. The product NA,B WB is the base's total doping per unit area, called its Gummel number. A thinner or lighter base gives more collector current for the same VBE.

## Worked Example 8.1

Declare an emitter area A = 100 µm² (1.0×10⁻⁶ cm²), a base doping NA,B = 1.0×10¹⁷ cm⁻³, a base width WB = 0.10 µm (1.0×10⁻⁵ cm), and an electron diffusion constant in the base Dn = 10 cm²/s, lower than Appendix A's hand value because heavy doping lowers mobility. Use ni = 1.0×10¹⁰ cm⁻³. Find IS, and the base transit time WB²/(2Dn).

IS = q A Dn ni²/(NA,B WB) = (1.602×10⁻¹⁹)(1.0×10⁻⁶)(10)(1.0×10²⁰)/((1.0×10¹⁷)(1.0×10⁻⁵)) = 1.6×10⁻¹⁶ A, or 0.16 fA. That is the same order as the 1 fA this book uses for its teaching junction; real small-signal transistors land between about 0.1 fA and 10 fA. The transit time is (1.0×10⁻⁵)²/(2 × 10) = 5.0×10⁻¹² s, 5 ps. An electron spends five picoseconds crossing this base. That number returns in Chapter 9 as the floor under the transistor's speed.

## 8.3 Where the base current comes from

The base contact must supply two currents. The first is holes injected backward from the base into the emitter, because the forward-biased emitter-base junction is a diode in both directions. The second is holes consumed by the few electrons that do recombine on their way across the base. A good transistor makes both small.

The backward injection is controlled by the same physics as the forward one, with the roles exchanged: holes diffuse into the emitter, whose doping is NE and whose width is WE. If backward injection dominates, the ratio of the two currents, the current gain β = IC/IB, is approximately

β ≈ (Dn NE WE) / (Dp NA,B WB)

This is why the emitter is doped far more heavily than the base. Every factor of ten in the doping ratio is a factor of ten in gain.

## Worked Example 8.2

Keep the base of Worked Example 8.1. Declare an emitter doping NE = 1.0×10¹⁹ cm⁻³, an emitter width WE = 0.20 µm, and a hole diffusion constant in the emitter Dp = 2 cm²/s. Estimate β from the injection ratio alone.

β ≈ (10 × 1.0×10¹⁹ × 0.20×10⁻⁴)/(2 × 1.0×10¹⁷ × 0.10×10⁻⁴) = (2.0×10¹⁵)/(2.0×10¹²) = 1000. Real silicon transistors with similar dimensions show β of roughly 100 to 300. The simple ratio overestimates because heavy doping narrows the emitter's bandgap, which raises its effective ni² and therefore the backward hole injection, and because recombination in the base and at the surface adds base current. The ratio is the right guide to design direction. The data sheet is the guide to the number.

## 8.4 Kirchhoff and the declared transistor

Whatever happens inside, charge is conserved at the terminals: IE = IC + IB. The fraction of emitter current that reaches the collector is α = IC/IE = β/(β + 1).

Declared picture for this part: β = 100, IB = 10 µA. Then IC = 1.00 mA, IE = 1.01 mA, and α = 0.990. The emitter-base voltage that produces that collector current, from the same law as Chapter 5, is VBE = VT ln(IC/IS + 1) = 0.718 V with IS = 1 fA. That is the usual hand value of 0.7 V with this book's own constants underneath it.

The order of cause and effect matters. VBE sets IC through the exponential; IB follows as IC/β. Book 2 can treat the transistor as a current-controlled amplifier for convenience, but the physics is voltage control. That distinction explains why β varies widely from part to part while the relation between VBE and IC holds closely: β depends on recombination and backward injection, which are hard to control, while IC depends on the base's Gummel number, which a process controls well.

Key idea. The collector current is set by VBE through the base's Gummel number. The base current is a loss that β measures. A thin, lightly doped base and a heavily doped emitter make that loss small.

## 8.5 Saturation and stored charge

If the collector voltage falls far enough that the collector-base junction becomes forward-biased, the collector begins injecting electrons back into the base. The transistor is saturated. The collector-emitter voltage collapses to roughly 0.1 V to 0.2 V, IC no longer follows βIB, and extra base current simply piles up as stored minority charge in the base and collector. A saturated transistor is a good switch in the on state. It is a slow one to turn off, because that stored charge must be removed before the collector junction can block, exactly the reverse-recovery story of Chapter 6. Logic and switching circuits avoid deep saturation for this reason, for example with a Schottky diode from base to collector that diverts the excess base drive.

## 8.6 PNP, SiGe, and the compound HBT

A PNP transistor is the same device with every doping and every sign reversed: holes cross the base, and currents and voltages flip direction. Because holes diffuse more slowly than electrons, a PNP of the same geometry has a lower β and a longer transit time, which is why fast circuits favor NPN devices. A silicon-germanium heterojunction bipolar transistor grades germanium into the base, which lowers the base bandgap, raises IS and β, and builds in a field that speeds electrons across. It is still a bipolar transistor with the same equations. The GaAs and InP heterojunction transistors of Chapter 7 belong to the same family, for radio rather than for logic.

## Practice

1. For the transistor of Worked Example 8.1, find IS if the base width is reduced to 0.05 µm with the same doping. What happens to the transit time?

2. A transistor has β = 150. Find α, and find IE when IC = 2.0 mA.

3. With IS = 1 fA, find VBE for IC = 0.1 mA and for IC = 10 mA. How many millivolts separate them, and why?

4. Using the ratio of Section 8.3, by what factor does β change if the emitter doping is raised from 1.0×10¹⁹ to 3.0×10¹⁹ cm⁻³, all else equal? Why would a real device gain less than that?

5. In one or two sentences, explain why a saturated transistor turns off more slowly than one held in forward-active mode.

# Chapter 9. Bias, Early Effect, and the Hybrid-π

Chapter 8 gave the transistor's large-signal law: collector current exponential in VBE, base current smaller by β. A circuit designer needs two more things from the device. The first is a way to hold the operating point steady when β and VBE refuse to be constant. The second is a linear model of how the device responds to a small signal riding on that operating point. This chapter derives both from the physics, then adds the capacitances that limit how fast the device can respond.

## 9.1 Why neither β nor VBE can set the bias

β varies by a factor of two or three among transistors of the same part number, and it changes with temperature and current. A circuit that sets IB and relies on β to produce IC inherits all of that spread. VBE is no better a handle. At constant collector current, VBE falls by roughly 2 mV for every degree Celsius, because IS rises steeply with temperature. Turn that around and a fixed VBE produces a collector current that climbs as the transistor warms.

## Worked Example 9.1

A transistor is biased by a fixed VBE that gives IC = 1.0 mA at 25 °C. Using −2 mV/°C and the 60 mV per decade of Chapter 5, estimate IC at 35 °C if VBE is held fixed.

Over 10 °C, the VBE needed for 1.0 mA falls by about 20 mV. Holding VBE fixed therefore leaves the junction 20 mV more forward-biased than it needs to be, which multiplies the current by 10^(20/59.9) ≈ 2.16. The current more than doubles for a ten-degree rise. The extra current heats the junction further, which can run away in a power stage. A bias scheme must set the current by some means other than the junction voltage.

## 9.2 Emitter degeneration

The standard cure puts a resistor RE in the emitter and sets the base at a fixed voltage VB, typically from a stiff divider. Kirchhoff around the base-emitter loop gives

IE = (VB − VBE) / RE

If VB is several times VBE, a change of tens of millivolts in VBE changes the numerator only slightly, and β enters only through α = β/(β + 1), which is close to one for any reasonable β. The resistor converts an exponential device into one whose current is set by Ohm's law. This is the device-side view of the bias networks Book 2 uses.

## Worked Example 9.2

VB = 2.0 V, RE = 1.3 kΩ, and VBE = 0.718 V at the operating point. Find IE, and find IC for β = 100 and for β = 300. Then estimate the change in IE if VBE falls by 100 mV over a 50 °C rise. Ignore the divider's loading by the base current.

IE = (2.0 − 0.718)/1300 = 0.986 mA. For β = 100, IC = α IE = (100/101)(0.986) = 0.976 mA. For β = 300, IC = (300/301)(0.986) = 0.983 mA. Tripling β moved IC by under 1%. A 100 mV drop in VBE raises IE by 0.100/1300 = 77 µA, about 7.8%. Degeneration has made the bias nearly independent of β and has reduced the thermal sensitivity from a doubling per ten degrees to under 8% over fifty. A larger VB and RE would reduce the thermal term further, at the cost of supply headroom.

## 9.3 The Early effect

The collector-base depletion layer widens as VCE rises, and it widens partly into the base. The effective base width WB shrinks, the Gummel number falls, and by Chapter 8's expression IS rises. The collector current therefore increases slightly with VCE even at fixed VBE. Extrapolate the collector characteristics backward and they meet the voltage axis near a single point, −VA, where VA is the Early voltage. The large-signal law gains a factor:

IC = IS e^(VBE / VT) (1 + VCE / VA),    ro = VA / IC

The slope of that characteristic is a small-signal output resistance ro. Declared: IC = 1 mA and VA = 100 V give ro = 100 kΩ. A lightly doped base widens the depletion region's encroachment and lowers VA; a heavily doped base resists it. The same trade appears again in the MOSFET as channel-length modulation.

## 9.4 The hybrid-π model

For a small signal vbe on top of the bias, the exponential can be replaced by its tangent. Its slope is the transconductance:

gm = dIC/dVBE = IC / VT

The base current is IC/β, so the small-signal input resistance looking into the base is rπ = β/gm. The output is the current gm vbe in parallel with ro. Those three elements, rπ from base to emitter, a controlled source gm vbe from collector to emitter, and ro across it, are the hybrid-π model at low frequency.

At the declared IC = 1 mA: gm = 1 mA/26 mV = 38.5 mS. rπ = 100/0.0385 ≈ 2.60 kΩ. ro = 100 kΩ. The product gm ro = VA/VT ≈ 3850, or about 72 dB, is the largest voltage gain a single transistor can provide with an ideal current-source load, and it depends only on the Early voltage and the temperature, not on the bias current.

## Worked Example 9.3

The declared transistor drives a collector resistor RC = 5.0 kΩ to the supply, with the emitter at small-signal ground. Find the small-signal voltage gain. Then find the input amplitude at which the second harmonic reaches 1% of the fundamental, using the exponential's expansion, for which the ratio is vbe/(4VT).

The load is RC in parallel with ro: 5.0k ‖ 100k ≈ 4.76 kΩ. The gain is −gm (RC ‖ ro) = −0.0385 × 4760 ≈ −183. For 1% second harmonic, vbe/(4VT) = 0.01, so vbe = 0.04 × 26 mV ≈ 1.04 mV peak. A transistor is "small-signal" only for input swings of about a millivolt. That is the deeper reason Book 2's amplifiers wrap transistors in feedback: the loop of Book 2's Chapter 11 divides this distortion by its loop gain.

Key idea. gm = IC/VT. Raise the current and the transconductance rises in proportion. The thermal voltage is still 26 mV, and an input swing comparable to it is no longer small.

## 9.5 Capacitances and fT

Two capacitances limit the transistor's speed. Charge stored in the base in transit, the 5 ps of Worked Example 8.1, behaves like a capacitance gm τF from base to emitter, where τF is the forward transit time, base transit plus smaller delays in the junctions. Added to it is the emitter-base depletion capacitance Cje, the Chapter 6 junction capacitance. Together they form Cπ. The collector-base depletion capacitance Cμ links input and output. The transition frequency fT, where the short-circuit current gain falls to one, is

fT = gm / (2π (Cπ + Cμ)),    Cπ = gm τF + Cje

## Worked Example 9.4

Declare τF = 10 ps, Cje = 50 fF, and Cμ = 20 fF. Find fT at IC = 0.1 mA, 1 mA, and 5 mA.

At 1 mA, gm = 38.5 mS, Cπ = 0.0385 × 10 ps + 50 fF ≈ 435 fF, and fT = 0.0385/(2π × 455 fF) ≈ 13.5 GHz. At 0.1 mA, gm = 3.85 mS, Cπ ≈ 88.5 fF, and fT ≈ 5.6 GHz, because the fixed junction capacitances now dominate. At 5 mA, fT ≈ 15.4 GHz, approaching the ceiling 1/(2πτF) ≈ 15.9 GHz set by transit time alone. A data sheet's fT curve rises with current, flattens near that ceiling, and in a real device falls again at high current when the injected electrons overwhelm the base doping, an effect this simple model leaves out.

## Practice

1. Find gm and rπ at IC = 0.25 mA with β = 120.

2. A transistor with VA = 60 V is biased at IC = 0.5 mA. Find ro and the intrinsic gain gm ro.

3. In the circuit of Worked Example 9.2, VB is raised to 4.0 V and RE chosen for the same IE. Find RE and the change in IE for the same 100 mV fall in VBE.

4. For the transistor of Worked Example 9.4, find fT at IC = 2 mA.

5. Explain in one or two sentences why the intrinsic gain gm ro does not depend on the bias current.

# Part III. The MOSFET, From Square Law to Below 20 nm

The bipolar transistor controls current by injecting carriers across a junction. The MOS transistor controls it by inducing them with an electric field through an insulator, so its gate draws no steady current at all. That one difference is why every digital chip is built from MOSFETs. This part starts at the capacitor that makes the field, derives the long-channel square law, shows where it fails, and ends at the fin and the nanosheet. A note on symbols: this book reserves VT for the thermal voltage, 26 mV, so the MOSFET's threshold voltage is written Vth throughout.

# Chapter 10. The MOSFET From the Capacitor Up

A MOSFET is a capacitor with a conducting channel for one plate. Put a positive voltage on a metal gate above a thin oxide over p-type silicon, and the field first pushes holes away from the surface, then, past a threshold, pulls enough electrons to the surface to form a thin n-type layer. Connect that layer at each end to an n-type contact, a source and a drain, and the gate voltage now controls a resistor. Everything else about the device is the details of how much charge that layer holds and how fast it moves.

## 10.1 The gate capacitance

The oxide is a parallel-plate capacitor. Its capacitance per unit area is the oxide permittivity divided by its thickness:

Cox = εox / tox,    εox = 3.9 ε0 = 3.45×10⁻¹³ F/cm

## Worked Example 10.1

Find Cox for an oxide thickness of 2.0 nm, in F/cm² and in fF/µm².

Cox = 3.45×10⁻¹³ F/cm / 2.0×10⁻⁷ cm ≈ 1.73×10⁻⁶ F/cm². Since 1 µm² is 10⁻⁸ cm², that is 1.73×10⁻¹⁴ F per µm², or about 17.3 fF/µm². A transistor gate one micrometer square on such an oxide is a 17 fF capacitor. Modern processes replace silicon dioxide with high-permittivity hafnium-based dielectrics and quote an equivalent oxide thickness, the SiO₂ thickness that would give the same Cox.

## 10.2 Threshold and inversion

Below threshold, the gate voltage mostly widens a depletion layer of ionized acceptors under the oxide, just as reverse bias widens a junction in Chapter 4. At threshold Vth, the surface electron concentration reaches the bulk hole concentration, the surface is said to be strongly inverted, and further gate voltage goes almost entirely into adding electrons to the inversion layer. Above threshold, the channel charge per unit area is approximately Cox (VGS − Vth). The quantity VGS − Vth is the overdrive voltage, Vov. Vth itself is set by the doping under the gate, the oxide thickness, and the work-function difference between gate and silicon, and the process designer tunes all three.

## 10.3 The long-channel square law

With a small drain voltage, the channel is a uniform sheet of charge Cox Vov moving with mobility μn, and the device is a resistor whose conductance is μn Cox (W/L) Vov. As VDS rises, the channel charge near the drain falls, because the gate-to-channel voltage there is smaller. When VDS reaches Vov, the charge at the drain end goes to zero, the channel is pinched off, and the current stops increasing. Integrating along the channel gives the two regions:

Triode (VDS < Vov):  ID = μn Cox (W/L) [Vov VDS − VDS²/2]

Saturation (VDS ≥ Vov):  ID = ½ μn Cox (W/L) Vov²,    gm = 2 ID / Vov

The MOSFET's "saturation" is the bipolar transistor's forward-active region, the amplifying region. The bipolar transistor's "saturation" corresponds to the MOSFET's triode region. The two vocabularies grew up separately, and the collision is permanent.

## Worked Example 10.2

A long-channel NMOS has μn Cox = 200 µA/V² and W/L = 10. It is biased in saturation at Vov = 0.2 V. Find ID and gm. Compare gm with that of a bipolar transistor at the same current.

ID = ½ × 200 µA/V² × 10 × (0.2)² = 40 µA. gm = 2 × 40 µA/0.2 V = 0.40 mS. A bipolar transistor at 40 µA has gm = 40 µA/26 mV ≈ 1.54 mS, almost four times more. The comparison is clearest as transconductance efficiency, gm/ID: 2/Vov = 10 V⁻¹ for this MOSFET, 1/VT ≈ 38.5 V⁻¹ for any bipolar transistor. The MOSFET can approach the bipolar figure only by lowering Vov toward the subthreshold region of Section 10.5.

## 10.4 Channel-length modulation

Past pinch-off, the pinched region near the drain grows with VDS and the effective channel shortens, so ID rises slightly, the MOSFET's version of the Early effect. A parameter λ, roughly inversely proportional to L, models it: ID is multiplied by (1 + λ VDS), and the output resistance is ro ≈ 1/(λ ID). With λ = 0.1 V⁻¹ at 40 µA, ro = 250 kΩ. Short channels have large λ, which is one reason analog designers draw transistors longer than the minimum the process allows.

## 10.5 Below threshold

Below Vth the current does not stop. A small population of electrons still reaches the surface, they move by diffusion, and the current is exponential in VGS, like a bipolar transistor with a weaker grip:

ID ∝ e^(VGS / (n VT)),    subthreshold swing S = n VT ln(10) ≈ n × 60 mV/decade

The factor n, typically 1.2 to 1.5 for a planar device, reflects the fact that the gate controls the surface potential only through a capacitive divider between the oxide and the depletion layer. At 300 K no MOSFET of this kind can switch faster than 60 mV per decade of current, because the thermal spread of carrier energies sets that floor. This has a hard consequence for digital design: lowering Vth by 60 to 90 mV to gain speed multiplies the off-state leakage of every transistor on the chip by ten.

## 10.6 Velocity saturation

The square law assumes carrier velocity is proportional to the lateral field. In silicon, electron velocity saturates near vsat ≈ 1×10⁷ cm/s once the field reaches a few volts per micrometer. In a channel 100 nm long with a few tenths of a volt across it, the field is already there. In the fully velocity-saturated limit, the channel charge moves at vsat regardless of length, and the current becomes linear in overdrive:

ID ≈ W Cox vsat Vov,    gm ≈ W Cox vsat

## Worked Example 10.3

A short-channel NMOS has the Cox of Worked Example 10.1 and a width of 1.0 µm, and is fully velocity-saturated. Find ID at Vov = 0.3 V, using vsat = 1.0×10⁷ cm/s.

ID ≈ (1.0×10⁻⁴ cm)(1.73×10⁻⁶ F/cm²)(1.0×10⁷ cm/s)(0.3 V) ≈ 5.2×10⁻⁴ A, about 0.52 mA per micrometer of width. Real short-channel devices deliver somewhat less than this limit at the same overdrive, because the carriers are not velocity-saturated over the whole channel. Drive current per micrometer of width, not μn Cox W/L, is how short-channel processes are compared.

Key idea. A MOSFET is a field-controlled sheet of charge. Its long-channel current is quadratic in overdrive, its short-channel current approaches linear, and below threshold it is exponential with a swing no better than 60 mV per decade at 300 K.

## Practice

1. Find Cox for an equivalent oxide thickness of 1.0 nm, in fF/µm².

2. The transistor of Worked Example 10.2 is rebiased at Vov = 0.4 V. Find ID, gm, and gm/ID.

3. A MOSFET is at VDS = 0.1 V with Vov = 0.5 V. Is it in triode or saturation, and what does that mean in bipolar vocabulary?

4. A process has a subthreshold swing of 80 mV/decade. By what factor does off-state leakage rise if Vth is lowered by 120 mV?

5. Explain in one or two sentences why velocity saturation makes ID depend only weakly on channel length.

# Chapter 11. The Fin, Then the Sheet

Chapter 10 ended with two walls. Below threshold, a MOSFET cannot switch more sharply than about 60 mV per decade, so threshold cannot keep falling. And as the channel shortens, the drain's field reaches under the gate and starts to control the channel itself, which makes the swing worse, not better. By about the 22 nm generation, a planar transistor, gated from one side only, had run out of ways to keep the gate in charge. The answer was geometric: wrap the gate around the channel.

## 11.1 Why one gate is not enough

In a short planar device, the drain's electric field lowers the barrier at the source end of the channel. That effect is drain-induced barrier lowering, DIBL, quoted in millivolts of threshold shift per volt of drain voltage. It makes the off-state current depend on drain voltage and degrades the subthreshold swing well above its 60 mV floor. The cure is a channel so thin that the gate's field reaches all of it, from more than one side, before the drain's field can. Thin silicon, surrounded by gate, is the whole idea behind the fin and the nanosheet.

## 11.2 The fin

A FinFET stands the channel on its edge: a narrow vertical ridge of silicon, the fin, with the gate draped over its top and both sides. The fin's width, under 10 nm in advanced nodes, is what the gate must control, and three-sided gating controls it well. The penalty is that width is no longer a continuous design variable. Each fin carries a fixed effective width, about twice its height plus its top width, and a transistor's drive is set by how many fins it has.

W_eff = N_fin (2 H_fin + W_fin)

## Worked Example 11.1

A FinFET process has fins 50 nm tall and 7 nm wide. Find the effective width of one fin and of a four-fin transistor. Using roughly 0.5 mA/µm from Worked Example 10.3 as an order-of-magnitude drive, estimate the four-fin transistor's on-current.

One fin: 2 × 50 + 7 = 107 nm. Four fins: 428 nm. At about 0.5 mA/µm, the on-current is roughly 0.21 mA. The designer cannot ask for 300 nm or 500 nm; the choices are 107, 214, 321, 428 nm, and so on. Weste and Harris still teach how to draw the circuit, but sizing now counts fins.

## 11.3 The nanosheet and gate-all-around

A fin cannot grow taller indefinitely, because tall, thin fins are mechanically fragile and hard to etch uniformly. The next step turns the fin on its side and slices it: a stack of thin horizontal silicon sheets, each completely surrounded by gate. Gate-all-around control is the best electrostatics a channel can have, and the sheet width, unlike fin width, can be chosen within a range, which returns some continuous sizing to the designer.

W_eff = N_sheet × 2 (W_sheet + T_sheet)

Three sheets 30 nm wide and 5 nm thick give 3 × 2 × 35 = 210 nm of effective width in a footprint not much larger than a single fin's. Beyond the nanosheet, the complementary FET (CFET) stacks an nFET directly over a pFET to shrink a logic cell further. That is published research and roadmap material, IRDS and conference papers, and it should be read as such.

## 11.4 What changes for the designer

At these nodes the transistor is only part of the problem. The resistance and capacitance of the tiny copper wires above the transistors, the back end of line, often dominate delay. Random variation in a few dopant atoms or a few nanometers of line edge shifts threshold from one device to the next, so matching, the subject of analog design, gets harder even as logic density improves. Leakage is a power budget, not a footnote. Taur and Ning, 3rd edition, is the device book for this territory. Veendrick surveys what else changes: leakage, back-end RC, variability, and yield.

Above all, below about 20 nm the foundry's process design kit outranks any textbook. The PDK's device models, layout rules, and corner files describe the transistors that will actually be built; the equations of Chapter 10 are a way of understanding those models, not a substitute for them.

One material belongs to this chapter by name only. Aluminum scandium nitride, AlScN, is a piezoelectric film that CMOS-compatible lines now deposit, and it is reshaping MEMS resonators and filters. This book names it as a film. The 2026 products that use it are surveyed in Book 7.

Key idea. The fin and the nanosheet exist to keep the gate in control of a channel too short for a planar gate to hold. Width became quantized with the fin and partly continuous again with the sheet. Below 20 nm, the PDK is the authority.

## Practice

1. A fin is 42 nm tall and 6 nm wide. Find the effective width of a three-fin transistor.

2. A nanosheet transistor has four sheets, each 25 nm wide and 6 nm thick. Find its effective width.

3. A device shows a threshold shift of 50 mV when VDS rises from 0.05 V to 0.75 V. Find its DIBL in mV/V.

4. In one or two sentences, explain why a thinner channel improves the gate's control over the barrier at the source.

# Part IV. Inside the Amplifier

Book 2 used the op-amp as a block with a declared open-loop gain of 10⁵, a gain-bandwidth product of 1 MHz, and a dominant pole at 10 Hz. This part opens the block. It builds the two circuits every analog chip is made from, the current mirror and the differential pair, puts them together into a two-stage op-amp whose gain, bandwidth, and slew rate can be calculated from the transistors of Parts II and III, and then derives the noise those transistors add. The circuit check for this part is the op-amp chapter of Tietze, Schenk, and Gamm, 16th edition, with Gray, Hurst, Lewis, and Meyer for the integrated details.

# Chapter 12. Mirrors, Pairs, and Matching

An integrated circuit cannot rely on any one transistor's absolute values. β varies by a factor of two across a wafer, Vth by tens of millivolts, and resistors by twenty percent. What a chip does control is how closely two transistors drawn side by side resemble each other. Analog IC design is the craft of making every important quantity depend on a ratio of matched devices, and the two circuits in this chapter are where that craft begins.

## 12.1 The current mirror

Connect a bipolar transistor's collector to its own base and feed it a reference current IREF. It settles at whatever VBE carries that current. Apply the same VBE to a second, identical transistor, and it carries the same collector current, because IC = IS e^(VBE/VT) and both share IS and VBE. The second transistor is a current source whose value was set by the first. The only error in the ideal picture is the base current: IREF must supply the collector current of the reference transistor and the base currents of both.

IOUT = IREF / (1 + 2/β)

## Worked Example 12.1

A simple bipolar mirror uses the declared transistor of Part II (β = 100, VA = 100 V) with IREF = 1 mA. Find the output current. Then find how much it changes when the output collector voltage rises by 5 V above the reference transistor's VBE.

IOUT = 1 mA/(1 + 0.02) = 0.980 mA, a 2% error from base current alone. The output resistance is ro = VA/IC = 100 V/1 mA = 100 kΩ, so 5 V more across the output transistor adds 5/100 kΩ = 50 µA, another 5%. Both errors have standard fixes. A third transistor that supplies the base current (the beta helper) reduces the base-current error to 2/(β(β + 1)), about 0.02%. A cascode, a second transistor stacked on the output that holds its collector voltage fixed, multiplies the output resistance by roughly β for a bipolar device or gm ro for a MOSFET.

The MOS mirror works the same way with VGS in place of VBE and has no gate current, so the base-current error disappears. Its ratio is set by device geometry: IOUT/IREF = (W/L)₂/(W/L)₁. Designers set ratios by repeating one unit transistor, so that a ratio of 4 is four identical devices in parallel, because a single transistor drawn four times wider does not scale its edge effects by exactly four.

## 12.2 Mismatch

Two transistors that are drawn identically still differ. In a bipolar pair the main difference is IS, from slight differences in emitter area and base doping, and it appears as an offset in VBE: a 1% difference in IS is VT ln(1.01) = 0.26 mV. In a MOS pair the main difference is Vth, from random variations in the number and position of dopant atoms under the gate and in the gate edge. Because the current in saturation depends on the square of the overdrive, a threshold difference ΔVth causes a fractional current difference that grows as the overdrive falls.

ΔID / ID ≈ 2 ΔVth / Vov

Random mismatch shrinks with device area. Pelgrom's law gives the standard deviation of the threshold difference between two identical devices of gate width W and length L, with a process constant AVt that has fallen from about 10 mV·µm in 1990s processes to 1–3 mV·µm in recent ones.

σ(ΔVth) = AVt / √(W L)

## Worked Example 12.2

A process has AVt = 3 mV·µm. A MOS mirror uses two 1 µm × 1 µm devices at Vov = 0.2 V. Find the standard deviation of the threshold mismatch and of the current ratio. What device area brings σ(ΔVth) to 1 mV?

σ(ΔVth) = 3 mV·µm/√(1 µm²) = 3 mV. The current ratio then has a standard deviation of 2 × 3 mV/0.2 V = 3%. To reach 1 mV, √(WL) must be 3 µm, so WL = 9 µm², nine times the area. Matching improves only with the square root of area, which is why precision analog circuits are large while digital logic on the same chip is drawn at minimum size. Raising Vov also helps the current matching, at the cost of headroom and transconductance efficiency.

Key idea. A mirror copies a current by copying a voltage. Its accuracy is set by how well two devices match, and random matching improves only as the square root of the area spent on them.

## 12.3 The differential pair

Two matched transistors with their emitters, or sources, tied together and fed from a single tail current IEE form a differential pair. The tail current must divide between the two sides, and the division depends only on the difference between the two input voltages. For bipolar transistors, the exponential law gives a closed form.

IC1 − IC2 = IEE tanh(Vd / (2 VT)),    Vd = VB1 − VB2

For small inputs, tanh(x) ≈ x, and each side's collector current changes by ± gm Vd/2, where gm is the transconductance of one transistor at its half of the tail current, IEE/(2VT). With load resistors RC on both collectors, the differential output voltage is gm RC Vd. A common-mode input moves both bases together and leaves the split unchanged, so the pair rejects it, up to the limit set by the tail source's finite output resistance and by mismatch.

## Worked Example 12.3

A bipolar pair has IEE = 2 mA and RC = 5 kΩ on each collector. Find the small-signal differential gain, and the input difference that steers 90% of the tail current difference to one side.

Each transistor runs at 1 mA, so gm = 1 mA/26 mV = 38.5 mS, and the differential gain is 38.5 mS × 5 kΩ = 192. The current difference reaches 90% of IEE when tanh(Vd/2VT) = 0.9, so Vd = 2 × 26 mV × atanh(0.9) = 2 × 26 × 1.472 = 76.6 mV. A bipolar pair is linear only for inputs well below about 26 mV; beyond 77 mV it is a switch. A MOS pair at Vov = 0.2 V steers its tail current completely at √2 Vov = 0.28 V, a much wider linear range bought with lower gm/ID.

## 12.4 Offset of a pair

If the two halves of a pair are mismatched, a nonzero input is needed to balance them. That input is the pair's offset voltage, and it is the offset the op-amp datasheets of Book 2 quote. A bipolar pair's offset is the VBE mismatch, which for a 1% IS mismatch is 0.26 mV, plus a contribution from mismatched load resistors divided by the gain. A MOS pair's offset is ΔVth directly, plus (Vov/2) times the fractional mismatch in W/L. With AVt = 3 mV·µm and 1 µm² devices that is several millivolts, which is why bipolar-input amplifiers have historically led on offset and why precision CMOS amplifiers use large input devices, trimming, or the chopper and auto-zero techniques that measure the offset and subtract it continuously.

Systematic offset is the other kind. It comes from the circuit, not from randomness: a load mirror whose two sides sit at different collector voltages carries slightly different currents through the Early effect, and the pair must be unbalanced to compensate. Systematic offset is removed at design time by making the two sides operate at the same voltages, which is part of the reason the two-stage op-amp of the next chapter sizes its second stage the way it does.

## Practice

1. A bipolar mirror has β = 50 and IREF = 200 µA. Find IOUT, ignoring the Early effect.

2. A MOS mirror operates at Vov = 0.15 V. Its devices have σ(ΔVth) = 2 mV. Find the standard deviation of the current ratio.

3. With AVt = 4 mV·µm, what gate area is needed for σ(ΔVth) = 0.5 mV?

4. A bipolar pair has IEE = 400 µA and RC = 20 kΩ. Find the differential gain.

5. Two transistors in a bipolar pair differ in IS by 2%. Find the resulting offset voltage.

6. Explain in one or two sentences why a ratio of four in a mirror is drawn as four unit devices rather than one device four times wider.

# Chapter 13. The Two-Stage Op-Amp From Its Transistors

Book 2 declared an op-amp with A₀ = 10⁵, a gain-bandwidth product of 1 MHz, and a dominant pole at 10 Hz, and built twenty chapters of circuits on those three numbers. This chapter derives all three from transistors. The architecture is the two-stage amplifier with Miller compensation, the structure behind the 741 and behind most general-purpose CMOS op-amps since. Each number turns out to depend on one or two device quantities, and seeing which ones explains most of what an op-amp datasheet shows.

## 13.1 The architecture

The first stage is a differential pair with a current-mirror load. The mirror copies the current of one side to the other, so the full differential signal current appears at a single output node; the stage converts a differential input into a single-ended current gm1 Vd. The second stage is a common-emitter or common-source amplifier with a current-source load, which provides most of the remaining gain. A compensation capacitor Cc connects the second stage's input to its output. An output stage, usually a class-AB follower, provides current to the load without adding voltage gain.

## 13.2 DC gain

Each gain stage is a transconductance driving its own output resistance. For the first stage that resistance is the parallel combination of the output resistance of the pair transistor and of the mirror transistor on the output side; for the second stage it is the gain transistor in parallel with its current-source load. With equal output resistances in each pair, the gains are gm ro/2 per stage.

A₀ = (gm1 ro1/2) × (gm6 ro6/2)

## Worked Example 13.1

A CMOS two-stage op-amp has a tail current of 20 µA (10 µA per input transistor) and a second stage at 100 µA. All devices run at Vov = 0.2 V and have λ = 0.02 V⁻¹. Find each stage's gain and A₀.

Input stage: gm1 = 2 × 10 µA/0.2 V = 0.10 mS, ro = 1/(0.02 × 10 µA) = 5 MΩ, and the gain is 0.10 mS × 5 MΩ/2 = 250. Second stage: gm6 = 2 × 100 µA/0.2 V = 1.0 mS, ro = 1/(0.02 × 100 µA) = 500 kΩ, and the gain is 1.0 mS × 500 kΩ/2 = 250. A₀ = 250 × 250 = 62,500, or 95.9 dB, the same order as Book 2's declared 10⁵. The current cancels out of each stage's gain, because gm rises with current and ro falls with it; the gain is set by Vov and λ, that is, by device length and bias choice.

## 13.3 Miller compensation and the gain-bandwidth product

Without compensation, the two high-impedance nodes form two poles at similar frequencies, and the amplifier oscillates when feedback is applied. The Miller capacitor connects across the second stage, whose gain multiplies its effect at the first stage's output, creating a dominant pole at a very low frequency. Above that pole, the first stage's signal current gm1 Vd flows almost entirely into Cc, and the output voltage is that current divided by the capacitor's impedance. The amplifier's gain therefore falls as gm1/(ωCc) and crosses unity at a frequency set by just two quantities.

GBW = gm1 / (2π Cc),    fp1 = GBW / A₀

## Worked Example 13.2

For the op-amp of Worked Example 13.1, choose Cc for GBW = 1 MHz and find the dominant pole. Repeat the capacitor choice for a bipolar input pair at the same 10 µA per side.

Cc = gm1/(2π × 1 MHz) = 0.10 mS/6.283 × 10⁶ = 15.9 pF. The dominant pole is 1 MHz/62,500 = 16 Hz, close to Book 2's 10 Hz. A bipolar pair at 10 µA has gm1 = 10 µA/26 mV = 0.385 mS, so it needs Cc = 61 pF for the same GBW. Higher transconductance per unit current means a larger capacitor for the same bandwidth, and that capacitor costs die area.

## 13.4 Slew rate

For a large input step, the differential pair switches completely, and the whole tail current IEE flows into or out of Cc. The output can then change no faster than that current charges the capacitor. This is the slew rate Book 2 measured; here it has a formula.

SR = IEE / Cc = 2π GBW × (IEE / gm1)

The second form is the useful one. For a fixed GBW, slew rate is proportional to the ratio of tail current to input transconductance. For a bipolar pair, IEE/gm1 = 2VT = 52 mV, and that is fixed by physics. For a MOS pair, IEE/gm1 = Vov, which the designer chooses, and an emitter resistor added to a bipolar pair raises the ratio the same way.

## Worked Example 13.3

Find the slew rate of the two op-amps of Worked Example 13.2.

Bipolar input: SR = 20 µA/61 pF = 0.33 V/µs, which is also 2π × 1 MHz × 52 mV. CMOS input at Vov = 0.2 V: SR = 20 µA/15.9 pF = 1.26 V/µs, about four times faster for the same bandwidth and current. This is why the 741's slew rate, about 0.5 V/µs, is an architectural limit, and why fast-slewing bipolar op-amps degenerate their input pairs.

Key idea. Gain is set by device length and bias, bandwidth by the input transconductance and the compensation capacitor, and slew rate by the tail current and the same capacitor. Raising IEE/gm1 is the one lever that improves slew rate without giving up bandwidth.

## 13.5 The second pole and the capacitive load

The output node has its own pole, at roughly gm6/(2π CL) with Miller compensation, where CL is the load capacitance plus the node's own capacitance. For a phase margin near 60° in a unity-gain follower, the second pole must lie at about 2.2 times the GBW. This is the transistor-level origin of Book 2's warning that a follower driving a large capacitor rings.

## Worked Example 13.4

For the op-amp of Worked Example 13.1 (gm6 = 1.0 mS, GBW = 1 MHz), find the largest load capacitance compatible with a second pole at 2.2 MHz.

CL = gm6/(2π × 2.2 MHz) = 1.0 mS/1.38 × 10⁷ = 72 pF. An unbuffered two-stage CMOS amplifier at this current cannot drive even a short cable without losing phase margin. Raising the second-stage current raises gm6 and the allowed load proportionally, which is why the output stage often draws most of an op-amp's supply current.

## 13.6 Input range and output stage

The input pair needs headroom: its tail current source must stay out of saturation, and its transistors must stay in their active region. A PMOS input pair works down to the negative rail but not up to the positive one, and an NMOS pair the reverse. Rail-to-rail input op-amps use both pairs in parallel and hand over from one to the other in the middle of the range, which is why their datasheets show an offset step and a change in bias current at one common-mode voltage. The output stage, usually a complementary pair biased slightly on (class AB) to avoid the crossover distortion of a pure class-B stage, sets the output swing; a rail-to-rail output uses common-source output devices whose drains can reach within one saturation voltage of each rail.

## Practice

1. A two-stage CMOS op-amp has stage gains of 180 and 220. Find A₀ in decibels.

2. The first stage has gm1 = 0.25 mS. Find Cc for a GBW of 5 MHz.

3. A bipolar-input op-amp has a tail current of 40 µA and Cc = 30 pF. Find its slew rate.

4. A CMOS op-amp has GBW = 10 MHz and its input pair runs at Vov = 0.15 V. Find its slew rate.

5. The second stage of an op-amp has gm6 = 4 mS. Find the largest load capacitance for a second pole at 2.2 times a GBW of 3 MHz.

6. Explain in one or two sentences why doubling the bias current of every stage, at the same overdrive, leaves A₀ unchanged but raises the allowed load capacitance.

# Chapter 14. The Noise a Device Makes

Book 2 treated op-amp noise as two numbers from a datasheet, a voltage density en and a current density in, and combined them with the thermal noise of the resistors around the amplifier. This chapter derives those numbers from the transistors. Three mechanisms account for nearly all of it: thermal noise in anything that has resistance, shot noise in any current that crosses a barrier, and flicker noise from charges trapped and released at imperfections. Each has a formula, and the formulas explain why bipolar and CMOS amplifiers have such different noise specifications.

## 14.1 Thermal and shot noise

Any resistance R at absolute temperature T generates a random voltage with a flat power spectral density 4kTR, in V²/Hz. Book 2 fixed the reference case: 100 kΩ at 290 K has 40 nV/√Hz. This part uses the same 290 K for noise, where 4kT = 1.60 × 10⁻²⁰ J, so that the numbers carry over directly; the 26 mV thermal voltage remains the hand value for device equations.

vn = √(4kTR),    in = √(2qI)

Shot noise arises because current crossing a junction is a stream of individual charges arriving independently. The current fluctuates about its mean with a flat density 2qI, in A²/Hz. A forward-biased diode, the collector current of a bipolar transistor, and the base current all carry full shot noise. A plain resistor does not: its electrons drift collectively and it has only thermal noise.

## Worked Example 14.1

Find the shot-noise current density of a 1 mA collector current and of the 10 µA base current that accompanies it at β = 100.

√(2 × 1.602 × 10⁻¹⁹ × 10⁻³) = 17.9 pA/√Hz for the collector current, and √(2 × 1.602 × 10⁻¹⁹ × 10⁻⁵) = 1.79 pA/√Hz for the base current. Over a 10 kHz bandwidth the collector figure integrates to 17.9 pA × √10,000 = 1.79 nA rms, roughly two parts per million of the current.

## 14.2 The bipolar transistor's input noise

Referred to the input, the collector's shot noise becomes a voltage: divide the current noise by gm. Because gm = IC/VT, the result falls as the square root of the collector current. The base current's shot noise flows in the source resistance and appears as the input current noise. The base spreading resistance rb adds its own thermal noise in series with the input, which matters for very low source resistances.

en = √(2qIC)/gm = VT √(2q/IC),    in = √(2q IB)

## Worked Example 14.2

Find en and in for the declared transistor at IC = 1 mA, ignoring rb. Find the source resistance for which the two contributions are equal, and the total noise with a 1 kΩ source.

en = 26 mV × √(2 × 1.602 × 10⁻¹⁹/10⁻³) = 0.47 nV/√Hz. in = 1.79 pA/√Hz from Worked Example 14.1. The two are equal when the source resistance is en/in = 0.465 nV/1.79 pA = 260 Ω, which is rπ/√β = 2.60 kΩ/10. With a 1 kΩ source, the source's own thermal noise is √(1.60 × 10⁻²⁰ × 1000) = 4.0 nV/√Hz, and adding en² and (in × 1 kΩ)² gives √(4.0² + 0.47² + 1.79²) = 4.4 nV/√Hz. The transistor adds only 10% to the noise of the source.

The optimum source resistance rπ/√β can be moved by choosing the collector current. Lowering IC raises en and lowers in, in proportion, so high-impedance sources call for low-current input stages and low-impedance sources such as moving-coil microphones or strain-gauge bridges call for high-current ones, sometimes several transistors in parallel to lower rb as well.

## 14.3 The MOSFET's channel noise

The channel of a MOSFET in saturation is a resistor-like region whose thermal noise appears as a drain current density 4kTγgm, where γ is 2/3 for a long-channel device and somewhat larger for short channels. Referred to the gate, the voltage noise is set by gm alone. The gate draws no steady current, so in at low frequency is only the shot noise of the gate leakage, femtoamperes per root hertz, which is why CMOS-input amplifiers are the choice for high-impedance sources such as photodiodes and piezoelectric sensors.

en² = 4kTγ/gm   (V²/Hz)

## Worked Example 14.3

The MOSFET of Chapter 10's Worked Example 10.2 has gm = 0.40 mS. Find its gate-referred thermal noise with γ = 2/3, and the gm needed to match the bipolar transistor's 0.47 nV/√Hz.

en = √(1.60 × 10⁻²⁰ × 0.667/0.40 × 10⁻³) = 5.2 nV/√Hz. To reach 0.47 nV/√Hz, gm must grow by (5.17/0.465)² ≈ 124 to about 49 mS, which at a gm/ID of 10 V⁻¹ means 4.9 mA, more than the bipolar transistor's 1 mA. A bipolar transistor reaches low voltage noise with less current because its gm/ID of 38.5 V⁻¹ is the highest of any device.

## 14.4 Flicker noise

At low frequencies a third mechanism dominates. Charges trapped and released at defects, most of all at the MOSFET's oxide interface, modulate the current with a density that rises as 1/f. The frequency at which flicker noise equals the white noise is the corner frequency fc. In a voltage-density plot the flicker part rises as 1/√f, so for f below fc the total density is en,white √(1 + fc/f). MOSFET flicker noise is inversely proportional to gate area, so large input devices are quieter; PMOS devices typically have lower flicker noise than NMOS because their carriers flow slightly away from the interface. Bipolar transistors, whose current flows in the bulk, have corners of a few hertz to a few hundred hertz, against kilohertz to megahertz for small MOSFETs.

## Worked Example 14.4

A CMOS amplifier has a white voltage noise of 10 nV/√Hz and a flicker corner of 1 kHz. Find the noise density at 10 Hz, and the corner if the input devices are made four times larger in area at the same gm.

At 10 Hz, en = 10 × √(1 + 1000/10) = 10 × √101 = 100 nV/√Hz, ten times the white value. Flicker power is inversely proportional to area, so four times the area cuts the flicker power by four and moves the corner to 250 Hz; at 10 Hz the density becomes 10 × √26 = 51 nV/√Hz. Chopper-stabilized amplifiers move the signal above the corner before amplifying it and avoid flicker noise almost entirely.

Key idea. Voltage noise is set by transconductance, current noise by the input bias current, and flicker noise by area and interface quality. A bipolar input wins on voltage noise per unit current; a CMOS input wins on current noise by orders of magnitude.

## 14.5 The differential pair and the op-amp

An op-amp's input stage is a differential pair, and both transistors contribute uncorrelated noise. The input-referred voltage noise is therefore √2 times that of one transistor, before the load mirror adds its own share referred back through the ratio of its gm to the input gm. Designers keep the mirror's gm small relative to the input pair's for exactly this reason. Popcorn noise, abrupt switching between two current levels caused by a single trap, appears in some devices as a random telegraph signal; it is a process-quality issue, screened in production, not something a circuit can design away.

## Practice

1. Find the shot-noise density of a 100 µA current.

2. Find en for a bipolar transistor at IC = 100 µA, ignoring rb.

3. A bipolar transistor has β = 200 and runs at 1 mA. Find its optimum source resistance rπ/√β.

4. A MOSFET has gm = 4 mS and γ = 2/3. Find its gate-referred thermal noise density.

5. An amplifier has 8 nV/√Hz white noise and a 200 Hz flicker corner. Find the density at 2 Hz.

6. Explain in one or two sentences why a CMOS-input op-amp is preferred for a 10 MΩ photodiode source even though its voltage noise is higher.

# Part V. CMOS as a Chip

Part III ended with a single transistor below 20 nm. A processor holds tens of billions of them, and at that scale the questions change. How fast does a gate switch, where does the power go, and why does a chip that is doing nothing still get warm? How does a transistor remember a bit? What breaks a chip in the field? This part answers those questions with the device equations already derived, and stops where layout, packaging, and system power begin, which are Books 6 and 7.

# Chapter 15. The CMOS Inverter: Threshold, Delay, and Power

Every digital gate is a variation on one circuit: an NMOS transistor pulling the output to ground and a PMOS transistor pulling it to the supply, with their gates tied together as the input. When the input is high the NMOS conducts and the PMOS is off; when it is low the reverse. In either steady state one device is off, so no current flows from supply to ground except leakage. That property, almost zero static power for a gate holding its state, is why CMOS displaced every other logic family. This chapter puts numbers on the three things that remain: where the gate switches, how long it takes, and the power it spends doing so.

## 15.1 The switching threshold

The inverter's switching threshold VM is the input voltage at which the output equals the input, with both transistors in saturation and carrying the same current. Setting the two square-law currents equal shows that VM sits at mid-supply when the PMOS is made stronger by the ratio of the mobilities, because holes move more slowly than electrons.

VM = VDD/2 when (W/L)p / (W/L)n = μn/μp   (with equal |Vth|)

## Worked Example 15.1

Using the declared working mobilities μn = 1000 and μp = 480 cm²/V·s, find the PMOS-to-NMOS width ratio for a symmetrical inverter with equal channel lengths.

(W/L)p/(W/L)n = 1000/480 = 2.08. The PMOS is drawn about twice as wide as the NMOS. In short-channel devices velocity saturation narrows the difference, because both carriers approach similar saturated velocities, and in FinFET and nanosheet processes strain engineering brings hole and electron drive close enough that ratios near 1 to 1.5 are common. The principle, size the pull-up and pull-down to equal strength, does not change.

## 15.2 Delay

A gate's delay is the time it takes to charge or discharge the capacitance it drives, CL, which is the input capacitance of the gates it feeds plus the wiring and its own drain capacitance. To first order the transistor delivers a constant on-current Ion while the output swings halfway, so the propagation delay is the charge needed for half the swing divided by that current.

tp ≈ CL VDD / (2 Ion)

## Worked Example 15.2

A minimum inverter in a modern process drives CL = 1.0 fF at VDD = 0.8 V with Ion = 50 µA. Find its propagation delay.

tp = 1.0 fF × 0.8 V/(2 × 50 µA) = 8 ps. Real gates drive more than one input and a length of wire, so a standard benchmark is the delay of an inverter driving four copies of itself, the fanout-of-four (FO4) delay. A clock period of 0.3 ns corresponds to only a few tens of FO4 delays, and that number, not the transistor alone, sets how much logic fits between two flip-flops.

## 15.3 Dynamic power

Each time the output rises, the supply delivers a charge CL VDD at voltage VDD, an energy CL VDD². Half of it is stored on the capacitor and half is dissipated in the PMOS. When the output falls, the stored half is dissipated in the NMOS. Each full cycle of the output therefore costs CL VDD², independent of the transistors' resistance. A chip's switched capacitance C, its clock frequency f, and the activity factor α, the fraction of nodes that make a full transition per clock cycle, give the familiar power equation.

Pdyn = α C VDD² f

## Worked Example 15.3

A logic block contains one million gates, each loading its output with 1 fF. The activity factor is 0.1, VDD = 0.8 V, and the clock is 2 GHz. Find the dynamic power. Then find the effect of lowering VDD to 0.6 V, if the threshold voltage is 0.3 V and the on-current follows (VDD − Vth)^1.3.

C = 10⁶ × 1 fF = 1 nF. Pdyn = 0.1 × 10⁻⁹ × 0.64 × 2 × 10⁹ = 0.128 W. At 0.6 V the energy per transition falls by (0.6/0.8)² = 0.56. Delay scales as VDD/(VDD − Vth)^1.3, which rises by a factor (0.6/0.3^1.3)/(0.8/0.5^1.3) = 1.46, so the clock must fall to 2/1.46 = 1.37 GHz. Power at the new maximum clock is 0.128 × 0.56/1.46 = 0.049 W, 38% of the original, for 69% of the speed. This trade, slower and much cooler, is why chips run many slower cores rather than one fast one, and why voltage is the first knob in every power-management scheme.

Key idea. Dynamic energy is CV² per transition, so voltage is worth twice as much as capacitance. Lowering the supply saves energy quadratically but slows the gate as VDD approaches Vth.

## 15.4 Leakage power

Chapter 10 showed that a MOSFET below threshold still conducts, with a current that falls by a decade for every subthreshold swing S, 60 mV at the ideal limit and 65–80 mV in practice. An "off" transistor at VGS = 0 therefore leaks a current proportional to 10^(−Vth/S). Every off transistor on a chip leaks at once, and the total is a static power that flows whether the chip is computing or idle.

Ioff ∝ 10^(−Vth / S),    Pleak = VDD × Σ Ioff

## Worked Example 15.4

A chip has one billion transistors, each leaking 1 nA at VDD = 0.8 V. Find the static power. Then find how it changes if Vth is lowered by 70 mV to recover speed, with S = 70 mV/decade.

Pleak = 10⁹ × 1 nA × 0.8 V = 0.8 W, before the chip does any work. Lowering Vth by one subthreshold swing raises every off-current tenfold, to 8 W. Leakage also roughly doubles for every 10 to 20 °C, which couples it to the cooling. Chips therefore use several threshold flavors: fast, leaky devices only on the critical paths and slow, tight ones everywhere else, and they cut the supply to idle blocks altogether (power gating). The fin and the nanosheet of Chapter 11 earned their place by restoring S toward 60 mV/decade, which allows a lower Vth for the same leakage.

## 15.5 From gate to chip

The same three equations scale to a full design. Delay sets the clock, dynamic power sets the cooling, and leakage sets the idle power and the battery life. Static timing analysis computes path delays from characterized gate models; power analysis sums α C V² f from simulated activity. Both rest on transistor models supplied with the process design kit, and below 20 nm the kit outranks any textbook value, including the hand numbers of this chapter.

## Practice

1. With μn/μp = 2.5, what PMOS-to-NMOS width ratio gives a symmetrical inverter?

2. A gate drives 3 fF at VDD = 0.75 V with Ion = 60 µA. Find tp.

3. A block has C = 5 nF of switched capacitance, α = 0.15, VDD = 0.9 V, and f = 1.5 GHz. Find Pdyn.

4. What fraction of dynamic energy per transition is saved by lowering VDD from 1.0 V to 0.7 V at the same frequency?

5. A chip leaks 0.5 W with S = 75 mV/decade. Estimate its leakage if Vth is raised by 150 mV.

6. Explain in one or two sentences why a CMOS gate's dynamic energy per transition does not depend on the on-resistance of its transistors.

# Chapter 16. How a Transistor Remembers: SRAM, DRAM, and Flash

Logic forgets its inputs as soon as they change. Memory must not. There are three standard ways to make transistors hold a bit, and each trades area, speed, and permanence in a different way. The static cell holds a bit in a loop of two inverters, the dynamic cell in a charge on a capacitor, and the flash cell in electrons trapped on an insulated gate. Each has one governing equation, and each of those equations sets a hard limit that the memory industry engineers around.

## 16.1 The six-transistor SRAM cell

Two inverters connected in a ring have two stable states: if the first output is high, the second is low, which holds the first high. That latch is the bit. Two access transistors connect its two nodes to a pair of bit lines when the word line is raised, for six transistors in all. Reading precharges both bit lines high and lets the cell discharge one of them slightly; a sense amplifier detects the difference. Writing drives one bit line low hard enough to overpower the latch.

The two operations pull the sizing in opposite directions. During a read, the access transistor and the cell's pull-down NMOS form a divider on the node that holds a zero. If the access device is too strong, the node rises past the other inverter's switching threshold and the read flips the cell. The cell ratio, the pull-down's W/L divided by the access device's, is therefore kept at about 1.5 to 2. During a write, the access transistor must overpower the PMOS pull-up, so the pull-up is made weakest of the three. Random threshold mismatch from Chapter 12 makes these margins statistical: a large SRAM has billions of cells, and the worst cell, not the average, must work, so read and write margins are designed to many standard deviations.

## Worked Example 16.1

During a read, the access transistor (treated as a resistance Racc) and the pull-down transistor (Rpd) divide the 0.8 V precharged bit line onto the storage node that holds zero. The opposite inverter switches at 0.35 V. Find the largest ratio Rpd/Racc that keeps the node below half the switching threshold, a common margin target.

The node voltage is 0.8 × Rpd/(Rpd + Racc). Setting this to 0.175 V gives Rpd/(Rpd + Racc) = 0.219, so Rpd/Racc = 0.219/0.781 = 0.28. The pull-down must be about 3.6 times stronger than the access device in this resistive picture. Square-law and velocity-saturation effects make the real ratio smaller, which is why the practical cell ratio is 1.5 to 2, but the direction is the same: the read margin is bought with a stronger pull-down.

## 16.2 The one-transistor DRAM cell

A DRAM cell is a capacitor Cs and one access transistor. A one is a charged capacitor and a zero an empty one. To read, the bit line, with its much larger capacitance Cbl, is precharged to half the array voltage, and the access transistor connects the cell to it. The charge shares between the two capacitors and moves the bit line by a small amount that a sense amplifier detects and restores. The read is destructive, so every read is followed by a write-back.

ΔVbl = (Vcell − Vpre) × Cs / (Cs + Cbl)

## Worked Example 16.2

A DRAM cell has Cs = 20 fF on a bit line of Cbl = 100 fF. The array stores 1.0 V for a one and precharges to 0.5 V. Find the read signal. Then find the largest cell leakage that lets the cell lose no more than 0.15 V in the standard 64 ms refresh interval.

ΔVbl = 0.5 × 20/120 = 83 mV, a small signal that the sense amplifier must resolve against its own offset and the noise of neighboring lines. A leakage I discharges the cell at I/Cs, so the limit is I = Cs ΔV/t = 20 fF × 0.15 V/64 ms = 47 fA. Leakage roughly doubles with every 10 °C, and the DRAM standards halve the refresh interval to 32 ms above 85 °C for that reason. Every row must be refreshed within the interval whether or not it is used, which is a share of the chip's power and bandwidth that grows with capacity.

The ratio Cs/Cbl is the central problem of DRAM scaling. The cell area shrinks, but the capacitance cannot shrink with it without losing the read signal, so DRAM capacitors have become tall cylinders with aspect ratios beyond 50 to 1 and high-permittivity dielectrics. The access transistor must leak femtoamperes when off, so it is built quite unlike a logic transistor, with a buried, recessed gate and a long effective channel.

Key idea. SRAM stores a bit in feedback and needs power to keep it but no refresh; DRAM stores it in charge, which leaks and must be refreshed; flash stores it in trapped charge that stays without power. Each cell's density is set by the element that holds the bit.

## 16.3 The flash cell

A flash cell is a MOSFET with an extra gate, floating and completely surrounded by insulator, or, in most current NAND flash, a charge-trapping silicon-nitride layer that plays the same role. Electrons placed on the floating gate or in the trap layer shield the channel from the control gate and raise the threshold voltage seen from it. Programming forces electrons through the thin tunnel oxide by Fowler-Nordheim tunneling with a high field, about 15 to 20 V across the stack; erasing pulls them out. The stored state is read by applying a control-gate voltage between the programmed and erased thresholds and asking whether the cell conducts.

ΔVth = −Q / Ccg   (Q is the trapped charge, negative for electrons)

## Worked Example 16.3

A NAND cell's effective coupling capacitance between control gate and storage layer is 0.1 fF. How many electrons shift its threshold by 1 V? A triple-level cell (TLC) places 8 threshold levels in a 6 V window. Find the level spacing in volts and in electrons.

N = Ccg ΔVth/q = 0.1 × 10⁻¹⁵ × 1/1.602 × 10⁻¹⁹ = 624 electrons per volt. Eight levels have seven spaces, so the spacing is 6/7 = 0.86 V, about 535 electrons. Losing a few tens of electrons over ten years of retention moves the threshold a noticeable fraction of the spacing, which is why multi-level cells need strong error-correcting codes and why each additional bit per cell (QLC has 16 levels and spacing of 0.4 V in the same window) costs endurance and retention.

Each program-erase cycle drives electrons through the tunnel oxide and leaves some trapped damage, so flash wears out: endurance is quoted in program-erase cycles, tens of thousands for single-level cells and roughly a thousand for TLC, and controllers spread writes evenly across the chip (wear leveling). Modern NAND stacks more than two hundred layers of cells vertically, because shrinking the cell laterally had reached the point where a few electrons decided a bit.

## Practice

1. A DRAM cell has Cs = 15 fF and Cbl = 90 fF, stores 1.1 V, and precharges to 0.55 V. Find the read signal.

2. For the cell of the previous problem, find the leakage that discharges it by 0.2 V in 32 ms.

3. A flash cell has Ccg = 0.2 fF. How many electrons shift its threshold by 0.5 V?

4. A two-bit (MLC) cell uses a 5 V threshold window. Find the level spacing.

5. In an SRAM read, the storage node is 0.9 V × Rpd/(Rpd + Racc). Find the node voltage when Rpd/Racc = 0.2.

6. Explain in one or two sentences why a DRAM read must be followed by a write-back while an SRAM read need not be.

# Chapter 17. What Breaks a Chip: ESD, Latch-up, and Aging

A chip that passes every test on the day it is made can still fail in three ways. It can be destroyed in an instant by a static discharge from a person or a machine. It can lock itself into a short circuit across its own supply through a parasitic device built into every CMOS process. Or it can wear out slowly as atoms in its wires drift and charges accumulate in its gate oxides. Each failure has a physical mechanism from Parts I to III and a design rule that follows from it.

## 17.1 Electrostatic discharge

A person walking across a carpet can charge to several kilovolts. Touching a pin, they discharge through the chip in a pulse lasting about a hundred nanoseconds. The standard human-body model (HBM) represents the person as a 100 pF capacitor discharging through 1.5 kΩ. The peak current is the test voltage divided by 1.5 kΩ, and it decays with a 150 ns time constant. The charged-device model (CDM) covers the other common case: the chip itself becomes charged during handling and discharges through one pin to a grounded tool, in a pulse lasting about a nanosecond with peak currents of several amperes.

HBM: Ipeak = VESD / 1.5 kΩ,    τ = 1.5 kΩ × 100 pF = 150 ns

## Worked Example 17.1

A pin is rated for 2 kV HBM. Find the peak current, the energy stored in the model capacitor, and the pin voltage during the pulse if the on-chip protection clamp triggers at 1.5 V and has 2 Ω of on-resistance.

Ipeak = 2000/1500 = 1.33 A. The stored energy is ½ × 100 pF × (2000 V)² = 0.2 mJ, nearly all dissipated in the 1.5 kΩ resistor, which is why the chip survives at all. The clamp holds the pin at about 1.5 + 1.33 × 2 = 4.2 V. A thin gate oxide connected to that pin must withstand 4.2 V for 100 ns, which sets the clamp's required size: a thinner oxide or a higher test level needs a larger, lower-resistance clamp, and that clamp's capacitance loads a high-speed pin, the central trade in I/O design.

The usual protection network places a diode from each pin to each supply rail and a large clamp transistor between the rails that turns on during a fast transient. A positive pulse into a pin passes through the upper diode onto the supply rail and through the rail clamp to ground; a negative pulse passes through the lower diode. The diodes are the junctions of Chapter 6 used in forward bias, where they handle amperes in a small area, and the design sets their size and the width of the metal that carries the current.

## 17.2 Latch-up

A CMOS inverter on a p-type substrate puts the PMOS in an n-type well. The PMOS source (p+), the well (n), and the substrate (p) form a parasitic PNP transistor. The well (n), the substrate (p), and the NMOS source (n+) form a parasitic NPN. The two are connected collector to base in a loop, the four-layer structure of a thyristor, which Chapter 19 treats as a deliberate device. Normally both are off. If a current injected into the substrate or well, from an overshooting I/O pin, a supply transient, or a radiation strike, develops enough voltage across the well or substrate resistance to forward-bias one emitter, that transistor turns on, its collector current turns on the other, and the loop sustains itself. A low-resistance path then connects VDD to ground until the power is removed, often after the metal has melted.

The loop can latch only if the product of the two parasitic current gains exceeds unity and the shunt resistances let the emitter junctions forward-bias. Design rules attack both conditions: substrate and well contacts placed frequently, so that injected current is collected before it can build a voltage, and guard rings around I/O transistors, diffusions that collect injected carriers before they reach the core. Silicon-on-insulator and the fully isolated fin and nanosheet devices reduce the risk because the parasitic loop has nowhere to form.

## Worked Example 17.2

An I/O pin can inject up to 10 mA into the substrate during an overshoot. An emitter junction of the parasitic NPN forward-biases at about 0.6 V. Find the largest substrate resistance between the injection point and the nearest substrate contact that keeps that junction off.

R ≤ 0.6 V/10 mA = 60 Ω. A lightly doped substrate has a resistivity of order 10 Ω·cm, so tens of micrometers of distance already exceed 60 Ω; the design rule that places a substrate contact within a short, specified distance of every NMOS, and a guard ring around every I/O device, is this inequality made into geometry. Production qualification tests inject a specified current, typically 100 mA, into every pin to confirm that it cannot latch.

Key idea. ESD is a failure of energy in nanoseconds, latch-up a failure of a parasitic thyristor, and aging a failure of slow drift. The first two are handled by protection structures and layout rules, the third by designing with margin for the end of life, not the start.

## 17.3 Electromigration

Electrons flowing through a thin copper or aluminum line transfer momentum to its atoms, and at high current density the atoms migrate downstream. Voids open where atoms leave and hillocks grow where they accumulate, until the line opens or shorts to a neighbor. The median time to failure follows Black's equation, with a current-density exponent n near 2 and an activation energy Ea of about 0.7 to 0.9 eV for copper interconnect.

MTTF = A J⁻ⁿ exp(Ea / kT)

## Worked Example 17.3

A copper line with Ea = 0.9 eV and n = 2 is rated for its life at 105 °C. Find the life factor if the line runs at 125 °C instead, and if the current density is doubled at the original temperature.

The temperature factor is exp((0.9/8.617 × 10⁻⁵) × (1/378.15 − 1/398.15)) = 4.0, so the line lasts one quarter as long at 125 °C. Doubling J shortens life by 2² = 4. A 20 °C rise costs as much as doubling the current. Design rules therefore set maximum current densities per wire width at a specified temperature, and power-delivery networks, which carry the largest currents, are drawn wide enough to meet them with margin.

## 17.4 Transistor aging

Transistors drift too. Under a sustained gate bias at high temperature, charges build up at the oxide interface and in the dielectric, and the threshold voltage magnitude rises. In PMOS devices under negative gate bias this is negative-bias temperature instability (NBTI); NMOS devices with high-permittivity dielectrics show the positive-bias version. The shift grows roughly as a power of time, ΔVth ∝ t^n with n near 0.2, so it is fast at first and slows, and it partly recovers when the stress is removed. Hot-carrier injection, in which electrons accelerated near the drain gain enough energy to enter the oxide, degrades devices that switch with high drain voltage. Time-dependent dielectric breakdown, the slow creation of a conducting path through the gate oxide, sets the maximum gate voltage a process allows.

## Worked Example 17.4

A PMOS device shows a 10 mV threshold shift after one year of stress, following ΔVth ∝ t^0.2. Predict the shift at ten years. If the gate delay changes by 0.12% per millivolt of threshold shift, what timing margin must the design keep for ten years?

ΔVth(10 years) = 10 mV × 10^0.2 = 15.8 mV. The delay increase is 15.8 × 0.12% = 1.9%. A clock path designed with zero margin at the start of life would fail timing after a few months; designers sign off with aging-aware models at the end-of-life point, and some chips monitor their own delay and raise the supply voltage slightly as they age.

## Practice

1. Find the HBM peak current at 8 kV.

2. Find the energy stored in the HBM capacitor at 4 kV.

3. A substrate injection of 25 mA must not raise any emitter above 0.5 V. Find the maximum substrate resistance.

4. A line with Ea = 0.7 eV is stressed at 105 °C instead of 85 °C. Find the electromigration life factor.

5. A threshold shift of 5 mV after one year follows t^0.2. Find the shift at ten years.

6. Explain in one or two sentences why the HBM 1.5 kΩ resistor, not the chip, absorbs most of the stored energy.

# Part VI. Light and Power

The last part takes the junction of Part I in two directions the earlier parts did not. The first is light: the recombination that the diode equations treated as a bookkeeping term turns out to emit photons in some crystals and absorb them in all of them, which gives the LED, the photodiode, and the solar cell. The second is high voltage: the breakdown mechanism Chapter 6 introduced becomes the design constraint of every power device, from the power MOSFET to the thyristor and the IGBT. Appendix C then hands the reader on to the rest of the series.

# Chapter 18. Recombination and Light: LEDs, Photodiodes, and Solar Cells

Chapter 5 derived the diode law by counting minority carriers injected across a junction and noting that they disappear by recombination within a diffusion length. Where the energy of each recombination goes, nothing in the circuit needed to know. In silicon it goes into heat. In gallium arsenide or gallium nitride much of it leaves as light. Run the process backward, and absorbed light creates carriers that a junction collects as current. One set of numbers, the bandgap and the carrier lifetime, connects all three devices in this chapter.

## 18.1 Lifetime and diffusion length

An excess minority carrier survives for an average time τ, the minority-carrier lifetime, before it recombines. While it survives it diffuses, and the average distance it covers is the diffusion length. Lifetimes range from nanoseconds in heavily doped or deliberately damaged silicon to milliseconds in the purest solar-grade material. The shortest lifetimes are engineered on purpose: fast-recovery diodes are doped with gold or platinum or irradiated, creating recombination centers that shorten the reverse-recovery tail of Chapter 6.

L = √(D τ)

## Worked Example 18.1

Using the declared diffusion constants Dn = 26.0 and Dp = 12.48 cm²/s, find the diffusion length of electrons with τ = 1 µs and of holes with τ = 10 µs.

Ln = √(26.0 × 10⁻⁶) cm = 5.1 × 10⁻³ cm = 51 µm. Lp = √(12.48 × 10⁻⁵) cm = 1.12 × 10⁻² cm = 112 µm. Both are long compared with the base of a transistor, which is why nearly every carrier injected into a thin base survives to the collector, and short compared with a solar wafer's thickness, which is why solar silicon needs long lifetimes.

## 18.2 Three ways to recombine

Recombination happens by three routes. In Shockley-Read-Hall recombination an electron and a hole meet at a defect level in the gap and their energy goes into lattice vibrations, heat. It dominates in silicon and depends on purity. In radiative recombination the electron drops directly across the gap and emits a photon of energy close to the bandgap. In Auger recombination the energy is handed to a third carrier, which matters at very high carrier densities, as in a heavily driven LED. Whether radiative recombination is efficient depends on the band structure. In a direct-gap semiconductor such as GaAs or GaN, the lowest conduction state and the highest valence state have the same crystal momentum, and an electron can fall straight down emitting only a photon. In an indirect-gap semiconductor such as silicon, a lattice vibration must take part to conserve momentum, the process is thousands of times slower, and defect recombination wins.

λ (nm) ≈ 1240 / E (eV)

## Worked Example 18.2

Find the wavelength corresponding to the bandgaps of silicon (1.12 eV), GaAs (1.42 eV), and GaN (3.4 eV), and say what each means for light emission and detection.

Silicon: 1240/1.12 = 1107 nm, in the near infrared. Silicon cannot emit efficiently, but it absorbs every wavelength shorter than about 1.1 µm, so it detects visible and near-infrared light and is transparent beyond that. GaAs: 873 nm, the classic infrared LED and laser material. GaN: 365 nm, ultraviolet; alloying it with indium lowers the gap into the blue, around 450 nm, which with a phosphor makes the white LED. Telecommunication wavelengths of 1310 and 1550 nm lie beyond silicon's edge, so their detectors are InGaAs grown on InP.

## 18.3 The LED

An LED is a forward-biased junction in a direct-gap material, usually a heterostructure that confines electrons and holes in a thin quantum-well layer where they recombine. Its forward voltage is roughly the photon energy in electron-volts plus resistive and heterojunction drops: about 1.8 to 2.2 V for red, about 3 V for blue. Its external quantum efficiency (EQE) is the fraction of injected electrons that produce a photon leaving the chip, and its wall-plug efficiency is the optical power out divided by the electrical power in.

Popt = EQE × (I/q) × hν,    ηwall = Popt / (I VF)

## Worked Example 18.3

A blue LED emits at 450 nm, runs at 20 mA with VF = 3.0 V, and has an EQE of 50%. Find the optical power and the wall-plug efficiency.

The photon energy is 1240/450 = 2.76 eV. The photon rate is 0.5 × 0.020/q, and each photon carries 2.76 eV, so Popt = 0.5 × 0.020 A × 2.76 V = 27.6 mW. The electrical input is 0.020 × 3.0 = 60 mW, so ηwall = 46%. The remaining 32 mW heats the junction, and because efficiency falls with temperature and at high current density (efficiency droop, mostly Auger recombination), LED drivers regulate current and LED luminaires are designed around heat sinking.

Key idea. The bandgap sets the color an LED emits and the longest wavelength a detector absorbs, through λ ≈ 1240/E. Whether a material emits efficiently is a question of direct versus indirect gap, not of the gap's size.

## 18.4 The photodiode

A reverse-biased junction that absorbs a photon in or near its depletion region collects the electron-hole pair as current. The quantum efficiency η is the fraction of incident photons that produce a collected pair, and the responsivity is the current per watt of light. The photocurrent is linear in light power over many decades, which is why photodiodes are run into the transimpedance amplifiers of Book 2 rather than read as voltages. Reverse bias widens the depletion region, which lowers the capacitance and speeds the response; an intrinsic layer between p and n (the PIN diode) does the same by design. The dark current, the junction's reverse leakage, sets the noise floor through its shot noise.

R = η q λ / (h c) ≈ η λ(nm) / 1240   (A/W)

## Worked Example 18.4

A silicon photodiode has η = 0.6 at 940 nm. An InGaAs photodiode has η = 0.7 at 1310 nm. Find both responsivities and the photocurrent from 50 µW on the InGaAs diode.

Silicon: 0.6 × 940/1240 = 0.45 A/W. InGaAs: 0.7 × 1310/1240 = 0.74 A/W. The photocurrent is 0.74 × 50 µW = 37 µA. Longer wavelengths give more current per watt because each watt contains more photons, each with less energy, until the wavelength passes the bandgap edge and the response falls to zero.

## 18.5 The solar cell

A solar cell is a large-area photodiode operated in the fourth quadrant, where it delivers power. Under illumination its current is the diode law minus a photocurrent IL. At open circuit the photocurrent is balanced entirely by forward diode current, which sets the open-circuit voltage. The maximum-power point lies a little below it, at about 80% of Voc in a good silicon cell. The Shockley-Queisser limit for a single junction, set by the bandgap and the solar spectrum, is about 33%; commercial silicon modules reach 20–24%.

Voc = VT ln(IL/IS + 1)

## Worked Example 18.5

A small silicon cell has IS = 1 pA and delivers IL = 35 mA in full sun. Find Voc. How much does Voc change if the light is halved?

Voc = 26 mV × ln(3.5 × 10¹⁰) = 26 mV × 24.3 = 0.631 V. Halving the light halves IL and lowers Voc by VT ln 2 = 18 mV, to 0.613 V. The voltage depends only logarithmically on light, while the current is proportional to it, which is why a panel's voltage stays near its rating from bright sun to overcast and its power falls mostly through current.

## Practice

1. Find the diffusion length of holes with Dp = 12.48 cm²/s and τ = 100 ns.

2. An LED emits at 625 nm. Find its photon energy.

3. A detector must respond at 1550 nm. Can it be silicon? Explain using the bandgap.

4. A photodiode has η = 0.8 at 850 nm. Find its responsivity and the current from 10 µW.

5. A red LED at 630 nm runs at 30 mA, VF = 2.1 V, EQE = 35%. Find its optical power and wall-plug efficiency.

6. A solar cell has Voc = 0.62 V at a given light level. Find Voc when the light is ten times stronger.

# Chapter 19. Breakdown and the Power Devices

Chapter 6 introduced breakdown as the knee where a reverse-biased diode suddenly conducts. For a power device, that knee is the specification everything else is traded against. A switch that must block 600 V needs a thick, lightly doped region to hold the field, and that same region is the resistance the current must cross when the switch is on. This chapter derives the trade, shows why it favors silicon carbide and gallium nitride at high voltage, and introduces the two silicon devices that work around it by flooding the drift region with carriers: the thyristor and the IGBT.

## 19.1 Avalanche and the critical field

In a reverse-biased junction, carriers crossing the depletion region are accelerated by the field. Above a certain field, a carrier gains enough energy between collisions to knock an electron out of a bond, creating a new pair, and each of those can do the same. The multiplication runs away at a critical field Ec, about 3 × 10⁵ V/cm in silicon at the doping levels of power devices (it rises slowly with doping). The wide-gap materials need far more energy per ionization and have critical fields about ten times higher: roughly 2.5 × 10⁶ V/cm for 4H-SiC and 3.3 × 10⁶ V/cm for GaN. Avalanche is not destructive in itself, and rated "avalanche-rugged" devices survive it for a specified energy, but it ends the device's ability to block. Narrow, heavily doped junctions break down at a few volts by a different mechanism, Zener tunneling, which is why reference diodes below about 5 V behave differently from those above.

## 19.2 Designing a drift region

Consider a one-sided junction: a heavily doped p+ region against a lightly doped n-type drift region of doping ND. The field is largest at the junction and falls linearly through the depletion region, with a slope set by the doping. Breakdown occurs when the peak field reaches Ec. Integrating the field over the depletion width gives the breakdown voltage, and solving for the doping and width that just support a required voltage gives the design of the drift region.

ND = εs Ec² / (2 q BV),    W = 2 BV / Ec,    Ron,sp = W / (q μn ND) = 4 BV² / (εs μn Ec³)

The last expression is the essential result. The specific on-resistance, the resistance of one square centimeter of drift region, rises as the square of the blocking voltage and falls as the cube of the critical field. The product εs μn Ec³ is Baliga's figure of merit, and it is the reason a material with a tenfold higher critical field can have a drift region hundreds of times less resistive for the same voltage.

## Worked Example 19.1

Design the drift region of an ideal silicon device that blocks 600 V, using εs = 11.7 ε0, Ec = 3 × 10⁵ V/cm, and μn = 1000 cm²/V·s. Find ND, W, and Ron,sp, and the chip area needed for 100 mΩ.

ND = (1.036 × 10⁻¹² × 9 × 10¹⁰)/(2 × 1.602 × 10⁻¹⁹ × 600) = 4.8 × 10¹⁴ cm⁻³. W = 2 × 600/3 × 10⁵ = 4.0 × 10⁻³ cm = 40 µm. Ron,sp = 4.0 × 10⁻³/(1.602 × 10⁻¹⁹ × 1000 × 4.8 × 10¹⁴) = 51 mΩ·cm². For 100 mΩ the area is 0.051/0.1 = 0.51 cm², a die more than 7 mm on a side, for the drift region alone. At 1200 V the square law gives four times the resistance, 206 mΩ·cm². The same calculation for 4H-SiC (εs = 9.7 ε0, Ec = 2.5 × 10⁶ V/cm, μn ≈ 900) at 1200 V gives about 0.48 mΩ·cm², over four hundred times lower; real SiC MOSFETs reach a few mΩ·cm² because their channel and substrate add resistance, which still makes them the standard high-voltage switch.

Key idea. A drift region's on-resistance rises as the square of the voltage it blocks and falls as the cube of the material's critical field. High-voltage switching therefore belongs either to wide-gap materials or to bipolar devices that flood the drift region with carriers.

## 19.3 The power MOSFET and the superjunction

A vertical power MOSFET places the channel at the top surface and the drain at the bottom of the die, with the drift region between them, and parallels hundreds of thousands of cells. Below about 200 V, the channel resistance is a large part of the total, and trench gates that pack the cells densely win. Above it, the drift region dominates. The superjunction MOSFET breaks the square law by building the drift region from alternating, charge-balanced columns of n- and p-type silicon: at high voltage the columns deplete each other sideways, so the field is flat and the n-columns can be doped much more heavily than a plain drift region for the same blocking voltage. Superjunction devices brought 600 V silicon MOSFETs well below the limit of Worked Example 19.1, and they remain the cost-effective choice at 600–900 V where SiC is not required.

## 19.4 The thyristor

A thyristor is a four-layer pnpn device, the same structure Chapter 17 found as the parasitic latch-up path, now made on purpose. It can be modeled as a PNP and an NPN transistor connected so that each one's collector drives the other's base. A gate current into the NPN base starts the loop. Once the sum of the two transistors' common-base gains reaches one, α₁ + α₂ = 1, the loop sustains itself and the gate loses control: the device stays on until the current falls below a holding current, normally at the next zero crossing of an AC supply. Because both carrier types flood the lightly doped regions, the on-state voltage is only one to two volts even in devices that block several kilovolts, and thyristors still switch the largest currents in power engineering, from light dimmers (as a triac, two thyristors in antiparallel) to high-voltage DC converter stations.

## 19.5 The IGBT

The insulated-gate bipolar transistor combines a MOSFET's gate with a bipolar output. Structurally, it is a vertical power MOSFET with an extra p+ layer at the bottom. When the MOSFET channel turns on, its electron current flows into the drift region and forward-biases the bottom junction, which injects holes. The drift region fills with both carriers far above its doping, the conductivity modulation that also gives the thyristor its low drop, so its resistance falls. The on-state looks like a diode voltage in series with a small resistance, and the current does not stop instantly at turn-off: the stored holes must recombine, producing a tail current that limits IGBTs to switching frequencies of tens of kilohertz. The structure also contains a parasitic thyristor, which modern designs suppress so thoroughly that IGBT latch-up is now rare.

MOSFET: Pcond = I² Ron,    IGBT: Pcond ≈ VCE0 I + rCE I²

## Worked Example 19.2

A 650 V MOSFET has Ron = 50 mΩ at operating temperature. A 650 V IGBT of similar die cost has VCE0 = 1.0 V and rCE = 20 mΩ. Compare their conduction losses at 20 A and 50 A, and find the current at which they are equal.

At 20 A: MOSFET 20² × 0.05 = 20 W; IGBT 1.0 × 20 + 0.02 × 400 = 28 W. At 50 A: MOSFET 125 W; IGBT 50 + 50 = 100 W. They are equal when 0.05 I² = 1.0 I + 0.02 I², which gives I = 1.0/0.03 = 33 A. Below that current the MOSFET wins and also switches faster; above it the IGBT's fixed junction drop is cheaper than the MOSFET's rising I²R. That is the usual division: MOSFETs, and increasingly SiC MOSFETs, for high frequency and moderate current; IGBTs for high current at low switching frequency, such as motor drives and traction inverters.

## Practice

1. Find ND, W, and Ron,sp for an ideal silicon drift region blocking 100 V, using the constants of Worked Example 19.1.

2. By what factor does Ron,sp change when a silicon design moves from 600 V to 1200 V?

3. Material A has a critical field three times that of material B, with equal permittivity and mobility. Compare their Ron,sp at the same blocking voltage.

4. A thyristor's two transistors have α₁ = 0.45 and α₂ = 0.40 at a given current. Is the device latched? What must happen for it to latch?

5. An IGBT has VCE0 = 0.9 V and rCE = 15 mΩ. Find its conduction loss at 40 A.

6. Explain in one or two sentences why an IGBT has a turn-off tail current and a MOSFET does not.

# Part VII. Making, Modeling, and New Materials

The earlier parts treated the device as given: a doping profile, a gate oxide, a fin, a junction. This last part asks where those come from and how designers use them. Chapter 20 follows a transistor through the fab, with the numbers that limit each step. Chapter 21 turns to a different kind of device the radio books rely on, the piezoelectric thin film and the acoustic resonator built from it, including the aluminum scandium nitride films of 2026. Chapter 22 explains the compact models and process design kits through which every device in this book reaches a circuit simulator.

# Chapter 20. How a Transistor Is Made

Every device in this book begins as a polished wafer of nearly perfect silicon, 300 mm across in a modern fab. It becomes a chip through a few hundred steps of a small number of kinds: growing or depositing films, patterning them with light, etching away what is not wanted, adding dopants, and heating to activate and move them. This chapter explains the steps whose physics sets the limits of the devices: diffusion, which blurs doping profiles with every heat cycle; ion implantation, which places dopants precisely; and lithography, whose resolution decides how small anything can be. The process engineer's details are in Plummer, Deal, and Griffin; the designer needs the numbers below.

## 20.1 Oxidation and films

Silicon's great advantage is its oxide. Heating silicon in oxygen or steam grows a layer of silicon dioxide that is an excellent insulator with a clean interface, and for every 100 nm of oxide grown, about 44 nm of silicon is consumed. Below the 45 nm node, the gate dielectric became a high-κ film, hafnium oxide, deposited atom layer by atom layer, because a silicon dioxide gate thin enough for the required capacitance leaked too much by tunneling. Other films, polysilicon, nitride, and metals, are deposited by chemical vapor deposition, atomic-layer deposition, or sputtering.

## 20.2 Diffusion and the thermal budget

Dopants move through the crystal by diffusion, the same random walk that Chapter 3 described for carriers, but millions of times slower and strongly dependent on temperature. The diffusivity follows an Arrhenius law, D = D0 exp(−Ea/kT); for boron in silicon a hand value is D0 = 1.0 cm²/s and Ea = 3.5 eV. A profile spreads by a characteristic length of about 2√(Dt). Every high-temperature step after the dopants are placed adds to that spreading, so the total of time and temperature, the thermal budget, must be kept small for shallow junctions. Modern processes use spike and flash anneals that hold the peak temperature for a second or less.

D = D0 exp(−Ea/kT),   diffusion length ≈ 2√(Dt)

## Worked Example 20.1

Find boron's diffusivity at 1000 °C and the diffusion length after 30 minutes. Compare it with a 10 s anneal at 1050 °C.

At 1273 K, kT = 0.1097 eV and D = 1.0 × exp(−3.5/0.1097) = 1.40 × 10⁻¹⁴ cm²/s. After 1800 s, 2√(Dt) = 2√(2.51 × 10⁻¹¹) cm = 0.10 µm, larger than the whole junction depth of a modern transistor. At 1050 °C, D = 4.66 × 10⁻¹⁴ cm²/s, and in 10 s the diffusion length is 2√(4.66 × 10⁻¹³) cm = 13.7 nm. The short, hot anneal activates the dopants while moving them only a few nanometers.

## 20.3 Ion implantation

Dopants are placed by ion implantation: ionized boron, phosphorus, or arsenic atoms are accelerated to kilo-electron-volt energies and shot into the wafer, where they stop at a depth set by their energy and spread around it in a roughly Gaussian profile. The dose, in atoms per square centimeter, is counted precisely by measuring the beam current, which is why implantation replaced diffusion from surface sources. The impacts damage the crystal, so an anneal must repair it and move the dopants onto lattice sites where they become active, which is the step Example 20.1 limits.

## Worked Example 20.2

A source-drain implant delivers 10¹⁵ atoms/cm² into a layer about 0.1 µm deep. Estimate the average concentration.

10¹⁵ cm⁻² / 10⁻⁵ cm = 10²⁰ cm⁻³, about one dopant per 500 silicon atoms, far above the doping of Chapter 2's examples and enough to make the region nearly metallic.

Key idea. The fab places dopants by implantation and then fights to keep them in place, because every heat step spreads them by about 2√(Dt). Lithography sets how small a feature can be drawn; diffusion sets how sharp it stays.

## 20.4 Lithography

Each layer's pattern is printed by projecting light through a mask onto a photoresist. The smallest half-pitch that can be printed follows the Rayleigh criterion, R = k1λ/NA, with λ the wavelength, NA the numerical aperture of the lens, and k1 a process factor whose physical limit for a single exposure is 0.25. The depth of focus, about k2λ/NA², shrinks faster than the resolution, which is why wafers must be extremely flat and are polished between layers. Argon-fluoride immersion lithography at 193 nm reached its limit around 40 nm half-pitch, and finer features were made by multiple patterning: printing a layer in two, three, or four exposures. Extreme ultraviolet lithography at 13.5 nm, in production since 2019, and high-NA EUV with NA = 0.55 restore single exposures for the finest layers.

R = k1 λ / NA,   DOF ≈ k2 λ / NA²

## Worked Example 20.3

Find the half-pitch resolution of 193 nm immersion at NA = 1.35 and k1 = 0.28, of EUV at NA = 0.33 and k1 = 0.4, and of high-NA EUV at NA = 0.55 and the same k1. Compare the depths of focus with k2 = 0.5.

Immersion: 0.28 × 193/1.35 = 40 nm, DOF 0.5 × 193/1.35² = 53 nm. EUV: 0.4 × 13.5/0.33 = 16.4 nm, DOF 62 nm. High-NA EUV: 0.4 × 13.5/0.55 = 9.8 nm, DOF only 22 nm. The finest metal and gate layers of 2026 nodes need high-NA EUV or EUV with double patterning, and the shallow focus of high NA demands thinner resists and flatter wafers.

## 20.5 From front end to back end

The front end of line builds the transistors: isolation, wells, the fin or nanosheet stack of Chapter 11, the gate, and the source and drain. The back end of line builds the wiring: a dozen or more layers of copper, or of cobalt and ruthenium at the finest pitches, separated by low-κ dielectrics and connected by vias. The wiring's resistance and capacitance, not the transistor, now limit much of a chip's speed and power, and the electromigration of Chapter 17 limits its current. The finished wafer is tested, diced, and packaged, which is Book 7's subject.

## Practice

1. How much silicon is consumed in growing 20 nm of thermal oxide?

2. Find boron's diffusivity at 1100 °C with the hand values.

3. Find the diffusion length for 1 hour at 1100 °C.

4. An implant of 5 × 10¹³ cm⁻² is spread over 0.5 µm. Find the average concentration.

5. Find the half-pitch for EUV at NA = 0.33 with k1 = 0.3.

6. Why did the industry adopt multiple patterning before EUV was ready?

# Chapter 21. Piezoelectric Films and Acoustic Resonators

A phone contains dozens of radio filters, and almost none of them are made from inductors and capacitors. They are acoustic: a piezoelectric crystal or film converts the electrical signal into a sound wave, the sound resonates in a structure a few micrometers in size, and the film converts it back. Because sound travels about 26,000 times slower than light in this material, an acoustic resonator at 2.45 GHz is micrometers long where an electromagnetic one would be centimeters, with a Q in the thousands. This chapter explains piezoelectricity, the bulk and surface acoustic resonators built on it, and why aluminum scandium nitride, AlScN, became the film of choice for the newest high-frequency filters. The survey of 2026 MEMS products is Book 7's.

## 21.1 Piezoelectricity

In a crystal without a center of symmetry, such as quartz, lithium tantalate, or the wurtzite form of aluminum nitride, straining the lattice shifts the positive and negative ions differently and produces a polarization; conversely, an applied field strains the crystal. The strength of this coupling between electrical and mechanical energy is expressed by an electromechanical coupling coefficient, k², the fraction of stored energy that converts between the two forms. Aluminum nitride, sputtered as a thin film compatible with CMOS fabs, has a thickness-mode k² of about 6.5%. Replacing part of the aluminum with scandium softens the lattice and raises the piezoelectric response substantially, so that AlScN films with scandium fractions of 20% to 30% reach roughly twice that coupling, at the cost of more difficult deposition and somewhat higher acoustic loss.

## 21.2 The bulk acoustic resonator

A film bulk acoustic resonator, FBAR, or its solidly mounted cousin, SMR, is a thin piezoelectric layer between two metal electrodes. A voltage across it launches a longitudinal sound wave through its thickness, and the structure resonates when the thickness is half an acoustic wavelength. In an FBAR the film is suspended over an air gap; in an SMR it sits on an acoustic mirror of alternating quarter-wave layers. Both trap the energy in the film. The electrodes add mass and lower the frequency, so real films are thinner than the simple formula gives, and the final frequency is trimmed by adjusting a thin layer on top.

f ≈ v / (2t),   λacoustic = v / f

## Worked Example 21.1

Aluminum nitride has a longitudinal sound velocity of about 11,350 m/s. Find the film thickness for a 2.45 GHz and a 5.5 GHz resonator, ignoring the electrodes, and compare the acoustic and electromagnetic wavelengths at 2.45 GHz.

t = 11,350/(2 × 2.45 × 10⁹) = 2.32 µm, and 11,350/(2 × 5.5 × 10⁹) = 1.03 µm. The acoustic wavelength at 2.45 GHz is 4.63 µm; the free-space wavelength is 122 mm, 26,400 times longer. That ratio is the whole reason acoustic filters fit in a phone.

## 21.3 The resonator as a circuit

Near resonance, an acoustic resonator behaves like the Butterworth-Van Dyke circuit: a series branch of Lm, Cm, and Rm, representing the mechanical resonance, in parallel with the plate capacitance C0 of the electrodes. The series branch resonates at fs, where the impedance is minimum, and with C0 it produces a parallel resonance at fp = fs√(1 + Cm/C0), where the impedance is maximum. The spacing between them is set by the coupling: kt² ≈ (π²/4)(fp − fs)/fp. A ladder filter of series and shunt resonators, offset in frequency, places its passband between these resonances, so the coupling sets the widest bandwidth a filter can have.

fp = fs √(1 + Cm/C0),   kt² ≈ (π²/4)(fp − fs)/fp

## Worked Example 21.2

A 2.45 GHz resonator has a series resonance fs = 2.45 GHz. Find fp and the spacing for kt² = 6.5% (AlN) and 13% (AlScN). Find Cm/C0 for the AlN case. Then size an AlN resonator, εr = 9.5, so that C0 has a reactance of 50 Ω at 2.45 GHz.

For AlN, (fp − fs)/fp = 0.065 × 4/π² = 0.0263, so fp = 2.45/(1 − 0.0263) = 2.516 GHz, a spacing of 66 MHz, and Cm/C0 = (2.516/2.45)² − 1 = 0.055. For AlScN at 13%, fp = 2.586 GHz, a spacing of 136 MHz, wide enough for the broad 5G bands that AlN cannot cover. For 50 Ω, C0 = 1/(2π × 2.45 × 10⁹ × 50) = 1.30 pF; with t = 2.32 µm, the area is C0t/(ε0εr) = 0.036 mm², a square 189 µm on a side.

Key idea. An acoustic resonator is a piezoelectric film half a sound wavelength thick. Its coupling coefficient sets the spacing of its two resonances and so the widest filter it can make, which is why higher-coupling AlScN films matter for wideband 5G and Wi-Fi bands.

## 21.4 Surface acoustic waves

A surface acoustic wave, SAW, device launches a wave along the surface of a piezoelectric crystal, usually lithium tantalate or lithium niobate, with interleaved comb electrodes. The electrode period is one acoustic wavelength, so with a velocity near 4000 m/s a 1 GHz device has a 4 µm period and 1 µm fingers. SAW filters are cheap and dominate below about 2 GHz; above that, the fingers become too fine and lossy, and BAW takes over. Temperature-compensated SAW adds a silicon dioxide layer whose stiffness changes in the opposite direction, and thin-film SAW on bonded substrates extends the technology upward.

## 21.5 Temperature and power

Acoustic velocities change with temperature, so filters drift: aluminum nitride's temperature coefficient of frequency is about −25 ppm/K, which moves a 2.45 GHz filter by 4.9 MHz over an 80 K range, a significant fraction of a narrow guard band. Oxide compensation layers reduce the drift. Power handling is the other limit: a transmit filter carries watts in a volume of a fraction of a cubic millimeter, and heating and acoustic stress at the edges set its maximum power, which is why duplexers for high-power user equipment use larger or cascaded resonators.

## Practice

1. Find the AlN film thickness for a 3.5 GHz resonator, ignoring electrodes.

2. Find the finger width, a quarter of the period, of a 2 GHz SAW device with v = 4000 m/s.

3. For a 3.5 GHz resonator with kt² = 10%, find the spacing between fs and fp.

4. A resonator has Cm/C0 = 0.05. Find fp/fs.

5. Why are acoustic resonators so much smaller than electromagnetic ones?

6. Why does adding scandium to AlN widen the filters it can make?

# Chapter 22. Compact Models and the Process Design Kit

This book has derived device behavior from hand equations with declared values. A designer at a foundry's 3 nm node uses none of them directly. The simulator calls a compact model, a set of equations with hundreds of fitted parameters, delivered with the process design kit, the PDK, that also contains the layout rules, the corner and statistical models, and the verified cells. Appendix C's rule that the PDK outranks every textbook below about 20 nm is a statement about these models. This chapter explains what a compact model is, how its parameters are extracted, and how corners and Monte Carlo statistics turn device variation into design margins.

## 22.1 What a compact model is

A compact model is a set of closed-form equations that give a device's terminal currents and charges as functions of its terminal voltages, smooth and continuous in every region so that the simulator's Newton iterations converge. The diode model is the simplest: the Shockley equation of Chapter 5 with a saturation current Is and an ideality factor n, plus series resistance, junction capacitance, and breakdown. The Gummel-Poon model extends the bipolar transistor of Chapter 8 with high-injection roll-off, the Early effect of Chapter 9, and charge storage. For MOSFETs the Berkeley BSIM family dominates: BSIM4 for planar devices and BSIM-CMG for FinFETs and nanosheets, with hundreds of parameters each for short-channel effects, velocity saturation, quantum confinement, and parasitic resistances.

## 22.2 Extracting parameters

Parameters are fitted to measurements of test structures across sizes, voltages, and temperatures. For the diode, two points on the exponential region determine n and Is directly: the ideality factor from the slope, n = ΔV/(VT ln(I2/I1)), and Is from either point. For a full MOSFET model, extraction proceeds in stages, long and wide devices first for the basic threshold and mobility, then short and narrow ones for the short-channel terms, then capacitance and noise, each stage holding the earlier parameters fixed. The result is accurate inside the measured range and can be badly wrong outside it, so a model's validity limits are part of the PDK.

n = ΔV / (VT ln(I2/I1)),   Is = I1 exp(−V1 / (n VT))

## Worked Example 22.1

A diode carries 1 mA at 0.65 V and 10 mA at 0.71 V at room temperature. Extract n and Is with VT = 26 mV.

n = 0.06/(0.026 × ln 10) = 0.06/0.0599 = 1.002, essentially ideal. Is = 10⁻³ × exp(−0.65/(1.002 × 0.026)) = 1.47 × 10⁻¹⁴ A. A third point at higher current that falls below the line would be fitted by the series resistance.

Key idea. A compact model is a smooth set of equations with fitted parameters, valid inside the range it was fitted over. Corners cover global process shifts; Monte Carlo covers local mismatch; the PDK supplies both.

## 22.3 Corners

Wafers differ from one another and from lot to lot, shifting every transistor's threshold and mobility together. The PDK captures this global variation in corner models: typical, fast, and slow for the NMOS and PMOS devices, giving the corners TT, FF, SS, FS, and SF, usually set at about three standard deviations. A design is simulated at every corner combined with the extremes of supply and temperature, typically −40 °C to 125 °C for automotive parts. A three-sigma corner on one side excludes only 0.13% of parts, so a design that works at all corners will work in nearly every manufactured chip, as far as global variation goes.

## 22.4 Local mismatch and Monte Carlo

Neighboring transistors on the same die also differ, randomly, because of the discrete dopant atoms, line-edge roughness, and the grain structure of the gate metal. Chapter 12 described this with Pelgrom's law, σ(ΔVt) = AVt/√(WL). Statistical models in the PDK represent it, and Monte Carlo simulation draws random parameter sets for every device to find the spread of a circuit's performance. For a structure repeated millions of times, like an SRAM cell, the margins must cover far more than three sigma: if a cache of 128 Mbit may contain no failing cell, the per-cell failure probability must be below 1/134,217,728 = 7.5 × 10⁻⁹, which corresponds to 5.7 sigma. Plain Monte Carlo cannot reach such tails in reasonable time, so specialized importance-sampling methods are used.

σ(ΔVt) = AVt / √(WL);   one-sided tail Q(z): Q(3) = 1.35 × 10⁻³,  Q(6) = 9.9 × 10⁻¹⁰

## Worked Example 22.2

With AVt = 3 mV·µm, find the threshold mismatch of a minimum-size pair with W = 0.1 µm and L = 0.05 µm, and of a pair with W = L = 1 µm. How many Monte Carlo runs would be needed just to observe about three failures if the failure rate is 10⁻³?

For the small pair, σ = 3/√0.005 = 42.4 mV, larger than many circuits' entire offset budget. For the 1 µm² pair, σ = 3 mV. Area buys matching, which is why analog input pairs are large. To see three failures at a rate of 10⁻³ requires about 3000 runs; a 10⁻⁹ rate would need billions, which is why the tails are estimated rather than sampled.

## Practice

1. A diode carries 0.1 mA at 0.60 V and 1 mA at 0.66 V. Find n.

2. Find σ(ΔVt) for a pair with W = 0.5 µm and L = 0.2 µm, using AVt = 3 mV·µm.

3. What area gives σ(ΔVt) = 1 mV with AVt = 3 mV·µm?

4. What fraction of parts lies inside ±3σ?

5. Why must a compact model be continuous and smooth in all regions?

6. Why do SRAM designers work at 5 to 6 sigma instead of 3?

# Appendix A. Constants Used in This Book

Thermal voltage VT: 26 mV. Declared room-temperature hand value.

Intrinsic concentration ni of silicon: 1.0×10^10 cm⁻³. Declared hand value.

Elementary charge q: 1.602×10⁻¹⁹ C.

Relative permittivity of silicon: 11.7.

Working mobilities, when a problem states none: μn = 1000 cm²/V·s, μp = 480 cm²/V·s.

Einstein, from those values: Dn = 26.0 cm²/s, Dp = 12.48 cm²/s.

Voltage per decade of ideal forward current: 59.9 mV.

Worked-diode saturation current: IS = 1.0×10⁻¹⁵ A.

Silicon bandgap cited in Chapter 1 is about 1.12 eV. Germanium about 0.66 eV. Gallium arsenide about 1.42 eV. Those three are orientation numbers, not inputs to the worked arithmetic.

Declared bipolar transistor (Part II): β = 100, IS = 1 fA, VA = 100 V, operating point IC = 1 mA, which gives VBE = 0.718 V, gm = 38.5 mS, rπ = 2.60 kΩ, ro = 100 kΩ.

VBE temperature coefficient at constant IC, hand value: −2 mV/°C.

Oxide permittivity εox = 3.9 ε0 = 3.45×10⁻¹³ F/cm (Part III).

Electron saturation velocity in silicon, hand value: vsat = 1.0×10⁷ cm/s.

Symbol note: VT is the thermal voltage; the MOSFET threshold is Vth.

Noise temperature for Part IV: 290 K, so 4kT = 1.60×10⁻²⁰ J (the same reference as Book 2). Device equations keep VT = 26 mV.

Channel-noise factor of a long-channel MOSFET: γ = 2/3.

Pelgrom matching constant, working value for a recent process: AVt = 3 mV·µm.

Critical field for avalanche, hand values: silicon 3×10⁵ V/cm, 4H-SiC 2.5×10⁶ V/cm, GaN 3.3×10⁶ V/cm.

Photon energy and wavelength: λ (nm) ≈ 1240 / E (eV).

Human-body ESD model: 100 pF through 1.5 kΩ.

Boron diffusion in silicon, hand values: D = D0 exp(−Ea/kT), D0 = 1.0 cm²/s, Ea = 3.5 eV; diffusion length ≈ 2√(Dt).

Lithography: half-pitch R = k1λ/NA (k1 ≥ 0.25 for one exposure); DOF ≈ k2λ/NA². ArF 193 nm, EUV 13.5 nm.

Acoustic: AlN longitudinal velocity ≈ 11,350 m/s, kt² ≈ 6.5%, εr ≈ 9.5, TCF ≈ −25 ppm/K. BAW f ≈ v/(2t); kt² ≈ (π²/4)(fp − fs)/fp.

Statistics: one-sided Q(3) = 1.35×10⁻³, Q(6) = 9.9×10⁻¹⁰; ±3σ holds 99.73%.

# Appendix B. Answers to the Practice Problems

## Chapter 1

1. p = ni²/n = 1.0e+05 cm⁻³. σ = q n μn = 0.1602 (Ω·cm)⁻¹. ρ = 6.242 Ω·cm. Ten times fewer electrons, ten times the resistivity.

2. A wider gap means fewer bonds broken at a given temperature, so ni is smaller. Reverse current of a junction tracks that minority population, so the leakage falls.

3. σ = q ni (μn + μp) = 2.371e-06 (Ω·cm)⁻¹. The hole term is μp/(μn+μp) = 0.32 of the total, about a third. It is not ignorable. It was ignorable in the worked example only because doping had made n huge and p tiny.

## Chapter 2

1. n ≈ 1.0×10^15 cm⁻³. p = ni²/n = 1.0e+05 cm⁻³.

2. The hole concentration is ni²/ND, not zero. At ND = 10^16 cm⁻³ it is 1.0e+04 cm⁻³.

3. Donors exceed acceptors by 9.0×10^15 cm⁻³, so the material is n-type. The majority electron concentration is approximately ND − NA = 9.0×10^15 cm⁻³. The minority hole concentration is ni² divided by that majority, about 1.1×10^4 cm⁻³. The subtraction is charge neutrality with the small carrier terms dropped. Dropping them is fair here because both are far below the doping difference.

## Chapter 3

1. Dn = 26.0 cm²/s. Dp = 12.48 cm²/s.

2. |Jp| = q Dp |dp/dx| = 199.952 A/cm². Conventional current points the same way the holes are spreading.

3. Jn,drift = q n μn E = 16.022 A/cm². The diffusion example was 374.909 A/cm². A modest field of 10 V/cm in this doping already beats that particular one-micron gradient. Steeper gradients, the kind inside a junction, put diffusion back in front.

## Chapter 4

1. NA ND / ni² = 1.0×10^34 / 1.0×10^20 = 1.0×10^14. Vbi = 0.026 × ln(10^14) = 0.026 × 32.236 = 0.838 V.

2. Only somewhat larger. Width follows the square root of (Vbi − V). A few volts of reverse bias increases the term inside the root by a factor of several, and the width by the square root of that factor, not by the factor itself.

3. Vbi depends on ln(1/ni²). A larger ni shrinks the argument of the logarithm, so Vbi falls.

## Chapter 5

1. I(0.65 V) = 7.200e-05 A = 72.00 μA.

2. One decade is 59.9 mV, so the rule says 59.9 mV. Exact voltages: 0.6585 V at 0.1 mA and 0.7184 V at 1 mA, a difference of 59.9 mV. The rule is the exact ideal-diode result once I is far above IS, which it is.

3. I(−0.2 V) = -9.995e-16 A, which is −IS times (1 − e^(−0.2/VT)). e^(−0.2/VT) is about 4.56e-04, so the factor in parentheses is already indistinguishable from 1 at three significant figures, and I ≈ −IS. A real diode adds surface leakage and generation inside the depletion layer, both missing from the ideal law.

## Chapter 6

1. 1 − V/Vbi = 3.784. Cj = 20 / 1.945 = 10.28 pF.

2. The time is waiting on minority charge stored by the forward current. A Schottky junction does not store that minority population in the same way, so the recovery tail is largely absent.

3. The resistor sets the current through the knee. The junction sets the voltage. Without the resistor the supply would try to hold 12 V across a 5.1 V knee.

## Chapter 7

1. Ropt = 40 / 0.50 = 80 Ω. P = (40)(0.50)/2 = 10 W.

2. Open an InP HEMT first. Do not open a GaN power switch, and do not open a handset HBT. A survivable limiter in front of that LNA can be GaN. The noise-figure deficit of 0.4 dB is an InP problem.

3. The 650 V part is a GaN-on-silicon power HEMT. InP is a small, fragile, poorly heat-spreading substrate with a 1 to 5 V RF habit. It is not in that market.

## Chapter 8

1. IS doubles to 3.2×10⁻¹⁶ A, because IS is inversely proportional to WB. The transit time WB²/(2Dn) falls by a factor of four, to 1.25 ps.

2. α = 150/151 = 0.993. IE = IC/α = 2.0 × 151/150 = 2.013 mA.

3. VBE = 0.026 ln(10⁻⁴/10⁻¹⁵) = 0.659 V and 0.026 ln(10⁻²/10⁻¹⁵) = 0.778 V, a difference of 120 mV. Two decades of current at about 60 mV per decade.

4. β is proportional to NE, so the ratio predicts three times the gain. A real device gains less because heavier doping narrows the emitter bandgap and lowers the hole diffusion constant, both of which increase backward injection.

5. In saturation the forward-biased collector junction stores excess minority charge in the base and collector, and that charge must be removed or recombine before the collector junction can block; in forward-active mode that excess charge was never stored.

## Chapter 9

1. gm = 0.25 mA/26 mV ≈ 9.62 mS; rπ = 120/0.00962 ≈ 12.5 kΩ.

2. ro = 60 V/0.5 mA = 120 kΩ; gm ro = VA/VT = 60/0.026 ≈ 2310, about 67 dB.

3. RE = (4.0 − 0.718)/0.986 mA ≈ 3.33 kΩ. ΔIE = 0.100/3330 ≈ 30 µA, about 3.0% instead of 7.8%.

4. gm = 76.9 mS; Cπ = 0.0769 × 10 ps + 50 fF ≈ 819 fF; fT = 0.0769/(2π × 839 fF) ≈ 14.6 GHz.

5. gm is proportional to IC and ro is inversely proportional to IC, so their product VA/VT contains no current at all.

## Chapter 10

1. Cox = 3.45×10⁻¹³/1.0×10⁻⁷ ≈ 3.45×10⁻⁶ F/cm² ≈ 34.5 fF/µm².

2. ID = ½ × 200 µA/V² × 10 × 0.16 = 160 µA; gm = 2 × 160 µA/0.4 V = 0.80 mS; gm/ID = 5 V⁻¹. Doubling Vov quadrupled the current but only doubled gm.

3. VDS < Vov, so it is in triode, where it behaves as a gate-controlled resistor; in bipolar vocabulary this corresponds to saturation.

4. 120/80 = 1.5 decades, a factor of 10^1.5 ≈ 32.

5. Once carriers move at a fixed saturated velocity, the current is the channel charge per unit width times that velocity, which does not involve L; shortening the channel only raises the field in a region where velocity can no longer increase.

## Chapter 11

1. 3 × (2 × 42 + 6) = 3 × 90 = 270 nm.

2. 4 × 2 × (25 + 6) = 248 nm.

3. 50 mV/0.70 V ≈ 71 mV/V.

4. In a thin channel, every point is close to a gate electrode, so the gate's field sets the potential throughout the channel; the drain's field, which must reach in from the far end, has little channel left to influence.

## Chapter 12

1. IOUT = 200 µA/(1 + 2/50) = 192.3 µA.

2. 2 × 2 mV/0.15 V = 2.7%.

3. √(WL) = 4/0.5 = 8 µm, so WL = 64 µm².

4. gm = 200 µA/26 mV = 7.69 mS; gain = 7.69 mS × 20 kΩ = 154.

5. VT ln(1.02) = 26 mV × 0.0198 = 0.51 mV.

6. Edge effects such as diffusion under the gate edge and corner rounding scale with perimeter, not area, so one wide device does not carry exactly four times the current of a narrow one; four copies of the same unit device reproduce every edge effect four times.

## Chapter 13

1. 180 × 220 = 39,600, which is 92.0 dB.

2. Cc = 0.25 mS/(2π × 5 MHz) = 7.96 pF.

3. SR = 40 µA/30 pF = 1.33 V/µs.

4. SR = 2π × 10 MHz × 0.15 V = 9.4 V/µs.

5. CL = 4 mS/(2π × 6.6 MHz) = 96 pF.

6. Each stage's gain gm ro is independent of current because gm rises and ro falls in proportion, but the second pole gm6/(2πCL) moves up with gm6, so a larger load capacitance keeps the same pole frequency.

## Chapter 14

1. √(2 × 1.602 × 10⁻¹⁹ × 10⁻⁴) = 5.7 pA/√Hz.

2. en = 26 mV × √(3.20 × 10⁻¹⁹/10⁻⁴) = 1.47 nV/√Hz.

3. rπ = 200 × 26 Ω = 5.2 kΩ; rπ/√β = 5.2 kΩ/14.1 = 368 Ω.

4. √(1.60 × 10⁻²⁰ × 0.667/0.004) = 1.6 nV/√Hz.

5. 8 × √(1 + 100) = 80 nV/√Hz.

6. At 10 MΩ, a bipolar input's current noise of order 1 pA/√Hz produces 10 µV/√Hz in the source resistance, far above any voltage noise, while a CMOS input's femtoampere current noise produces only nanovolts; the source resistance's own thermal noise, about 400 nV/√Hz, then dominates.

## Chapter 15

1. (W/L)p/(W/L)n = 2.5.

2. tp = 3 fF × 0.75 V/(2 × 60 µA) = 18.8 ps.

3. 0.15 × 5 × 10⁻⁹ × 0.81 × 1.5 × 10⁹ = 0.91 W.

4. (0.7/1.0)² = 0.49, so 51% is saved.

5. 150/75 = 2 decades, so leakage falls to 0.5/100 = 5 mW.

6. Charging a capacitor to VDD through any resistance dissipates ½CV² in that resistance, because a smaller resistance carries a larger current for a shorter time and the product is fixed; the resistance sets the speed, not the energy.

## Chapter 16

1. ΔVbl = 0.55 × 15/105 = 78.6 mV.

2. I = 15 fF × 0.2 V/32 ms = 94 fA.

3. N = 0.2 × 10⁻¹⁵ × 0.5/1.602 × 10⁻¹⁹ = 624 electrons.

4. Four levels have three spaces, so 5/3 = 1.67 V.

5. 0.9 × 0.2/1.2 = 0.15 V.

6. Charge sharing with the bit line leaves the DRAM cell at nearly the bit-line voltage, so its stored value is lost and must be restored; an SRAM cell's latch actively holds its state through a correctly sized read and restores its own nodes.

## Chapter 17

1. 8000/1500 = 5.33 A.

2. ½ × 100 pF × (4000)² = 0.8 mJ.

3. 0.5/0.025 = 20 Ω.

4. exp((0.7/8.617 × 10⁻⁵)(1/358.15 − 1/378.15)) = 3.3, so the life falls to about 30%.

5. 5 × 10^0.2 = 7.9 mV.

6. The protection clamp holds the pin at a few volts while the capacitor starts at kilovolts, so almost all of the voltage, and therefore almost all of the I²R energy, appears across the 1.5 kΩ resistor in series.

## Chapter 18

1. L = √(12.48 × 10⁻⁷) cm = 1.12 × 10⁻³ cm = 11.2 µm.

2. 1240/625 = 1.98 eV.

3. No. 1550 nm corresponds to 1240/1550 = 0.80 eV, below silicon's 1.12 eV gap, so the photons pass through without creating carriers; InGaAs or germanium is used.

4. R = 0.8 × 850/1240 = 0.55 A/W; I = 5.5 µA.

5. hν = 1240/630 = 1.97 eV; Popt = 0.35 × 0.030 × 1.97 = 20.7 mW; input 63 mW; ηwall = 33%.

6. Voc rises by VT ln 10 = 60 mV, to 0.68 V.

## Chapter 19

1. ND = 2.9 × 10¹⁵ cm⁻³; W = 6.7 µm; Ron,sp = 1.43 mΩ·cm².

2. Ron,sp ∝ BV², so it rises by 2² = 4.

3. Ron,sp ∝ 1/Ec³, so material A has 1/27 of the resistance of material B.

4. α₁ + α₂ = 0.85 < 1, so it is not latched. A gate current, or a current rise that increases both gains, must bring the sum to 1.

5. 0.9 × 40 + 0.015 × 1600 = 36 + 24 = 60 W.

6. The IGBT conducts by flooding its drift region with injected holes, which must recombine or be swept out after the channel turns off, while a MOSFET's current is carried by majority electrons alone, which stop when the channel closes.

## Chapter 20

1. About 8.8 nm.

2. 1.42 × 10⁻¹³ cm²/s.

3. 2√(1.42 × 10⁻¹³ × 3600) cm = 0.45 µm.

4. 10¹⁸ cm⁻³.

5. 0.3 × 13.5/0.33 = 12.3 nm.

6. 193 nm immersion lithography could not print features below about 40 nm half-pitch in one exposure, so smaller pitches were split across several aligned exposures until EUV tools reached production.

## Chapter 21

1. 1.62 µm.

2. 0.5 µm.

3. About 148 MHz.

4. √1.05 = 1.0247.

5. Sound in these materials travels tens of thousands of times slower than light, so its wavelength at the same frequency is that much shorter.

6. It raises the piezoelectric coupling, which increases the spacing between the series and parallel resonances that bounds the filter bandwidth.

## Chapter 22

1. 0.06/(0.026 × ln 10) = 1.00.

2. 3/√0.1 = 9.5 mV.

3. 9 µm².

4. 99.73%.

5. The simulator solves the circuit by Newton iteration, which fails to converge if currents or their derivatives jump between regions.

6. A memory has millions or billions of identical cells, and all must work, so the per-cell failure probability must be extremely small.

# Appendix C. Where This Book Hands Off

This book now runs from the crystal to the chip: carriers and junctions (Part I), the bipolar transistor (Part II), the MOSFET to the nanosheet (Part III), the amplifier built from them (Part IV), CMOS as a chip (Part V), the junction as a source and detector of light and as a high-voltage switch (Part VI), and how devices are made, modeled, and extended to piezoelectric films (Part VII). Each later book of the series starts from a device this one has explained and stops explaining it.

Part VII adds the fab steps that set every device's limits (Chapter 20), the piezoelectric thin films and acoustic resonators behind the filters that Book 4 assumes (Chapter 21), and the compact models and process design kits through which all of these devices reach a simulator (Chapter 22). Book 7 surveys the 2026 MEMS products built on the same films.

Book 4 takes the transistor's small-signal model and the GaAs, GaN, and InP devices of Chapter 7 into matching networks, transmission lines, and antennas. Book 5 uses the converter and sample-and-hold limits of Book 2 and the noise of Chapter 14 to build modems and software-defined radios. Book 4 uses the load line of Chapter 7, the noise of Chapter 14, and the power-device limits of Chapter 19 for the transmitter chain, the PLL, and the receiver noise cascade. Book 7 treats the ESD and latch-up of Chapter 17 as system-level immunity and test, together with emissions. Book 6 takes the power MOSFET, superjunction, SiC, GaN, and IGBT of Chapter 19 into converters, magnetics, and the grid. Book 7 picks up the chip of Part V at the package: interposers, high-bandwidth memory built from the DRAM cells of Chapter 16, the 875 A of a modern accelerator, and MEMS.

Two habits travel with the reader. First, every quantity in this book was computed from a declared hand value; when a vendor model or a process design kit gives a different number, the vendor number wins for that part. Second, below about 20 nm the PDK outranks every textbook, including this one.

# Appendix D. Sources for Chapter 7

The prose above is this book's. The shelf it was checked against is short, and it is not a substitute for a foundry kit.

Cripps, RF Power Amplifiers for Wireless Communications, and Advanced Techniques in RF Power Amplifier Design. The load line is technology-agnostic. GaN adds trapping and memory.

Grebennikov, RF and Microwave Transmitter Design, and the switch-mode RF power-amplifier book with Sokal and Franco.

Quay, Gallium Nitride Electronics. Mishra and Rajan, and Morkoç, for the nitride heterostructure. Sze for the vocabulary.

Vendelin, Pavio, and Rohde for GaAs-era linear and nonlinear microwave circuits. Gonzalez and Maas remain the amplifier and mixer references.

InP devices: Rodwell's HBT and HEMT papers, and the millimeter-wave MMIC papers from JPL and Fraunhofer IAF. Foundry kits from Teledyne, WIN, Northrop Grumman, and Fraunhofer matter more than a textbook.

GaN models: Wolfspeed and Qorvo application notes. Power-conversion GaN, which Chapter 7 only points at: Erickson and Maksimović, plus the vendor power-loop note.

Tietze, Schenk, and Gamm, 16th edition (2019), remains the silicon circuit check for the bipolar and MOS chapters of Parts II and III.

# Appendix E. Sources for Parts II to VII

Tietze, Schenk, and Gamm, 16th edition (2019), bipolar and amplifier chapters.

Sedra and Smith; Gray and Meyer (or Gray, Hurst, Lewis, and Meyer) for analog bipolar and IC.

Taur and Ning, Fundamentals of Modern VLSI Devices, 3rd. Veendrick, Nanometer CMOS ICs. Saha on FinFET. The PDK for anything you will actually tape out.

Part IV: Gray, Hurst, Lewis, and Meyer, Analysis and Design of Analog Integrated Circuits; Razavi, Design of Analog CMOS Integrated Circuits; Johns and Martin, Analog Integrated Circuit Design. Pelgrom, Duinmaijer, and Welbers, "Matching Properties of MOS Transistors" (IEEE JSSC, 1989). Motchenbacher and Connelly, Low-Noise Electronic System Design, for Chapter 14.

Part V: Rabaey, Chandrakasan, and Nikolić, Digital Integrated Circuits; Weste and Harris, CMOS VLSI Design. Jacob, Ng, and Wang, Memory Systems, for Chapter 16. Amerasekera and Duvvury, ESD in Silicon Integrated Circuits; the JEDEC/ESDA JS-001 (HBM), JS-002 (CDM), and JESD78 (latch-up) standards; Black's 1969 electromigration papers.

Part VI: Sze and Ng, Physics of Semiconductor Devices, for recombination, photodetectors, and solar cells; Schubert, Light-Emitting Diodes; Baliga, Fundamentals of Power Semiconductor Devices, for Chapter 19.

Part VII: Plummer, Deal, and Griffin, Silicon VLSI Technology, for Chapter 20. Lakin, "Thin Film Resonator Technology" (IEEE UFFC, 2005); Akiyama et al., "Enhancement of Piezoelectric Response in Scandium Aluminum Nitride Alloy Thin Films" (Advanced Materials, 2009); Hashimoto, RF Bulk Acoustic Wave Filters for Communications, for Chapter 21. Chauhan et al., FinFET Modeling for IC Simulation and Design: Using the BSIM-CMG Standard; Gummel and Poon, "An Integral Charge Control Model of Bipolar Transistors" (Bell System Technical Journal, 1970), for Chapter 22.