Transceivers and High-Power RF

Book 6 of the Electrical Engineering Series

Lothar J. Musiol

Copyright © 2026 Lothar J. Musiol. All rights reserved. Kindle edition.

Power Amplifiers, Load Lines, Doherty, Receiver Noise, the PLL, and Radar

# How to Use This Book

This book owns the watts. It follows a transmitter from the device to the antenna: the device families, the load line, the amplifier classes, efficiency under modulated signals and the Doherty amplifier, and then the heat sink, combiners, filters, and feed line that stand between the transistor and the radiated power. It then turns to the receiving half of the radio, its noise figure and dynamic range, and to the oscillator and phase-locked loop that tune both halves. The last chapter, radar, brings transmitter and receiver together.

Each chapter states its question, develops the idea in words, and works it through with declared numbers. Every number was computed, and the answers to every practice problem are in Appendix B. The reader should know the matching and S-parameter material of Book 4 and the modulation and EVM material of Book 5.

The series gives each subject one home. Device physics, including why gallium nitride withstands high voltage, is Book 3's. Antenna gain, the Friis path factor, and matching networks are Book 4's. The waveform, OFDM, and the band plan are Book 5's. Emission masks, test chambers, and certification are Book 7's. Power conversion, including pulse modulators for tube transmitters, is Book 8's. This book's anchor numbers are the load-line pair for 100 W, Ropt = 10.13 Ω at 50 V with a 5 V knee and 3.13 Ω at 28 V with a 3 V knee, and the rule that 1 dB of feed loss leaves 0.794 of the power. The texts in Appendix C go deeper; this book's explanations, examples, and problems are its own.

# Contents

Chapter 1. The Transmitter Chain and Its Devices

Chapter 2. The Load Line

Chapter 3. Amplifier Classes and Efficiency

Chapter 4. Linearity, Back-Off, and the Doherty Amplifier

Chapter 5. Heat, Combining, and the Path to the Antenna

Chapter 6. Receiver Noise and Dynamic Range

Chapter 7. Oscillators, Phase Noise, and the PLL

Chapter 8. Radar

Appendix A. Constants and Formulas Used in This Book

Appendix B. Answers to the Practice Problems

Appendix C. Sources

# Chapter 1. The Transmitter Chain and Its Devices

Book 5 built the waveform. This book makes it powerful enough to cross a room, a city, or the distance to a satellite, and then receives it again when it arrives weak and noisy. The transmitter is a chain, and every link in it either delivers watts or loses them: the device, the load line it is designed to, the amplifier class, the output match, the bias, the heat sink, the combiner, the isolator, the harmonic filter, the feed line, the duplexer, and the antenna, whose output is the effective isotropic radiated power that the regulator limits. The receiver half of a radio, the low-noise amplifier, the noise cascade, and the oscillator that tunes both halves sit in this book as well, because they share the same box and the same engineering. Why gallium nitride withstands the voltage is Book 3's question, antenna gain is Book 4's, and the test chamber is Book 7's. This book owns the watts.

## 1.1 Power in decibels

Radio engineers count power in dBm, decibels relative to one milliwatt: P(dBm) = 10 log₁₀(P/1 mW). One watt is 30 dBm, 100 W is 50 dBm, and 1 kW is 60 dBm. Gains and losses in decibels add. The effective isotropic radiated power is the transmitter output, minus the loss between the amplifier and the antenna, plus the antenna gain in dBi. A loss of 1 dB is a factor of 10^(−0.1) = 0.794 in power: a fifth of the watts, gone as heat in a cable.

EIRP (dBm) = Pout (dBm) − Lfeed (dB) + Gant (dBi)

## Worked Example 1.1

Problem. A 100 W amplifier feeds a 17 dBi sector antenna through 1 dB of cable and connectors. Find the power at the antenna and the EIRP in dBm and watts.

Solution. 100 W is 50 dBm. After 1 dB the power is 49 dBm, or 100 × 0.794 = 79.4 W. The EIRP is 49 + 17 = 66 dBm, which is 10^(3.6) mW = 3981 W, about 4 kW. The 20.6 W lost in the feed heats the cable and buys nothing; one decibel saved after the last amplifier is worth exactly as much as one decibel of extra amplifier output, and it is almost always cheaper.

Key idea. One decibel after the last amplifier is one decibel of EIRP. Every loss between the device and the antenna must be paid for by a bigger, hotter, more expensive amplifier.

## 1.2 The device families

Five technologies cover nearly all power amplifiers in 2026. Silicon LDMOS, the laterally diffused MOS transistor, dominates from HF to about 3 GHz: it is cheap, rugged, and runs from 28 or 50 V supplies, and it powers broadcast transmitters, industrial heating, and most sub-3 GHz cellular base stations built before gallium nitride took over. Gallium nitride on silicon carbide, GaN-on-SiC, covers roughly 0.5 to 40 GHz. Its wide bandgap lets it run at 28 to 50 V with power densities several times those of gallium arsenide per millimeter of gate, and its silicon-carbide substrate conducts heat well; it is the default for new base stations, radar, and satellite links. Gallium arsenide, as heterojunction bipolar transistors and pHEMTs, supplies drivers and the few-watt amplifiers in phones and Wi-Fi. Indium phosphide offers the highest transistor speed and is used for sub-terahertz work and low noise, not for kilowatts. Vacuum tubes, the traveling-wave tube, klystron, and magnetron, remain where kilowatts to megawatts are needed at high frequency, because combining enough transistors becomes more expensive than one tube.

## 1.3 Supply voltage and power

A transistor's output power is set by how much voltage and current it can swing. The voltage is limited by breakdown, which sets the supply, and the current by the device's size. Chapter 2 makes this precise with the load line, but the consequence is already clear: at a given power, a higher supply means a lower current and a higher optimum load resistance, which is closer to the 50 Ω of the outside world and easier to match. That is the main reason high-voltage gallium nitride displaced gallium arsenide in base stations, and why a phone amplifier running from a 3.4 V battery must transform its load by a large ratio, losing efficiency in the process.

## Worked Example 1.2

Problem. A radar transmitter needs 2 kW peak at the antenna through 0.6 dB of loss. Its GaN modules each deliver 250 W at the combiner input, and the combiner loses 0.4 dB. How many modules are needed?

Solution. The total loss is 1.0 dB, a factor of 0.794, so the modules together must supply 2000/0.794 = 2518 W. At 250 W each, that is 10.07 modules, so eleven, or in a binary combiner tree, sixteen. A tree of eight would deliver only 8 × 250 × 0.794 = 1589 W. The arithmetic shows why combining loss limits the solid-state route: every doubling of the module count adds a combining stage and its loss.

## Practice

1. Convert 5 W, 250 W, and 2 kW to dBm.

2. A 20 W amplifier feeds a 3 dB cable run to a 12 dBi antenna. Find the EIRP in dBm and the power lost in the cable.

3. What fraction of the power is left after 2 dB of loss?

4. Which device family would you choose for a 200 W amplifier at 3.5 GHz, and why?

5. An amplifier loses 0.5 dB in its output isolator and 0.5 dB in its harmonic filter. What output power does it need to deliver 40 W after both?

6. Why does a higher supply voltage make the output match easier?

# Chapter 2. The Load Line

A small-signal amplifier is matched for gain: its load is chosen so that the most power is transferred, which means a conjugate match to the device's output impedance. A power amplifier is matched for power, and that is a different load. The device can swing its voltage only between a knee, below which its current collapses, and a maximum set by the supply and breakdown, and it can swing its current only between zero and a maximum. The load that lets it use the whole of both ranges at once is the optimum load, Ropt, and finding it is the first step of every power-amplifier design. The method is due to Steve Cripps, whose RF Power Amplifiers for Wireless Communications remains the reference.

## 2.1 The optimum load

With a supply VDD and a knee voltage Vknee, the drain voltage can swing by VDD − Vknee on either side of the supply, so the peak fundamental voltage is VDD − Vknee. With a sinusoidal current, the fundamental output power is the product of the peak voltage and peak current divided by two. For a target output power P, the optimum load resistance follows directly, and it is a resistance at the intrinsic drain, behind the device's output capacitance and package parasitics, which the matching network must also absorb.

Ropt = (VDD − Vknee)² / (2P)

## Worked Example 2.1

Problem. Find Ropt for a 100 W amplifier on a 50 V GaN process with a 5 V knee, and on a 28 V LDMOS process with a 3 V knee. Find the ratio of 50 Ω to each, and the peak fundamental current in each case.

Solution. At 50 V, Ropt = 45²/200 = 10.125 Ω, which this book writes as 10.13 Ω. At 28 V, Ropt = 25²/200 = 3.125 Ω, written 3.13 Ω. The transformation ratios to 50 Ω are 4.9 and 16. The peak fundamental currents are 2P/(VDD − Vknee) = 200/45 = 4.44 A and 200/25 = 8.0 A. The 28 V design needs nearly twice the current and so a larger device, with more output capacitance to tune out, and a matching network with three times the impedance ratio.

## 2.2 Why the conjugate match is wrong

The device's output resistance, the slope of its current against drain voltage, is typically hundreds of ohms for a large transistor. A conjugate match would present that resistance as the load, and the voltage swing would hit the knee and the supply long before the current reached its maximum: the device would deliver a small fraction of the power it could. The load-line match accepts a lower gain in exchange for using the full current and voltage range. The difference between the two, often 2 to 3 dB of output power, is the reason power amplifiers are designed by load-pull measurements, which sweep the load and map power and efficiency contours, rather than by small-signal S-parameters.

Key idea. A power amplifier is matched for power, not gain. The optimum load is set by the voltage and current the device can swing, and at a given power it scales with the square of the voltage.

## 2.3 The matching network

The output network must transform 50 Ω down to Ropt and absorb the device's output capacitance. For a single L-section, Book 4's result gives Q = √(Rhigh/Rlow − 1). For the 50 V design, Q = √(4.94 − 1) = 1.98; for the 28 V design, Q = √(16 − 1) = 3.87. A higher Q means a narrower bandwidth, larger circulating currents, and more loss in the network's finite-Q components. Wideband or high-ratio designs use two or more sections, each with a lower ratio, or a transmission-line transformer. In a phone the problem is worse: at 3.4 V with a 0.3 V knee, 1 W needs Ropt = 3.1²/2 = 4.8 Ω, and 2 W needs 2.4 Ω, a 21:1 ratio.

## Worked Example 2.2

Problem. A 200 W design on the 50 V process uses the same 5 V knee. Find Ropt, the single-section Q to 50 Ω, and compare it with the 100 W design.

Solution. Ropt = 45²/400 = 5.06 Ω. The ratio is 50/5.06 = 9.9, so Q = √(9.9 − 1) = 2.98. Doubling the power at the same voltage halves Ropt and raises the single-section Q from 1.98 to 2.98. To get back to the 100 W impedance level, the designer would combine two 100 W devices, Chapter 5's subject, or move to a higher supply.

## 2.4 Raising the rail

The scaling Ropt ∝ V²/P is the main argument of high-voltage design. Doubling the supply at constant power quadruples the optimum load and halves the current. Gallium nitride's breakdown field, about ten times that of silicon, permits 48 to 50 V supplies at microwave frequencies, and some designs use 65 V or more. The cost is that the knee voltage, the drain-source capacitance, and the trapping effects that cause memory in the amplifier's response all change with voltage, and the designer must characterize them at the operating point.

## Practice

1. Find Ropt for 200 W at 50 V with a 5 V knee, and for 50 W at 28 V with a 3 V knee.

2. Find the peak fundamental drain current for 100 W at 50 V with a 5 V knee.

3. Find the single-section L-network Q to transform 50 Ω to 3.13 Ω.

4. A phone amplifier at 3.4 V has a 0.3 V knee. Find Ropt for 1 W.

5. By what factor does Ropt change when the supply is doubled at constant power and negligible knee?

6. Why is a power amplifier's output not conjugately matched?

# Chapter 3. Amplifier Classes and Efficiency

Chapter 2 fixed the load. This chapter fixes the bias, and with it the fraction of each RF cycle during which the transistor conducts. That choice, named by the amplifier's class, decides the efficiency: how much of the DC supply power becomes RF and how much becomes heat. In a base station that heat is the operator's largest electricity bill after air conditioning; in a phone it is battery life. The classes fall into two families: the current-source classes A, AB, B, and C, which trade linearity for efficiency by conducting for less of the cycle, and the switching and harmonic-tuned classes D, E, and F, which shape the voltage and current waveforms so that they do not overlap.

## 3.1 Efficiency measures

Drain efficiency is the RF output power divided by the DC supply power. Power-added efficiency subtracts the input drive, which matters when the gain is low, as it is at millimeter wave. The power dissipated in the device is the DC input plus the RF input minus the RF output, and it is the number the heat sink of Chapter 5 must handle.

ηD = Pout / PDC,    PAE = (Pout − Pin) / PDC,    Pdiss = PDC + Pin − Pout

## Worked Example 3.1

Problem. A 100 W amplifier has 15 dB of gain and 65% drain efficiency. Find the input power, DC power, PAE, and dissipation.

Solution. The gain is a factor of 31.6, so Pin = 100/31.6 = 3.16 W. PDC = 100/0.65 = 153.8 W. PAE = (100 − 3.16)/153.8 = 62.9%. The dissipation is 153.8 + 3.16 − 100 = 57.0 W. At 12 dB of gain and 70% drain efficiency, a 40 W millimeter-wave stage would have PAE = (40 − 2.52)/57.1 = 65.6%, already four and a half points below its drain efficiency; at lower gains the gap grows quickly.

## 3.2 Conduction angle: classes A, AB, B, and C

In class A the transistor is biased at half its maximum current and conducts for the whole cycle. Its peak theoretical efficiency is 50%, and it is the most linear. Lowering the bias reduces the conduction angle, the part of the cycle over which current flows. In class B, biased at the threshold, the device conducts for half the cycle, 180°, and the theoretical peak efficiency is π/4 = 78.5%. Class AB lies between. In class C the angle is less than 180° and the efficiency rises further, but the fundamental output falls and the gain falls with it. For a conduction angle α, with an ideal linear device and a harmonic short at the output, the peak efficiency is given below.

η(α) = (α − sin α) / [4 (sin(α/2) − (α/2) cos(α/2))]

The formula gives 50.0% for class A at 360°, 60.2% at 270°, 78.5% for class B at 180°, 89.7% at 120°, and 94.0% at 90°. The fundamental output power, relative to class A with the same peak current and voltage, is +0.55 dB at 270°, 0 dB at 180°, −2.1 dB at 120°, and −4.2 dB at 90°. Deep class C therefore buys efficiency with power and gain, and is used for constant-envelope signals such as FM broadcast, where linearity does not matter. Most linear amplifiers run in class AB, a little above the threshold, to keep the gain flat at low drive.

## Worked Example 3.2

Problem. A device biased for a 120° conduction angle delivers its maximum output with a drain efficiency at the theoretical limit. If the same device in class B delivers 100 W, what does it deliver in this class C condition, and how much dissipation is saved?

Solution. At 120° the output is 2.14 dB below class B, so 100 × 10^(−0.214) = 61.1 W. In class B the DC power is 100/0.785 = 127.3 W and the dissipation 27.3 W. At 120°, the DC power is 61.1/0.897 = 68.1 W and the dissipation 7.0 W. Efficiency rose from 78.5% to 89.7%, but the device now delivers 39% less power: class C is a trade, not a free gain.

Key idea. Efficiency comes from keeping voltage and current from being large at the same time. Reducing the conduction angle does it at a cost in power and gain; shaping the waveforms with harmonics or switching does it without that cost, in principle reaching 100%.

## 3.3 Switching and harmonic-tuned classes

If the transistor acts as a switch, it has either zero voltage across it or zero current through it, and an ideal switch dissipates nothing. Class D uses two switches and a filter; it is common at audio and low radio frequencies. Class E uses one switch with a shunt capacitor and a tuned load designed so that the voltage returns to zero, with zero slope, at the instant the switch closes; this removes the loss of discharging the capacitor and makes class E practical into the gigahertz range. Class F adds open circuits at the odd harmonics and short circuits at the even ones, so the drain voltage approaches a square wave and the current a half-sine; inverse class F swaps them. Real devices have finite on-resistance, output capacitance, and transition times, and practical class E and F amplifiers reach 70 to 85% drain efficiency at a few gigahertz. All of these are efficient only at full drive. Chapter 4 addresses what happens when the signal's envelope varies.

## Practice

1. An amplifier delivers 50 W from 75 W of DC with 1 W of drive. Find its drain efficiency, PAE, and dissipation.

2. Find the theoretical efficiency at a conduction angle of 270°.

3. Why does class C reduce gain?

4. A class AB amplifier runs at 60% drain efficiency at 200 W output. Find the dissipation, neglecting the input.

5. A 30 dBm stage has 8 dB of gain and 50% drain efficiency. Find its PAE.

6. What condition at switch closing defines class E?

# Chapter 4. Linearity, Back-Off, and the Doherty Amplifier

Chapter 3's efficiencies are peak values, reached only when the amplifier is driven to its full output. The signals of Book 5 are not like that. An OFDM signal has a peak-to-average power ratio of 8 to 11 dB, so a linear amplifier that must pass its peaks spends most of its time at a fraction of its rated power, where its efficiency is poor. This chapter measures the linearity an amplifier needs, computes how much efficiency back-off costs, and shows how the Doherty amplifier, digital predistortion, and envelope tracking recover it. The Doherty is the default architecture of cellular base stations in 2026.

## 4.1 Compression and intermodulation

As drive rises, a real amplifier's gain begins to fall. The 1 dB compression point, P1dB, is the output power at which the gain is 1 dB below its small-signal value. With two closely spaced tones at the input, the amplifier's curvature generates third-order intermodulation products at 2f1 − f2 and 2f2 − f1, which fall inside or next to the wanted band and cannot be filtered out. Their power rises 3 dB for every 1 dB of input. Extrapolating the fundamental and third-order lines to their intersection defines the output third-order intercept, OIP3, a figure of merit that is typically about 10 dB above P1dB for a well-behaved amplifier.

OIP3 = Pout,tone + ΔIM3 / 2,  where ΔIM3 is the carrier-to-IM3 ratio in dB

## Worked Example 4.1

Problem. With two tones of 30 dBm each at the output, the third-order products are 40 dB below each tone. Find OIP3, and the IM3 ratio after backing the drive off by 3 dB.

Solution. OIP3 = 30 + 40/2 = 50 dBm. At 27 dBm per tone, the tones have fallen 3 dB and the products 9 dB, so the ratio becomes 40 + 6 = 46 dBc. Each decibel of back-off buys two decibels of intermodulation ratio, which is why the first remedy for a failing spectral mask, a subject for Book 7's chamber, is often less drive, at a cost in efficiency.

## 4.2 What back-off costs

A class A amplifier draws constant DC power, so its efficiency falls in proportion to output power: at 8 dB below peak it is 50% × 10^(−0.8) = 7.9%. A class B amplifier's DC current falls with the signal amplitude, so its efficiency falls only with the square root of output power: η = (π/4)√(Pout/Pmax), giving 55.6% at 3 dB back-off, 39.4% at 6 dB, and 31.3% at 8 dB. For a 100 W-peak class B stage carrying a signal with 8 dB of PAPR, the average output is 15.8 W and the DC input 50.7 W.

## 4.3 The Doherty amplifier

William Doherty's 1936 design uses two amplifiers. The main, or carrier, amplifier runs in class AB or B and handles the signal up to a set back-off; the auxiliary, or peaking, amplifier runs in class C and turns on only for the peaks. A quarter-wave line between them acts as an impedance inverter. At low power the main amplifier sees twice its optimum load, so it saturates and reaches peak efficiency at half its full voltage swing, which is 6 dB below the combined peak. Above that, the peaking amplifier's current pulls the main amplifier's load back down toward Ropt, a mechanism called load modulation, and both reach full power together. In the ideal symmetric Doherty the efficiency is 78.5% both at peak and at 6 dB back-off, dips only to about 70% between, and falls linearly with amplitude below 6 dB.

Below 6 dB back-off:  η = (π/4) (v / 0.5),  v = normalized output amplitude

## Worked Example 4.2

Problem. A 100 W-peak amplifier carries a signal with 8 dB of PAPR. Compare the efficiency and DC power at the average output for class B and for an ideal symmetric Doherty.

Solution. At 8 dB back-off the normalized amplitude is v = 10^(−8/20) = 0.398. Class B gives (π/4)(0.398) = 31.3%; the Doherty gives (π/4)(0.398/0.5) = 62.5%. For the 15.8 W average output, the class B stage draws 50.7 W and the Doherty 25.3 W, halving the DC power and cutting the dissipation from 34.8 W to 9.5 W. Real Doherty amplifiers reach 45 to 55% average efficiency on such signals, below the ideal because the peaking device turns on gradually and the inverter has loss and limited bandwidth. Asymmetric versions, with a larger peaking device, place the second efficiency peak at 8 to 10 dB to match the signal.

Key idea. A linear amplifier's average efficiency is set by the signal's peak-to-average ratio. The Doherty restores efficiency at back-off by modulating the main amplifier's load, and digital predistortion then restores the linearity the Doherty gives up.

## 4.4 Predistortion and envelope tracking

A Doherty amplifier is efficient but not linear: its gain and phase change with drive, the AM-to-AM and AM-to-PM distortion, and they depend on the recent history of the signal through heating and charge trapping, which are memory effects. Digital predistortion measures the amplifier's output through a feedback receiver, fits a model, often a memory polynomial, and pre-distorts the baseband signal with the inverse, so that the cascade is linear. It typically improves the adjacent-channel leakage ratio by 15 to 20 dB. Envelope tracking takes a different route: it varies the supply voltage with the signal's envelope, so the amplifier always runs close to saturation. It is widely used in phones, where a fast DC-DC converter can follow the envelope of a 100 MHz signal, and less in base stations, where Doherty is simpler at high power.

## Practice

1. Two tones of 20 dBm each produce IM3 products at −46 dBc. Find OIP3.

2. For the amplifier of the previous problem, what IM3 ratio results when each tone is reduced to 17 dBm?

3. Find a class B amplifier's efficiency at 10 dB back-off.

4. Find an ideal symmetric Doherty's efficiency at 10 dB back-off.

5. A 200 W-peak Doherty carries a signal with 9 dB of PAPR at 55% average efficiency. Find the average output and the DC power.

6. Why does an amplifier's IM3 ratio improve by 2 dB for each decibel of back-off?

# Chapter 5. Heat, Combining, and the Path to the Antenna

An amplifier that meets its power and efficiency targets on the bench can still fail in service, by overheating, by a single failed device taking down a combined array, or by losing its gains in the hardware between the device and the antenna. This chapter follows the power out of the transistor: through the thermal path that removes its waste heat, the combiners that add several devices together, the isolator and harmonic filter that protect and clean it, and the feed line that carries it to the antenna.

## 5.1 The thermal path

Heat flows from the transistor's channel, or junction, through the die and package to the flange, then through an interface material to the heat sink and into the air or coolant. Each step has a thermal resistance θ in °C/W, and the temperature rises add like voltage drops in a series circuit. The junction temperature sets the device's lifetime: GaN-on-SiC parts are typically rated for channel temperatures up to about 225 °C, and designers usually hold them well below 200 °C to keep the predicted lifetime long. Pulsed operation, as in radar, lets the peak temperature stay below the steady-state value, and transient thermal impedance curves give the peak rise for a given pulse width and duty cycle.

Tj = Tambient + Pdiss (θjc + θcs + θsa)

## Worked Example 5.1

Problem. The 100 W amplifier of Worked Example 3.1 dissipates 57.0 W. Its θjc is 0.8 °C/W, the interface adds 0.1 °C/W, and the heat sink 0.3 °C/W to air at 40 °C. Find the junction temperature. What sink resistance would hold Tj to 200 °C?

Solution. The total resistance is 1.2 °C/W, so the junction rises 57.0 × 1.2 = 68.4 °C, to 108.4 °C. The flange sits at 40 + 57.0 × 0.4 = 62.8 °C. For Tj = 200 °C, the total may be (200 − 40)/57.0 = 2.81 °C/W, so the sink could be as poor as 2.81 − 0.9 = 1.91 °C/W. The design has ample margin; the margin is what allows the amplifier to survive a hot day, a blocked fan, or a mismatched antenna.

## 5.2 Combining

When one device cannot supply the power, several are combined. The Wilkinson divider and combiner, the 90° hybrid, and spatial or radial combiners all add N equal, in-phase signals into one port without loss in the ideal case, and each dissipates the difference in an isolation resistor or load when the inputs are unequal. The output of an ideal combiner with N inputs is the square of the sum of the input amplitudes, divided by N. A phase error θ between two equal inputs leaves a fraction cos²(θ/2) of the power: 99.2% at 10°, 97.0% at 20°, and 93.3% at 30°. A failed device is more costly: with one of N inputs dead, the output falls to (N − 1)²/N² of full power, which is 25% for N = 2, 56% for N = 4, and 77% for N = 8. The rest is dissipated in the isolation loads, which must be rated for it.

Pout = (Σ √Pi)² / N   (ideal N-way in-phase combiner)

## Worked Example 5.2

Problem. Four 100 W modules feed a combiner with 0.3 dB of loss. Find the output with all four working and with one failed.

Solution. With all four, the output is 400 × 10^(−0.03) = 373 W. With one failed, the ideal output is (3√100)²/4 = 225 W, 56% of 400 W, and after the combiner loss 210 W. The isolation loads absorb 75 W, and must be rated for at least that continuously. The combined transmitter loses 2.5 dB, not the 1.25 dB that a quarter of the hardware might suggest.

Key idea. Combining multiplies power but also adds loss and creates a failure mode. A combiner's output falls by more than the failed fraction, because the surviving inputs no longer add in amplitude.

## 5.3 Isolator, harmonic filter, and duplexer

A ferrite isolator passes power forward and diverts reflected power into a load, so the amplifier always sees close to its design impedance even when the antenna is damaged or iced. A 2:1 VSWR at the antenna reflects 11.1% of the power; without an isolator, that reflection shifts the load seen by the device and can push its voltage beyond breakdown. The harmonic filter, a lowpass after the amplifier, attenuates the second and third harmonics, which a class AB stage generates at 20 to 30 dB below the carrier and which emission limits, the subject of Book 7, typically require at 60 dB or more below. A duplexer in a frequency-division system adds the receive filter, so the transmitter and receiver can share one antenna. Each element typically costs 0.2 to 0.5 dB.

## 5.4 The feed line

The last loss is the line to the antenna, which can be tens of meters on a tower. A cable with 6.6 dB of loss per 100 m at 2 GHz loses 1.98 dB in 30 m, leaving 63% of the power. Mounting the amplifier at the top of the mast, as in a remote radio head or an active antenna unit, removes that loss, and the same choice on the receive side improves the noise figure, as Chapter 6 shows. That is why modern base stations put the radio beside the antenna and carry the signal down the mast on optical fiber.

## Worked Example 5.3

Problem. A transmitter delivers 100 W through a 0.3 dB isolator, a 0.5 dB harmonic filter and duplexer, and 1.98 dB of cable. Find the power at the antenna. How much is recovered by mounting the radio at the antenna, keeping only 0.2 dB of jumper?

Solution. The losses total 2.78 dB, leaving 100 × 10^(−0.278) = 52.7 W. With the radio at the antenna, the loss is 1.0 dB, leaving 79.4 W. Moving the radio recovers 26.7 W, more than a quarter of the amplifier's output, with no change to the amplifier.

## Practice

1. A device dissipates 30 W with a total thermal resistance of 2.5 °C/W in 45 °C air. Find Tj.

2. Two equal amplifiers combine with a 15° phase error. What fraction of the power is delivered?

3. An 8-way combiner loses one input. By how many decibels does the output fall?

4. What fraction of the power is reflected at a VSWR of 1.5?

5. Find the loss of 45 m of cable rated at 6.6 dB per 100 m, and the power left from 40 W.

6. Why must a combiner's isolation resistors be rated for more than the normal imbalance power?

# Chapter 6. Receiver Noise and Dynamic Range

A transmitter's job ends at the antenna; a receiver's begins there, with a signal that may be a billionth of a billionth of a watt. Whether that signal can be recovered depends on how much noise the receiver adds to it, and whether the receiver survives the strong signals that arrive alongside. This chapter defines the noise figure and noise temperature, derives the cascade formula that shows why the first amplifier matters most, and sets the receiver's dynamic range between its noise floor and its intercept point. Book 5 used the resulting sensitivity; this is where it comes from. The receiver belongs in this book because it shares the radio, the antenna, the duplexer, and often the heat with the transmitter.

## 6.1 Noise figure and noise temperature

A resistor at temperature T delivers available noise power kTB into a matched load, which at the reference temperature T0 = 290 K is −174 dBm in each hertz. A two-port's noise factor F is the ratio of the input SNR to the output SNR when the source is at T0; the noise figure is NF = 10 log₁₀F. Equivalently, the two-port adds noise as if its input were connected to an extra resistor at the effective noise temperature Te = T0(F − 1). A 3 dB noise figure corresponds to 288.6 K, almost exactly T0; a 1 dB noise figure to 75.1 K; and 0.5 dB to 35.4 K. Noise temperature is the more natural measure for low-noise systems such as satellite ground stations and radio telescopes, where the sky itself may be at only a few tens of kelvin.

Te = T0 (F − 1),   T0 = 290 K,   kT0 = −174 dBm/Hz

## 6.2 The cascade

In a chain of stages, each stage's noise is divided by the total gain ahead of it when referred to the input. The result, Friis's formula for noise factor, shows that once the first stage has enough gain, the later stages hardly matter, and that any loss ahead of the first amplifier adds directly to the noise figure. A passive loss of L dB at T0 has a noise figure of L dB.

F = F1 + (F2 − 1)/G1 + (F3 − 1)/(G1G2) + …

## Worked Example 6.1

Problem. A receiver has an LNA with NF 1 dB and gain 15 dB, a filter with 2 dB loss, a mixer with NF 10 dB and gain 8 dB, and an IF amplifier with NF 4 dB and gain 20 dB. Find the cascade noise figure. Repeat with the filter ahead of the LNA.

Solution. In linear terms the LNA contributes F1 = 1.259. The filter contributes (1.585 − 1)/31.6 = 0.018, the mixer (10 − 1)/(31.6 × 0.631) = 0.451, and the IF amplifier 0.0012. The total is 1.740, a noise figure of 2.41 dB. With the filter first, its 2 dB adds directly and the LNA's gain no longer shields the mixer as well, giving 3.91 dB. The filter-first arrangement costs 1.5 dB of sensitivity, and it is used anyway when strong out-of-band signals would otherwise overload the LNA: the trade between noise and survival is the central decision of receiver design.

## Worked Example 6.2

Problem. The same receiver sits at the bottom of a mast with 3 dB of cable between the antenna and the LNA. Find the noise figure, and the noise figure with the LNA moved to the top of the mast.

Solution. With the cable first, its 3 dB adds to the cascade: 5.41 dB. With the LNA at the top, the cable follows the LNA's 15 dB of gain and the cascade is 3.52 dB. The tower-mounted amplifier recovers 1.9 dB. The transmit side of Chapter 5 made the same argument for power; the remote radio head serves both.

Key idea. The first stage sets the noise figure. Loss ahead of the first amplifier adds decibel for decibel, and gain in the first stage hides the noise of everything after it.

## 6.3 Sensitivity and dynamic range

The noise floor of a receiver is −174 dBm/Hz + 10 log₁₀B + NF. Sensitivity adds the SNR the demodulator needs, as in Book 5. The upper end of the receiver's range is set by distortion: two strong signals generate third-order products, Chapter 4's mechanism, and they become a problem when they rise to the noise floor. The spurious-free dynamic range is the range of input levels over which the products remain below the floor. Two-thirds appears because the products rise 3 dB per decibel while the signals rise one.

SFDR = (2/3) (IIP3 − Nfloor)

## Worked Example 6.3

Problem. The receiver of Worked Example 6.1, with a 2.41 dB noise figure, uses a 1 MHz channel and has an input third-order intercept of −10 dBm. Find the noise floor and the spurious-free dynamic range.

Solution. The floor is −174 + 60 + 2.41 = −111.6 dBm. SFDR = (2/3)(−10 + 111.6) = 67.7 dB. Raising the LNA gain lowers the noise figure but lowers the cascade's IIP3 too, because every later stage sees bigger signals; the designer sets the gain to balance the two. A phone receiver faces a further limit: its own transmitter, a few tens of megahertz away, leaks through the duplexer and can desensitize the receiver, which is why duplexer isolation of 50 dB or more is standard.

## 6.4 Blocking and reciprocal mixing

A strong signal outside the channel can also harm reception through the local oscillator. The oscillator's phase noise, Chapter 7's subject, spreads the oscillator's power around its nominal frequency, and a strong blocker mixes with that skirt into the wanted channel. This reciprocal mixing sets a floor that no amount of low-noise gain can remove: the blocker power, plus the phase noise in dBc/Hz at the blocker's offset, plus 10 log₁₀B.

## Practice

1. Find the noise temperature corresponding to a 2 dB noise figure.

2. A two-stage chain has an LNA with NF 0.8 dB and gain 20 dB, then a stage with NF 12 dB. Find the cascade noise figure.

3. Find the noise floor of a 20 MHz receiver with a 2 dB noise figure.

4. Find the SFDR for the receiver of the previous problem with an IIP3 of 0 dBm.

5. A blocker at −30 dBm meets oscillator phase noise of −120 dBc/Hz at its offset. Find the reciprocal-mixing noise in a 1 MHz channel.

6. Why does a filter placed ahead of the LNA raise the noise figure by its full loss?

# Chapter 7. Oscillators, Phase Noise, and the PLL

Every radio needs a frequency reference: a local oscillator that moves the signal up to the carrier in the transmitter and back down in the receiver. Its frequency must be exact and adjustable in fine steps, and its spectrum must be clean, because any noise on it is copied onto every signal it mixes with. Real oscillators spread their power into a skirt of phase noise around the carrier. This chapter quantifies that skirt, shows how a phase-locked loop combines a clean but fixed reference with a tunable but noisy oscillator, and connects the result to the EVM and reciprocal-mixing limits of Book 5 and Chapter 6. The loop's mathematics is Book 2's feedback, but the PLL lives here, because its specifications come from the radio.

## 7.1 Phase noise

An oscillator's phase noise is written L(Δf), the noise power in a 1 Hz band at an offset Δf from the carrier, relative to the carrier power, in dBc/Hz. Leeson's model explains its shape. The resonator, with loaded quality factor QL, filters the amplifier's noise; within the resonator's half-bandwidth f0/(2QL), the feedback turns amplitude noise into phase noise that rises as 1/Δf², and closer in, flicker noise steepens it to 1/Δf³. Far from the carrier the noise is flat. The model shows the two levers: a higher QL, and a higher signal power Ps in the resonator.

L(Δf) ≈ 10 log₁₀{ (2FkT/Ps) [1 + (f0 / (2QL Δf))²] }   (without the flicker term)

## Worked Example 7.1

Problem. A 2.4 GHz oscillator has QL = 20, a resonator power of 1 mW, and an amplifier noise factor of 10. Find its phase noise at 1 MHz and 100 kHz offsets, and at 1 MHz if QL falls to 10.

Solution. 2FkT/Ps = 2 × 10 × 4.0 × 10⁻²¹/10⁻³ = 8.0 × 10⁻¹⁷, which is −161 dBc/Hz. At 1 MHz, f0/(2QLΔf) = 60, and the bracket is 3601, adding 35.6 dB: L = −125.4 dBc/Hz. At 100 kHz the factor is 600, adding 20 dB more: −105.4 dBc/Hz. Halving QL to 10 raises the noise at 1 MHz by 6 dB, to −119.4 dBc/Hz. The integrated LC resonators of a CMOS chip have QL of about 10 to 20, which is why on-chip oscillators are noisier than discrete ones.

## 7.2 The phase-locked loop

A PLL divides the oscillator output by N, compares its phase with a crystal reference at fref, and steers the voltage-controlled oscillator so that the two stay in phase; the output is then N × fref. Inside the loop bandwidth, the output follows the reference, and the reference's phase noise appears multiplied by N, which is 20 log₁₀N in decibels. Outside the loop bandwidth, the VCO runs free and its own noise appears. The designer chooses the loop bandwidth near the offset where the two contributions cross. An integer-N synthesizer steps in multiples of fref, so fine steps need a low reference and therefore a large N. A fractional-N synthesizer dithers N between integers with a delta-sigma modulator, allowing a high reference, a small N, and fine steps; the dithering's quantization noise is shaped to high offsets, where the loop filters it.

fout = N fref,    in-band noise = Lref + 20 log₁₀N

## Worked Example 7.2

Problem. A 2.4 GHz synthesizer must step in 1 MHz increments. The reference, referred to the phase detector, contributes −150 dBc/Hz. Compare the in-band phase noise of an integer-N design with a 1 MHz reference and a fractional-N design with a 40 MHz reference.

Solution. The integer-N design needs N = 2400, so the in-band noise is −150 + 20 log₁₀2400 = −150 + 67.6 = −82.4 dBc/Hz. The fractional-N design has N ≈ 60, adding 35.6 dB, for −114.4 dBc/Hz, 32 dB better. Fractional-N synthesis is the reason modern radios can have both fine channel steps and clean in-band noise.

Key idea. A PLL copies the reference inside its bandwidth, multiplied by N, and leaves the VCO's own noise outside it. Keep N small, the reference clean, and the loop bandwidth where the two noise curves cross.

## 7.3 Settling

A PLL must also settle quickly when the channel changes. For a second-order loop with natural frequency ωn and damping ζ, the frequency error after a step decays roughly as e^(−ζωnt), so the time to settle to a fraction ε of the step is about −ln(ε√(1 − ζ²))/(ζωn). With fn = 100 kHz and ζ = 0.707, settling to one part per million of the step takes 31.9 µs. A wider loop settles faster and suppresses more VCO noise, but passes more reference and delta-sigma noise, and the loop must remain stable with margin: Book 2's phase-margin analysis applies directly.

## 7.4 From phase noise to EVM and jitter

Integrating the phase noise over the offsets that matter gives the oscillator's rms phase error σφ = √(2∫L(f)df), in radians, with L in linear units. The factor 2 counts both sidebands. The phase error rotates every constellation point, and for small errors it acts as an EVM of σφ. In a clock, the same quantity divided by 2πf0 is the rms jitter, which sets the converter limit of Book 5.

## Worked Example 7.3

Problem. An oscillator at 2.4 GHz has flat phase noise of −110 dBc/Hz from 10 kHz to 10 MHz offset. Find its rms phase error, the EVM floor it sets, and its rms jitter.

Solution. The integral is 2 × 10⁻¹¹ × (10⁷ − 10⁴) = 2.0 × 10⁻⁴ rad², so σφ = 0.0141 rad, or 0.81°. As an EVM floor that is 1.41%, or −37.0 dB, just short of the −38 dB that Book 5 found for Wi-Fi 7's 4096-QAM, so this oscillator would limit that mode before any other impairment. The jitter is 0.0141/(2π × 2.4 × 10⁹) = 0.94 ps rms.

## Practice

1. By how much does the in-band reference noise rise in a PLL with N = 145?

2. In Leeson's model, how much does doubling the resonator power change the far-out phase noise?

3. An oscillator has flat phase noise of −100 dBc/Hz from 1 kHz to 1 MHz at 1 GHz. Find σφ in degrees and the rms jitter.

4. A synthesizer with a 40 MHz reference must produce 5.8 GHz. Find N and the in-band noise if the reference contributes −150 dBc/Hz.

5. Find the approximate settling time of a loop with fn = 50 kHz and ζ = 0.707 to one part per million.

6. Why does a small N improve in-band phase noise?

# Chapter 8. Radar

Radar is the application in which this book's two halves meet most directly. A transmitter sends a pulse of hundreds of watts to megawatts, a target reflects a tiny fraction of it, and a receiver with the lowest possible noise must detect the echo. Because the signal travels out and back, the received power falls as the fourth power of range, not the square, and that one fact shapes everything else: the high peak powers, the long pulses compressed after reception, and the integration of many echoes. This chapter derives the radar equation, computes range, resolution, and Doppler, and finishes with the 77 GHz FMCW radar of a modern car. Antenna gain is taken from Book 4; if the last stage is a tube, its pulse modulator is Book 8's.

## 8.1 The radar equation

A transmitter of power Pt with antenna gain G produces a power density of PtG/(4πR²) at range R. A target of radar cross-section σ intercepts and reradiates σ times that density, which spreads again over 4πR² on the way back, and the receiving antenna, of effective area Gλ²/(4π), collects part of it. Multiplying gives the monostatic radar equation, with one antenna for both directions.

Pr = Pt G² λ² σ / [(4π)³ R⁴]

## Worked Example 8.1

Problem. A 10 GHz radar transmits 1 kW peak through a 30 dBi antenna at a 1 m² target 10 km away. Its receiver has a 1 MHz bandwidth and a 3 dB noise figure, and detection needs 13 dB of SNR. Find the received power and the maximum range.

Solution. λ = 3.00 cm. Pr = 10³ × 10⁶ × (0.0300)² × 1/[(4π)³ × 10¹⁶] = 4.53 × 10⁻¹⁴ W, or −103.4 dBm. The noise floor is −174 + 60 + 3 = −111 dBm, so the required power is −98 dBm. The echo at 10 km is 5.4 dB too weak; since Pr ∝ R⁻⁴, the maximum range is 10 km × 10^(−5.4/40) = 7.31 km. Doubling the transmitter power would extend it only by 2^(1/4) = 1.19, to 8.69 km.

Key idea. Radar's echo falls as 1/R⁴. Doubling the range takes sixteen times the power, so radar designers buy range with antenna gain, pulse energy, and integration rather than with peak watts alone.

## 8.2 Pulses, energy, and resolution

Detection depends on the energy of the pulse, not its peak power: a matched filter's output SNR is the received pulse energy divided by kTF, as in Book 5. The range resolution, the smallest separation at which two targets appear distinct, is ΔR = c/(2B), where B is the pulse bandwidth; for a simple pulse of length τ, B ≈ 1/τ. A long pulse has more energy but poor resolution. Pulse compression removes the conflict: the transmitter sends a long pulse with a wide bandwidth, usually a linear frequency chirp, and the receiver's matched filter compresses it into a short one. The compression ratio is the time-bandwidth product Bτ.

ΔR = c / (2B),    Runamb = c × PRI / 2,    duty = τ / PRI

## Worked Example 8.2

Problem. The radar of Example 8.1 uses a 1 µs pulse at a pulse repetition interval of 1 ms. Find its range resolution, unambiguous range, duty cycle, and average power. It is changed to a 10 µs pulse chirped over 10 MHz at the same peak power. Find the new resolution, the compression ratio, and the new maximum range.

Solution. With 1 µs, B ≈ 1 MHz and ΔR = 150 m. The unambiguous range is c × 1 ms/2 = 150 km, the duty cycle 0.1%, and the average power 1 W. The chirped pulse has ΔR = c/(2 × 10 MHz) = 15 m and Bτ = 100, or 20 dB of compression. Its energy is ten times larger, so its SNR rises by 10 dB, and the maximum range becomes 7.31 × 10^(10/40) = 13.0 km, while the resolution improves tenfold. The average power rises to 10 W at 1% duty cycle, which the transmitter's thermal design of Chapter 5 must accept.

## 8.3 Doppler

A target moving with radial velocity v shifts the echo by fd = 2v/λ, twice the one-way shift because both paths shorten. Pulse-Doppler radars measure that shift across a train of pulses to separate moving targets from stationary clutter. At 10 GHz, a target closing at 100 m/s shifts the echo by 6.67 kHz.

fd = 2v / λ

## 8.4 The 77 GHz automotive radar

Automotive radars use the 76 to 81 GHz band and a frequency-modulated continuous wave instead of pulses. The transmitter sweeps linearly over a bandwidth B in a time Tc, a slope S = B/Tc. The echo from a target at range R arrives 2R/c later, so mixing it with the current transmit signal produces a beat frequency fb = 2RS/c, which an FFT turns into a range profile. The resolution is again c/(2B): a 4 GHz sweep resolves 3.75 cm, enough to separate a pedestrian from a nearby car. A sequence of chirps gives the Doppler, and an array of receive antennas, Book 4's subject, gives the angle. The whole radar is a single CMOS or SiGe chip with a few milliwatts per transmit channel; the R⁴ law is overcome by the short ranges and by integrating hundreds of chirps.

## Worked Example 8.3

Problem. A 77 GHz FMCW radar sweeps 1 GHz in 50 µs. Find the slope, the beat frequency for a target at 50 m, the range resolution, and the Doppler shift for a closing speed of 30 m/s.

Solution. S = 10⁹/(50 × 10⁻⁶) = 2 × 10¹³ Hz/s. fb = 2 × 50 × 2 × 10¹³/2.998 × 10⁸ = 6.67 MHz, within reach of an inexpensive converter. ΔR = c/(2 × 1 GHz) = 15.0 cm. λ = 3.89 mm, so fd = 2 × 30/0.00389 = 15.4 kHz.

## Practice

1. By what factor must the transmitter power rise to double a radar's range, all else equal?

2. Find the range resolution of a 150 MHz chirp.

3. Find the unambiguous range at a pulse repetition interval of 500 µs.

4. A radar transmits 1 µs pulses of 1 kW peak every 1 ms. Find its average power.

5. Find the Doppler shift at 77 GHz for a closing speed of 30 m/s.

6. An FMCW radar sweeps 4 GHz in 40 µs. Find the beat frequency for a target at 100 m.

# Appendix A. Constants and Formulas Used in This Book

dBm = 10 log₁₀(P/1 mW); 1 W = 30 dBm; 1 dB of loss leaves 0.794 of the power. EIRP = Pout − Lfeed + Gant.

Load line: Ropt = (VDD − Vknee)²/(2P). 100 W: 10.13 Ω at 50 V/5 V knee; 3.13 Ω at 28 V/3 V knee.

Efficiency: ηD = Pout/PDC; PAE = (Pout − Pin)/PDC. Class A 50%; class B π/4 = 78.5%; η(α) = (α − sin α)/[4(sin(α/2) − (α/2)cos(α/2))].

Back-off: class A η ∝ Pout; class B η = (π/4)√(Pout/Pmax); ideal symmetric Doherty 78.5% at peak and at 6 dB back-off.

OIP3 = Ptone + ΔIM3/2. SFDR = (2/3)(IIP3 − Nfloor).

Thermal: Tj = Tamb + Pdiss Σθ. Ideal N-way combiner: Pout = (Σ√Pi)²/N; one of N failed leaves (N − 1)²/N².

Noise: kT0 = −174 dBm/Hz at 290 K; Te = T0(F − 1); F = F1 + (F2 − 1)/G1 + (F3 − 1)/(G1G2) + ….

PLL: fout = N fref; in-band noise = Lref + 20 log₁₀N. Leeson: L(Δf) ≈ 10 log₁₀{(2FkT/Ps)[1 + (f0/(2QLΔf))²]}. σφ = √(2∫L df); jitter = σφ/(2πf0).

Radar: Pr = PtG²λ²σ/[(4π)³R⁴]; ΔR = c/(2B); Runamb = c·PRI/2; fd = 2v/λ; FMCW fb = 2RS/c.

# Appendix B. Answers to the Practice Problems

## Chapter 1

1. 37.0 dBm, 54.0 dBm, and 63.0 dBm.

2. 43.0 − 3 + 12 = 52.0 dBm; the cable delivers 10.0 W and loses 10.0 W.

3. 10^(−0.2) = 0.631.

4. GaN-on-SiC: it operates well above 3 GHz at 28 to 50 V with high power density and good heat removal; LDMOS is near its upper frequency limit there.

5. 1 dB in total, so 40/0.794 = 50.4 W.

6. At a given power, a higher voltage swing means a smaller current and a higher optimum load resistance, closer to 50 Ω, so the matching network needs a smaller transformation ratio, a lower Q, and less loss.

## Chapter 2

1. 45²/400 = 5.06 Ω; 25²/100 = 6.25 Ω.

2. 200/45 = 4.44 A.

3. Q = √(50/3.125 − 1) = 3.87.

4. 3.1²/2 = 4.8 Ω.

5. By four.

6. A conjugate match would present the device's high output resistance as the load, so the voltage would limit at the knee and supply long before the current reached its maximum, delivering much less power than the load-line match.

## Chapter 3

1. ηD = 50/75 = 66.7%; PAE = 49/75 = 65.3%; Pdiss = 75 + 1 − 50 = 26 W.

2. 60.2%.

3. The device conducts for only part of the cycle, so for a given drive amplitude the fundamental output current is smaller; reaching full current needs a larger input swing, and the gain falls.

4. PDC = 333.3 W, so 133.3 W is dissipated.

5. Pout = 1 W, Pin = 0.158 W, PDC = 2 W; PAE = (1 − 0.158)/2 = 42.1%.

6. The switch voltage is zero, and its slope is zero, at the instant the switch closes, so no charge is dumped from the shunt capacitor.

## Chapter 4

1. 20 + 46/2 = 43 dBm.

2. 46 + 2 × 3 = 52 dBc.

3. (π/4) × 10^(−0.5) = 24.8%.

4. (π/4) × 0.316/0.5 = 49.7%.

5. Average 200 × 10^(−0.9) = 25.2 W; DC 25.2/0.55 = 45.8 W.

6. The tones fall 1 dB per decibel of input while third-order products fall 3 dB, so their ratio grows by 2 dB.

## Chapter 5

1. 45 + 30 × 2.5 = 120 °C.

2. cos²(7.5°) = 98.3%.

3. (7/8)² = 0.766, a fall of 1.16 dB.

4. |Γ| = 0.5/2.5 = 0.2, so 4.0%.

5. 2.97 dB, leaving 40 × 10^(−0.297) = 20.2 W.

6. When a device fails, the isolation loads absorb a large part of the surviving devices' power, which is far more than the small imbalance of normal operation.

## Chapter 6

1. 290 × (1.585 − 1) = 170 K.

2. F = 1.202 + (15.85 − 1)/100 = 1.351, or 1.31 dB.

3. −174 + 73.0 + 2 = −99.0 dBm.

4. (2/3)(0 + 99.0) = 66.0 dB.

5. −30 − 120 + 60 = −90 dBm.

6. A passive loss at T0 attenuates the signal but delivers the same thermal noise power kT0B, so the SNR falls by the loss, and nothing before it provides gain to hide it.

## Chapter 7

1. 20 log₁₀145 = 43.2 dB.

2. It falls by 3 dB, since the noise term is divided by Ps.

3. σφ = √(2 × 10⁻¹⁰ × 999,000) = 0.0141 rad = 0.81°; jitter = 0.0141/(2π × 10⁹) = 2.25 ps.

4. N = 145; −150 + 43.2 = −106.8 dBc/Hz.

5. Twice the 100 kHz value: 63.8 µs.

6. The divider makes the loop multiply the reference's phase deviations by N, so the reference's noise appears at the output raised by 20 log₁₀N; a smaller N multiplies it less.

## Chapter 8

1. 2⁴ = 16.

2. c/(2 × 150 MHz) = 1.0 m.

3. c × 500 µs/2 = 75 km.

4. 1 kW × 0.001 = 1 W.

5. 15.4 kHz.

6. S = 10¹⁴ Hz/s, so fb = 2 × 100 × 10¹⁴/2.998 × 10⁸ = 66.7 MHz.

# Appendix C. Sources

Steve C. Cripps, RF Power Amplifiers for Wireless Communications, 2nd edition, and Advanced Techniques in RF Power Amplifier Design, for the load line, classes, and Doherty (Chapters 2 to 4).

Andrei Grebennikov, Nathan O. Sokal, and Marc J. Franco, Switchmode RF and Microwave Power Amplifiers, for classes E and F.

Peter B. Kenington, High-Linearity RF Amplifier Design, for intermodulation and predistortion.

Kevin McClaning, Wireless Receiver Design for Digital Communications, and Behzad Razavi, RF Microelectronics, 2nd edition, for noise cascades, dynamic range, and oscillators (Chapters 6 and 7).

Floyd M. Gardner, Phaselock Techniques, 3rd edition, and Roland E. Best, Phase-Locked Loops, for the PLL; D. B. Leeson, "A Simple Model of Feedback Oscillator Noise Spectrum," Proceedings of the IEEE, 1966.

Merrill I. Skolnik, Introduction to Radar Systems, 3rd edition, and Mark A. Richards, Fundamentals of Radar Signal Processing, for Chapter 8.

W. H. Doherty, "A New High Efficiency Power Amplifier for Modulated Waves," Proceedings of the IRE, 1936.