RF, Microwave, and Transceivers

Book 4 of the Electrical Engineering Series

Lothar J. Musiol

Copyright © 2026 Lothar J. Musiol. All rights reserved. Kindle edition.

Lines, Matching, Filters, Antennas, Power Amplifiers, Receivers, and Synthesizers, from 1 GHz to Millimeter Wave

# How to Use This Book

Books 1 to 3 built circuits on the assumption that a wire has no length. This book drops that assumption and follows a radio from the copper to the air. Part I, Chapters 1 to 19, is the passive structure: the transmission line and reflection, lumped and distributed matching and the Smith chart, scattering parameters, resonators, filters and waveguide, the antenna and the array, couplers and dividers, microstrip and stripline, broadband matching, cavities, small antennas and horns, propagation, ferrites and circulators, antenna measurement, noise in passive networks, a filter carried from specification to layout, and RF switches.

Part II, Chapters 20 to 40, is the active radio. It follows a transmitter from the device to the antenna, through the load line, the amplifier classes, the Doherty amplifier, heat and combining, and then the receiver's noise and dynamic range, the oscillator and the PLL, and radar. Chapters 28 to 40 deepen the chain: the low-noise amplifier, mixers and frequency planning, the link budget, bias and ruggedness, vacuum tubes, RF exposure, transceiver isolation, class E and F design, envelope tracking and outphasing, amplifier measurement, digital predistortion, frequency synthesis, and power detection and control.

Each chapter states the question it answers, develops the picture in words, and then works it through with declared numbers. Every number in a worked example was computed, not estimated, and the answers to every practice problem are in Appendix B. The book uses one set of reference values throughout: the speed of light is 2.998 × 10⁸ m/s, the system reference impedance is 50 Ω, and the reference board has εr = 3.55. The link-budget anchor is the Friis path factor at 2.45 GHz over 100 m between isotropic antennas, 9.48 × 10⁻⁹, or −80.2 dB, and the load-line anchors for 100 W are Ropt = 10.13 Ω at 50 V with a 5 V knee and 3.13 Ω at 28 V with a 3 V knee.

The series gives each subject one home. Device physics, including why gallium nitride withstands high voltage, is Book 3's. The waveform, OFDM, and the band plan are Book 5's. Power conversion, including pulse modulators for tube transmitters, is Book 6's. Laminate properties at 10 GHz, packages, emission limits, test chambers, and certification are Book 7's. Readers who want the next level of depth should turn to the texts in Appendix C; this book's explanations, examples, and problems are its own.

# Contents

Part I. Passive Structures: Lines, Matching, Filters, and Antennas

Chapter 1. When a Wire Stops Being a Wire

Chapter 2. Transmission Lines and Reflection

Chapter 3. Lumped Matching: The L-Network and Its Q

Chapter 4. Distributed Matching and the Smith Chart at Work

Chapter 5. Scattering Parameters

Chapter 6. Resonators, Filters, and Waveguide

Chapter 7. The Antenna

Chapter 8. Arrays, Beam Steering, and Millimeter Wave

Chapter 9. Couplers, Dividers, and Hybrids

Chapter 10. Microstrip and Stripline by Design

Chapter 11. Broadband Matching and the Bode-Fano Limit

Chapter 12. Cavities and Dielectric Resonators

Chapter 13. Small Antennas, Monopoles, Loops, and Horns

Chapter 14. Propagation Beyond Free Space

Chapter 15. Ferrites, Circulators, and Isolators

Chapter 16. Measuring Antennas: Ranges and Gain

Chapter 17. Noise in Passive Networks

Chapter 18. A Bandpass Filter from Specification to Layout

Chapter 19. RF Switches

Part II. Active Circuits: Amplifiers, Receivers, Oscillators, and Synthesizers

Chapter 20. The Transmitter Chain and Its Devices

Chapter 21. The Load Line

Chapter 22. Amplifier Classes and Efficiency

Chapter 23. Linearity, Back-Off, and the Doherty Amplifier

Chapter 24. Heat, Combining, and the Path to the Antenna

Chapter 25. Receiver Noise and Dynamic Range

Chapter 26. Oscillators, Phase Noise, and the PLL

Chapter 27. Radar

Chapter 28. The Low-Noise Amplifier

Chapter 29. Mixers and Frequency Planning

Chapter 30. The Link Budget

Chapter 31. Bias, Ruggedness, and Protection

Chapter 32. Vacuum Tubes: Klystrons, TWTs, and Magnetrons

Chapter 33. RF Exposure and Safety

Chapter 34. Transceiver Architectures and Isolation

Chapter 35. Designing Class E and Class F Amplifiers

Chapter 36. Envelope Tracking, Polar, and Outphasing

Chapter 37. Measuring a Power Amplifier

Chapter 38. Digital Predistortion

Chapter 39. Frequency Synthesis: Fractional-N and DDS

Chapter 40. Power Detection and Power Control

Appendix A. Constants and Formulas Used in This Book

Appendix B. Answers to the Practice Problems

Appendix C. Sources

# Part I. Passive Structures: Lines, Matching, Filters, and Antennas

# Chapter 1. When a Wire Stops Being a Wire

Book 1 solved circuits in which a wire has no length: the voltage at one end equals the voltage at the other, at every instant. That assumption is excellent as long as a signal's wavelength is much longer than the circuit. At radio frequencies it fails, and the failure is the whole subject of this book. A trace becomes a transmission line, a capacitor becomes an inductor, and a connection becomes a radiator. This chapter sets out where the boundary lies, fixes the units the rest of the book uses, and makes one claim that shapes the series: millimeter wave is not a separate subject. It is this subject at a shorter wavelength, with less forgiveness.

## 1.1 Wavelength sets the scale

A signal of frequency f travels at the speed of light in vacuum, c = 2.998 × 10⁸ m/s, and more slowly in a dielectric. In a line fully surrounded by a dielectric of relative permittivity εr, such as a stripline buried inside a circuit board, the speed falls by √εr, and so does the wavelength. A partially surrounded line, such as a microstrip on the surface of a board, uses an effective permittivity εeff between 1 and εr, which Chapter 2 calculates.

λ0 = c / f,    λg = λ0 / √εeff

## Worked Example 1.1

Problem. Find the free-space wavelength at 2.45 GHz, 28 GHz, and 77 GHz, and the guided wavelength at 2.45 GHz in a stripline on a laminate with εr = 3.55.

Solution. λ0 = 2.998 × 10⁸/2.45 × 10⁹ = 12.24 cm at 2.45 GHz, 10.7 mm at 28 GHz, and 3.89 mm at 77 GHz. In the stripline, λg = 12.24/√3.55 = 12.24/1.884 = 6.49 cm. A Wi-Fi signal's wavelength on the board is the size of a credit card; a 77 GHz automotive radar's wavelength in air is smaller than a fingernail.

A common rule of thumb treats a circuit as lumped when its largest dimension is below about one tenth of the guided wavelength. A 1 cm trace in the stripline above reaches that limit at the frequency where λg = 10 cm, which is 2.998 × 10⁸/(1.884 × 0.10) = 1.6 GHz. Below that, Book 1's circuit laws work with small corrections. Above it, the trace must be treated as a line with a characteristic impedance and a delay.

Key idea. A wire is a wire only when it is short compared with a wavelength. Shorter wavelength is not a new subject; it is the same subject with less forgiveness.

## 1.2 Decibels and dBm

Radio engineering adds and subtracts in decibels because gains and losses multiply along a chain and span many orders of magnitude. A power ratio in decibels is 10 log₁₀(P₂/P₁). Absolute power is expressed relative to one milliwatt: P(dBm) = 10 log₁₀(P/1 mW). In a 50 Ω system, 0 dBm is 0.224 V rms, and every 20 dB is a factor of ten in voltage. Antenna gain relative to an isotropic radiator is written dBi. A decibel figure is always a ratio; adding two dBm values is meaningless, while adding a dBm value and a dB gain gives a dBm result.

## Worked Example 1.2

Problem. A transmitter delivers 20 dBm. It passes through a cable with 3 dB of loss and an antenna with 6 dBi of gain. Find the power at the antenna input in milliwatts and the effective isotropic radiated power in dBm. Then express −100 dBm as a voltage across 50 Ω.

Solution. 20 dBm is 100 mW. After the cable, 20 − 3 = 17 dBm, which is 50 mW. The radiated power referred to an isotropic antenna is 17 + 6 = 23 dBm. For the received signal, −100 dBm is 10⁻¹³ W, and V = √(PR) = √(10⁻¹³ × 50) = 2.24 µV rms. A receiver works with microvolts while the transmitter beside it handles volts, a range of 120 dB that every layout decision must respect.

## 1.3 Parts that are not what they say

Every real component carries parasitic elements that a schematic leaves out, and at radio frequencies those parasitics decide what the part does. A capacitor in a small surface-mount package has an equivalent series inductance (ESL) of roughly 0.5 to 1 nH from its terminations and mounting. A resistor has a shunt capacitance across its body. An inductor has capacitance between its turns. Each part therefore has a self-resonant frequency (SRF), above which it behaves as its opposite.

fSRF = 1 / (2π √(ESL × C))

## Worked Example 1.3

Problem. A 10 pF capacitor in an 0603 package has 0.8 nH of ESL as mounted. Find its self-resonant frequency and its net reactance at 2.45 GHz.

Solution. fSRF = 1/(2π √(0.8 × 10⁻⁹ × 10 × 10⁻¹²)) = 1.78 GHz. At 2.45 GHz the capacitive reactance is 1/(2π × 2.45 GHz × 10 pF) = 6.50 Ω and the inductive reactance is 2π × 2.45 GHz × 0.8 nH = 12.32 Ω, so the net reactance is +5.82 Ω: the part is an inductor of about 0.38 nH. As a DC block that is harmless; in a matching network it is a different component from the one drawn. RF designers choose capacitors by their SRF, often using a value whose series resonance falls at the operating frequency, where the part is nearly a short circuit.

## 1.4 The plan of this book

Chapter 2 develops the transmission line, reflection, and standing waves. Chapters 3 and 4 match one impedance to another, first with lumped components and then with lines and the Smith chart, which Book 1 introduced and this book uses as a working tool. Chapter 5 describes networks by their scattering parameters, the language of every RF datasheet and network analyzer. Chapter 6 builds resonators, filters, and waveguide. Chapter 7 treats the antenna, and Chapter 8 the array and the millimeter-wave link. Chapters 9 to 19 extend the toolkit: couplers, dividers, and hybrids; microstrip and stripline design; broadband matching and the Bode-Fano limit; cavities and dielectric resonators; small antennas, monopoles, loops, and horns; propagation beyond free space; ferrites, circulators, and isolators; antenna measurement; noise in passive networks; a bandpass filter carried from specification to layout; and RF switches. Part II, Chapters 20 to 40, turns to the active radio: amplifiers, receivers, oscillators, synthesizers, and the measurements that prove them. Transmitter power, power-amplifier design, and receiver noise belong to Part II; laminate properties at 10 GHz, the package, and emissions limits belong to Book 7. This book supplies the passive structure that connects them, and it owns the antenna gains that every link budget in the series uses.

## Practice

1. Find the free-space and stripline wavelengths at 5.8 GHz for εr = 3.55.

2. At what frequency does a 2 cm trace in the stripline of Worked Example 1.1 reach one tenth of a guided wavelength?

3. Convert 27 dBm to milliwatts and −90 dBm to microvolts rms in 50 Ω.

4. A 100 pF capacitor has 0.6 nH of ESL. Find its self-resonant frequency.

5. A signal of −30 dBm passes a 20 dB amplifier, a 2 dB filter, and a 6 dB attenuator. Find the output power.

6. Explain in one or two sentences why a capacitor is sometimes chosen so that its self-resonance falls at the operating frequency.

# Chapter 2. Transmission Lines and Reflection

A transmission line is two conductors with a fixed geometry along their length: a coaxial cable, a pair of traces, a trace over a ground plane. A signal launched into it travels as a wave, and the ratio of voltage to current in that wave is fixed by the geometry, not by what is connected at the far end. When the wave arrives at a load that does not have the same ratio, part of it reflects. Everything in RF design, from matching to filters to antennas, is a way of controlling those reflections, and this chapter gives the four or five formulas that describe them.

## 2.1 Characteristic impedance and velocity

A short piece of line has series inductance L and shunt capacitance C per unit length. A wave on a lossless line sees the ratio of voltage to current set by those two quantities alone, and travels at a speed set by their product.

Z0 = √(L / C),    vp = 1 / √(L C) = c / √εeff

## Worked Example 2.1

Problem. A 50 Ω coaxial cable has a velocity factor of 0.66. Find its capacitance and inductance per meter, and the delay of a 2 m length.

Solution. vp = 0.66 × 2.998 × 10⁸ = 1.98 × 10⁸ m/s. From Z0 = √(L/C) and vp = 1/√(LC), C = 1/(Z0 vp) = 1/(50 × 1.98 × 10⁸) = 101 pF/m and L = Z0/vp = 253 nH/m. A 2 m length delays the signal by 2/1.98 × 10⁸ = 10.1 ns. The same 2 m of cable at DC is 202 pF of capacitance, which is how an oscilloscope probe cable looks to a slow circuit; at 2.45 GHz it is twenty wavelengths of line.

A microstrip, a trace of width W on a dielectric of thickness h over a ground plane, is the standard line on a circuit board. Part of its field is in the dielectric and part in the air above, so its effective permittivity lies between 1 and εr. Closed-form approximations by Hammerstad and Jensen give Z0 and εeff to about 1%, and field solvers do better. On a 0.508 mm laminate with εr = 3.55, a 50 Ω microstrip is about 1.14 mm wide and has εeff ≈ 2.79, so its guided wavelength at 2.45 GHz is 12.24/√2.79 = 7.33 cm, longer than the 6.49 cm of the stripline in Chapter 1 because some of its field travels in air.

## 2.2 Reflection coefficient, VSWR, and return loss

At a load ZL, the reflected wave's voltage divided by the incident wave's voltage is the reflection coefficient Γ. It is a complex number of magnitude between 0 (a perfect match) and 1 (an open, a short, or a pure reactance). The incident and reflected waves add along the line into a standing wave whose maximum-to-minimum ratio is the voltage standing-wave ratio (VSWR). Three further figures express the same information: return loss, the reflected power below the incident power; mismatch loss, the power that fails to reach the load; and the transmitted fraction 1 − |Γ|².

Γ = (ZL − Z0) / (ZL + Z0),    VSWR = (1 + |Γ|) / (1 − |Γ|)

RL = −20 log₁₀ |Γ| dB,    ML = −10 log₁₀ (1 − |Γ|²) dB

## Worked Example 2.2

Problem. A 50 Ω line is terminated in 100 Ω. Find Γ, the VSWR, the return loss, and the mismatch loss.

Solution. Γ = (100 − 50)/(100 + 50) = 1/3. VSWR = (4/3)/(2/3) = 2.0. RL = −20 log₁₀(1/3) = 9.54 dB. ML = −10 log₁₀(1 − 1/9) = 0.51 dB. A VSWR of 2 sounds alarming but costs only half a decibel of delivered power; what it more often costs is the behavior of whatever drives the line, because an amplifier designed for 50 Ω sees a load that moves around the Smith chart with cable length and frequency.

Key idea. A reflection depends only on the ratio of load to line impedance. A line of the right impedance carries any amount of power to a matched load without reflection, and a mismatch at either end reflects part of every wave that reaches it.

## 2.3 What a line does to an impedance

Looking into a line of length l terminated in ZL, the impedance is not ZL. The reflected wave returns with a phase delay of 2βl, where β = 2π/λg, and the input impedance rotates accordingly. Three special cases are worth remembering. A half-wavelength line repeats its load. A quarter-wavelength line inverts it, Zin = Z0²/ZL, which is the basis of the quarter-wave transformer of Chapter 4. A short-circuited line shorter than a quarter wavelength is an inductor, and an open-circuited one a capacitor.

Zin = Z0 (ZL + j Z0 tan βl) / (Z0 + j ZL tan βl)

## Worked Example 2.3

Problem. Find the input impedance of a 50 Ω line one tenth of a wavelength long terminated in 100 Ω. Then find the inductance that a short-circuited 50 Ω stub one eighth of a wavelength long represents at 2.45 GHz.

Solution. βl = 36°, tan βl = 0.727. Zin = 50 (100 + j36.3)/(50 + j72.7) = 49.1 − j35.0 Ω. Only 0.1λ of line has turned a resistive 100 Ω load into an impedance with half the resistance and a large capacitive reactance, which is why a load must be specified at a reference plane. For the stub, βl = 45° and Zin = jZ0 tan 45° = j50 Ω, an inductance of 50/(2π × 2.45 × 10⁹) = 3.25 nH, realized with no wire-wound part at all.

## 2.4 Loss on the line

Real lines lose power in the conductors, through skin effect, and in the dielectric, through its loss tangent. Loss is quoted in decibels per meter and rises with frequency: conductor loss roughly as √f and dielectric loss as f. The skin-depth numbers belong to Book 6 and the laminate loss tangents at 10 GHz to Book 7; this book uses their consequences. One consequence surprises people: a lossy cable improves the apparent match of whatever is at its far end, because the reflected wave is attenuated twice.

## Worked Example 2.4

Problem. An antenna with a 9.54 dB return loss (VSWR 2.0) is fed through a cable with 3 dB of loss. What return loss does the transmitter measure, and what fraction of the transmitter's power is radiated, ignoring antenna efficiency?

Solution. The reflection passes the cable twice, so the measured return loss is 9.54 + 2 × 3 = 15.5 dB, which looks like a VSWR of 1.4. The power is worse than it looks: half is lost in the cable on the way out, and of the half that arrives, 1 − 1/9 = 89% enters the antenna, so the antenna receives 44% of the transmitter's power. Measuring the match at the transmitter end of a long cable flatters the antenna; the measurement belongs at the antenna's terminals.

## Practice

1. A 75 Ω cable has a velocity factor of 0.85. Find its capacitance and inductance per meter.

2. A 50 Ω line is terminated in 75 Ω. Find Γ, VSWR, return loss, and mismatch loss.

3. A 50 Ω line is terminated in 25 Ω. Find Γ and the VSWR.

4. Find the input impedance of a quarter-wave 50 Ω line terminated in 200 Ω.

5. An open-circuited 50 Ω stub is one eighth of a wavelength long at 2.45 GHz. What capacitance does it represent?

6. Explain in one or two sentences why a load impedance must always be quoted with its reference plane at RF.

# Chapter 3. Lumped Matching: The L-Network and Its Q

A source delivers its maximum power to a load whose impedance is the complex conjugate of its own. Most loads are not: a transistor's output wants a few ohms, an antenna offers whatever its geometry gives, and a cable is 50 Ω. A matching network transforms one impedance into another without dissipating power, which means it is built only from reactances. This chapter builds the simplest one, the two-element L-network, shows that its bandwidth is fixed by the ratio it must transform, and then adds a third element to trade bandwidth for control.

## 3.1 The L-network

An L-network has one series and one shunt reactance. The shunt element always sits across the larger resistance. Its job is to convert that large resistance, in parallel with a reactance, into an equivalent series combination whose resistance equals the smaller one; the series element then cancels the leftover reactance. Working through the conversion gives a single number that fixes both reactances: the network's Q.

Q = √(Rhigh / Rlow − 1),    Xseries = Q × Rlow,    Xshunt = Rhigh / Q

Each reactance may be an inductor or a capacitor, provided the two are of opposite type. A series inductor with a shunt capacitor is a low-pass network, which also attenuates harmonics, the usual choice at an amplifier output. A series capacitor with a shunt inductor is high-pass, which blocks DC and can carry bias through the inductor.

## Worked Example 3.1

Problem. Design a low-pass L-network that matches a 10 Ω load to a 50 Ω source at 2.45 GHz. Then find the frequencies between which the return loss stays above 10 dB.

Solution. Q = √(50/10 − 1) = 2.0. The series reactance next to the 10 Ω load is 2.0 × 10 = 20 Ω, an inductor of 20/(2π × 2.45 × 10⁹) = 1.30 nH. The shunt reactance across the 50 Ω side is 50/2.0 = 25 Ω, a capacitor of 1/(2π × 2.45 × 10⁹ × 25) = 2.60 pF. At 2.45 GHz the network presents exactly 50 Ω. A frequency sweep of the input reflection shows the return loss stays above 10 dB from 1.94 to 2.87 GHz, a bandwidth of 38% of the center frequency.

Key idea. An L-network's Q is set entirely by the impedance ratio it must transform. A large ratio forces a high Q, and a high Q means a narrow bandwidth and large circulating currents in the components.

## 3.2 Three elements: pi and T networks

When the bandwidth of an L-network is too wide, which matters when the network must also filter, or the component values it demands are impractical, a third element provides a free choice of Q. A pi network is two back-to-back L-networks that transform both terminations down to a smaller virtual resistance Rv in the middle; a T network transforms them up to a larger one. The designer chooses the Q of the higher-resistance side, which fixes Rv, and then designs each half as an L-network.

Pi network: Rv = Rhigh / (Q² + 1),    Q₂ = √(Rlow / Rv − 1)

## Worked Example 3.2

Problem. Design a pi network that matches 10 Ω to 50 Ω at 2.45 GHz with a Q of 5 on the 50 Ω side, using shunt capacitors and a series inductor. Compare its bandwidth with that of Worked Example 3.1.

Solution. Rv = 50/(5² + 1) = 1.92 Ω. The 50 Ω half has a shunt reactance of 50/5 = 10 Ω (6.50 pF) and a series reactance of 5 × 1.92 = 9.62 Ω. The 10 Ω half has Q₂ = √(10/1.92 − 1) = 2.05, a series reactance of 2.05 × 1.92 = 3.94 Ω, and a shunt reactance of 10/2.05 = 4.88 Ω (13.3 pF). The two series inductors combine into 9.62 + 3.94 = 13.56 Ω, which is 0.88 nH. A sweep gives a 10 dB return-loss bandwidth from 2.31 to 2.57 GHz, about 10%, against 38% for the L-network. The narrower match is also a better harmonic filter. The 0.88 nH inductor is no larger than the inductance of a component pad and its vias, so at this frequency it is built as a short length of narrow line, not as a part.

## 3.3 Real components and loss

The matching network's own losses come from the finite Q of its components, which Chapter 1 defined through their parasitics. An inductor with quality factor QL = X/R has a series resistance X/QL. In a network that transforms down to a low resistance, that resistance sits in series with the low-resistance side, where a fraction of an ohm is a significant fraction of the load. The loss rises with the network Q, which is the second reason, after bandwidth, to keep the transformation ratio of each section modest. Large ratios are often split into two cascaded L-networks, each with a Q near the square root of the single-section value.

## Worked Example 3.3

Problem. The 1.30 nH inductor of Worked Example 3.1 has QL = 40 at 2.45 GHz. Estimate the loss it adds.

Solution. Its series resistance is 20 Ω/40 = 0.50 Ω, in series with the 10 Ω load. The fraction of the power delivered to the network that reaches the load is 10/10.5 = 0.952, a loss of 0.21 dB. For a power amplifier delivering 10 W into the network, 0.48 W heats the inductor. With a load of 2 Ω instead of 10 Ω, the same inductor Q would cost about 1 dB.

## 3.4 Matching complex loads

A load with a reactive part is matched by first absorbing or canceling its reactance. A series reactance in the load can be absorbed into the network's series element; a shunt reactance into the shunt element. If the load is 10 + j5 Ω and the L-network calls for a series reactance of +j20 Ω, the network supplies only j15 Ω, because the load already contains j5 Ω. When the load's reactance is larger than the network needs, a component of the opposite sign cancels the excess. The Smith chart of the next chapter makes these steps visual, and it is how most designers carry them out.

## Practice

1. Design a low-pass L-network to match 5 Ω to 50 Ω at 900 MHz. Give Q, the series inductance, and the shunt capacitance.

2. Design a high-pass L-network to match 200 Ω to 50 Ω. Find Q, and say which side the shunt element goes on.

3. For the pi network of Worked Example 3.2, find Rv if Q on the 50 Ω side is raised to 8.

4. The series inductor of Practice Problem 1 has QL = 30. Estimate the loss.

5. A load of 10 + j8 Ω is matched by the network of Worked Example 3.1. What series reactance must the network itself supply?

6. Explain in one or two sentences why a large impedance ratio is often matched with two cascaded L-networks.

# Chapter 4. Distributed Matching and the Smith Chart at Work

Above a few gigahertz the lumped components of Chapter 3 run out: the values become fractions of a nanohenry or a picofarad, comparable to the parasitics of the pads they sit on. The cure is to build the matching network from the transmission line itself. A quarter-wave section transforms resistance, a stub supplies reactance, and a length of line moves an impedance to wherever it is most convenient to correct. Book 1 introduced the Smith chart as a map of impedance; this chapter uses it as the drawing board on which such designs are made.

## 4.1 The quarter-wave transformer

Chapter 2 showed that a quarter-wavelength line inverts its load: Zin = Zq²/ZL. Choosing the section's impedance as the geometric mean of the two resistances makes the input equal the source impedance exactly.

Zq = √(Z0 RL),    length = λg / 4

## Worked Example 4.1

Problem. Match a 25 Ω load to a 50 Ω line at 2.45 GHz with a quarter-wave stripline section on εr = 3.55. Find its impedance and length, and the fractional bandwidth over which |Γ| stays below 0.1.

Solution. Zq = √(25 × 50) = 35.36 Ω. From Chapter 1, λg = 6.49 cm, so the section is 1.62 cm long. Away from the design frequency the section is no longer a quarter wave, and the reflection grows. For a single section, the bandwidth for a maximum reflection Γm is Δf/f0 = 2 − (4/π) arccos[(Γm/√(1 − Γm²)) × 2√(Z0RL)/|RL − Z0|]. With Γm = 0.1 this gives 0.367, so the match holds from about 2.00 to 2.90 GHz. Two sections in a binomial (maximally flat) design, of 42.0 Ω and 29.7 Ω, widen the same specification to 72%, at the cost of twice the length.

The quarter-wave transformer matches only resistances. A complex load is first moved along a line to a point where its impedance is purely resistive, which happens twice in every half wavelength, at the maximum and the minimum of the standing wave, and the transformer is placed there.

## 4.2 Reading the Smith chart as a tool

On the Smith chart, a point is a normalized impedance z = Z/Z0, and its distance from the center is |Γ|. Three moves account for nearly every design. Moving along a lossless line toward the generator rotates the point clockwise around the center at constant |Γ|, a full turn for every half wavelength. Adding a series reactance moves the point along a circle of constant resistance. Adding a shunt susceptance moves it along a circle of constant conductance, which is easiest to see by switching to the admittance chart, the same chart rotated by 180°. A match is the center. A design is a path from the load to the center using only those moves.

Key idea. A line changes an impedance's angle, a series element moves it along a resistance circle, and a shunt element along a conductance circle. Any match is a sequence of those three moves that ends at the center of the chart.

## 4.3 The single-stub match

The most common distributed match uses one length of line and one shunt stub. Starting at the load, the line rotates the admittance until its real part equals 1/Z0, which on the admittance chart means reaching the unit-conductance circle. At that point the admittance is 1/Z0 + jB. A shunt stub, short-circuited or open-circuited, supplies the susceptance −jB and lands the point at the center. There are always two solutions within a half wavelength, one on each side of the chart, and the shorter combination is normally preferred because it has the wider bandwidth.

## Worked Example 4.2

Problem. A load of 100 + j50 Ω on a 50 Ω line is to be matched with a short-circuited shunt stub at 2.45 GHz in the stripline of Chapter 1. Find the reflection coefficient, the stub position, and the stub length.

Solution. Γ = (100 + j50 − 50)/(100 + j50 + 50) = 0.447 at an angle of 26.6°, a VSWR of 2.62. The line length d that brings the normalized admittance onto the unit-conductance circle satisfies tan βd = 3 or tan βd = −1. The first solution, βd = 71.6°, is d = 0.199λ, where the normalized admittance is y = 1 + j1. A short-circuited stub has normalized admittance −j cot βl, so −j1 requires cot βl = 1 and l = λ/8. In the stripline, d = 0.199 × 6.49 = 1.29 cm and l = 0.81 cm. The second solution is d = 0.375λ, where y = 1 − j1, matched by a 3λ/8 short-circuited stub or a λ/8 open-circuited one.

## 4.4 Why stubs are short, and when they are radial

A stub's reactance changes with frequency faster the longer it is, because its electrical length grows in proportion to frequency. Choosing the shortest stub and the shortest line keeps the match broad. Open stubs avoid the via a short needs, but their end radiates a little and adds fringing capacitance, so the physical length is trimmed shorter than the calculation. Where a stub must present a very low impedance over a wide band, as when it feeds DC bias to an amplifier while blocking the RF, a radial stub, a fan-shaped open stub, gives a broader short circuit than a straight one.

## Worked Example 4.3

Problem. A bias feed must present a high impedance to the RF at its connection to a 50 Ω line. It uses a quarter-wave high-impedance line ending in a capacitor that is a short circuit at 2.45 GHz. Explain why the arrangement works and find the line's length in the stripline of Chapter 1.

Solution. A quarter-wave line inverts its termination. A short circuit at the far end, provided by the bypass capacitor or a radial stub, becomes an open circuit at the near end, so the bias line does not load the RF path, while DC passes freely along it. The length is λg/4 = 1.62 cm. Using a narrow, high-impedance line keeps the impedance high across a wider band around the design frequency, and the inversion works only near the frequencies at which the line is an odd number of quarter wavelengths long.

## Practice

1. Find the impedance of a quarter-wave transformer matching 100 Ω to 50 Ω.

2. Find the single-section bandwidth for |Γ| < 0.1 when matching 100 Ω to 50 Ω.

3. A load is 25 − j25 Ω on a 50 Ω line. Show that a short-circuited shunt stub placed directly at the load can match it, and find the stub length.

4. On the Smith chart, how many degrees does a point rotate when it moves 0.1λ toward the generator?

5. A two-section binomial transformer matches 25 Ω to 50 Ω. Which section impedance sits next to the load?

6. Explain in one or two sentences why the shorter of the two single-stub solutions usually gives a wider bandwidth.

# Chapter 5. Scattering Parameters

At low frequency a two-port network is described by impedances or admittances measured with its ports open or shorted. At radio frequencies those measurements are impossible: an open circuit radiates, a short circuit has inductance, and an active device presented with either may oscillate. Scattering parameters avoid the problem by describing a network in terms of waves going in and waves coming out with every port terminated in the reference impedance, normally 50 Ω. Every RF datasheet, every simulator, and every network analyzer speaks this language, and this chapter teaches enough of it to read, convert, and use the numbers.

## 5.1 Waves in, waves out

At each port n, an incident wave aₙ and a reflected wave bₙ are defined so that |aₙ|² and |bₙ|² are the powers traveling toward and away from the port. For a two-port the outgoing waves are linear combinations of the incoming ones, and the four coefficients are the S-parameters.

b₁ = S₁₁ a₁ + S₁₂ a₂,    b₂ = S₂₁ a₁ + S₂₂ a₂

With the output terminated in Z0, a₂ = 0, so S₁₁ is the input reflection coefficient and S₂₁ the forward transmission coefficient. S₂₂ and S₁₂ are the same quantities seen from the output. Each is a complex number, a magnitude and a phase. In decibels, the return loss is −20 log₁₀|S₁₁|, the gain or insertion loss is 20 log₁₀|S₂₁|, and the reverse isolation is −20 log₁₀|S₁₂|. Because S₁₁ is a reflection coefficient, the input impedance follows from Chapter 2's formula turned around.

Zin = Z0 (1 + S₁₁) / (1 − S₁₁)

## Worked Example 5.1

Problem. A network analyzer measures S₁₁ = 0.5 at 90° on a component in a 50 Ω system. Find the input impedance and the return loss. A second component has S₁₁ = 0.2 at 0°; find its impedance.

Solution. Zin = 50 (1 + j0.5)/(1 − j0.5) = 50 (1 + j0.5)²/1.25 = 50 (0.75 + j1.0)/1.25 = 30 + j40 Ω. The return loss is −20 log₁₀ 0.5 = 6.0 dB. For S₁₁ = 0.2, Zin = 50 × 1.2/0.8 = 75 Ω, purely resistive because the reflection has zero phase.

## 5.2 Lossless, reciprocal, and matched

Three properties can be read directly from the matrix. A network is reciprocal, as every passive network without ferrites or active parts is, when S₁₂ = S₂₁. It is lossless when the power that comes out equals the power that goes in, which for a two-port driven from port 1 means |S₁₁|² + |S₂₁|² = 1. And it is matched at a port when that port's reflection coefficient is zero. A lossless network that is not matched must reflect whatever it does not pass, so measuring S₁₁ alone tells how much a lossless filter passes.

## Worked Example 5.2

Problem. A lossless filter has |S₁₁| = 0.3 at some frequency in its passband. Find its insertion loss there. Then a lossy attenuator has |S₂₁| = 0.5 and |S₁₁| = 0.1; find the fraction of incident power it absorbs.

Solution. For the lossless filter, |S₂₁|² = 1 − 0.09 = 0.91, an insertion loss of −10 log₁₀ 0.91 = 0.41 dB, all of it reflected rather than dissipated. For the attenuator, |S₂₁|² = 0.25 and |S₁₁|² = 0.01, so 1 − 0.25 − 0.01 = 0.74 of the incident power is absorbed. Its insertion loss is 6.0 dB and its return loss 20 dB, the specification of a good 6 dB pad.

## 5.3 Mismatch uncertainty in a chain

S-parameters are defined with ideal Z0 terminations, but real components are cascaded with imperfect ones. When a source with reflection coefficient Γs drives a load with Γl, the wave bounces between them, and the power delivered differs from the matched value by a factor that depends on the phase of the product, which changes with cable length and frequency. The limits of the uncertainty are easy to state.

Mismatch uncertainty = 20 log₁₀ (1 ± |Γs| |Γl|) dB

## Worked Example 5.3

Problem. A signal generator with a VSWR of 1.5 drives a power sensor with a VSWR of 1.5. Find the range of the mismatch uncertainty. Repeat for a generator VSWR of 2.0 and a sensor VSWR of 1.2.

Solution. VSWR 1.5 corresponds to |Γ| = 0.5/2.5 = 0.2, so |Γs||Γl| = 0.04 and the uncertainty is +0.34 dB to −0.35 dB. With VSWR 2.0 (|Γ| = 1/3) and 1.2 (|Γ| = 0.091), the product is 0.030, giving +0.26 to −0.27 dB. The uncertainty is often the largest term in an RF power measurement, and the standard remedy is a well-matched attenuator between the two, which reduces the effective reflection on each side by twice its loss in decibels.

Key idea. S-parameters describe a network by its waves with every port matched. They are measured, not calculated from opens and shorts, and they are only as meaningful as the reference impedance and reference planes they were measured at.

## 5.4 Stability of an amplifier from its S-parameters

A transistor at RF is usually specified as a table of S-parameters at its bias point. Before matching networks are designed, the designer must know whether some passive source or load impedance could make the device oscillate. The Rollett stability factor K and the determinant Δ answer the question: a two-port is unconditionally stable, stable for every passive termination, when K > 1 and |Δ| < 1. When it is, its maximum available gain (MAG), the gain with both ports conjugately matched, follows from the same quantities.

Δ = S₁₁S₂₂ − S₁₂S₂₁,    K = (1 − |S₁₁|² − |S₂₂|² + |Δ|²) / (2 |S₁₂ S₂₁|)

MAG = (|S₂₁| / |S₁₂|) (K − √(K² − 1))

## Worked Example 5.4

Problem. A transistor at 2.4 GHz has S₁₁ = 0.6∠−120°, S₂₁ = 3.5∠80°, S₁₂ = 0.06∠40°, and S₂₂ = 0.4∠−60°. Determine its stability and its maximum available gain, and compare with its gain in a plain 50 Ω system.

Solution. S₁₁S₂₂ = 0.24∠−180°, and S₁₂S₂₁ = 0.21∠120°. Their difference has magnitude |Δ| = 0.226. K = (1 − 0.36 − 0.16 + 0.051)/(2 × 0.21) = 0.531/0.42 = 1.265. Since K > 1 and |Δ| < 1, the device is unconditionally stable. MAG = (3.5/0.06)(1.265 − √(1.265² − 1)) = 58.3 × 0.490 = 28.6, or 14.6 dB. In 50 Ω without matching, the gain is |S₂₁|² = 12.25, or 10.9 dB, so conjugate matching networks of the kind in Chapters 3 and 4 add about 3.7 dB. The power-amplifier version of this design, with load lines and heat, is Part II.

## 5.5 Measuring S-parameters

A vector network analyzer measures incident and reflected waves with directional couplers and compares them in magnitude and phase. Its raw measurement includes the errors of its own cables, connectors, and couplers, which are removed by calibration: the analyzer measures known standards, such as a short, an open, a load, and a through connection (SOLT), and solves for its error terms. Calibration moves the reference plane to the end of the cables. Fixtures and board launches between that plane and the device are removed by de-embedding, and the quality of an S-parameter file depends on how carefully that last step was done.

## Practice

1. A device has |S₁₁| = 0.1 and |S₂₁| = 0.995 and is passive. Find its return loss and insertion loss, and state whether it is lossless.

2. Find Zin for S₁₁ = 0.3∠−45° in a 50 Ω system.

3. A source with |Γ| = 0.3 drives a load with |Γ| = 0.2. Find the mismatch uncertainty.

4. A transistor has S₁₁ = 0.5∠−60°, S₂₁ = 2.0∠100°, S₁₂ = 0.1∠30°, S₂₂ = 0.5∠−40°. Given |Δ| = 0.41, find K and state whether it is unconditionally stable.

5. A reciprocal network has S₂₁ = 0.7∠−30°. What is S₁₂?

6. Explain in one or two sentences why S-parameters, rather than open- and short-circuit parameters, are measured at microwave frequencies.

# Chapter 6. Resonators, Filters, and Waveguide

A filter is a matching network that refuses to match outside its passband. Inside the band it presents the source impedance and passes the signal; outside it presents a reactance and reflects it. This chapter builds filters in three steps: the low-pass prototype that sets the shape of the response, the transformations that turn it into a real low-pass or band-pass circuit, and the resonator quality factor that decides how much of the passband signal the filter loses. The last of those steps leads from microstrip to cavities and waveguide, which is where high-power and low-loss filters live.

## 6.1 The low-pass prototype

Filter design starts from a normalized ladder of alternating series inductors and shunt capacitors, designed for a 1 Ω termination and a cutoff of 1 rad/s. Its element values gₖ are tabulated for each response type. A Butterworth (maximally flat) filter of order n has a monotonic response that falls by 3 dB at cutoff and then by 20n dB per decade. A Chebyshev filter allows a specified ripple in the passband in exchange for a steeper fall just beyond cutoff. The third-order values are g = 1, 2, 1 for Butterworth and g = 1.0316, 1.1474, 1.0316 for a Chebyshev response with 0.1 dB ripple. Scaling to a real impedance R0 and cutoff frequency ωc gives the components.

Lₖ = gₖ R0 / ωc,    Cₖ = gₖ / (R0 ωc),    Butterworth: |S₂₁|² = 1 / (1 + (ω/ωc)^(2n))

## Worked Example 6.1

Problem. Design a third-order Butterworth low-pass filter with a 1 GHz cutoff in a 50 Ω system, as shunt capacitor, series inductor, shunt capacitor. Find its attenuation at 2 GHz and at 2.45 GHz.

Solution. ωc = 2π × 10⁹ rad/s. C₁ = C₃ = 1/(50 × 2π × 10⁹) = 3.18 pF; L₂ = 2 × 50/(2π × 10⁹) = 15.9 nH. At 2 GHz, ω/ωc = 2 and the attenuation is 10 log₁₀(1 + 2⁶) = 18.1 dB. At 2.45 GHz it is 10 log₁₀(1 + 2.45⁶) = 23.4 dB. A harmonic filter after a 1 GHz amplifier must suppress the second harmonic by more than this, so a real design would use a higher order or a Chebyshev response, and would check the inductor's self-resonance, which for 15.9 nH in a small package can fall near the frequencies the filter is meant to stop.

## 6.2 Band-pass filters and resonator Q

A band-pass filter replaces each series inductor of the prototype with a series resonator and each shunt capacitor with a parallel resonator, all tuned to the center frequency f0. The fractional bandwidth FBW = Δf/f0 scales the element values: the narrower the band, the higher the stored energy in each resonator relative to the power passing through. That is why narrow filters are lossy. Each resonator has an unloaded quality factor Qu, the ratio of stored energy to energy lost per radian, set by its conductors and dielectric. Cohn's estimate gives the midband insertion loss of a filter from its prototype values, its bandwidth, and its resonators' Qu.

IL₀ ≈ 4.343 × Σ gₖ / (FBW × Qu)   dB

## Worked Example 6.2

Problem. A three-pole 0.1 dB Chebyshev band-pass filter is centered at 2.45 GHz with a 100 MHz bandwidth. Estimate its midband loss with microstrip resonators of Qu = 200 and with cavity resonators of Qu = 2000.

Solution. FBW = 100/2450 = 0.0408 and Σgₖ = 3.21. With Qu = 200, IL₀ ≈ 4.343 × 3.21/(0.0408 × 200) = 1.71 dB. With Qu = 2000 it is 0.17 dB. In a receiver the 1.5 dB difference adds directly to the noise figure; after a transmitter it is heat in the filter: at 100 W, a 1.71 dB loss burns 33 W, and a 0.17 dB loss 3.9 W. This is why filters that follow power amplifiers are cavities, as Part II assumes.

Key idea. A filter's shape comes from its prototype, but its loss comes from its resonators. Narrowing the bandwidth or adding poles multiplies the stored energy, and the loss rises in proportion unless the resonator Q rises with it.

## 6.3 Waveguide

A hollow metal pipe guides waves without a center conductor, with lower loss and higher power capacity than any coaxial line of similar size. It has a cutoff frequency below which nothing propagates. In a rectangular guide of broad dimension a, the dominant TE₁₀ mode cuts off where half a free-space wavelength equals a. Above cutoff the wave travels with a guide wavelength longer than the free-space wavelength, and the guide is used from about 1.25 to 1.9 times its cutoff, where only the dominant mode propagates and the loss is low.

fc = c / (2a),    λg = λ0 / √(1 − (fc/f)²)

## Worked Example 6.3

Problem. WR-90 waveguide has a = 22.86 mm. Find its cutoff frequency and its guide wavelength at 10 GHz. Then find the length d of a rectangular TE₁₀₁ cavity in the same guide that resonates at 10 GHz.

Solution. fc = 2.998 × 10⁸/(2 × 0.02286) = 6.56 GHz, so the guide is used across the X band, 8.2 to 12.4 GHz. At 10 GHz, λ0 = 29.98 mm and λg = 29.98/√(1 − 0.430) = 39.7 mm. The cavity resonates when its length is half a guide wavelength, d = λg/2 = 19.9 mm; equivalently f = (c/2)√((1/a)² + (1/d)²) = 10 GHz with d = 19.85 mm. Such a cavity in copper has an unloaded Q of several thousand, ten times a microstrip resonator. At millimeter wave the same formula sets the size of the guide: WR-28, for 26.5 to 40 GHz, has a = 7.11 mm and a cutoff of 21.1 GHz.

## 6.4 Which technology at which frequency

Lumped LC filters work to a few gigahertz, limited by component self-resonance and Q near 50 to 100. Microstrip and stripline filters, built from coupled line resonators, work from about 1 GHz into the millimeter range, with Qu of 100 to 300. Ceramic coaxial resonators reach Qu near 500 to 1000 in a small volume for handset and base-station front ends. Acoustic filters, surface and bulk acoustic wave devices, are tiny and steep up to about 6 GHz and dominate mobile phones; their resonators are mechanical, which is why the MEMS survey in Book 7 claims them too. Cavities and waveguide, with Qu in the thousands, take the high-power and lowest-loss jobs. A filter is chosen by the loss it can afford, the power it must carry, and the space it may occupy, roughly in that order.

## Practice

1. Scale the third-order Butterworth prototype to a 500 MHz cutoff in 50 Ω.

2. Find the attenuation of a third-order Butterworth low-pass filter at three times its cutoff frequency.

3. A three-pole 0.1 dB Chebyshev filter has a 2% fractional bandwidth and resonators with Qu = 500. Estimate its midband loss.

4. Find the guide wavelength of WR-90 at 12 GHz.

5. Find the cutoff frequency of a rectangular waveguide with a = 15.8 mm.

6. Explain in one or two sentences why a narrowband filter after a power amplifier is usually a cavity rather than microstrip.

# Chapter 7. The Antenna

An antenna is the matching network between a transmission line and free space. Its terminals present an impedance like any other load, and the power that enters them leaves as a wave whose strength depends on direction. This chapter defines the handful of numbers that describe an antenna, directivity, efficiency, gain, effective aperture, and polarization, derives the link equation that combines two antennas and a distance, and sizes the two antennas most boards actually use, the dipole and the patch. The antenna gains in this chapter are the ones every link budget in the series uses; transmitter power is Part II's and the legal limit on what is radiated is Book 7's.

## 7.1 Directivity, efficiency, and gain

An isotropic radiator, a reference that no real antenna achieves, spreads its power equally over a sphere. A real antenna concentrates it. Directivity D is the ratio of the power density in the strongest direction to the density an isotropic radiator would produce with the same total radiated power. Radiation efficiency η is the fraction of the power accepted at the terminals that is radiated rather than lost in conductors and dielectric. Gain is their product, and in decibels it is quoted in dBi.

G = η D,    η = Rrad / (Rrad + Rloss)

The resistance seen at an antenna's terminals has two parts: the radiation resistance Rrad, which represents power leaving as a wave, and the loss resistance Rloss. A half-wave dipole has Rrad ≈ 73 Ω and D = 1.64, or 2.15 dBi. An antenna much smaller than a wavelength has a radiation resistance that falls as the square of its size; for a short dipole of length l it is 80π²(l/λ)².

## Worked Example 7.1

Problem. A short dipole one twentieth of a wavelength long has 2 Ω of conductor loss. Find its radiation resistance and efficiency. Then find the efficiency of a dipole one tenth of a wavelength long with 1.5 Ω of loss.

Solution. For l = λ/20, Rrad = 80π² × (0.05)² = 1.97 Ω, and η = 1.97/(1.97 + 2) = 0.50, so half the accepted power heats the wire: the gain is 3 dB below the directivity. For l = λ/10, Rrad = 80π² × 0.01 = 7.90 Ω and η = 7.90/9.40 = 0.84. Small antennas are inefficient because their radiation resistance shrinks faster than their loss, and they are also hard to match, because a 2 Ω resistance must be transformed to 50 Ω with the high-Q network that Chapter 3 warned against.

## 7.2 Effective aperture and the link equation

A receiving antenna captures the power flowing through an effective area Ae, related to its gain by a factor that depends only on wavelength. The relation, applied to both ends of a link at distance R, gives the Friis transmission equation. The factor (λ/4πR)² is the free-space path factor between two isotropic antennas; antenna gains multiply it.

Ae = G λ² / (4π),    Pr / Pt = Gt Gr (λ / 4πR)²

## Worked Example 7.2

Problem. Find the free-space path factor at 2.45 GHz over 100 m between isotropic antennas. Then find the received power if a 20 dBm transmitter and the receiver both use 10 dBi antennas, and the effective aperture of an isotropic antenna at this frequency.

Solution. λ = 0.1224 m, so (λ/4πR)² = (0.1224/1256.6)² = 9.48 × 10⁻⁹, or −80.2 dB. This is the book's reference number. With 10 dBi at each end, Pr = 20 + 10 + 10 − 80.2 = −40.2 dBm, about 95 nW. An isotropic antenna's aperture is λ²/4π = 0.0150/12.57 = 1.19 × 10⁻³ m², about 12 cm². Increasing the distance tenfold to 1 km lowers the path factor to −100.2 dB: free-space loss grows 20 dB per decade of distance. One decibel lost in the feed between transmitter and antenna is one decibel of radiated power lost, the rule Part II uses for EIRP.

Key idea. Gain does not create power; it concentrates it. A higher-gain antenna both radiates more strongly in its main direction and captures more power from it, because its effective aperture is proportional to its gain.

## 7.3 Apertures and beamwidth

For an antenna that is large compared with a wavelength, such as a reflector dish, the gain follows from its physical area and an aperture efficiency ηap, typically 0.55 to 0.7. Its half-power beamwidth is roughly 70 λ/D degrees. Gain and beamwidth trade directly: a narrower beam covers less sky and must be pointed more precisely.

G = ηap (π D / λ)²,    θ₃dB ≈ 70 λ / D degrees

## Worked Example 7.3

Problem. A 0.6 m satellite-television dish has ηap = 0.65 at 12 GHz. Find its gain and beamwidth.

Solution. λ = 2.50 cm. G = 0.65 × (π × 0.6/0.025)² = 0.65 × 5,690 = 3,700, or 35.7 dBi. θ₃dB ≈ 70 × 0.025/0.6 = 2.9°. The dish must be aimed to within about a degree, which is why its mounting bracket is the most carefully specified part of the installation.

## 7.4 Polarization

The electric field of a radiated wave points in a definite direction, its polarization, which may be linear (vertical, horizontal, or slanted) or circular (rotating once per cycle, left- or right-handed). A receiving antenna captures only the component matching its own polarization. Between two linear antennas misaligned by an angle ψ, the polarization loss factor is cos²ψ. Between a linear and a circular antenna it is 0.5, a fixed 3 dB loss in exchange for indifference to orientation, which is why satellite navigation and many mobile links use circular polarization.

## Worked Example 7.4

Problem. Two linearly polarized antennas are misaligned by 30° and by 45°. Find the polarization loss in each case.

Solution. At 30°, cos² 30° = 0.75, a loss of 1.25 dB. At 45°, cos² 45° = 0.5, a loss of 3.0 dB. At 90° the loss is in theory infinite and in practice 20 to 30 dB, limited by the antennas' cross-polarized radiation, which is the basis of polarization reuse in point-to-point links.

## 7.5 The dipole and the patch

A half-wave dipole is cut slightly shorter than half a free-space wavelength, about 0.47 to 0.48 λ, so that its input reactance is zero; at that length its resistance is near 70 Ω and matches a 50 Ω line with VSWR about 1.4. A dipole is balanced, while coaxial cable is not, so it is fed through a balun to keep current off the outside of the cable. The microstrip patch, a rectangle of copper over a ground plane, is the dipole of circuit boards. It resonates when its length is about half a guided wavelength in the substrate, slightly reduced because the fields fringe past its edges, and it radiates broadside with a gain of about 5 to 7 dBi. The Balanis design equations give its width, effective permittivity, and edge extension ΔL.

W = (c / 2f) √(2 / (εr + 1)),    L = c / (2f √εeff) − 2ΔL

## Worked Example 7.5

Problem. Size a rectangular patch for 2.45 GHz on a 0.813 mm laminate with εr = 3.55.

Solution. W = (0.1224/2) × √(2/4.55) = 6.12 cm × 0.663 = 40.6 mm. The effective permittivity is (εr + 1)/2 + ((εr − 1)/2)(1 + 12h/W)^(−1/2) = 3.42, and the edge extension from the Balanis formula is ΔL = 0.39 mm. Then L = 2.998 × 10⁸/(2 × 2.45 × 10⁹ × √3.42) − 2 × 0.39 mm = 33.1 − 0.78 = 32.3 mm. A simulator and one trimmed prototype normally adjust the length by a few percent, because the patch's resonance, and therefore its narrow bandwidth of a few percent, depends sensitively on the substrate's actual permittivity.

## Practice

1. Find the length of a half-wave dipole cut to 0.48λ at 5.8 GHz.

2. A small antenna has Rrad = 5 Ω and Rloss = 3 Ω. Find its efficiency and the gain in dBi if its directivity is 1.5 (1.76 dBi).

3. Find the free-space path factor at 2.45 GHz over 1 km between isotropic antennas.

4. A 0.3 m dish has ηap = 0.6 at 10 GHz. Find its gain and beamwidth.

5. Find the effective aperture of a 6 dBi antenna at 2.45 GHz.

6. Explain in one or two sentences why a linear antenna receiving a circularly polarized signal loses 3 dB regardless of its orientation.

# Chapter 8. Arrays, Beam Steering, and Millimeter Wave

A single antenna's beam is fixed by its shape. An array of identical antennas, each fed with a controlled amplitude and phase, has a beam that can be shaped and steered electronically, with no moving parts, as fast as the phases can be changed. Arrays are how 5G base stations, automotive radars, satellite terminals, and Wi-Fi access points point their energy. They are also the reason millimeter-wave links work at all, and this chapter ends by showing, with the Friis equation of Chapter 7, why higher frequency can mean a stronger link rather than a weaker one.

## 8.1 The array factor

Consider N identical elements along a line with spacing d, each fed with equal amplitude and a progressive phase step β from one element to the next. In a direction θ from broadside, the waves from adjacent elements differ in phase by ψ = kd sin θ + β, where k = 2π/λ. The array's pattern is the element's pattern multiplied by the array factor, which depends only on N and ψ. The main beam points where ψ = 0, so the phase step that steers it to an angle θ₀ follows directly.

AF(ψ) = sin(Nψ/2) / (N sin(ψ/2)),    ψ = kd sin θ + β,    β = −kd sin θ₀

## Worked Example 8.1

Problem. An array has half-wavelength spacing. Find the phase step between elements that steers its beam to 30° and to 20° from broadside.

Solution. With d = λ/2, kd = π, which is 180°. For θ₀ = 30°, β = −180° × sin 30° = −90° per element. For 20°, β = −180° × 0.342 = −61.6°. A 16-element array steered to 30° needs phases of 0°, −90°, −180°, and so on to −1,350°, which modulo 360° repeats the four values 0°, −90°, −180°, and −270°.

## 8.2 Gain, beamwidth, and grating lobes

With uniform excitation and negligible coupling, an array of N elements has about N times the gain of one element, so 16 elements add 12 dB and 64 add 18 dB. Its broadside half-power beamwidth narrows in proportion to its length: for a uniform linear array of length Nd, it is about 0.886 λ/(Nd) radians, and it widens by 1/cos θ₀ as the beam is steered away from broadside, because the array's projected length shrinks. The element pattern also falls off with angle, so the gain drops roughly as cos θ₀, about 3 dB at 60°, the scan loss.

The spacing has an upper limit. If the elements are too far apart, the array factor has other directions where ψ is a multiple of 2π, and full-strength copies of the main beam, grating lobes, appear in visible space. To keep them out while scanning to θmax, the spacing must satisfy a simple inequality, which is why arrays are built at about half a wavelength.

d / λ < 1 / (1 + |sin θmax|)

## Worked Example 8.2

Problem. A linear array of 16 patch elements, each 5 dBi, uses half-wavelength spacing. Find its broadside gain and beamwidth. What is the largest spacing that allows scanning to ±45° without grating lobes?

Solution. G ≈ 5 + 10 log₁₀ 16 = 17.0 dBi. The beamwidth is 0.886/(16 × 0.5) = 0.111 rad = 6.3° at broadside, and about 9° at a 45° scan. For scanning to 45°, d/λ < 1/(1 + 0.707) = 0.586, so the spacing may grow to 0.59λ, which reduces the number of elements, and of their amplifiers, needed for a given aperture. Scanning to 60° would require d < 0.54λ.

Key idea. An array's gain grows with the number of elements and its beamwidth shrinks with its length. Half-wavelength spacing is not a tradition; it is the largest spacing that keeps grating lobes out of a wide scan.

## 8.3 Phase shifters and their quantization

An active electronically scanned array puts a phase shifter, and usually a small amplifier, behind every element; those amplifiers are Part II's subject. A digital phase shifter with b bits sets phases in steps of 360°/2^b. The rounding errors are close to uniformly distributed, with a standard deviation of the step divided by √12, and a random phase error of σ radians reduces the array's gain by about a factor of exp(−σ²). Beamforming chips commonly use five or six bits, partly for this reason and partly to keep the sidelobes low.

## Worked Example 8.3

Problem. Estimate the gain loss of a large array from phase quantization with 3-bit and with 4-bit phase shifters.

Solution. A 3-bit shifter has 45° steps, so σ = 45°/√12 = 13.0° = 0.227 rad, and the loss is −10 log₁₀ exp(−0.0514) = 0.22 dB. A 4-bit shifter has 22.5° steps, σ = 6.5° = 0.113 rad, and a loss of 0.056 dB. The gain loss is small even at three bits; what more bits buy is lower quantization sidelobes and finer pointing.

## 8.4 Millimeter wave is the same subject

Above about 24 GHz the wavelength is around a centimeter or less, and three things change. The free-space path factor between isotropic antennas falls as λ², so it looks 20 dB worse for every factor of ten in frequency. Losses rise: line and dielectric loss per meter, which is why the laminate choices of Book 7 matter, and the atmosphere's oxygen absorption near 60 GHz, about 15 dB/km, which limits 60 GHz links to short range and makes the band easy to reuse. And every interconnect becomes significant: a bond wire carries roughly 1 nH per millimeter, and a 0.3 mm wire at 77 GHz has a reactance of 145 Ω, which is why millimeter-wave chips are flip-chip mounted or put the antenna in the package, as Book 7 describes. None of this needs new physics. What changes is the size of every mistake.

## Worked Example 8.4

Problem. Two terminals 100 m apart each have a fixed antenna aperture of 0.01 m², a square 10 cm on a side, with full aperture efficiency. Compare the link factor Gt Gr (λ/4πR)² at 2.45 GHz and at 28 GHz. Check that 100 m is in the far field at 28 GHz.

Solution. At 2.45 GHz, G = 4πAe/λ² = 4π × 0.01/0.1224² = 8.39, or 9.2 dBi, and the path factor is −80.2 dB, so the link factor is 9.2 + 9.2 − 80.2 = −61.8 dB. At 28 GHz, λ = 1.07 cm, G = 1,096, or 30.4 dBi, and the path factor is −101.4 dB, so the link factor is 30.4 + 30.4 − 101.4 = −40.6 dB, 21 dB stronger. With equal apertures, each antenna's gain rises as f² while the path factor falls as 1/f², and the link improves by 20 log₁₀(28/2.45) = 21 dB. The price is beamwidth: each 28 GHz beam is a few degrees wide and must be steered at the other terminal, which is exactly what the arrays of this chapter do. The far-field distance 2D²/λ = 2 × 0.1²/0.0107 = 1.9 m is far shorter than 100 m.

## 8.5 Where this book stops

This book has supplied the passive structure of a radio: lines, matches, filters, antennas, and arrays, and the gain of every antenna in a link budget. Book 5 builds the modem that decides what waveform these structures carry and how the band plan divides the spectrum. Part II supplies the transmitter power, the power amplifier with its load line, the receiver's noise, and the oscillator. Book 7 measures what the antenna actually radiated against the legal masks and the chamber methods. Book 7 supplies the laminates, packages, and antenna-in-package structures that turn the millimeter-wave designs of this chapter into hardware.

## Practice

1. An array with 0.5λ spacing must steer to 45°. Find the phase step between elements.

2. Find the approximate gain of a 64-element array of 6 dBi elements.

3. Find the broadside half-power beamwidth of a 32-element uniform linear array at half-wavelength spacing.

4. What is the largest element spacing for scanning to ±30° without grating lobes?

5. Find the gain loss from phase quantization with 5-bit phase shifters.

6. Repeat the fixed-aperture comparison of Worked Example 8.4 for 2.45 GHz and 60 GHz, ignoring atmospheric absorption. By how many decibels does the link factor change?

# Chapter 9. Couplers, Dividers, and Hybrids

Many RF systems need to split a signal into parts, combine parts into one, or sample a wave traveling in one direction without disturbing it. The structures that do these jobs, power dividers, hybrids, and directional couplers, are built from the transmission lines of Chapter 2 and described by the scattering parameters of Chapter 5. They appear in every balanced amplifier, Doherty combiner, phased-array feed, and reflectometer. This chapter explains the main types and the figures that describe them.

## 9.1 What a three-port can and cannot do

A lossless, reciprocal three-port cannot be matched at all three ports, a consequence of the unitary S-matrix of Chapter 5. A simple T-junction splits power but reflects from its outputs and couples them to each other. The Wilkinson divider avoids that by adding a resistor between the outputs. It is matched at all ports and isolates the outputs, because the resistor absorbs power only when the outputs are unbalanced; when they are driven equally, no current flows through it and the divider is lossless.

## 9.2 The Wilkinson divider

The equal-split Wilkinson uses two quarter-wave lines of impedance √2 Z0 from the input to the outputs and a resistor of 2Z0 between the outputs. Each output receives half the power, a 3.01 dB split, in phase. As a combiner it adds two equal, in-phase signals without loss and dissipates the difference if they are unequal, which is why Part II's combiners degrade gracefully when one amplifier fails. Its bandwidth is limited by the quarter-wave lines; multisection versions extend it to an octave or more.

Wilkinson: Zline = √2 Z0,   R = 2Z0,   |S21|² = |S31|² = ½

## Worked Example 9.1

Problem. Design an equal-split Wilkinson divider for a 50 Ω system at 2.45 GHz on a microstrip with an effective permittivity of 3.3. Find the line impedance, the resistor, and the physical line length.

Solution. The lines are 50√2 = 70.7 Ω and the resistor is 100 Ω. The guided wavelength is 299.8/(2.45 × √3.3) = 67.4 mm, so each quarter-wave line is 16.8 mm long. Splitting four ways with two stages loses 6.02 dB per output, the same as 3.01 dB twice.

## 9.3 Quadrature hybrids

The branch-line hybrid is a square of quarter-wave lines: two series arms of Z0/√2 and two shunt arms of Z0. A signal at port 1 divides equally between ports 2 and 3, 90° apart, and port 4 is isolated. Balanced amplifiers use a pair of these hybrids: reflections from the two identical amplifiers cancel at the input and go to the isolated port's load, so the pair presents a good match even when each amplifier alone is poorly matched. The rat-race, or 180° hybrid, does the same with in-phase and anti-phase outputs and serves balanced mixers and baluns.

branch-line: series arms Z0/√2 = 35.4 Ω,  shunt arms Z0 = 50 Ω  (for Z0 = 50 Ω)

Key idea. Dividers and hybrids are networks of quarter-wave lines arranged so that waves add at the wanted ports and cancel at the isolated one. Their resistors or isolated loads absorb only the imbalance.

## 9.4 The directional coupler

A directional coupler samples a small part of a wave traveling one way. Two parallel lines close together couple through their mutual capacitance and inductance; a wave on the main line couples into the adjacent line traveling backward, toward the coupled port. The coupling sets the sample's level, and the directivity, how much more strongly the coupler responds to the forward wave than to the reverse one, sets how well it distinguishes the two. The isolation is the coupling plus the directivity. A coupled-line coupler is designed from its even- and odd-mode impedances, which for a voltage coupling factor C are Z0e = Z0√((1 + C)/(1 − C)) and Z0o = Z0√((1 − C)/(1 + C)).

isolation = coupling + directivity,   Z0e Z0o = Z0²

## Worked Example 9.2

Problem. Find the even- and odd-mode impedances of a 10 dB and a 20 dB coupled-line coupler in 50 Ω.

Solution. For 10 dB, C = 10^(−10/20) = 0.316, Z0e = 50√(1.316/0.684) = 69.4 Ω and Z0o = 36.0 Ω. For 20 dB, C = 0.1, Z0e = 55.3 Ω and Z0o = 45.2 Ω. Looser coupling needs the two impedances closer together, which means the lines are farther apart.

## Worked Example 9.3

Problem. A reflectometer uses a coupler with 30 dB of directivity to measure a load with a true return loss of 20 dB. What range of return loss might it report?

Solution. The true |Γ| is 0.1. The leakage of the forward wave into the reverse port, 30 dB down, is 0.0316 in amplitude and adds with an unknown phase, so the measured |Γ| lies between 0.068 and 0.132: a return loss reading anywhere from 17.6 to 23.3 dB. Measuring a good match needs a coupler whose directivity is well beyond the return loss being measured, or vector error correction as in Chapter 5.

## Practice

1. Find the line impedance and resistor of a Wilkinson divider in a 75 Ω system.

2. What is the loss to each output of an eight-way divider built from equal Wilkinson stages, ignoring dissipation?

3. A coupler has 20 dB coupling and 25 dB directivity. Find its isolation.

4. Find the possible return-loss readings for a true 20 dB return loss with 25 dB directivity.

5. Why does a balanced amplifier present a good input match even if each amplifier is poorly matched?

6. Why can't a lossless reciprocal three-port be matched at all ports?

# Chapter 10. Microstrip and Stripline by Design

Chapter 2 treated a transmission line through its characteristic impedance and velocity without asking how a given geometry produces them. On a circuit board the line is a copper trace over a ground plane, microstrip, or a trace buried between two planes, stripline. This chapter gives the closed-form design equations for both, shows how to find the trace width for a wanted impedance, and estimates the dielectric loss. The laminate's measured properties at 10 GHz are Book 7's; here a typical value stands in, and the method is the subject.

## 10.1 Microstrip and its effective permittivity

In microstrip, part of the field lies in the substrate and part in the air above it, so the wave travels at a speed set by an effective permittivity between 1 and εr. Wider traces put more of the field in the substrate and raise εeff; they also lower the impedance. Hammerstad's equations, as given in Pozar, express both in terms of the width-to-height ratio u = W/h, accurate to about 1% for most practical boards. Designers usually need the inverse: the width that gives 50 Ω. Wheeler's synthesis formula provides it in closed form.

εeff = (εr + 1)/2 + (εr − 1)/(2√(1 + 12/u));   Z0 = 120π / [√εeff (u + 1.393 + 0.667 ln(u + 1.444))]  (u ≥ 1)

## Worked Example 10.1

Problem. A laminate has εr = 3.55 and a thickness of 0.508 mm. Find the trace widths for 50 Ω, 70.7 Ω, and 35.4 Ω, the impedances the Wilkinson and branch-line designs of Chapter 9 need, and the guided wavelength of the 50 Ω line at 10 GHz.

Solution. The synthesis gives u = 2.24 for 50 Ω, a width of 1.14 mm, with εeff = 2.78; checking with Hammerstad's analysis gives 50.3 Ω. For 70.7 Ω, u = 1.22, a width of 0.62 mm, εeff = 2.66. For 35.4 Ω, u = 3.74, a width of 1.90 mm, εeff = 2.90. At 10 GHz the 50 Ω line's guided wavelength is 30.0/√2.78 = 18.0 mm. On a 1.6 mm FR-4 board with εr = 4.4, the 50 Ω width would be 3.06 mm, with εeff = 3.33.

## 10.2 Stripline

In stripline the trace is fully embedded in dielectric between two ground planes, so the wave is a true TEM wave with εeff = εr, and the line is shielded, which keeps its fields away from neighbors and from the outside world. For a thin strip of width W centered between planes separated by b, Pozar's formula gives Z0 = 30π b /(√εr (We + 0.441b)), with We ≈ W for wide strips. Stripline is slower than microstrip on the same laminate and its traces are narrower for the same impedance, but it radiates less and couples less, which suits dense digital and RF boards alike.

stripline: Z0 = 30π b / [√εr (W + 0.441 b)]  (for √εr Z0 < 120 Ω)

## Worked Example 10.2

Problem. Find the width of a 50 Ω stripline with εr = 3.55 and a 1.0 mm plane spacing. Compare its velocity with the microstrip of Example 10.1.

Solution. W/b = 30π/(√3.55 × 50) − 0.441 = 0.559, so W = 0.56 mm, half the microstrip's width. Its velocity is c/√3.55 = 0.531c, against c/√2.78 = 0.600c for the microstrip, so a stripline delay line is about 12% shorter for the same delay.

Key idea. A line's impedance is set by the ratio of its width to its height above the ground, and its velocity by how much of the field lies in the dielectric. Closed-form formulas get within about a percent; a field solver and a test coupon confirm the rest.

## 10.3 Dielectric and conductor loss

The dielectric's loss tangent, tan δ, converts part of the stored electric energy to heat every cycle, giving an attenuation that rises in proportion to frequency. For microstrip, only the part of the field in the substrate contributes, so αd = k0 εr (εeff − 1) tan δ / (2√εeff (εr − 1)) nepers per meter. Conductor loss rises with the square root of frequency through the skin effect of Book 6's Chapter 6 and is increased by copper roughness, which at tens of gigahertz can add half again to the smooth-copper value. Above a few gigahertz on low-cost laminates, dielectric loss dominates; on low-loss laminates, conductor loss often does.

αd = k0 εr (εeff − 1) tan δ / [2 √εeff (εr − 1)]  Np/m;   1 Np = 8.686 dB

## Worked Example 10.3

Problem. The 50 Ω microstrip of Example 10.1 is on a laminate with tan δ = 0.0027. Find its dielectric loss at 10 GHz.

Solution. k0 = 2π × 10¹⁰/2.998 × 10⁸ = 209.6 rad/m. αd = 209.6 × 3.55 × 1.78 × 0.0027 / (2 × 1.667 × 2.55) = 0.421 Np/m, or 3.65 dB/m, 0.037 dB/cm. A 10 cm feed line therefore loses about 0.37 dB to the dielectric alone, before conductor loss is added.

## 10.4 Dispersion, modes, and discontinuities

Microstrip is not perfectly TEM, so its εeff rises slowly with frequency, an effect called dispersion that matters for wideband pulses and at millimeter waves. Thick substrates and wide traces can also support higher-order and surface-wave modes above a cutoff, so millimeter-wave boards use thin substrates. Bends, steps in width, T-junctions, and vias add parasitic capacitance and inductance; mitered bends and teardrop transitions reduce them, and circuit simulators model each discontinuity explicitly. The via between layers, discussed in Book 7, is often the largest single discontinuity on a fast board.

## Practice

1. Find εeff for a microstrip with εr = 3.55 and u = 2.24.

2. Find the guided wavelength at 28 GHz on a line with εeff = 2.78.

3. Find the 50 Ω stripline width for εr = 3.0 and b = 0.8 mm.

4. A line loses 0.037 dB/cm to the dielectric at 10 GHz. Estimate the loss at 20 GHz.

5. Why is a stripline's effective permittivity equal to εr?

6. Why do millimeter-wave boards use thin substrates?

# Chapter 11. Broadband Matching and the Bode-Fano Limit

Chapters 3 and 4 matched one impedance to another at a single frequency. Real signals occupy bands, and many systems, from wideband radios to test equipment, must match over an octave or more. This chapter asks how wide a match can be. Multisection transformers and low-Q networks widen it, and the Bode-Fano criterion sets a hard limit that no network, however clever, can exceed when the load stores energy in a reactance.

## 11.1 Q and bandwidth

An L-network's Q, from Chapter 3, is fixed by the ratio of the resistances: Q = √(Rhigh/Rlow − 1). A higher Q means a narrower band, roughly f0/Q for the match to stay within a few decibels of return loss. Matching 50 Ω to 100 Ω needs Q = 1 and is naturally broadband; matching 50 Ω to 500 Ω needs Q = 3. The cure for a high ratio is to split the transformation into steps, each with a lower Q: two L-sections through an intermediate resistance, or several quarter-wave sections whose impedances step gradually.

## 11.2 Multisection transformers

A single quarter-wave transformer is matched exactly only at its design frequency. A multisection transformer uses several quarter-wave sections whose small reflections are chosen to cancel over a band. The binomial design makes the response maximally flat at the center; the Chebyshev design allows a ripple of chosen size in return for a wider band. Pozar gives the bandwidth for a maximum tolerated reflection Γm in closed form. For a single section, Δf/f0 = 2 − (4/π) arccos[(Γm/√(1 − Γm²)) × 2√(Z0ZL)/|ZL − Z0|]; for an N-section binomial design, Δf/f0 = 2 − (4/π) arccos[½(Γm/A)^(1/N)], with A = 2^(−N)(ZL − Z0)/(ZL + Z0).

binomial: ln(Zn+1/Zn) ≈ 2^(−N) C(N, n) ln(ZL/Z0)

## Worked Example 11.1

Problem. Match 50 Ω to 100 Ω with a maximum reflection of 0.1 (20 dB return loss). Find the fractional bandwidth of a single quarter-wave section and of a two-section binomial transformer, and the section impedances.

Solution. The single section has impedance √(50 × 100) = 70.7 Ω and a fractional bandwidth of 0.367, or 36.7%. For two binomial sections, A = ¼ × 50/150 = 0.0833, and the bandwidth is 2 − (4/π) arccos(½ × (0.1/0.0833)^½) = 0.738, or 73.8%, twice as wide. The section impedances are 50 × 2^(1/4) = 59.5 Ω and 50 × 2^(3/4) = 84.1 Ω.

## 11.3 The Bode-Fano limit

Many loads are not resistive: a transistor's input looks like a resistance in parallel or series with a capacitance, and an antenna has stored reactive energy. Bode and Fano showed that for a parallel RC load matched by any lossless network, the integral of ln(1/|Γ|) over angular frequency cannot exceed π/(RC). If the match is held at a constant reflection Γ over a band Δω and is total reflection elsewhere, the best achievable is ln(1/Γ) = π/(RC Δω). A wider band forces a worse match, and a larger capacitance makes it worse still. The limit applies to every possible network, so it tells a designer when to stop trying.

∫ ln(1/|Γ(ω)|) dω ≤ π/(RC);   rectangular best case: Γ = exp[−π/(RC Δω)]

## Worked Example 11.2

Problem. A transistor's input is 50 Ω in parallel with 5 pF. What is the best return loss any lossless network can achieve over a 1 GHz band, and over a 2 GHz band?

Solution. For Δf = 1 GHz, π/(RC Δω) = π/(50 × 5 × 10⁻¹² × 2π × 10⁹) = 2.0, so Γ = e^(−2) = 0.135, a return loss of 17.4 dB and a mismatch loss of 0.08 dB. For 2 GHz the exponent halves to 1.0, so Γ = 0.368, a return loss of only 8.7 dB and a mismatch loss of 0.63 dB. Doubling the bandwidth halves the logarithm of the achievable return loss.

Key idea. A reactive load limits the product of bandwidth and match quality. The Bode-Fano integral sets that limit for every possible lossless network, so wideband designs must accept a poorer match, add loss, or reduce the load's reactance.

## 11.4 Ways around the limit

Since the limit applies to lossless matching of a fixed load, designers escape it in three ways. They can change the load: smaller transistors or distributed amplifiers, which absorb the device capacitance into an artificial transmission line, achieve multioctave bandwidths. They can add loss: resistive matching or feedback flattens the response at the cost of gain and noise figure, a trade that Part II's LNA chapter weighs. Or they can use balanced structures, like the balanced amplifier of Chapter 9, which hide each device's poor match behind hybrids. Test equipment and wideband receivers use all three.

## Practice

1. Find the L-network Q to match 10 Ω to 50 Ω.

2. Find the section impedances of a two-section binomial transformer from 50 Ω to 200 Ω.

3. For a 50 Ω ∥ 2 pF load, what is the best return loss over a 2 GHz band?

4. What is the widest band over which a 50 Ω ∥ 2 pF load can be matched to a 10 dB return loss?

5. Why does a Chebyshev transformer give more bandwidth than a binomial one with the same number of sections?

6. Why does a distributed amplifier escape the Bode-Fano limit of a single transistor?

# Chapter 12. Cavities and Dielectric Resonators

Chapter 6 showed that a filter's insertion loss depends on the unloaded Q of its resonators. A microstrip resonator has a Q of a few hundred; the narrowband filters of base stations, satellites, and radars need thousands. Those come from cavities, closed metal boxes whose walls confine the field, and from dielectric resonators, pucks of low-loss ceramic in which the field is concentrated. This chapter computes a cavity's resonant frequency and Q, shows what that Q buys in a filter, and explains how dielectric resonators shrink the result.

## 12.1 The rectangular cavity

A length of the rectangular waveguide of Chapter 6, shorted at both ends, becomes a cavity. The lowest mode, TE101, has a half wavelength across the broad wall a and another along the length d, so its resonant frequency is f = (c/2)√(1/a² + 1/d²). The fields are confined, nothing radiates, and the only losses are the currents in the walls, which flow in a skin depth set by the surface resistance Rs = √(ωμ0/(2σ)). Pozar gives the conductor Q of the TE101 mode in closed form, and for copper at X band it is several thousand.

f101 = (c/2)√(1/a² + 1/d²),   Rs = √(ωμ0 / 2σ)

Qc = (k a d)³ b η / (2π² Rs) × 1 / (2a³b + 2bd³ + a³d + ad³)

## Worked Example 12.1

Problem. A copper cavity is made from WR-90 waveguide, a = 22.86 mm and b = 10.16 mm. Find the length for a 10 GHz TE101 resonance and its conductor Q. How much lower would the Q be in aluminum, with σ = 3.5 × 10⁷ S/m?

Solution. Solving for d gives 1/d² = (2f/c)² − 1/a², so d = 19.85 mm. At 10 GHz copper's surface resistance is 0.0261 Ω, and the formula gives Qc = 7820, a 3 dB bandwidth of only 1.28 MHz. Q scales with √σ, so aluminum gives 7820 × √(3.5/5.8) = 6080. Real cavities, with seams, tuning screws, and surface roughness, reach perhaps 70% to 90% of the ideal value.

## 12.2 What the Q buys

A band-pass filter's midband insertion loss is about 4.343 Σgk/(FBW × Qu) dB, from Chapter 6. For a five-pole filter with Σgk near 5 and a 1% fractional bandwidth, resonators with a Q of 200 give 10.9 dB of loss, unusable; with a Q of 7800, the loss is 0.28 dB. Narrow filters therefore need high-Q resonators, and the narrower the band, the more the Q matters. That is why the duplexers and channel filters of cellular base stations, the output multiplexers of satellites, and the preselectors of radars are cavity or dielectric-resonator filters, while handsets use the acoustic filters of Book 3.

Key idea. A resonator's unloaded Q sets how narrow a filter can be without excessive loss. Confining the fields in a metal cavity or a low-loss dielectric raises the Q from hundreds to thousands.

## 12.3 Coupling and loaded Q

A cavity is useful only if energy can enter and leave it, through a probe, a loop, or an iris. The coupling coefficient β compares the power lost to the external circuit with that lost in the walls. The loaded Q, which sets the observed bandwidth, is Qu/(1 + β); at critical coupling, β = 1, the cavity is matched at resonance and its loaded Q is half its unloaded Q. In a multipole filter, the irises between cavities set the coupling coefficients that the prototype of Chapter 6 calls for, and tuning screws trim each resonance.

QL = Qu / (1 + β)

## 12.4 Temperature and the dielectric resonator

A cavity's frequency follows its dimensions, so thermal expansion detunes it: copper expands about 17 ppm/K, shifting a 10 GHz cavity by about 170 kHz per kelvin, a large fraction of a 1 MHz bandwidth over a 50 K range. Invar, at about 1.2 ppm/K, or temperature-compensating structures solve this at a cost. Dielectric resonators solve it and the size problem at once: a ceramic puck with εr of 30 to 45 shrinks the resonator by about √εr, has a loss tangent near 10⁻⁴ for a Q of up to about 10,000, and can be formulated to have a temperature coefficient of frequency near zero. A puck for 3.5 GHz with εr = 45 is about 13 mm across, against several centimeters for an air cavity.

## Worked Example 12.2

Problem. A copper cavity at 10 GHz has a 1.28 MHz bandwidth. How far does it drift over a 50 K temperature range, and how does an Invar cavity compare?

Solution. Copper: 17 ppm/K × 10 GHz = 170 kHz/K, so 8.5 MHz over 50 K, more than six bandwidths. Invar: 1.2 ppm/K gives 12 kHz/K, or 0.6 MHz over 50 K, half a bandwidth. Satellite multiplexers therefore use Invar or temperature-compensated designs, and terrestrial filters often use dielectric resonators with near-zero drift.

## Practice

1. Find the TE101 frequency of a WR-90 cavity 25 mm long.

2. A resonator's unloaded Q is 5000 and it is critically coupled. Find its loaded Q.

3. Find the insertion loss of a five-pole filter with Σgk = 5, FBW = 0.5%, and Qu = 10,000.

4. A cavity's Q is 6000 in copper. Estimate it in a metal with a quarter of copper's conductivity.

5. Why does a cavity have no radiation loss?

6. Why do handsets not use cavity or dielectric-resonator filters?

# Chapter 13. Small Antennas, Monopoles, Loops, and Horns

Chapter 7 treated the dipole, the patch, and the large aperture. Most antennas in products are none of these: they are monopoles over a circuit board, small loops in key fobs, chip antennas squeezed into a corner, or horns feeding reflectors and test ranges. This chapter covers those types and the physical limit that governs every small antenna: the smaller it is relative to the wavelength, the narrower its bandwidth and the harder it is to make efficient.

## 13.1 The monopole

A quarter-wave monopole over a large ground plane behaves like half of a dipole: its image in the plane completes the dipole. All its power goes into the half-space above the plane, so its directivity is twice the dipole's, 5.15 dBi, and its input resistance is half, about 36.5 Ω. On a real product the ground plane is the circuit board, which is often not large compared with a wavelength, so the board itself radiates and the pattern, gain, and tuning depend on its size and on the hand holding it. At 2.45 GHz a quarter wave is 30.6 mm in air, which is why most handheld antennas are folded, meandered, or loaded to fit.

## 13.2 The small-antenna limit

An antenna that fits inside a sphere of radius a much smaller than a wavelength stores far more reactive energy than it radiates, so it has a high Q and a narrow bandwidth. Chu's limit, refined by McLean, gives the minimum Q of any antenna inside that sphere: Q = 1/(ka)³ + 1/(ka), with k = 2π/λ. The fractional bandwidth for a given VSWR s is at most about (s − 1)/(Q√s). Matching networks cannot beat it, by the same argument as Chapter 11's Bode-Fano limit. Adding loss broadens the band but wastes power, which is how many tiny antennas meet their bandwidth specifications.

Qmin = 1/(ka)³ + 1/(ka),   FBW ≤ (s − 1)/(Q√s)

## Worked Example 13.1

Problem. A 915 MHz sensor's antenna must fit in a sphere of 20 mm radius. Find ka, the minimum Q, and the maximum bandwidth for a 2:1 VSWR. Repeat for a 2.45 GHz antenna in a 10 mm sphere.

Solution. At 915 MHz, k = 2π/0.3277 m = 19.2 rad/m, so ka = 0.384 and Qmin = 1/0.0564 + 1/0.384 = 20.3. The bandwidth for a 2:1 VSWR is at most 1/(20.3 × √2) = 3.5%, or 32 MHz, enough for the 26 MHz ISM band with little margin, and only if the antenna reaches the limit, which practical antennas rarely do. At 2.45 GHz in a 10 mm sphere, ka = 0.513 and Qmin = 9.3, so the bandwidth limit is about 7.6%.

Key idea. A small antenna's bandwidth is limited by its size in wavelengths: the minimum Q grows as 1/(ka)³. When a product shrinks its antenna, it pays in bandwidth, efficiency, or both.

## 13.3 The small loop

A loop much smaller than a wavelength has a radiation resistance of 31,171 (A/λ²)² ohms for one turn of area A, minute for practical sizes. Its conductor loss, set by the surface resistance of Chapter 12 and the wire's circumference, is usually much larger, so a small loop is an inefficient radiator. It is used anyway in key fobs, pagers, and hearing aids, because it is compact, responds to the magnetic field, and is less detuned by the body than a small electric antenna. As a receiving antenna at low frequencies, where atmospheric noise dominates, its inefficiency costs little sensitivity.

Rrad = 31,171 (A/λ²)² Ω   (one turn)

## Worked Example 13.2

Problem. A key fob uses a single-turn copper loop 10 mm in diameter, of wire 0.5 mm in radius, at 433 MHz. Find its radiation resistance, its loss resistance, and its efficiency.

Solution. A = π × 0.005² = 7.85 × 10⁻⁵ m² and λ = 0.692 m, so Rrad = 31,171 × (7.85 × 10⁻⁵/0.479)² = 0.84 mΩ. The surface resistance of copper at 433 MHz is 5.43 mΩ per square; the loss resistance is Rs times the circumference over the wire's perimeter, 5.43 mΩ × 31.4 mm / 3.14 mm = 54.3 mΩ. The efficiency is 0.84/(0.84 + 54.3) = 1.5%, or −18.2 dB. Short-range remote controls tolerate that, and regulations limit their radiated power anyway.

## 13.4 Horns

A horn flares a waveguide into an aperture, transforming the guide's impedance gradually to free space. Its gain follows the aperture formula of Chapter 7, G = εap 4πA/λ², with an aperture efficiency of about 0.5 for an optimum pyramidal horn. Horns are broadband, predictable, and low-loss, which makes them the standard gain references for antenna measurement and the feeds of reflector antennas. Corrugated horns produce symmetrical, low-sidelobe patterns for satellite feeds.

## Worked Example 13.3

Problem. A pyramidal horn at 10 GHz has an aperture of 100 × 80 mm and an aperture efficiency of 0.51. Find its gain.

Solution. λ = 30.0 mm, so G = 0.51 × 4π × 0.1 × 0.08/0.0300² = 57.0, or 17.6 dBi.

## Practice

1. Find the length of a quarter-wave monopole at 868 MHz.

2. An 868 MHz antenna fits in a 30 mm sphere. Find ka and the minimum Q.

3. Find the gain of a horn of 50 × 40 mm at 20 GHz with an aperture efficiency of 0.51.

4. How does a small loop's radiation resistance scale with its diameter?

5. Why is a quarter-wave monopole's directivity twice a half-wave dipole's?

6. Why do small antennas on products often include deliberate loss?

# Chapter 14. Propagation Beyond Free Space

The Friis equation of Chapter 7 assumes empty space between two antennas. Real paths run over the ground, past buildings and hills, and through air and rain. This chapter adds the corrections that matter most to a radio engineer: the Fresnel zone that a line-of-sight path must keep clear, the ground reflection that changes the distance law, diffraction over an obstacle, and absorption by oxygen and rain at millimeter waves. Each correction is a number of decibels added to the free-space loss.

## 14.1 The Fresnel zone

Energy does not travel along an infinitely thin ray. The first Fresnel zone is the ellipsoid around the direct path within which the excess path length is less than half a wavelength; obstacles inside it reduce the received power even if they do not block the line of sight. Its radius at a point a distance d1 from one end and d2 from the other is r1 = √(λd1d2/(d1 + d2)). Microwave links are designed to keep at least 60% of the first zone clear, allowing for the Earth's bulge, which with the standard effective-Earth-radius factor k = 4/3 rises by d1d2/(2kR) at the midpoint.

r1 = √(λ d1 d2 / (d1 + d2)),   bulge h = d1 d2 / (2 k R),  k = 4/3,  R = 6371 km

## Worked Example 14.1

Problem. A 5.8 GHz link spans 10 km. Find the first Fresnel radius at the midpoint, the 60% clearance, and the Earth bulge there.

Solution. λ = 51.7 mm, so r1 = √(0.0517 × 5000 × 5000/10,000) = 11.4 m, and 60% clearance is 6.8 m. The bulge is 5000²/(2 × 4/3 × 6.371 × 10⁶) = 1.47 m. The antennas must therefore be mounted high enough that the line between them clears the terrain at mid-path by at least 6.8 + 1.5 = 8.3 m.

## 14.2 The ground reflection

Over flat ground, the receiver sees the direct wave and a wave reflected from the ground, nearly 180° out of phase at grazing incidence. Close in, the two alternately add and cancel; beyond a breakpoint distance db = 4hthr/λ, they cancel more and more completely, and the received power falls as 1/d⁴ instead of 1/d². In that region the path loss is approximately 40 log10 d − 20 log10(hthr), independent of frequency. This two-ray model is the simplest explanation of why cellular path loss grows faster than free space, and why antenna height matters so much.

db = 4 ht hr / λ;   beyond db: L ≈ 40 log10 d − 20 log10(ht hr)  dB

## Worked Example 14.2

Problem. A 2.4 GHz base antenna is 10 m high and the handset 1.5 m. Find the breakpoint and the two-ray path loss at 2 km, compared with free space.

Solution. λ = 0.125 m, so db = 4 × 10 × 1.5/0.125 = 480 m. At 2 km the two-ray loss is 40 × 3.301 − 20 log10 15 = 132.0 − 23.5 = 108.5 dB, against 106.1 dB in free space. The difference grows by 20 dB per decade of distance beyond the breakpoint: at 20 km it would be 22.5 dB.

Key idea. Real propagation adds losses to free space: obstacles in the Fresnel zone, ground reflections that steepen the distance law beyond a breakpoint, and absorption by gases and rain that grows with frequency.

## 14.3 Diffraction over an obstacle

An obstacle that blocks the line of sight does not create a perfect shadow; the wave diffracts over its edge. The knife-edge model expresses the obstruction as a dimensionless parameter ν = h√(2(d1 + d2)/(λd1d2)), with h the height of the edge above the line of sight, and the diffraction loss as approximately 6.9 + 20 log10(√((ν − 0.1)² + 1) + ν − 0.1) dB for ν > −0.7. An edge exactly on the line of sight, ν = 0, costs 6.0 dB. Real terrain with several obstacles and rounded hills is handled by extensions of the model and by terrain databases in planning tools.

ν = h √(2(d1 + d2)/(λ d1 d2)),   J(ν) ≈ 6.9 + 20 log10[√((ν − 0.1)² + 1) + ν − 0.1]  dB

## Worked Example 14.3

Problem. On the link of Example 14.1, a ridge at mid-path rises 5 m above the line of sight. Find the diffraction loss.

Solution. ν = 5 × √(2 × 10,000/(0.0517 × 5000 × 5000)) = 0.622, so J = 6.9 + 20 log10(√(0.522² + 1) + 0.522) = 11.3 dB. The link's fade margin, 27.3 dB in Example 30.1, would fall to 16 dB.

## 14.4 Gases and rain

Above about 10 GHz the atmosphere absorbs. Water vapor has a line near 22 GHz, and oxygen a strong band near 60 GHz, where absorption reaches about 15 dB/km at sea level, which limits 60 GHz links to short ranges but also isolates neighboring links from one another. Rain attenuates more strongly with frequency and rain rate. The ITU-R P.838 model gives the specific attenuation as γ = kR^α dB/km with coefficients tabulated by frequency; at 30 GHz with horizontal polarization, k ≈ 0.240 and α ≈ 0.949, so a heavy 25 mm/h rain costs about 5.1 dB/km. Millimeter-wave link budgets must include these losses with the availability statistics of Chapter 30.

γrain = k R^α  dB/km

## Practice

1. Find the first Fresnel radius at the midpoint of a 2 km, 24 GHz path.

2. Find the two-ray breakpoint for 30 m and 2 m antennas at 3.5 GHz.

3. Find the free-space loss of a 500 m, 60 GHz link and the total with 15 dB/km of oxygen absorption.

4. What is the diffraction loss when the edge just grazes the line of sight?

5. How much rain attenuation does a 2 km, 30 GHz link suffer at 25 mm/h?

6. Why is path loss beyond the breakpoint independent of frequency in the two-ray model?

# Chapter 15. Ferrites, Circulators, and Isolators

Every network in this book so far has been reciprocal: a signal travels from port 1 to port 2 exactly as it travels from port 2 to port 1, and S₂₁ equals S₁₂. Magnetized ferrite breaks that symmetry. In a ferrite the electron spins precess about the bias field in one direction only, so a wave that rotates with the precession sees a different material from one that rotates against it. That single fact gives the circulator, which sends power around its ports in one direction, and the isolator, which passes power forward and absorbs it backward. This chapter explains how they work, what their datasheet numbers mean, and how they protect amplifiers and share antennas.

## 15.1 Gyromagnetic resonance

An electron spin in a magnetic field precesses like a top at a rate proportional to the field: about 28 GHz per tesla. In a ferrite, a ceramic of iron oxide with other metal oxides such as yttrium iron garnet, the spins act together, and a microwave field rotating in the same sense as the precession is absorbed strongly near the resonance, while a field rotating in the opposite sense passes almost untouched. Devices operate a safe distance from resonance, where the two senses still see different permeabilities but the loss is low. The bias field comes from a small permanent magnet built into the device.

f_res = γ H,   γ ≈ 28 GHz/T

## Worked Example 15.1

Problem. Find the gyromagnetic resonance for an internal field of 0.1 T.

Solution. f_res = 28 GHz/T × 0.1 T = 2.8 GHz. A device for the 2.45 GHz band biased at 0.1 T would be too close to resonance to have low loss, so the designer chooses either a weaker field, operating below resonance, or a stronger field, operating above it. The choice is made by the manufacturer; the user meets it as the device's specified band.

## 15.2 The three-port circulator

The junction circulator is a ferrite disk between two ground planes, or under a microstrip conductor, with three lines meeting it at 120°. Biased correctly, the disk sets up a field pattern rotated so that power entering port 1 leaves at port 2, power entering port 2 leaves at port 3, and power entering port 3 leaves at port 1. An ideal circulator's S-matrix has zeros on its diagonal and ones in a rotating pattern; a real one has an insertion loss of a few tenths of a decibel and an isolation, the leakage in the wrong direction, of about 20 dB over its band.

S = [[0, 0, 1], [1, 0, 0], [0, 1, 0]]  (ideal circulator)

## 15.3 The isolator

Terminating port 3 of a circulator in a matched resistor makes an isolator. Power from the source passes from port 1 to port 2 and on to the load; any power the load reflects enters port 2, is routed to port 3, and is dissipated in the resistor instead of returning to the source. The source therefore sees a good match regardless of the load. The resistor must be rated for the reflected power, which in a fault, such as a disconnected antenna, is all the forward power.

## Worked Example 15.2

Problem. A 10 W amplifier drives an antenna whose VSWR is 3:1 through an isolator with 20 dB isolation. Find the power reflected by the antenna, the power dissipated in the isolator's termination, and the effective reflection coefficient the amplifier sees.

Solution. VSWR 3:1 gives |Γ| = (3 − 1)/(3 + 1) = 0.5, so 25% of the power, 2.5 W, is reflected. Nearly all of it goes to the termination, which must therefore handle at least 2.5 W and, for the open-antenna fault, the full 10 W. The leakage back toward the amplifier is 0.25 × 0.01 = 0.0025 of the forward power, an effective |Γ| of √0.0025 = 0.05 or a VSWR of 1.11. The amplifier sees nearly a perfect load, which keeps its gain, linearity, and stability as designed in Part II.

Key idea. Biased ferrite is non-reciprocal. A circulator routes power around its ports in one direction, and an isolator, a circulator with a terminated port, sends reflected power into a resistor instead of back to the source.

## 15.4 Circulators as duplexers

A circulator can let a transmitter and receiver share one antenna at the same frequency: transmitter at port 1, antenna at port 2, receiver at port 3. Two leakage paths limit it. The first is the circulator's own isolation from port 1 to port 3. The second is usually larger: the antenna's mismatch reflects part of the transmitted power back into port 2, and the circulator dutifully routes it to the receiver. The antenna's return loss therefore sets the isolation as much as the ferrite does, and radar and full-duplex systems add cancellation or a limiter in front of the receiver.

## Worked Example 15.3

Problem. A 30 dBm transmitter feeds an antenna through a circulator with 20 dB isolation. The antenna's return loss is 15 dB. Estimate the power reaching the receiver port by each path.

Solution. Direct leakage is 30 − 20 = 10 dBm. The antenna reflection is 30 − 15 = 15 dBm, routed to the receiver with only the circulator's insertion loss. The reflection dominates, and 15 dBm is enough to compress or damage a low-noise amplifier. Improving the antenna's match to 25 dB return loss lowers that path to 5 dBm, which shows why the antenna is part of the isolation budget.

## 15.5 Practical limits

Ferrite devices are band-limited, typically to an octave or less, because the bias must stay in the low-loss region across the band. Their properties drift with temperature, since the ferrite's magnetization falls as it warms, and they can be detuned by nearby steel or magnets. High power heats the ferrite and can drive it nonlinear. At millimeter wave, and on chips where a magnet is not practical, designers turn to active circulators built from transistors or to time-modulated networks, which break reciprocity electronically at the cost of noise and linearity.

## Practice

1. Find the gyromagnetic resonance for an internal field of 0.05 T.

2. An antenna has VSWR 2:1. What fraction of the forward power does it reflect?

3. With 25 dB isolation, what effective |Γ| does a source see behind an isolator when the load reflects all its power?

4. What happens at a circulator's port 3 when power enters port 2?

5. A 33 dBm transmitter feeds an antenna with 12 dB return loss through a circulator. Find the power the antenna reflects toward the receiver.

6. Why must an isolator's termination be rated for the full forward power?

# Chapter 16. Measuring Antennas: Ranges and Gain

Chapter 7 defined an antenna's gain and pattern, and every link budget in the series uses those numbers. They come from measurement. Measuring an antenna is harder than measuring a circuit because the quantity of interest exists only at a distance, in the far field, and because the room itself reflects. This chapter explains the far-field condition, the ranges built to satisfy it, and the two standard ways to measure gain.

## 16.1 The far-field distance

Near an antenna, the field has reactive components and its shape changes with distance. Far away, the wave from every part of the aperture arrives nearly in phase, and the pattern stops changing. The usual criterion is that the phase difference between a ray from the center of the aperture and a ray from its edge be no more than π/8, or 22.5°. That gives the far-field distance in terms of the largest dimension D of the antenna, and it grows with the square of the size.

R_ff = 2D² / λ

## Worked Example 16.1

Problem. Find the far-field distance of a horn with a 0.2 m aperture at 10 GHz and of a 1 m dish at 30 GHz.

Solution. At 10 GHz, λ = 0.03 m, so R_ff = 2 × 0.04/0.03 = 2.67 m, which fits indoors. At 30 GHz, λ = 0.01 m, so R_ff = 2 × 1/0.01 = 200 m, which does not. Large, high-frequency antennas are measured outdoors, in a compact range, or with near-field scanning.

## 16.2 Ranges

An anechoic chamber lined with pyramidal absorber, reflecting −40 dB or better at the frequencies of use, provides a far-field range for small antennas. The region where the incident field is uniform enough for measurement is the quiet zone. A compact range uses a large, precise reflector to convert the spherical wave of a feed into a plane wave within a few meters, so a large antenna can be measured indoors. A near-field scanner samples the field's amplitude and phase on a surface close to the antenna, at spacings of no more than half a wavelength, and computes the far-field pattern by a Fourier transform; it trades room size for precise positioning and computation.

Key idea. Gain and pattern are far-field quantities. When 2D²/λ is too far for a room, a reflector or a near-field scan with a transform provides the plane wave instead.

## 16.3 Gain by comparison

The most common gain measurement compares the antenna under test with a reference antenna of known gain, usually a calibrated standard-gain horn. Each is placed in turn at the receiving position under identical conditions, and the received powers are recorded. The difference in received power in decibels is the difference in gain.

G_AUT (dBi) = G_ref (dBi) + P_AUT (dBm) − P_ref (dBm)

## Worked Example 16.2

Problem. A 15.0 dBi standard horn receives −32.4 dBm. With the antenna under test in its place, the received power is −29.8 dBm. Find the gain of the antenna under test.

Solution. G_AUT = 15.0 + (−29.8) − (−32.4) = 17.6 dBi. The method relies on both antennas being matched equally well, so the mismatch loss of each, from Chapter 2, is measured and corrected.

## 16.4 Gain from the Friis equation

Without a reference, the gain follows from the link equation of Chapter 7. With two identical antennas facing each other at a known distance, the measured ratio of received to transmitted power equals the product of the gains times the path factor. With three different antennas, measured in all three pairs, the three equations can be solved for each gain without assuming any two are identical. This three-antenna method is how standard horns are themselves calibrated.

Pr/Pt = G₁ G₂ (λ / 4πR)²

## Worked Example 16.3

Problem. Two identical horns are 5 m apart at 10 GHz. The measured Pr/Pt is −36.4 dB. Find each horn's gain.

Solution. The path factor is 20 log(0.03/(4π × 5)) = −66.4 dB. Then G₁G₂ = −36.4 − (−66.4) = 30.0 dB, and since the horns are identical, each is 15.0 dBi. With D = 0.2 m, the far-field distance is 2.67 m, so 5 m satisfies it.

## 16.5 Pattern, polarization, and efficiency

A pattern is measured by rotating the antenna under test on a positioner while recording the received power, usually in two principal planes, and then over a full sphere when the total radiated power is wanted. Rotating the source antenna's polarization measures the cross-polar level. Integrating the pattern over the sphere gives directivity; comparing it with the measured gain gives the radiation efficiency, which for a small antenna, as Chapter 13 explained, can be far below unity. For phones and IoT devices, over-the-air tests measure total radiated power and total isotropic sensitivity with the device's own radio running, which captures the antenna, the matching, and the body of the product together.

## Practice

1. Find the far-field distance of a 0.3 m antenna at 5 GHz.

2. Find the far-field distance of a 0.1 m antenna at 60 GHz.

3. A 20.0 dBi reference receives −40.0 dBm and the antenna under test receives −43.5 dBm. Find the antenna's gain.

4. Find the path factor at 10 GHz and 10 m.

5. Two identical antennas at 10 GHz and 10 m give Pr/Pt = −52.4 dB. Find each gain.

6. What maximum sample spacing does a near-field scanner use?

# Chapter 17. Noise in Passive Networks

A passive network adds no gain, but it does add noise. Any component that absorbs power at a physical temperature also emits thermal noise, and in a receiver the loss in front of the first amplifier is often the largest single term in the noise budget. Chapter 25 develops the receiver's noise cascade; this chapter supplies the passive parts of it: the noise of a lossy line or attenuator, the noise temperature of an antenna, and the rule that explains why the low-noise amplifier goes at the antenna.

## 17.1 Noise temperature

Noise power in a bandwidth B is kTB, where k is Boltzmann's constant, 1.381 × 10⁻²³ J/K. It is often convenient to describe a component's added noise by an equivalent temperature Te, the temperature a matched resistor at the input would need to produce the same added noise. Noise figure and noise temperature are two expressions of the same quantity, related through the reference temperature T₀ = 290 K.

N = kTB,   Te = T₀(F − 1),   F = 1 + Te/T₀

## 17.2 The lossy line and the attenuator

A matched attenuator with loss factor L (L > 1) at the physical temperature T passes 1/L of its input and emits noise of its own. Thermodynamics fixes the amount: if the input were a resistor at the same temperature T, the output must still look like a resistor at T. That condition gives the attenuator's equivalent input noise temperature and, at T₀, a noise figure equal to its loss. A cable with 2 dB of loss at room temperature has a noise figure of 2 dB.

Te = (L − 1)T,   F = L at T = T₀,   Tout = Tin/L + (1 − 1/L)T

## Worked Example 17.1

Problem. A 2 dB cable at 290 K feeds an amplifier with a 1 dB noise figure and high gain. Find the cascade noise figure.

Solution. By the cascade of Chapter 25, F = F_cable + (F_amp − 1)/G_cable = L + (F_amp − 1)L = L F_amp, so in decibels the noise figures simply add: 2 + 1 = 3.0 dB. Every decibel of loss in front of the first amplifier adds a decibel to the noise figure.

## 17.3 Antenna temperature

An antenna delivers the noise of whatever it sees, weighted by its pattern. Pointed at the clear sky at high elevation, a microwave antenna sees a cold background, a few kelvin from space plus the atmosphere's emission, and its antenna temperature may be tens of kelvin. Pointed at the ground or a wall, it sees about 290 K. This is why satellite ground stations, whose antennas look at the cold sky, can benefit from an LNA with a noise temperature of tens of kelvin, while a terrestrial antenna looking at the horizon gains little from an amplifier much better than its surroundings.

## Worked Example 17.2

Problem. An antenna with T_A = 50 K is connected through 0.5 dB of feed loss at 290 K to an LNA with a 0.5 dB noise figure and 20 dB gain. Find the system noise temperature referred to the antenna terminals, then repeat with the LNA moved ahead of the loss. Find the noise power in 1 MHz for each.

Solution. L = 10^0.05 = 1.122, so the loss and the LNA each have Te = 0.122 × 290 = 35.4 K. With the loss first, the system temperature at the antenna is T_A + Te,loss + L·Te,LNA = 50 + 35.4 + 39.7 = 125.1 K. With the LNA first, it is T_A + Te,LNA + Te,loss/G = 50 + 35.4 + 0.35 = 85.7 K. In 1 MHz, kTB is −117.6 dBm for 125.1 K and −119.3 dBm for 85.7 K. Moving the LNA ahead of only half a decibel of loss improves the noise floor by 1.6 dB, which is why masthead and feed-mounted LNAs exist.

Key idea. Loss at physical temperature T adds noise temperature (L − 1)T, so a passive loss at 290 K adds its full decibel value to the noise figure. Put the first amplifier before the loss.

## 17.4 Mismatch and noise

A low-noise transistor reaches its minimum noise figure only when it sees a particular source impedance, generally not the conjugate match that gives maximum gain. The input network of an LNA is therefore a compromise between noise match and power match, drawn on the Smith chart of Chapter 4 as circles of constant noise figure and constant gain. The input mismatch that results is often accepted, and an isolator or a balanced amplifier, Chapter 9's hybrids at each end, restores a good return loss without disturbing the noise match.

## Practice

1. Find the noise temperature of an amplifier with a 2 dB noise figure.

2. Find the noise figure of an amplifier with Te = 75 K.

3. Find the equivalent noise temperature of a 3 dB attenuator at 290 K.

4. Find kTB at 290 K in 1 MHz, in dBm.

5. A 1.5 dB cable precedes an LNA with a 0.8 dB noise figure. Find the cascade noise figure.

6. Find the output noise temperature of a 3 dB attenuator at 290 K whose input is a 10 K source.

# Chapter 18. A Bandpass Filter from Specification to Layout

Chapter 6 gave the prototype values and the loss formula for filters, and Chapter 12 gave the resonators. This chapter carries one filter all the way through, from a specification to the coupling numbers a layout must realize, so that the steps can be seen in order. The example is a third-order Chebyshev bandpass filter for the 2.45 GHz band on the book's reference microstrip, with a comparison against the same filter built from cavities.

## 18.1 The specification

The filter must pass 2.40 to 2.50 GHz with at most 0.1 dB of ripple, and it should reject a signal at 2.60 GHz as strongly as possible with three resonators. The center frequency is f₀ = 2.45 GHz, and taking the passband edges as about 2.40 and 2.50 GHz with a little margin gives a fractional bandwidth FBW of 4%, about 98 MHz.

## 18.2 From prototype to couplings

A coupled-resonator filter is described by two kinds of numbers: the external quality factor Qe, which sets how strongly the first and last resonators are loaded by the source and load, and the coupling coefficients Mᵢ,ᵢ₊₁ between adjacent resonators. Both follow directly from the lowpass prototype values gₖ and the fractional bandwidth. For the 0.1 dB Chebyshev prototype of third order, g₁ = g₃ = 1.0316 and g₂ = 1.1474.

Qe = g₀g₁ / FBW,   Mᵢ,ᵢ₊₁ = FBW / √(gᵢ gᵢ₊₁)

## Worked Example 18.1

Problem. Find Qe and the coupling coefficients for the 4% filter.

Solution. Qe = 1.0316/0.04 = 25.8. M₁₂ = M₂₃ = 0.04/√(1.0316 × 1.1474) = 0.04/1.088 = 0.0368. The layout must now produce a tap or gap that loads the end resonators to Qe = 25.8, and gaps between resonators that couple them by 3.68%. Electromagnetic simulation of two resonators at a time finds those dimensions: the coupling is half the fractional separation of the two resonant peaks a pair of coupled resonators shows.

## 18.3 The resonators

On the reference microstrip, with εeff = 2.79, the guided wavelength at 2.45 GHz is λ₀/√εeff = 12.24/1.670 = 7.33 cm, so a half-wave resonator is about 3.66 cm long before corrections for end effects. Folding the resonators into hairpins or squares shortens the filter and lets the coupling be set by the spacing of parallel arms. The unloaded Q of such microstrip resonators is typically about 200, set by conductor and dielectric loss, and it, not the topology, determines the passband loss.

## Worked Example 18.2

Problem. Find the midband loss with Qu = 200 and with Qu = 2000.

Solution. The formula of Chapter 6 gives loss ≈ 4.343 Σgₖ/(FBW Qu). Σgₖ = 3.2106, so with Qu = 200 the loss is 4.343 × 3.2106/(0.04 × 200) = 1.74 dB, and with Qu = 2000, as with Chapter 12's cavities or dielectric resonators, it is 0.17 dB. The tenfold Q costs volume and money, and it is needed only when the 1.6 dB matters, as it does ahead of an LNA by Chapter 17's rule.

Key idea. A coupled-resonator filter is specified by its external Q and coupling coefficients, which follow from the prototype and the bandwidth. The resonators' unloaded Q then sets the loss.

## 18.4 Rejection

The rejection of the Chebyshev response at a frequency f follows from the lowpass prototype after the bandpass transformation, which maps f to a normalized frequency Ω. For the 0.1 dB ripple, ε² = 10^0.01 − 1 = 0.0233, and the attenuation of an n-th order filter is computed from the Chebyshev polynomial.

Ω = (1/FBW)(f/f₀ − f₀/f),   A = 10 log₁₀[1 + ε² cosh²(n arccosh |Ω|)]

## Worked Example 18.3

Problem. Find the rejection of the third-order filter at 2.60 GHz and at 2.70 GHz.

Solution. At 2.60 GHz, f/f₀ − f₀/f = 1.0612 − 0.9423 = 0.1189, so Ω = 2.97 and A = 23.4 dB. At 2.70 GHz, Ω = 4.87 and A = 36.7 dB. If 2.60 GHz needs 40 dB, the designer must add a resonator, narrow the bandwidth, or add a transmission zero by cross-coupling the first and third resonators. Each choice raises the passband loss, so the specification and the loss are negotiated together.

## 18.5 Tuning and tolerance

A 4% filter is sensitive to its dimensions. The resonant frequency of a half-wave resonator scales with 1/√εeff, so a laminate whose εr varies by ±2% moves the center by about ±1%, a quarter of the passband. Production filters therefore either use laminates with tight εr control, as Book 7 discusses, or provide tuning screws or laser trimming. Simulation should include a tolerance sweep before the layout is released.

## Practice

1. Find Qe for a 0.1 dB, third-order filter with FBW = 5%.

2. Find M₁₂ for the same filter.

3. Find the half-wave resonator length on a microstrip with εeff = 2.79 at 5.0 GHz.

4. Find the midband loss of the 4% filter with Qu = 500.

5. Find Ω for the 4% filter at 2.30 GHz.

6. Find the rejection at 2.30 GHz.

# Chapter 19. RF Switches

Every radio selects paths: one antenna or another, one band filter or another, the transmit or the receive side. The element that does it is a switch, a PIN diode, a silicon-on-insulator transistor, or a MEMS contact, and at RF it is analyzed with the impedances and reflection coefficients of this book. This chapter computes a switch's insertion loss and isolation and shows how series and shunt elements combine. The passive mixer, which is also a switch, is treated with the other mixers in Chapter 29.

## 19.1 The series switch

A PIN diode, a silicon-on-insulator transistor, or a MEMS contact can act as a switch. In series with a 50 Ω line, the switch's on-state resistance R causes insertion loss, and its off-state capacitance C lets signal leak through, limiting isolation. For a series element of impedance Z between matched source and load, the transmission is 2Z₀/(2Z₀ + Z).

IL = 20 log₁₀|1 + Z/(2Z₀)|,   Z_on = R,   Z_off = 1/(jωC)

## Worked Example 19.1

Problem. A series switch has R = 1 Ω on and C = 0.1 pF off. Find the insertion loss and the isolation at 2.45 GHz.

Solution. On, IL = 20 log(1 + 1/100) = 0.086 dB. Off, the capacitance has a reactance of 1/(2π × 2.45×10⁹ × 0.1×10⁻¹²) = 650 Ω, and the isolation is 20 log|1 − j6.50| = 16.4 dB. That isolation is poor for a transmit-receive switch, and it worsens with frequency as the reactance falls.

## 19.2 Shunt switches and the series-shunt pair

A switch to ground across the line works the other way: off, its small capacitance barely loads the line; on, its low resistance shorts the line and reflects the signal. For a shunt element Z, the transmission is 2Z/(2Z + Z₀). Practical switches combine a series element in the through path with a shunt element that turns on when the series one turns off, so the off state both blocks and shorts the signal. A quarter-wave line between a shunt switch and the junction, Chapter 2's impedance inverter, turns the short into an open at the junction so the off arm does not load the on arm.

S₂₁(shunt) = 2Z / (2Z + Z₀)

## Worked Example 19.2

Problem. A shunt switch has R = 1 Ω when on. Find the isolation it provides.

Solution. |S₂₁| = 2/(2 + 50) = 0.0385, an isolation of 28.3 dB, in addition to the 16 dB of the series element if both are used. Isolations of 40 dB and more come from such series-shunt combinations, at the cost of slightly more insertion loss.

Key idea. A series switch's isolation is set by its off capacitance and falls with frequency; a shunt switch to ground, or a series-shunt pair, restores it.

## Practice

1. Find the insertion loss of a series switch with R = 2 Ω.

2. Find the isolation of a series switch with C = 0.1 pF at 5.8 GHz.

3. Find the isolation of a shunt switch with R = 2 Ω.

# Part II. Active Circuits: Amplifiers, Receivers, Oscillators, and Synthesizers

# Chapter 20. The Transmitter Chain and Its Devices

Book 5 built the waveform. This book makes it powerful enough to cross a room, a city, or the distance to a satellite, and then receives it again when it arrives weak and noisy. The transmitter is a chain, and every link in it either delivers watts or loses them: the device, the load line it is designed to, the amplifier class, the output match, the bias, the heat sink, the combiner, the isolator, the harmonic filter, the feed line, the duplexer, and the antenna, whose output is the effective isotropic radiated power that the regulator limits. The receiver half of a radio, the low-noise amplifier, the noise cascade, and the oscillator that tunes both halves sit in this book as well, because they share the same box and the same engineering. Why gallium nitride withstands the voltage is Book 3's question, antenna gain is Part I's, and the test chamber is Book 7's. This book owns the watts.

## 20.1 Power in decibels

Radio engineers count power in dBm, decibels relative to one milliwatt: P(dBm) = 10 log₁₀(P/1 mW). One watt is 30 dBm, 100 W is 50 dBm, and 1 kW is 60 dBm. Gains and losses in decibels add. The effective isotropic radiated power is the transmitter output, minus the loss between the amplifier and the antenna, plus the antenna gain in dBi. A loss of 1 dB is a factor of 10^(−0.1) = 0.794 in power: a fifth of the watts, gone as heat in a cable.

EIRP (dBm) = Pout (dBm) − Lfeed (dB) + Gant (dBi)

## Worked Example 20.1

Problem. A 100 W amplifier feeds a 17 dBi sector antenna through 1 dB of cable and connectors. Find the power at the antenna and the EIRP in dBm and watts.

Solution. 100 W is 50 dBm. After 1 dB the power is 49 dBm, or 100 × 0.794 = 79.4 W. The EIRP is 49 + 17 = 66 dBm, which is 10^(3.6) mW = 3981 W, about 4 kW. The 20.6 W lost in the feed heats the cable and buys nothing; one decibel saved after the last amplifier is worth exactly as much as one decibel of extra amplifier output, and it is almost always cheaper.

Key idea. One decibel after the last amplifier is one decibel of EIRP. Every loss between the device and the antenna must be paid for by a bigger, hotter, more expensive amplifier.

## 20.2 The device families

Five technologies cover nearly all power amplifiers in 2026. Silicon LDMOS, the laterally diffused MOS transistor, dominates from HF to about 3 GHz: it is cheap, rugged, and runs from 28 or 50 V supplies, and it powers broadcast transmitters, industrial heating, and most sub-3 GHz cellular base stations built before gallium nitride took over. Gallium nitride on silicon carbide, GaN-on-SiC, covers roughly 0.5 to 40 GHz. Its wide bandgap lets it run at 28 to 50 V with power densities several times those of gallium arsenide per millimeter of gate, and its silicon-carbide substrate conducts heat well; it is the default for new base stations, radar, and satellite links. Gallium arsenide, as heterojunction bipolar transistors and pHEMTs, supplies drivers and the few-watt amplifiers in phones and Wi-Fi. Indium phosphide offers the highest transistor speed and is used for sub-terahertz work and low noise, not for kilowatts. Vacuum tubes, the traveling-wave tube, klystron, and magnetron, remain where kilowatts to megawatts are needed at high frequency, because combining enough transistors becomes more expensive than one tube.

## 20.3 Supply voltage and power

A transistor's output power is set by how much voltage and current it can swing. The voltage is limited by breakdown, which sets the supply, and the current by the device's size. Chapter 21 makes this precise with the load line, but the consequence is already clear: at a given power, a higher supply means a lower current and a higher optimum load resistance, which is closer to the 50 Ω of the outside world and easier to match. That is the main reason high-voltage gallium nitride displaced gallium arsenide in base stations, and why a phone amplifier running from a 3.4 V battery must transform its load by a large ratio, losing efficiency in the process.

## Worked Example 20.2

Problem. A radar transmitter needs 2 kW peak at the antenna through 0.6 dB of loss. Its GaN modules each deliver 250 W at the combiner input, and the combiner loses 0.4 dB. How many modules are needed?

Solution. The total loss is 1.0 dB, a factor of 0.794, so the modules together must supply 2000/0.794 = 2518 W. At 250 W each, that is 10.07 modules, so eleven, or in a binary combiner tree, sixteen. A tree of eight would deliver only 8 × 250 × 0.794 = 1589 W. The arithmetic shows why combining loss limits the solid-state route: every doubling of the module count adds a combining stage and its loss.

## Practice

1. Convert 5 W, 250 W, and 2 kW to dBm.

2. A 20 W amplifier feeds a 3 dB cable run to a 12 dBi antenna. Find the EIRP in dBm and the power lost in the cable.

3. What fraction of the power is left after 2 dB of loss?

4. Which device family would you choose for a 200 W amplifier at 3.5 GHz, and why?

5. An amplifier loses 0.5 dB in its output isolator and 0.5 dB in its harmonic filter. What output power does it need to deliver 40 W after both?

6. Why does a higher supply voltage make the output match easier?

# Chapter 21. The Load Line

A small-signal amplifier is matched for gain: its load is chosen so that the most power is transferred, which means a conjugate match to the device's output impedance. A power amplifier is matched for power, and that is a different load. The device can swing its voltage only between a knee, below which its current collapses, and a maximum set by the supply and breakdown, and it can swing its current only between zero and a maximum. The load that lets it use the whole of both ranges at once is the optimum load, Ropt, and finding it is the first step of every power-amplifier design. The method is due to Steve Cripps, whose RF Power Amplifiers for Wireless Communications remains the reference.

## 21.1 The optimum load

With a supply VDD and a knee voltage Vknee, the drain voltage can swing by VDD − Vknee on either side of the supply, so the peak fundamental voltage is VDD − Vknee. With a sinusoidal current, the fundamental output power is the product of the peak voltage and peak current divided by two. For a target output power P, the optimum load resistance follows directly, and it is a resistance at the intrinsic drain, behind the device's output capacitance and package parasitics, which the matching network must also absorb.

Ropt = (VDD − Vknee)² / (2P)

## Worked Example 21.1

Problem. Find Ropt for a 100 W amplifier on a 50 V GaN process with a 5 V knee, and on a 28 V LDMOS process with a 3 V knee. Find the ratio of 50 Ω to each, and the peak fundamental current in each case.

Solution. At 50 V, Ropt = 45²/200 = 10.125 Ω, which this book writes as 10.13 Ω. At 28 V, Ropt = 25²/200 = 3.125 Ω, written 3.13 Ω. The transformation ratios to 50 Ω are 4.9 and 16. The peak fundamental currents are 2P/(VDD − Vknee) = 200/45 = 4.44 A and 200/25 = 8.0 A. The 28 V design needs nearly twice the current and so a larger device, with more output capacitance to tune out, and a matching network with three times the impedance ratio.

## 21.2 Why the conjugate match is wrong

The device's output resistance, the slope of its current against drain voltage, is typically hundreds of ohms for a large transistor. A conjugate match would present that resistance as the load, and the voltage swing would hit the knee and the supply long before the current reached its maximum: the device would deliver a small fraction of the power it could. The load-line match accepts a lower gain in exchange for using the full current and voltage range. The difference between the two, often 2 to 3 dB of output power, is the reason power amplifiers are designed by load-pull measurements, which sweep the load and map power and efficiency contours, rather than by small-signal S-parameters.

Key idea. A power amplifier is matched for power, not gain. The optimum load is set by the voltage and current the device can swing, and at a given power it scales with the square of the voltage.

## 21.3 The matching network

The output network must transform 50 Ω down to Ropt and absorb the device's output capacitance. For a single L-section, Part I's result gives Q = √(Rhigh/Rlow − 1). For the 50 V design, Q = √(4.94 − 1) = 1.98; for the 28 V design, Q = √(16 − 1) = 3.87. A higher Q means a narrower bandwidth, larger circulating currents, and more loss in the network's finite-Q components. Wideband or high-ratio designs use two or more sections, each with a lower ratio, or a transmission-line transformer. In a phone the problem is worse: at 3.4 V with a 0.3 V knee, 1 W needs Ropt = 3.1²/2 = 4.8 Ω, and 2 W needs 2.4 Ω, a 21:1 ratio.

## Worked Example 21.2

Problem. A 200 W design on the 50 V process uses the same 5 V knee. Find Ropt, the single-section Q to 50 Ω, and compare it with the 100 W design.

Solution. Ropt = 45²/400 = 5.06 Ω. The ratio is 50/5.06 = 9.9, so Q = √(9.9 − 1) = 2.98. Doubling the power at the same voltage halves Ropt and raises the single-section Q from 1.98 to 2.98. To get back to the 100 W impedance level, the designer would combine two 100 W devices, Chapter 24's subject, or move to a higher supply.

## 21.4 Raising the rail

The scaling Ropt ∝ V²/P is the main argument of high-voltage design. Doubling the supply at constant power quadruples the optimum load and halves the current. Gallium nitride's breakdown field, about ten times that of silicon, permits 48 to 50 V supplies at microwave frequencies, and some designs use 65 V or more. The cost is that the knee voltage, the drain-source capacitance, and the trapping effects that cause memory in the amplifier's response all change with voltage, and the designer must characterize them at the operating point.

## Practice

1. Find Ropt for 200 W at 50 V with a 5 V knee, and for 50 W at 28 V with a 3 V knee.

2. Find the peak fundamental drain current for 100 W at 50 V with a 5 V knee.

3. Find the single-section L-network Q to transform 50 Ω to 3.13 Ω.

4. A phone amplifier at 3.4 V has a 0.3 V knee. Find Ropt for 1 W.

5. By what factor does Ropt change when the supply is doubled at constant power and negligible knee?

6. Why is a power amplifier's output not conjugately matched?

# Chapter 22. Amplifier Classes and Efficiency

Chapter 21 fixed the load. This chapter fixes the bias, and with it the fraction of each RF cycle during which the transistor conducts. That choice, named by the amplifier's class, decides the efficiency: how much of the DC supply power becomes RF and how much becomes heat. In a base station that heat is the operator's largest electricity bill after air conditioning; in a phone it is battery life. The classes fall into two families: the current-source classes A, AB, B, and C, which trade linearity for efficiency by conducting for less of the cycle, and the switching and harmonic-tuned classes D, E, and F, which shape the voltage and current waveforms so that they do not overlap.

## 22.1 Efficiency measures

Drain efficiency is the RF output power divided by the DC supply power. Power-added efficiency subtracts the input drive, which matters when the gain is low, as it is at millimeter wave. The power dissipated in the device is the DC input plus the RF input minus the RF output, and it is the number the heat sink of Chapter 24 must handle.

ηD = Pout / PDC,    PAE = (Pout − Pin) / PDC,    Pdiss = PDC + Pin − Pout

## Worked Example 22.1

Problem. A 100 W amplifier has 15 dB of gain and 65% drain efficiency. Find the input power, DC power, PAE, and dissipation.

Solution. The gain is a factor of 31.6, so Pin = 100/31.6 = 3.16 W. PDC = 100/0.65 = 153.8 W. PAE = (100 − 3.16)/153.8 = 62.9%. The dissipation is 153.8 + 3.16 − 100 = 57.0 W. At 12 dB of gain and 70% drain efficiency, a 40 W millimeter-wave stage would have PAE = (40 − 2.52)/57.1 = 65.6%, already four and a half points below its drain efficiency; at lower gains the gap grows quickly.

## 22.2 Conduction angle: classes A, AB, B, and C

In class A the transistor is biased at half its maximum current and conducts for the whole cycle. Its peak theoretical efficiency is 50%, and it is the most linear. Lowering the bias reduces the conduction angle, the part of the cycle over which current flows. In class B, biased at the threshold, the device conducts for half the cycle, 180°, and the theoretical peak efficiency is π/4 = 78.5%. Class AB lies between. In class C the angle is less than 180° and the efficiency rises further, but the fundamental output falls and the gain falls with it. For a conduction angle α, with an ideal linear device and a harmonic short at the output, the peak efficiency is given below.

η(α) = (α − sin α) / [4 (sin(α/2) − (α/2) cos(α/2))]

The formula gives 50.0% for class A at 360°, 60.2% at 270°, 78.5% for class B at 180°, 89.7% at 120°, and 94.0% at 90°. The fundamental output power, relative to class A with the same peak current and voltage, is +0.55 dB at 270°, 0 dB at 180°, −2.1 dB at 120°, and −4.2 dB at 90°. Deep class C therefore buys efficiency with power and gain, and is used for constant-envelope signals such as FM broadcast, where linearity does not matter. Most linear amplifiers run in class AB, a little above the threshold, to keep the gain flat at low drive.

## Worked Example 22.2

Problem. A device biased for a 120° conduction angle delivers its maximum output with a drain efficiency at the theoretical limit. If the same device in class B delivers 100 W, what does it deliver in this class C condition, and how much dissipation is saved?

Solution. At 120° the output is 2.14 dB below class B, so 100 × 10^(−0.214) = 61.1 W. In class B the DC power is 100/0.785 = 127.3 W and the dissipation 27.3 W. At 120°, the DC power is 61.1/0.897 = 68.1 W and the dissipation 7.0 W. Efficiency rose from 78.5% to 89.7%, but the device now delivers 39% less power: class C is a trade, not a free gain.

Key idea. Efficiency comes from keeping voltage and current from being large at the same time. Reducing the conduction angle does it at a cost in power and gain; shaping the waveforms with harmonics or switching does it without that cost, in principle reaching 100%.

## 22.3 Switching and harmonic-tuned classes

If the transistor acts as a switch, it has either zero voltage across it or zero current through it, and an ideal switch dissipates nothing. Class D uses two switches and a filter; it is common at audio and low radio frequencies. Class E uses one switch with a shunt capacitor and a tuned load designed so that the voltage returns to zero, with zero slope, at the instant the switch closes; this removes the loss of discharging the capacitor and makes class E practical into the gigahertz range. Class F adds open circuits at the odd harmonics and short circuits at the even ones, so the drain voltage approaches a square wave and the current a half-sine; inverse class F swaps them. Real devices have finite on-resistance, output capacitance, and transition times, and practical class E and F amplifiers reach 70 to 85% drain efficiency at a few gigahertz. All of these are efficient only at full drive. Chapter 23 addresses what happens when the signal's envelope varies.

## Practice

1. An amplifier delivers 50 W from 75 W of DC with 1 W of drive. Find its drain efficiency, PAE, and dissipation.

2. Find the theoretical efficiency at a conduction angle of 270°.

3. Why does class C reduce gain?

4. A class AB amplifier runs at 60% drain efficiency at 200 W output. Find the dissipation, neglecting the input.

5. A 30 dBm stage has 8 dB of gain and 50% drain efficiency. Find its PAE.

6. What condition at switch closing defines class E?

# Chapter 23. Linearity, Back-Off, and the Doherty Amplifier

Chapter 22's efficiencies are peak values, reached only when the amplifier is driven to its full output. The signals of Book 5 are not like that. An OFDM signal has a peak-to-average power ratio of 8 to 11 dB, so a linear amplifier that must pass its peaks spends most of its time at a fraction of its rated power, where its efficiency is poor. This chapter measures the linearity an amplifier needs, computes how much efficiency back-off costs, and shows how the Doherty amplifier, digital predistortion, and envelope tracking recover it. The Doherty is the default architecture of cellular base stations in 2026.

## 23.1 Compression and intermodulation

As drive rises, a real amplifier's gain begins to fall. The 1 dB compression point, P1dB, is the output power at which the gain is 1 dB below its small-signal value. With two closely spaced tones at the input, the amplifier's curvature generates third-order intermodulation products at 2f1 − f2 and 2f2 − f1, which fall inside or next to the wanted band and cannot be filtered out. Their power rises 3 dB for every 1 dB of input. Extrapolating the fundamental and third-order lines to their intersection defines the output third-order intercept, OIP3, a figure of merit that is typically about 10 dB above P1dB for a well-behaved amplifier.

OIP3 = Pout,tone + ΔIM3 / 2,  where ΔIM3 is the carrier-to-IM3 ratio in dB

## Worked Example 23.1

Problem. With two tones of 30 dBm each at the output, the third-order products are 40 dB below each tone. Find OIP3, and the IM3 ratio after backing the drive off by 3 dB.

Solution. OIP3 = 30 + 40/2 = 50 dBm. At 27 dBm per tone, the tones have fallen 3 dB and the products 9 dB, so the ratio becomes 40 + 6 = 46 dBc. Each decibel of back-off buys two decibels of intermodulation ratio, which is why the first remedy for a failing spectral mask, a subject for Book 7's chamber, is often less drive, at a cost in efficiency.

## 23.2 What back-off costs

A class A amplifier draws constant DC power, so its efficiency falls in proportion to output power: at 8 dB below peak it is 50% × 10^(−0.8) = 7.9%. A class B amplifier's DC current falls with the signal amplitude, so its efficiency falls only with the square root of output power: η = (π/4)√(Pout/Pmax), giving 55.6% at 3 dB back-off, 39.4% at 6 dB, and 31.3% at 8 dB. For a 100 W-peak class B stage carrying a signal with 8 dB of PAPR, the average output is 15.8 W and the DC input 50.7 W.

## 23.3 The Doherty amplifier

William Doherty's 1936 design uses two amplifiers. The main, or carrier, amplifier runs in class AB or B and handles the signal up to a set back-off; the auxiliary, or peaking, amplifier runs in class C and turns on only for the peaks. A quarter-wave line between them acts as an impedance inverter. At low power the main amplifier sees twice its optimum load, so it saturates and reaches peak efficiency at half its full voltage swing, which is 6 dB below the combined peak. Above that, the peaking amplifier's current pulls the main amplifier's load back down toward Ropt, a mechanism called load modulation, and both reach full power together. In the ideal symmetric Doherty the efficiency is 78.5% both at peak and at 6 dB back-off, dips only to about 70% between, and falls linearly with amplitude below 6 dB.

Below 6 dB back-off:  η = (π/4) (v / 0.5),  v = normalized output amplitude

## Worked Example 23.2

Problem. A 100 W-peak amplifier carries a signal with 8 dB of PAPR. Compare the efficiency and DC power at the average output for class B and for an ideal symmetric Doherty.

Solution. At 8 dB back-off the normalized amplitude is v = 10^(−8/20) = 0.398. Class B gives (π/4)(0.398) = 31.3%; the Doherty gives (π/4)(0.398/0.5) = 62.5%. For the 15.8 W average output, the class B stage draws 50.7 W and the Doherty 25.3 W, halving the DC power and cutting the dissipation from 34.8 W to 9.5 W. Real Doherty amplifiers reach 45 to 55% average efficiency on such signals, below the ideal because the peaking device turns on gradually and the inverter has loss and limited bandwidth. Asymmetric versions, with a larger peaking device, place the second efficiency peak at 8 to 10 dB to match the signal.

Key idea. A linear amplifier's average efficiency is set by the signal's peak-to-average ratio. The Doherty restores efficiency at back-off by modulating the main amplifier's load, and digital predistortion then restores the linearity the Doherty gives up.

## 23.4 Predistortion and envelope tracking

A Doherty amplifier is efficient but not linear: its gain and phase change with drive, the AM-to-AM and AM-to-PM distortion, and they depend on the recent history of the signal through heating and charge trapping, which are memory effects. Digital predistortion measures the amplifier's output through a feedback receiver, fits a model, often a memory polynomial, and pre-distorts the baseband signal with the inverse, so that the cascade is linear. It typically improves the adjacent-channel leakage ratio by 15 to 20 dB. Envelope tracking takes a different route: it varies the supply voltage with the signal's envelope, so the amplifier always runs close to saturation. It is widely used in phones, where a fast DC-DC converter can follow the envelope of a 100 MHz signal, and less in base stations, where Doherty is simpler at high power.

## Practice

1. Two tones of 20 dBm each produce IM3 products at −46 dBc. Find OIP3.

2. For the amplifier of the previous problem, what IM3 ratio results when each tone is reduced to 17 dBm?

3. Find a class B amplifier's efficiency at 10 dB back-off.

4. Find an ideal symmetric Doherty's efficiency at 10 dB back-off.

5. A 200 W-peak Doherty carries a signal with 9 dB of PAPR at 55% average efficiency. Find the average output and the DC power.

6. Why does an amplifier's IM3 ratio improve by 2 dB for each decibel of back-off?

# Chapter 24. Heat, Combining, and the Path to the Antenna

An amplifier that meets its power and efficiency targets on the bench can still fail in service, by overheating, by a single failed device taking down a combined array, or by losing its gains in the hardware between the device and the antenna. This chapter follows the power out of the transistor: through the thermal path that removes its waste heat, the combiners that add several devices together, the isolator and harmonic filter that protect and clean it, and the feed line that carries it to the antenna.

## 24.1 The thermal path

Heat flows from the transistor's channel, or junction, through the die and package to the flange, then through an interface material to the heat sink and into the air or coolant. Each step has a thermal resistance θ in °C/W, and the temperature rises add like voltage drops in a series circuit. The junction temperature sets the device's lifetime: GaN-on-SiC parts are typically rated for channel temperatures up to about 225 °C, and designers usually hold them well below 200 °C to keep the predicted lifetime long. Pulsed operation, as in radar, lets the peak temperature stay below the steady-state value, and transient thermal impedance curves give the peak rise for a given pulse width and duty cycle.

Tj = Tambient + Pdiss (θjc + θcs + θsa)

## Worked Example 24.1

Problem. The 100 W amplifier of Worked Example 22.1 dissipates 57.0 W. Its θjc is 0.8 °C/W, the interface adds 0.1 °C/W, and the heat sink 0.3 °C/W to air at 40 °C. Find the junction temperature. What sink resistance would hold Tj to 200 °C?

Solution. The total resistance is 1.2 °C/W, so the junction rises 57.0 × 1.2 = 68.4 °C, to 108.4 °C. The flange sits at 40 + 57.0 × 0.4 = 62.8 °C. For Tj = 200 °C, the total may be (200 − 40)/57.0 = 2.81 °C/W, so the sink could be as poor as 2.81 − 0.9 = 1.91 °C/W. The design has ample margin; the margin is what allows the amplifier to survive a hot day, a blocked fan, or a mismatched antenna.

## 24.2 Combining

When one device cannot supply the power, several are combined. The Wilkinson divider and combiner, the 90° hybrid, and spatial or radial combiners all add N equal, in-phase signals into one port without loss in the ideal case, and each dissipates the difference in an isolation resistor or load when the inputs are unequal. The output of an ideal combiner with N inputs is the square of the sum of the input amplitudes, divided by N. A phase error θ between two equal inputs leaves a fraction cos²(θ/2) of the power: 99.2% at 10°, 97.0% at 20°, and 93.3% at 30°. A failed device is more costly: with one of N inputs dead, the output falls to (N − 1)²/N² of full power, which is 25% for N = 2, 56% for N = 4, and 77% for N = 8. The rest is dissipated in the isolation loads, which must be rated for it.

Pout = (Σ √Pi)² / N   (ideal N-way in-phase combiner)

## Worked Example 24.2

Problem. Four 100 W modules feed a combiner with 0.3 dB of loss. Find the output with all four working and with one failed.

Solution. With all four, the output is 400 × 10^(−0.03) = 373 W. With one failed, the ideal output is (3√100)²/4 = 225 W, 56% of 400 W, and after the combiner loss 210 W. The isolation loads absorb 75 W, and must be rated for at least that continuously. The combined transmitter loses 2.5 dB, not the 1.25 dB that a quarter of the hardware might suggest.

Key idea. Combining multiplies power but also adds loss and creates a failure mode. A combiner's output falls by more than the failed fraction, because the surviving inputs no longer add in amplitude.

## 24.3 Isolator, harmonic filter, and duplexer

A ferrite isolator passes power forward and diverts reflected power into a load, so the amplifier always sees close to its design impedance even when the antenna is damaged or iced. A 2:1 VSWR at the antenna reflects 11.1% of the power; without an isolator, that reflection shifts the load seen by the device and can push its voltage beyond breakdown. The harmonic filter, a lowpass after the amplifier, attenuates the second and third harmonics, which a class AB stage generates at 20 to 30 dB below the carrier and which emission limits, the subject of Book 7, typically require at 60 dB or more below. A duplexer in a frequency-division system adds the receive filter, so the transmitter and receiver can share one antenna. Each element typically costs 0.2 to 0.5 dB.

## 24.4 The feed line

The last loss is the line to the antenna, which can be tens of meters on a tower. A cable with 6.6 dB of loss per 100 m at 2 GHz loses 1.98 dB in 30 m, leaving 63% of the power. Mounting the amplifier at the top of the mast, as in a remote radio head or an active antenna unit, removes that loss, and the same choice on the receive side improves the noise figure, as Chapter 25 shows. That is why modern base stations put the radio beside the antenna and carry the signal down the mast on optical fiber.

## Worked Example 24.3

Problem. A transmitter delivers 100 W through a 0.3 dB isolator, a 0.5 dB harmonic filter and duplexer, and 1.98 dB of cable. Find the power at the antenna. How much is recovered by mounting the radio at the antenna, keeping only 0.2 dB of jumper?

Solution. The losses total 2.78 dB, leaving 100 × 10^(−0.278) = 52.7 W. With the radio at the antenna, the loss is 1.0 dB, leaving 79.4 W. Moving the radio recovers 26.7 W, more than a quarter of the amplifier's output, with no change to the amplifier.

## Practice

1. A device dissipates 30 W with a total thermal resistance of 2.5 °C/W in 45 °C air. Find Tj.

2. Two equal amplifiers combine with a 15° phase error. What fraction of the power is delivered?

3. An 8-way combiner loses one input. By how many decibels does the output fall?

4. What fraction of the power is reflected at a VSWR of 1.5?

5. Find the loss of 45 m of cable rated at 6.6 dB per 100 m, and the power left from 40 W.

6. Why must a combiner's isolation resistors be rated for more than the normal imbalance power?

# Chapter 25. Receiver Noise and Dynamic Range

A transmitter's job ends at the antenna; a receiver's begins there, with a signal that may be a billionth of a billionth of a watt. Whether that signal can be recovered depends on how much noise the receiver adds to it, and whether the receiver survives the strong signals that arrive alongside. This chapter defines the noise figure and noise temperature, derives the cascade formula that shows why the first amplifier matters most, and sets the receiver's dynamic range between its noise floor and its intercept point. Book 5 used the resulting sensitivity; this is where it comes from. The receiver belongs in this book because it shares the radio, the antenna, the duplexer, and often the heat with the transmitter.

## 25.1 Noise figure and noise temperature

A resistor at temperature T delivers available noise power kTB into a matched load, which at the reference temperature T0 = 290 K is −174 dBm in each hertz. A two-port's noise factor F is the ratio of the input SNR to the output SNR when the source is at T0; the noise figure is NF = 10 log₁₀F. Equivalently, the two-port adds noise as if its input were connected to an extra resistor at the effective noise temperature Te = T0(F − 1). A 3 dB noise figure corresponds to 288.6 K, almost exactly T0; a 1 dB noise figure to 75.1 K; and 0.5 dB to 35.4 K. Noise temperature is the more natural measure for low-noise systems such as satellite ground stations and radio telescopes, where the sky itself may be at only a few tens of kelvin.

Te = T0 (F − 1),   T0 = 290 K,   kT0 = −174 dBm/Hz

## 25.2 The cascade

In a chain of stages, each stage's noise is divided by the total gain ahead of it when referred to the input. The result, Friis's formula for noise factor, shows that once the first stage has enough gain, the later stages hardly matter, and that any loss ahead of the first amplifier adds directly to the noise figure. A passive loss of L dB at T0 has a noise figure of L dB.

F = F1 + (F2 − 1)/G1 + (F3 − 1)/(G1G2) + …

## Worked Example 25.1

Problem. A receiver has an LNA with NF 1 dB and gain 15 dB, a filter with 2 dB loss, a mixer with NF 10 dB and gain 8 dB, and an IF amplifier with NF 4 dB and gain 20 dB. Find the cascade noise figure. Repeat with the filter ahead of the LNA.

Solution. In linear terms the LNA contributes F1 = 1.259. The filter contributes (1.585 − 1)/31.6 = 0.018, the mixer (10 − 1)/(31.6 × 0.631) = 0.451, and the IF amplifier 0.0012. The total is 1.740, a noise figure of 2.41 dB. With the filter first, its 2 dB adds directly and the LNA's gain no longer shields the mixer as well, giving 3.91 dB. The filter-first arrangement costs 1.5 dB of sensitivity, and it is used anyway when strong out-of-band signals would otherwise overload the LNA: the trade between noise and survival is the central decision of receiver design.

## Worked Example 25.2

Problem. The same receiver sits at the bottom of a mast with 3 dB of cable between the antenna and the LNA. Find the noise figure, and the noise figure with the LNA moved to the top of the mast.

Solution. With the cable first, its 3 dB adds to the cascade: 5.41 dB. With the LNA at the top, the cable follows the LNA's 15 dB of gain and the cascade is 3.52 dB. The tower-mounted amplifier recovers 1.9 dB. The transmit side of Chapter 24 made the same argument for power; the remote radio head serves both.

Key idea. The first stage sets the noise figure. Loss ahead of the first amplifier adds decibel for decibel, and gain in the first stage hides the noise of everything after it.

## 25.3 Sensitivity and dynamic range

The noise floor of a receiver is −174 dBm/Hz + 10 log₁₀B + NF. Sensitivity adds the SNR the demodulator needs, as in Book 5. The upper end of the receiver's range is set by distortion: two strong signals generate third-order products, Chapter 23's mechanism, and they become a problem when they rise to the noise floor. The spurious-free dynamic range is the range of input levels over which the products remain below the floor. Two-thirds appears because the products rise 3 dB per decibel while the signals rise one.

SFDR = (2/3) (IIP3 − Nfloor)

## Worked Example 25.3

Problem. The receiver of Worked Example 25.1, with a 2.41 dB noise figure, uses a 1 MHz channel and has an input third-order intercept of −10 dBm. Find the noise floor and the spurious-free dynamic range.

Solution. The floor is −174 + 60 + 2.41 = −111.6 dBm. SFDR = (2/3)(−10 + 111.6) = 67.7 dB. Raising the LNA gain lowers the noise figure but lowers the cascade's IIP3 too, because every later stage sees bigger signals; the designer sets the gain to balance the two. A phone receiver faces a further limit: its own transmitter, a few tens of megahertz away, leaks through the duplexer and can desensitize the receiver, which is why duplexer isolation of 50 dB or more is standard.

## 25.4 Blocking and reciprocal mixing

A strong signal outside the channel can also harm reception through the local oscillator. The oscillator's phase noise, Chapter 26's subject, spreads the oscillator's power around its nominal frequency, and a strong blocker mixes with that skirt into the wanted channel. This reciprocal mixing sets a floor that no amount of low-noise gain can remove: the blocker power, plus the phase noise in dBc/Hz at the blocker's offset, plus 10 log₁₀B.

## Practice

1. Find the noise temperature corresponding to a 2 dB noise figure.

2. A two-stage chain has an LNA with NF 0.8 dB and gain 20 dB, then a stage with NF 12 dB. Find the cascade noise figure.

3. Find the noise floor of a 20 MHz receiver with a 2 dB noise figure.

4. Find the SFDR for the receiver of the previous problem with an IIP3 of 0 dBm.

5. A blocker at −30 dBm meets oscillator phase noise of −120 dBc/Hz at its offset. Find the reciprocal-mixing noise in a 1 MHz channel.

6. Why does a filter placed ahead of the LNA raise the noise figure by its full loss?

# Chapter 26. Oscillators, Phase Noise, and the PLL

Every radio needs a frequency reference: a local oscillator that moves the signal up to the carrier in the transmitter and back down in the receiver. Its frequency must be exact and adjustable in fine steps, and its spectrum must be clean, because any noise on it is copied onto every signal it mixes with. Real oscillators spread their power into a skirt of phase noise around the carrier. This chapter quantifies that skirt, shows how a phase-locked loop combines a clean but fixed reference with a tunable but noisy oscillator, and connects the result to the EVM and reciprocal-mixing limits of Book 5 and Chapter 25. The loop's mathematics is Book 2's feedback, but the PLL lives here, because its specifications come from the radio.

## 26.1 Phase noise

An oscillator's phase noise is written L(Δf), the noise power in a 1 Hz band at an offset Δf from the carrier, relative to the carrier power, in dBc/Hz. Leeson's model explains its shape. The resonator, with loaded quality factor QL, filters the amplifier's noise; within the resonator's half-bandwidth f0/(2QL), the feedback turns amplitude noise into phase noise that rises as 1/Δf², and closer in, flicker noise steepens it to 1/Δf³. Far from the carrier the noise is flat. The model shows the two levers: a higher QL, and a higher signal power Ps in the resonator.

L(Δf) ≈ 10 log₁₀{ (2FkT/Ps) [1 + (f0 / (2QL Δf))²] }   (without the flicker term)

## Worked Example 26.1

Problem. A 2.4 GHz oscillator has QL = 20, a resonator power of 1 mW, and an amplifier noise factor of 10. Find its phase noise at 1 MHz and 100 kHz offsets, and at 1 MHz if QL falls to 10.

Solution. 2FkT/Ps = 2 × 10 × 4.0 × 10⁻²¹/10⁻³ = 8.0 × 10⁻¹⁷, which is −161 dBc/Hz. At 1 MHz, f0/(2QLΔf) = 60, and the bracket is 3601, adding 35.6 dB: L = −125.4 dBc/Hz. At 100 kHz the factor is 600, adding 20 dB more: −105.4 dBc/Hz. Halving QL to 10 raises the noise at 1 MHz by 6 dB, to −119.4 dBc/Hz. The integrated LC resonators of a CMOS chip have QL of about 10 to 20, which is why on-chip oscillators are noisier than discrete ones.

## 26.2 The phase-locked loop

A PLL divides the oscillator output by N, compares its phase with a crystal reference at fref, and steers the voltage-controlled oscillator so that the two stay in phase; the output is then N × fref. Inside the loop bandwidth, the output follows the reference, and the reference's phase noise appears multiplied by N, which is 20 log₁₀N in decibels. Outside the loop bandwidth, the VCO runs free and its own noise appears. The designer chooses the loop bandwidth near the offset where the two contributions cross. An integer-N synthesizer steps in multiples of fref, so fine steps need a low reference and therefore a large N. A fractional-N synthesizer dithers N between integers with a delta-sigma modulator, allowing a high reference, a small N, and fine steps; the dithering's quantization noise is shaped to high offsets, where the loop filters it.

fout = N fref,    in-band noise = Lref + 20 log₁₀N

## Worked Example 26.2

Problem. A 2.4 GHz synthesizer must step in 1 MHz increments. The reference, referred to the phase detector, contributes −150 dBc/Hz. Compare the in-band phase noise of an integer-N design with a 1 MHz reference and a fractional-N design with a 40 MHz reference.

Solution. The integer-N design needs N = 2400, so the in-band noise is −150 + 20 log₁₀2400 = −150 + 67.6 = −82.4 dBc/Hz. The fractional-N design has N ≈ 60, adding 35.6 dB, for −114.4 dBc/Hz, 32 dB better. Fractional-N synthesis is the reason modern radios can have both fine channel steps and clean in-band noise.

Key idea. A PLL copies the reference inside its bandwidth, multiplied by N, and leaves the VCO's own noise outside it. Keep N small, the reference clean, and the loop bandwidth where the two noise curves cross.

## 26.3 Settling

A PLL must also settle quickly when the channel changes. For a second-order loop with natural frequency ωn and damping ζ, the frequency error after a step decays roughly as e^(−ζωnt), so the time to settle to a fraction ε of the step is about −ln(ε√(1 − ζ²))/(ζωn). With fn = 100 kHz and ζ = 0.707, settling to one part per million of the step takes 31.9 µs. A wider loop settles faster and suppresses more VCO noise, but passes more reference and delta-sigma noise, and the loop must remain stable with margin: Book 2's phase-margin analysis applies directly.

## 26.4 From phase noise to EVM and jitter

Integrating the phase noise over the offsets that matter gives the oscillator's rms phase error σφ = √(2∫L(f)df), in radians, with L in linear units. The factor 2 counts both sidebands. The phase error rotates every constellation point, and for small errors it acts as an EVM of σφ. In a clock, the same quantity divided by 2πf0 is the rms jitter, which sets the converter limit of Book 5.

## Worked Example 26.3

Problem. An oscillator at 2.4 GHz has flat phase noise of −110 dBc/Hz from 10 kHz to 10 MHz offset. Find its rms phase error, the EVM floor it sets, and its rms jitter.

Solution. The integral is 2 × 10⁻¹¹ × (10⁷ − 10⁴) = 2.0 × 10⁻⁴ rad², so σφ = 0.0141 rad, or 0.81°. As an EVM floor that is 1.41%, or −37.0 dB, just short of the −38 dB that Book 5 found for Wi-Fi 7's 4096-QAM, so this oscillator would limit that mode before any other impairment. The jitter is 0.0141/(2π × 2.4 × 10⁹) = 0.94 ps rms.

## Practice

1. By how much does the in-band reference noise rise in a PLL with N = 145?

2. In Leeson's model, how much does doubling the resonator power change the far-out phase noise?

3. An oscillator has flat phase noise of −100 dBc/Hz from 1 kHz to 1 MHz at 1 GHz. Find σφ in degrees and the rms jitter.

4. A synthesizer with a 40 MHz reference must produce 5.8 GHz. Find N and the in-band noise if the reference contributes −150 dBc/Hz.

5. Find the approximate settling time of a loop with fn = 50 kHz and ζ = 0.707 to one part per million.

6. Why does a small N improve in-band phase noise?

# Chapter 27. Radar

Radar is the application in which this book's two halves meet most directly. A transmitter sends a pulse of hundreds of watts to megawatts, a target reflects a tiny fraction of it, and a receiver with the lowest possible noise must detect the echo. Because the signal travels out and back, the received power falls as the fourth power of range, not the square, and that one fact shapes everything else: the high peak powers, the long pulses compressed after reception, and the integration of many echoes. This chapter derives the radar equation, computes range, resolution, and Doppler, and finishes with the 77 GHz FMCW radar of a modern car. Antenna gain is taken from Part I; if the last stage is a tube, its pulse modulator is Book 6's.

## 27.1 The radar equation

A transmitter of power Pt with antenna gain G produces a power density of PtG/(4πR²) at range R. A target of radar cross-section σ intercepts and reradiates σ times that density, which spreads again over 4πR² on the way back, and the receiving antenna, of effective area Gλ²/(4π), collects part of it. Multiplying gives the monostatic radar equation, with one antenna for both directions.

Pr = Pt G² λ² σ / [(4π)³ R⁴]

## Worked Example 27.1

Problem. A 10 GHz radar transmits 1 kW peak through a 30 dBi antenna at a 1 m² target 10 km away. Its receiver has a 1 MHz bandwidth and a 3 dB noise figure, and detection needs 13 dB of SNR. Find the received power and the maximum range.

Solution. λ = 3.00 cm. Pr = 10³ × 10⁶ × (0.0300)² × 1/[(4π)³ × 10¹⁶] = 4.53 × 10⁻¹⁴ W, or −103.4 dBm. The noise floor is −174 + 60 + 3 = −111 dBm, so the required power is −98 dBm. The echo at 10 km is 5.4 dB too weak; since Pr ∝ R⁻⁴, the maximum range is 10 km × 10^(−5.4/40) = 7.31 km. Doubling the transmitter power would extend it only by 2^(1/4) = 1.19, to 8.69 km.

Key idea. Radar's echo falls as 1/R⁴. Doubling the range takes sixteen times the power, so radar designers buy range with antenna gain, pulse energy, and integration rather than with peak watts alone.

## 27.2 Pulses, energy, and resolution

Detection depends on the energy of the pulse, not its peak power: a matched filter's output SNR is the received pulse energy divided by kTF, as in Book 5. The range resolution, the smallest separation at which two targets appear distinct, is ΔR = c/(2B), where B is the pulse bandwidth; for a simple pulse of length τ, B ≈ 1/τ. A long pulse has more energy but poor resolution. Pulse compression removes the conflict: the transmitter sends a long pulse with a wide bandwidth, usually a linear frequency chirp, and the receiver's matched filter compresses it into a short one. The compression ratio is the time-bandwidth product Bτ.

ΔR = c / (2B),    Runamb = c × PRI / 2,    duty = τ / PRI

## Worked Example 27.2

Problem. The radar of Example 27.1 uses a 1 µs pulse at a pulse repetition interval of 1 ms. Find its range resolution, unambiguous range, duty cycle, and average power. It is changed to a 10 µs pulse chirped over 10 MHz at the same peak power. Find the new resolution, the compression ratio, and the new maximum range.

Solution. With 1 µs, B ≈ 1 MHz and ΔR = 150 m. The unambiguous range is c × 1 ms/2 = 150 km, the duty cycle 0.1%, and the average power 1 W. The chirped pulse has ΔR = c/(2 × 10 MHz) = 15 m and Bτ = 100, or 20 dB of compression. Its energy is ten times larger, so its SNR rises by 10 dB, and the maximum range becomes 7.31 × 10^(10/40) = 13.0 km, while the resolution improves tenfold. The average power rises to 10 W at 1% duty cycle, which the transmitter's thermal design of Chapter 24 must accept.

## 27.3 Doppler

A target moving with radial velocity v shifts the echo by fd = 2v/λ, twice the one-way shift because both paths shorten. Pulse-Doppler radars measure that shift across a train of pulses to separate moving targets from stationary clutter. At 10 GHz, a target closing at 100 m/s shifts the echo by 6.67 kHz.

fd = 2v / λ

## 27.4 The 77 GHz automotive radar

Automotive radars use the 76 to 81 GHz band and a frequency-modulated continuous wave instead of pulses. The transmitter sweeps linearly over a bandwidth B in a time Tc, a slope S = B/Tc. The echo from a target at range R arrives 2R/c later, so mixing it with the current transmit signal produces a beat frequency fb = 2RS/c, which an FFT turns into a range profile. The resolution is again c/(2B): a 4 GHz sweep resolves 3.75 cm, enough to separate a pedestrian from a nearby car. A sequence of chirps gives the Doppler, and an array of receive antennas, Part I's subject, gives the angle. The whole radar is a single CMOS or SiGe chip with a few milliwatts per transmit channel; the R⁴ law is overcome by the short ranges and by integrating hundreds of chirps.

## Worked Example 27.3

Problem. A 77 GHz FMCW radar sweeps 1 GHz in 50 µs. Find the slope, the beat frequency for a target at 50 m, the range resolution, and the Doppler shift for a closing speed of 30 m/s.

Solution. S = 10⁹/(50 × 10⁻⁶) = 2 × 10¹³ Hz/s. fb = 2 × 50 × 2 × 10¹³/2.998 × 10⁸ = 6.67 MHz, within reach of an inexpensive converter. ΔR = c/(2 × 1 GHz) = 15.0 cm. λ = 3.89 mm, so fd = 2 × 30/0.00389 = 15.4 kHz.

## Practice

1. By what factor must the transmitter power rise to double a radar's range, all else equal?

2. Find the range resolution of a 150 MHz chirp.

3. Find the unambiguous range at a pulse repetition interval of 500 µs.

4. A radar transmits 1 µs pulses of 1 kW peak every 1 ms. Find its average power.

5. Find the Doppler shift at 77 GHz for a closing speed of 30 m/s.

6. An FMCW radar sweeps 4 GHz in 40 µs. Find the beat frequency for a target at 100 m.

# Chapter 28. The Low-Noise Amplifier

Chapter 25 showed that the first stage of a receiver sets its noise figure. This chapter designs that stage. A low-noise amplifier must add as little noise as possible, provide enough gain to hide the stages after it, remain stable, survive the strong signals and transmitter leakage around it, and match the antenna well enough not to reflect the signal away. Those goals conflict, because the source impedance that minimizes a transistor's noise is not the one that maximizes its gain or its input match. The LNA designer's craft is resolving that conflict. Stability from S-parameters, which every amplifier must satisfy, is treated in Chapter 5; this chapter assumes a stable device.

## 28.1 The noise parameters

A transistor's noise figure depends on the impedance of the source driving it, which the input matching network sets. Four noise parameters describe the dependence: the minimum noise factor Fmin, the optimum source reflection coefficient Γopt at which it occurs, and the noise resistance Rn, which says how fast the noise rises as the source moves away from Γopt. A device with small Rn is forgiving; one with large Rn must be matched precisely. Manufacturers give the four parameters at each frequency, and they are measured with a tuner that sweeps the source impedance.

F = Fmin + (4Rn/Z0) |Γs − Γopt|² / [(1 − |Γs|²) |1 + Γopt|²]

## Worked Example 28.1

Problem. A transistor has NFmin = 0.5 dB, Rn = 8 Ω, and Γopt = 0.5∠60° in a 50 Ω system. Find its noise figure with a 50 Ω source, Γs = 0, and with Γs = 0.3∠60° and 0.5∠30°.

Solution. With Γs = 0, F = 1.122 + (32/50) × 0.25/(1 × |1 + 0.5∠60°|²) = 1.122 + 0.64 × 0.25/1.75 = 1.213, or 0.84 dB. At Γs = Γopt the figure is the minimum, 0.50 dB. With Γs = 0.3∠60°, a partial step toward the optimum, it is 0.56 dB, and with 0.5∠30°, the right magnitude at the wrong angle, 0.62 dB. Driving the device straight from 50 Ω would cost 0.34 dB, which in a satellite ground receiver, where the antenna's noise temperature is only tens of kelvin, is a large fraction of the system noise.

## 28.2 Noise match versus gain match

The source impedance for minimum noise is generally not the conjugate of the device's input impedance, so a noise-matched LNA reflects part of the incoming power and has a poor input return loss. Two techniques reconcile them. Inductive source degeneration, a small inductance in the emitter or source, adds a real part to the input impedance without adding noise, moving the input match toward Γopt's conjugate; it is the standard narrowband CMOS and HEMT LNA. Balanced amplifiers place two identical noise-matched stages between 90° hybrids, so that their reflections cancel at the input port and appear instead in the hybrid's load, giving both low noise and good match at the cost of twice the devices.

Key idea. A transistor has one source impedance for lowest noise and another for best match. The LNA's input network chooses between them, and degeneration or a balanced pair lets it have both.

## 28.3 Gain and the cascade

The LNA's gain should be high enough that later stages hardly affect the system noise figure, but not so high that it overloads them or lowers the cascade's intercept point, as Chapter 25 showed. A lower-noise device with less gain may give a worse system than a noisier device with more gain. Any loss before the LNA, in a connector, a filter, or a switch, adds decibel for decibel to the noise figure, so in the most sensitive systems the LNA is mounted directly at the antenna, and sometimes cooled.

## Worked Example 28.2

Problem. An LNA is followed by a stage with a 6 dB noise figure. Compare device A, with 0.8 dB noise figure and 10 dB gain, and device B, with 0.6 dB and 6 dB gain. Then find the system noise figure if a 0.3 dB loss precedes device A.

Solution. For A, F = 1.202 + (3.981 − 1)/10 = 1.500, a system noise figure of 1.76 dB. For B, F = 1.148 + 2.981/3.981 = 1.897, or 2.78 dB. The lower-noise device gives the worse receiver by a full decibel. With 0.3 dB of loss before device A, the system figure becomes 2.06 dB, and the loss has cost as much as the whole difference between the two devices' own noise figures.

## 28.4 Linearity and protection

An LNA in a phone or base station sits beside its own transmitter, and in a radar beside a kilowatt pulse. It must tolerate out-of-band blockers without compressing, survive the leakage that the duplexer or switch lets through, Chapter 34's subject, and recover quickly after an overload. Limiters, diodes that clip large signals before the LNA, protect radar receivers, but they add loss and therefore noise. GaN LNAs, which tolerate several watts of input, are increasingly used where survivability matters more than the last tenth of a decibel.

## Practice

1. Find the noise factor of a device with NFmin = 1.0 dB, Rn = 10 Ω, and Γopt = 0.4∠90°, with a 50 Ω source.

2. A system has 0.5 dB of cable loss before an LNA with 1.0 dB noise figure and 20 dB gain, followed by a 10 dB stage. Find the system noise figure.

3. What does inductive source degeneration add to an LNA's input impedance?

4. An LNA has 15 dB gain and the next stage a 12 dB noise figure. How much does the second stage add to the noise factor?

5. Why might a lower-noise transistor give a worse receiver?

6. Why does a balanced LNA have a good input match even when each half is noise-matched?

# Chapter 29. Mixers and Frequency Planning

Every superheterodyne and zero-IF radio of Book 5 contains a mixer, the circuit that shifts a signal from one frequency to another by multiplying it with a local oscillator. A mixer is a deliberately nonlinear device, and it produces not only the wanted product but a family of unwanted ones. Choosing the intermediate and oscillator frequencies so that those products fall where filters can remove them is frequency planning, and it is one of the decisions that most shape a radio's architecture. This chapter covers mixer types and figures of merit, the image problem, and the spur chart.

## 29.1 What a mixer does

Multiplying a signal at fRF by an oscillator at fLO produces components at their sum and difference. The receiver keeps the difference, fIF = |fRF − fLO|; the transmitter keeps the sum or difference that lands at the carrier. A double-balanced mixer, a diode ring or a Gilbert cell of transistors, suppresses the oscillator and the input at its output port and cancels many even-order products. Passive mixers have a conversion loss of about 6 to 7 dB and a noise figure roughly equal to it, but very high intercept points; active Gilbert-cell mixers have conversion gain but lower linearity. The oscillator drive must be large enough to switch the mixer cleanly, and the oscillator's phase noise, Chapter 26's subject, is transferred onto every signal.

fIF = |fRF − fLO|,   spurs at |m fLO ± n fRF| = fIF

## 29.2 The image

Two radio frequencies produce the same difference with a given oscillator: the wanted one, and the image on the opposite side of the oscillator at the same distance. A mixer cannot tell them apart, so the image must be removed before the mixer by a filter, or cancelled by an image-reject mixer that uses quadrature paths, as in Book 5's I/Q receiver. A high intermediate frequency puts the image far from the wanted band, where a filter can remove it easily, but makes the later IF filtering harder; a low IF does the opposite. Many receivers use two conversions to get both.

fimage = fRF − 2fIF  (low-side LO),   fRF + 2fIF  (high-side LO)

## Worked Example 29.1

Problem. A 2.45 GHz receiver uses a 400 MHz IF with low-side injection. Find the oscillator frequency and the image. An interferer at the image is 40 dB stronger than the wanted signal, which needs 20 dB of SNR. How much image rejection is needed?

Solution. fLO = 2.45 − 0.40 = 2.05 GHz, and the image is at 2.05 − 0.40 = 1.65 GHz, 800 MHz below the wanted signal. The image must end up at least 20 dB below the wanted signal after conversion, so the filter and mixer together must reject it by 40 + 20 = 60 dB. At an 800 MHz separation a modest ceramic filter can provide that; with a 40 MHz IF the image would be only 80 MHz away and would need a much sharper filter or an image-reject architecture.

Key idea. A mixer converts every input that satisfies |m fLO ± n fRF| = fIF, not only the one you want. Frequency planning places the oscillator and IF so that the image and the strong spurs land where a filter can remove them.

## 29.3 The spur chart

Because a real mixer is not an ideal multiplier, it converts inputs at every frequency where some combination m fLO ± n fRF equals the IF, with m and n small integers. Each such response, a spur, is weaker for higher orders, but a strong interferer at a low-order spur frequency can still block the receiver. The half-IF spur, with m = n = 2, lies halfway between the wanted signal and the oscillator, where it is hard to filter. A frequency plan lists the low-order spur frequencies, checks them against the filter's response and the expected interferers, and moves the IF if a dangerous spur falls in band.

## Worked Example 29.2

Problem. For the receiver of Example 29.1, find the radio frequencies between 1 and 4 GHz that convert to the 400 MHz IF for m and n up to 3.

Solution. Solving m × 2.05 ± n fRF = ±0.40 gives, for (m, n) = (1, 1), the wanted 2.45 GHz and the image 1.65 GHz. For (1, 2), 1.225 GHz. For (2, 1), 3.70 GHz. For (2, 2), 2.25 GHz and 1.85 GHz; 2.25 GHz is the half-IF spur, only 200 MHz below the wanted signal. For (2, 3), 1.50 and 1.233 GHz; for (3, 2), 3.275 and 2.875 GHz; for (3, 3), 2.183 and 1.917 GHz. The front-end filter must reject the half-IF spur at 2.25 GHz and the (3, 3) response at 2.183 GHz, both near the band; the others are far enough away to be easy.

## 29.4 The passive mixer as a switch

Multiplying a signal by a square wave at the LO frequency is the same as switching its polarity at that rate, which is what a ring of four diodes or four transistors does in a double-balanced mixer. The square wave's fundamental has an amplitude of 4/π times its peak, and the product contains the sum and difference frequencies. For an ideal commutating mixer, the fraction of the RF power that reaches the wanted IF is (2/π)², a conversion loss of 3.9 dB. Real diode mixers show 6 to 7 dB because of diode resistance, the transformers or baluns, and imperfect switching. The switches themselves are the series and shunt elements of Chapter 19.

L_conv,ideal = −20 log₁₀(2/π) = 3.92 dB

The balance of a double-balanced mixer cancels the LO at the RF and IF ports and the RF at the IF port, so port-to-port isolation, often 30 to 40 dB, depends on how well the baluns and diodes are matched. Because the switches are driven hard by the LO, a passive mixer is quite linear; a rule of thumb for diode mixers is an input third-order intercept about 10 dB above the LO drive.

## 29.5 Transmit mixers and direct conversion

A transmitter that upconverts with a mixer produces the wanted sideband, the image sideband, and oscillator leakage, all of which the emission mask of Book 7 constrains. Quadrature upconverters cancel the image sideband to the extent that their I and Q paths match, the same imbalance arithmetic as in Book 5's receiver, and they null the oscillator leakage by trimming DC offsets. Direct conversion, with the oscillator at the carrier, avoids the image entirely but places the oscillator in band, where its leakage, pulling by the power amplifier, and radiation must be controlled. Many modern transmitters avoid analog mixers altogether by synthesizing the signal digitally at a high rate and converting it with a fast DAC directly at the carrier.

## Practice

1. Find the image of a 5.8 GHz signal with a high-side LO and a 1 GHz IF.

2. Find the half-IF spur for a 900 MHz receiver with a 70 MHz IF and low-side LO.

3. An image interferer 30 dB above the wanted signal must end 15 dB below it. How much rejection is needed?

4. A passive mixer has 6.5 dB conversion loss. Estimate its noise figure.

5. Why does a higher IF make image rejection easier?

6. Why does a direct-conversion transmitter need careful control of LO leakage?

7. What is the ideal conversion loss of a commutating mixer?

8. Estimate the IIP3 of a diode mixer driven with +7 dBm of LO.

# Chapter 30. The Link Budget

A link budget adds up every gain and loss between the transmitter's output and the receiver's detector and compares the result with what the demodulator needs. It ties together almost every topic in this book and Book 5: power amplifier output, feeder loss, antenna gain, path loss, receiver noise, and the SNR or Eb/N0 that a modulation needs. Because everything is in decibels, the budget is a column of additions, and the bottom line, the margin, says how much fading or degradation the link can tolerate. This chapter builds a terrestrial budget in dBm and a satellite budget in the G/T form.

## 30.1 The terrestrial budget

The received power is the transmitter power, minus the feeder losses, plus both antenna gains, minus the path loss. In free space the path loss is the Friis term, which grows 20 dB per decade of distance and 20 dB per decade of frequency. The fade margin is the received power minus the receiver's sensitivity, which Part I defined as the noise floor plus the noise figure plus the required SNR. Real paths add atmospheric absorption, rain at millimeter waves, obstruction of the Fresnel zone, and multipath fading, so a microwave link is usually designed for a margin of 20 to 40 dB depending on the availability required.

Pr = Pt − Lt + Gt − Lpath + Gr − Lr,   Lpath = 20 log10(4πd f / c)

## Worked Example 30.1

Problem. A 5.8 GHz point-to-point link spans 10 km. The transmitter delivers 27 dBm, each feeder loses 1 dB, each antenna has 25 dBi, and the receiver sensitivity is −80 dBm. Find the path loss, the received power, and the fade margin.

Solution. Path loss = 20 log10(4π × 10⁴ × 5.8 × 10⁹ / 2.998 × 10⁸) = 127.7 dB. Received power = 27 − 1 + 25 − 127.7 + 25 − 1 = −52.7 dBm. The fade margin is −52.7 − (−80) = 27.3 dB, comfortable for a clear line-of-sight path at this frequency. Doubling the distance to 20 km would add 6.0 dB of path loss and leave 21.3 dB.

Key idea. A link budget is a sum in decibels: transmit power plus antenna gains minus losses gives the received power, and the margin over the receiver's sensitivity is what protects the link against fading.

## 30.2 The satellite budget and G/T

Satellite links use a different bookkeeping. The transmitter's power and antenna are combined into the effective isotropic radiated power, EIRP, in dBW. The receiver's antenna gain and system noise temperature are combined into a figure of merit, G/T, in dB/K, because a larger antenna and a quieter receiver are interchangeable ways of improving the link. The carrier-to-noise-density ratio then follows from Boltzmann's constant, −228.6 dBW/K/Hz, and dividing by the noise bandwidth gives C/N. The system temperature is the antenna temperature, the sky noise the antenna sees, plus the receiver's equivalent temperature Te = 290(F − 1).

C/N0 = EIRP − Lpath + G/T − k  (dBHz),   C/N = C/N0 − 10 log10 B

## Worked Example 30.2

Problem. A Ku-band satellite at 12 GHz is 38,000 km from the ground station. Its EIRP is 52 dBW; the ground terminal's G/T is 15 dB/K. Find C/N0 and the C/N in a 30 MHz channel, clear sky and with a 3 dB rain fade.

Solution. Path loss = 20 log10(4π × 3.8 × 10⁷ × 1.2 × 10¹⁰ / 2.998 × 10⁸) = 205.6 dB. C/N0 = 52 − 205.6 + 15 + 228.6 = 90.0 dBHz. In 30 MHz, 10 log10(3 × 10⁷) = 74.8 dB, so C/N = 15.2 dB in clear sky and 12.2 dB in the rain fade (strictly, rain also raises the sky noise and so lowers G/T a little more). Both exceed the roughly 6 to 10 dB that common satellite modulation and coding formats need.

## Worked Example 30.3

Problem. A ground antenna has 40 dBi of gain and sees an antenna temperature of 50 K. Its LNA has a 0.8 dB noise figure. Find the system temperature and G/T.

Solution. Te = 290(10^0.08 − 1) = 58.7 K, so Tsys = 50 + 58.7 = 108.7 K, or 20.4 dBK. G/T = 40 − 20.4 = 19.6 dB/K. If the system temperature were 150 K, G/T would fall to 18.2 dB/K; the quieter LNA is worth about 1.4 dB, the same as a 17% larger dish diameter.

## 30.3 Margins and availability

The margin a link needs depends on how often it may fail. Rain fading at frequencies above about 10 GHz is described statistically: a link designed for 99.99% availability, about 53 minutes of outage per year, needs far more rain margin than one designed for 99.9%. Multipath fading on terrestrial links is countered by diversity, two antennas spaced vertically or two frequencies, rather than by power alone. Adaptive modulation, as in Book 5, trades throughput for margin, dropping to a robust format during a fade instead of losing the link.

## Practice

1. Find the free-space path loss at 24 GHz over 2 km.

2. A link has 20 dBm transmit power, 30 dBi antennas at both ends, no feeder loss, and 120 dB path loss. Find the received power.

3. A 20 GHz satellite at 36,000 km has an EIRP of 50 dBW. The terminal's G/T is 10 dB/K. Find C/N in 10 MHz.

4. An LNA has a noise figure of 1.5 dB. Find its equivalent noise temperature.

5. How much path loss is added by tripling the distance?

6. Why is G/T a useful single figure of merit for a receiving station?

# Chapter 31. Bias, Ruggedness, and Protection

A power amplifier that meets every specification on the bench can still fail in the field: an antenna is disconnected, a connector corrodes, the supply comes up in the wrong order, or the device heats and its bias drifts. This chapter covers the practical engineering that keeps the amplifier of Chapters 21 to 24 alive: setting and stabilizing the bias, surviving a mismatched load, and detecting faults fast enough to act on them.

## 31.1 Biasing the device

The quiescent current sets the amplifier's class and therefore its linearity and efficiency, so it must be set accurately and held there as temperature changes. A gallium nitride or LDMOS transistor is biased by its gate voltage; for depletion-mode GaN that voltage is negative, a few volts below zero, and the drain must not be powered while the gate is at zero, because the device is then fully on. The safe sequence is gate negative first, then drain on, and on shutdown drain off first, then gate. Bipolar devices bring the opposite hazard: the base-emitter voltage needed for a given current falls by about 2 mV for each kelvin of temperature rise, so a fixed bias voltage gives a current that rises as the device heats, which heats it further. Emitter ballast resistors and bias circuits that track the device's temperature, often a diode or a scaled transistor on the same die, break that loop.

ΔVBE ≈ −2 mV/K,   IC ∝ exp(ΔV/VT),   VT = kT/q ≈ 25.9 mV

## Worked Example 31.1

Problem. A bipolar stage is biased with a fixed base-emitter voltage. Its junction warms by 10 K. By what factor does the collector current rise if nothing compensates?

Solution. A 10 K rise is equivalent to a 20 mV increase in the effective drive. The current scales as exp(0.020 / 0.02585) = 2.17, more than doubling. The extra current raises dissipation and temperature further; with enough thermal resistance the loop runs away. A ballast resistor that drops even a few tens of millivolts at the operating current provides negative feedback that stops it.

## 31.2 The mismatched load

An amplifier designed for a 50 Ω load sees something else when the antenna is damaged, iced, or missing. The voltage standing-wave ratio describes the mismatch: the reflection coefficient is |Γ| = (VSWR − 1)/(VSWR + 1), and a fraction |Γ|² of the forward power returns toward the amplifier. Along the line the incident and reflected waves add, so at some points the voltage reaches (1 + |Γ|) times the matched value. Depending on the phase of the reflection, the device sees either that excess voltage or a corresponding excess current. Ruggedness specifications state the VSWR, often 10:1 at all phases, that a device survives without damage.

|Γ| = (VSWR − 1)/(VSWR + 1),   Prefl = |Γ|² Pfwd,   Vpeak,max = (1 + |Γ|) Vpeak

## Worked Example 31.2

Problem. A 100 W amplifier into 50 Ω faces a 10:1 VSWR. Find the reflection coefficient, the reflected power, and the worst-case peak voltage on the line.

Solution. |Γ| = 9/11 = 0.818. Reflected power = 0.818² × 100 = 66.9 W. The matched peak voltage is √(2 × 100 × 50) = 100 V, so the worst case is 100 × 1.818 = 181.8 V, nearly twice the design value, applied through the matching network to the device. The return loss is only 1.74 dB.

Key idea. A severe mismatch reflects most of the power and nearly doubles the peak voltage or current somewhere in the output network. The amplifier survives either because the device is rated for it, because an isolator absorbs the reflection, or because protection cuts the drive before damage occurs.

## 31.3 Detecting faults and acting on them

A directional coupler at the output samples the forward and reflected waves separately, and diode or log detectors convert them to voltages. Comparing the two gives the VSWR continuously. Protection logic reduces the drive, the supply, or both, when the reflected power or the device temperature exceeds a threshold, and shuts down entirely on a severe fault. The response must be fast: a GaN device can be destroyed in microseconds by an overvoltage, so the first line of defense is often analog, with digital supervision setting the limits and logging events. The same coupler feeds the closed-loop power control that holds the output at its set value as temperature and supply vary.

## Worked Example 31.3

Problem. The amplifier of Example 31.2 has a 30 dB reverse coupler. What power does the reflected-power detector see at a 10:1 VSWR, and at 1.5:1?

Solution. At 10:1 the reflected power is 66.9 W, 48.3 dBm, so the detector sees 18.3 dBm. At 1.5:1, |Γ| = 0.2 and the reflected power is 0.04 × 100 = 4 W, 36.0 dBm, so the detector sees 6.0 dBm. The 12 dB difference is easy to resolve, though the coupler's finite directivity, the forward signal leaking into the reverse port, limits how small a mismatch can be measured.

## 31.4 Supply, overdrive, and inrush

Other common failures come from the supply and the input. An overdrive pushes the device deep into compression, raising its dissipation and the voltage swing; input limiters and drive monitoring guard against it. A supply that overshoots at turn-on can exceed the drain breakdown rating, and the large decoupling capacitors at the drain draw a heavy inrush that soft-start circuits limit. Load pulling at turn-on, with the antenna switch or duplexer not yet settled, can momentarily present a bad load. A good design treats each of these as a specified condition, tested at the extremes of temperature and supply.

## Practice

1. Find |Γ|, the return loss, and the reflected fraction for a VSWR of 2:1.

2. A 50 W amplifier into 50 Ω sees a 5:1 VSWR. Find the matched and worst-case peak voltages.

3. At a 3:1 VSWR, how much of the forward power reaches the load, in dB?

4. Why must a depletion-mode GaN amplifier's gate be biased before its drain is powered?

5. What does a directional coupler's directivity limit?

6. Why do bipolar power stages use emitter ballast resistors?

# Chapter 32. Vacuum Tubes: Klystrons, TWTs, and Magnetrons

Solid-state amplifiers have replaced vacuum tubes in most radios, but not everywhere. Where the power is kilowatts to megawatts, the frequency is very high, or the device must survive radiation and heat in orbit, electron beams in vacuum still win. Satellite transponders use traveling-wave tubes, particle accelerators and broadcast transmitters use klystrons and inductive output tubes, every microwave oven contains a magnetron, and fusion experiments heat plasma with gyrotrons. This chapter explains how a beam of electrons in vacuum amplifies a microwave signal and where each type fits.

## 32.1 The electron beam

A heated cathode emits electrons, and a high voltage accelerates them into a beam. The beam's DC power is its voltage times its current, and the tube's job is to convert part of that kinetic energy into RF. The current a gun can draw at a given voltage follows the Child-Langmuir law, I = K V^(3/2), where the constant K, the perveance, depends only on the gun's geometry; perveances around 0.5 to 2 microperveance (µperv, 10⁻⁶ A/V^1.5) are typical. At tens of kilovolts the electrons move at a sizable fraction of the speed of light, which is what lets the beam interact with a slow electromagnetic wave.

Pbeam = V I,   I = K V^(3/2),   v = c√(1 − 1/γ²),  γ = 1 + eV/(mc²)

## Worked Example 32.1

Problem. A tube's gun runs at 10 kV and 0.5 A. Find the beam power, the perveance, the electron velocity, and the RF output at 40% efficiency.

Solution. Beam power = 10⁴ × 0.5 = 5 kW. Perveance = 0.5 / (10⁴)^1.5 = 0.5 × 10⁻⁶, or 0.5 µperv. With mc² = 511 keV, γ = 1.0196 and v = 0.195c = 5.85 × 10⁷ m/s. At 40% efficiency the RF output is 2 kW and 3 kW must be removed as heat, mostly in the collector.

## 32.2 Klystrons

In a klystron the RF input drives a resonant cavity whose gap speeds up some electrons and slows others. Downstream, the fast electrons catch up with the slow ones and the beam bunches; at an output cavity the bunched current induces a large RF voltage, delivering power. Intermediate cavities increase the bunching and the gain, which can exceed 40 dB. Klystrons are narrowband, because the cavities are resonant, but they reach very high power: multi-megawatt pulsed klystrons drive linear accelerators, and continuous-wave tubes of tens of kilowatts drive broadcast and industrial heating. The inductive output tube, a hybrid of a klystron and a triode, is common in UHF television transmitters.

## 32.3 Traveling-wave tubes

A traveling-wave tube slows an RF wave along a helix or a coupled-cavity structure until its phase velocity nearly matches the beam's. Over the length of the tube the wave and the beam exchange energy continuously, and the wave grows. Because the helix is not resonant, a TWT can amplify over an octave or more, with gains of 40 to 60 dB. Space TWTs add multistage depressed collectors that recover energy from the spent beam by slowing the electrons before they land, raising overall efficiency to 60% to 70%, more than a comparable solid-state amplifier at Ku and Ka band. That efficiency is why most high-power satellite transponders still use TWT amplifiers.

## Worked Example 32.2

Problem. The 5 kW beam of Example 32.1 delivers 2 kW of RF. A depressed collector recovers 60% of the 3 kW remaining in the spent beam. Find the overall efficiency.

Solution. The supply now provides 5000 − 0.6 × 3000 = 3200 W net, because the recovered energy returns to the collector supply. The efficiency rises from 40% to 2000 / 3200 = 62.5%, and the heat to be removed falls from 3 kW to 1.2 kW.

Key idea. A vacuum tube converts the kinetic energy of an electron beam into RF, so its power is limited by the beam, not by a semiconductor's breakdown field and junction temperature. Depressed collectors recover much of the energy the RF did not take.

## 32.4 Magnetrons and gyrotrons

The magnetron is an oscillator rather than an amplifier: electrons spiral in crossed electric and magnetic fields past a ring of resonant cavities and give up energy to the oscillation. It is cheap and efficient, about 65% to 75% at 2.45 GHz, which is why a microwave oven delivers about a kilowatt from a component costing a few dollars. Its frequency and phase are not well controlled, which limits it to heating and older radars. The gyrotron uses electrons gyrating in a strong magnetic field to generate megawatts at frequencies of 100 GHz and above, where conventional structures would be too small, and it heats the plasma in fusion experiments.

## Worked Example 32.3

Problem. A 250 W Ku-band satellite amplifier is built either as a TWT at 60% efficiency or as a solid-state amplifier at 40%. Find the DC power and the heat in each case.

Solution. TWT: 250 / 0.6 = 417 W from the bus, 167 W of heat. Solid state: 250 / 0.4 = 625 W, 375 W of heat. On a spacecraft, where every watt of solar array and radiator costs mass, the tube saves 208 W of DC power and more than halves the heat, outweighing its high-voltage supply and finite cathode life.

## 32.5 Tubes against solid state

Solid-state amplifiers win on lifetime, low voltage, graceful degradation, and cost at moderate power. Reaching 2 kW from 150 W GaN devices takes 16 of them and a combiner; with 0.5 dB of combining loss that yields about 2.14 kW, a large, complex assembly. Tubes win at the highest powers and frequencies, in high-efficiency space amplifiers, and in harsh radiation environments. The boundary keeps moving as GaN improves, but in 2026 it still lies around the kilowatt level at X band and above.

## Practice

1. A tube runs at 20 kV and 2 A. Find the beam power and perveance.

2. Find the electron velocity, as a fraction of c, at 20 kV.

3. A magnetron delivers 1 kW at 70% efficiency. Find the DC input and the heat.

4. Why is a klystron narrowband while a helix TWT is broadband?

5. What does a depressed collector do?

6. Combining 16 amplifiers of 150 W with 0.5 dB of loss gives how much power?

# Chapter 33. RF Exposure and Safety

A transmitter that radiates kilowatts of EIRP is also a source of radio-frequency energy absorbed by people nearby. At the frequencies in this book the established effect of that energy is heating, and exposure limits are set with large safety factors below the levels that cause measurable heating. Every licensed transmitter, base station, and phone must show compliance. This chapter explains the two quantities the limits use, power density and specific absorption rate, and how an engineer finds the distance at which an antenna becomes compliant.

## 33.1 The limits

Two bodies of rules dominate. In the United States the FCC limits of 47 CFR 1.1310, based on the IEEE and NCRP work of the 1980s and 1990s, set a maximum permissible exposure for the general public of 1 mW/cm², which is 10 W/m², from 1.5 to 100 GHz, and f/1500 mW/cm² between 300 and 1500 MHz, with f in megahertz. Occupational limits, for trained workers aware of the exposure, are five times higher. Internationally, the ICNIRP 2020 guidelines set 10 W/m² for the general public from 2 to 300 GHz, averaged over 30 minutes for whole-body exposure. Near the body, for phones and other devices held close, the limits are on the specific absorption rate: 1.6 W/kg averaged over 1 g of tissue under the FCC rules, and 2 W/kg over 10 g under ICNIRP.

FCC general public: S ≤ f/1500 mW/cm² (300–1500 MHz);  1 mW/cm² = 10 W/m² (1.5–100 GHz)

## 33.2 Power density in the far field

Far from an antenna the power density is the EIRP spread over a sphere, S = EIRP/(4πR²). Solving for R gives the distance beyond which the exposure is below a limit. Near the ground, the direct and ground-reflected waves can add; FCC guidance in OET Bulletin 65 uses a reflection factor of 2.56 in power density, corresponding to a field 1.6 times the free-space value, for a conservative estimate. The far-field formula overestimates the density in the near field of a large aperture antenna, where the beam has not yet spread; there the density is bounded by roughly four times the power divided by the aperture area.

S = EIRP/(4πR²),   Rcompliance = √(F × EIRP / (4π Slimit)),  F = 1 or 2.56

## Worked Example 33.1

Problem. A sector antenna radiates an EIRP of 36 dBW (3981 W) at 3.5 GHz. Find the general-public compliance distance in the main beam, with and without the ground-reflection factor, and the occupational distance.

Solution. The limit at 3.5 GHz is 10 W/m². Without reflection, R = √(3981 / (4π × 10)) = 5.63 m. With the 2.56 factor, R = 5.63 × 1.6 = 9.0 m. For the occupational limit of 50 W/m², R = √(3981 / (4π × 50)) = 2.52 m. Because sector antennas are mounted high and the beam is narrow in elevation, people at ground level are usually far outside the main beam and well below the limit; the exclusion zone matters mostly to workers on the roof or tower.

Key idea. The power density from a transmitter falls as 1/R², so a compliance distance is the square root of the EIRP over 4π times the limit. Workers near the antenna, not the public on the ground, are the usual concern.

## Worked Example 33.2

Problem. A 900 MHz transmitter has an EIRP of 100 W. Find the general-public limit and the compliance distance with the 2.56 factor.

Solution. The limit is 900/1500 = 0.6 mW/cm², 6 W/m². Without reflection R = √(100 / (4π × 6)) = 1.15 m; with the factor, 1.84 m.

## 33.3 Specific absorption rate

Close to the body, a device's field couples directly into tissue, and the far-field density is meaningless. SAR, the power absorbed per kilogram, is measured with a probe scanning inside a tissue-simulating liquid in a phantom of the head or body, while the device transmits at full power. The limit of 1.6 W/kg over 1 g is a heating rate: in tissue with a heat capacity near 3600 J/kg/K, it would raise the temperature by about 0.027 K per minute if no heat were carried away, and blood flow removes much of it. Phones meet the limit by antenna placement and by reducing power when sensors detect the head or body nearby. Above about 6 GHz, where energy is absorbed close to the skin surface, the newer rules switch from SAR to absorbed power density over a small area of skin.

## 33.4 Compliance in practice

For a new site, the engineer computes the exposure from every transmitter there, adds their contributions as fractions of their own limits, and checks that the sum stays below one in accessible areas. Where it does not, the site uses signs, barriers, and procedures, and workers power down or reduce the antennas before working in front of them. Measurement with broadband or frequency-selective probes confirms the calculation. Massive MIMO base stations, whose beams move, are assessed using the actual time-averaged power in each direction, which is usually well below the theoretical maximum.

## Practice

1. Convert 1 mW/cm² to W/m².

2. Find the general-public limit at 600 MHz under the FCC rules.

3. An antenna radiates 50 W EIRP. Find the power density at 3 m.

4. Find the compliance distance for 1000 W EIRP at 28 GHz, without the reflection factor.

5. What electric field, in V/m, corresponds to a far-field power density of 10 W/m²?

6. Why is SAR used instead of power density for a phone held to the head?

# Chapter 34. Transceiver Architectures and Isolation

A transceiver puts a transmitter that delivers watts next to a receiver that must detect signals near −100 dBm, often sharing one antenna. The ratio between them can exceed 120 dB, and keeping the transmitter from deafening its own receiver is a central problem of transceiver design. This chapter compares the ways of sharing the antenna, time division, frequency division, and full duplex, and works out the isolation each requires.

## 34.1 Time division and frequency division

In time-division duplex, TDD, the transmitter and receiver take turns on the same frequency, and a switch connects the antenna to one or the other. The receiver never has to listen while the transmitter is on, so the isolation required is modest, but the switch must change state quickly and guard times must separate the bursts. Wi-Fi, most 5G mid-band and millimeter-wave systems, and many radars use TDD. In frequency-division duplex, FDD, the transmitter and receiver work at the same time on different frequencies, separated by a duplex gap, and a duplexer of two filters, the subject of Chapter 24, separates them. FDD needs the duplexer to provide high isolation in two places: at the transmit frequency, to keep the carrier from overloading the receiver, and at the receive frequency, to keep the transmitter's noise out of the receive band.

## 34.2 Transmitter noise in the receive band

A power amplifier's output contains not only the carrier but broadband noise, and part of that noise falls in the receive band. The receiver cannot filter it, because it is at the wanted frequency, so the duplexer must attenuate it before it reaches the LNA. If the leaked noise equals the receiver's own noise floor, the floor rises by 3 dB, a 3 dB loss of sensitivity; designs usually keep the leak about 10 dB below the floor, which costs about half a decibel.

Nfloor = −174 + NF dBm/Hz,   desense = 10 log10(1 + 10^((Nleak − Nfloor)/10))

## Worked Example 34.1

Problem. An FDD receiver has a 3 dB noise figure. The transmitter's noise in the receive band is −135 dBm/Hz at the antenna port. What duplexer isolation at the receive frequency keeps the leak 9 dB below the receiver floor, and what is the desense?

Solution. The receiver floor is −174 + 3 = −171 dBm/Hz. The leak must be −180 dBm/Hz, so the isolation must be −135 − (−180) = 45 dB. The desense is 10 log10(1 + 10^(−0.9)) = 0.51 dB. With only 40 dB of isolation, the leak would be −175 dBm/Hz, 4 dB below the floor, and the desense would grow to 1.46 dB.

Key idea. In FDD, transmitter noise that lands in the receive band cannot be filtered at the receiver, so the duplexer's isolation at the receive frequency sets how much sensitivity the transmitter costs.

## 34.3 The transmit carrier as a blocker

The transmitter's carrier, attenuated by the duplexer's isolation at the transmit frequency, arrives at the LNA as a strong blocker just outside the receive band. It can compress the LNA, generate intermodulation with other signals, and, mixed with the oscillator's phase noise as in Chapter 25's reciprocal mixing, raise the noise floor. Its own modulation can also leak through second-order nonlinearity in a direct-conversion receiver, creating a baseband interferer. The receiver's linearity must be specified against this known blocker.

## Worked Example 34.2

Problem. A handset transmits 25 dBm. The duplexer provides 50 dB of isolation at the transmit frequency. What carrier power reaches the LNA, and what does that imply?

Solution. 25 − 50 = −25 dBm at the LNA input. The LNA's input compression point must be well above −25 dBm, typically by 10 dB or more, and its IIP3 must keep intermodulation between this carrier and other blockers below the noise floor. A receiver that was designed for weak signals alone would be overloaded by its own transmitter.

## 34.4 Full duplex on one frequency

Transmitting and receiving at once on the same frequency, full duplex, would double spectral efficiency, but the receiver must then remove its own transmitted signal almost completely. The cancellation is done in stages: antenna separation or a circulator gives a few tens of decibels, an analog canceller that subtracts a tapped copy of the transmit signal gives more, and a digital canceller that models the residual channel and nonlinearities removes the rest. Each stage must track changes in the antenna and environment. Full duplex is used today in some cable systems and in-band relays, and it remains a research topic for cellular radios. The ferrite circulator of Chapter 15 is the passive starting point, and its leakage paths are worked there.

## Worked Example 34.3

Problem. A full-duplex radio transmits 25 dBm and receives in a 20 MHz channel with a 5 dB noise figure. How much self-interference cancellation brings its own signal down to the noise floor?

Solution. The noise floor is −174 + 10 log10(2 × 10⁷) + 5 = −96 dBm. The cancellation must be 25 − (−96) = 121 dB, all in the same band as the wanted signal, which is why it must combine passive isolation, analog cancellation, and digital cancellation, each contributing a few tens of decibels.

## Practice

1. Find the receiver noise floor in dBm/Hz for a 4 dB noise figure.

2. A transmitter's noise leak is 6 dB below the receiver floor. Find the desense.

3. A transmitter at 23 dBm, 10 MHz channel, receiver noise figure 4 dB. How much cancellation does full duplex need?

4. The duplexer gives 55 dB of transmit-band isolation. What blocker reaches the LNA from a 28 dBm transmitter?

5. Why does TDD need less duplexer isolation than FDD?

6. Why can't the receiver simply filter the transmitter's noise in the receive band?

# Chapter 35. Designing Class E and Class F Amplifiers

Chapter 22 introduced the switching and harmonic-tuned classes as ideas. This chapter designs them. Both aim at the same thing, a drain voltage and current that never overlap, so that the device dissipates almost nothing, but they reach it differently. Class E shapes the voltage with a shunt capacitor and a tuned load so that the switch closes at zero voltage and zero slope. Class F shapes it with harmonic terminations so that the voltage is nearly square and the current nearly a half sine. The design equations below are the standard ones from Sokal's and Raab's work.

## 35.1 The class E conditions

In class E the transistor acts as a switch, on for half of each cycle. A capacitor across the switch, which may be partly or wholly the device's own output capacitance, charges while the switch is off, and a series resonant load with some excess inductance shapes the voltage so that it returns to zero, with zero slope, just as the switch closes. No energy stored in the capacitor is dumped into the switch, which is the loss that ruins a hard-switched amplifier at high frequency. For a 50% duty cycle and an ideal switch, the design equations fix the load resistance, the shunt capacitance, and the excess reactance in terms of the supply and the output power.

R = 0.5768 VDD²/P,   C = 0.1836/(ωR),   Xexcess = 1.1525 R,   vpk = 3.562 VDD,   ipk = 2.862 IDC

## Worked Example 35.1

Problem. Design a class E stage for 100 W at 900 MHz from a 28 V supply. Find the load resistance, the shunt capacitance, the excess reactance, the peak switch voltage, and the peak current.

Solution. R = 0.5768 × 28² / 100 = 4.52 Ω. C = 0.1836 / (2π × 9 × 10⁸ × 4.52) = 7.18 pF. The excess reactance is 1.1525 × 4.52 = 5.21 Ω, an inductance of 0.92 nH at 900 MHz. The DC current is 100 / 28 = 3.57 A, so the peak current is 2.862 × 3.57 = 10.2 A, and the peak voltage is 3.562 × 28 = 99.7 V. The device must withstand about 100 V, which is why class E at this power suits GaN with its high breakdown voltage.

Key idea. Class E avoids switching loss by arranging for the switch voltage to fall to zero, with zero slope, at the instant the switch closes. The price is a peak voltage about 3.6 times the supply.

## 35.2 The frequency limit

The shunt capacitance in Example 35.1 must be at least the device's own output capacitance, because that capacitance is always there. The maximum frequency at which class E works at full power is therefore the one where the required C equals Cout. Above it, the design must reduce power, use a smaller device, or move to a different class. The power output capability, P/(vpk ipk), is 0.098 for class E against 0.125 for class B, so for the same device stresses class E delivers a little less power.

fmax = 0.1836/(2π R Cout),   cp = P/(vpk ipk) = 0.0981

## Worked Example 35.2

Problem. The device in Example 35.1 has an output capacitance of 10 pF. What is the highest frequency for the design?

Solution. fmax = 0.1836 / (2π × 4.52 × 10⁻¹¹) = 646 MHz. At 900 MHz the 10 pF device has more capacitance than the design allows, so the designer must accept less than 100 W, use a smaller device with less capacitance, or move to a class E variant or a different class.

## 35.3 Class F and its inverse

Class F leaves the device acting as a current source, as in class B, but presents short circuits at the even harmonics and open circuits at the odd ones. The current then contains the fundamental and even harmonics, approaching a half sine; the voltage contains the fundamental and odd harmonics, approaching a square wave. Neither waveform needs to be perfect: each added harmonic raises the efficiency. Inverse class F swaps the roles, a square current and a half-sine voltage, and suits devices whose voltage swing is limited by the knee. In practice, controlling the second and third harmonics captures most of the benefit.

η = ½ (V1/VDD)(I1/IDC)

## Worked Example 35.3

Problem. Find the maximum drain efficiency of an ideal class F amplifier whose voltage contains the fundamental and the third harmonic and whose current contains the fundamental and the second harmonic. Compare with half-sine current.

Solution. With a third harmonic, the largest fundamental voltage that keeps the waveform non-negative is V1/VDD = 2/√3 = 1.155. With a second harmonic, the largest fundamental current is I1/IDC = √2 = 1.414. The efficiency is ½ × 1.155 × 1.414 = 81.6%. With the full half-sine current, I1/IDC = π/2, it rises to ½ × 1.155 × 1.571 = 90.7%. For comparison, class B, with no voltage harmonics, gives π/4 = 78.5%, and controlling only the second harmonic of the current with a sinusoidal voltage gives 70.7%.

## 35.4 Practical harmonic terminations

Real terminations are made with transmission-line stubs or lumped resonators placed at the device's internal current source, which means compensating for its output capacitance and package parasitics. At microwave frequencies only the first two or three harmonics can be controlled, and the device's capacitance shorts the higher ones anyway, which partly helps. Load-pull measurements at the fundamental and harmonics show how sensitive the efficiency is to each termination. Continuous class F and continuous class B/J modes relax the terminations so that high efficiency holds over a band, not only at one frequency.

## Practice

1. Design a class E stage for 10 W from a 12 V supply at 13.56 MHz. Find R and C.

2. What peak voltage must the switch in the previous problem withstand?

3. Find the maximum frequency for a 4.52 Ω class E design if Cout = 5 pF.

4. What is the efficiency of a class F amplifier with half-sine current and voltage limited to the fundamental?

5. Why does class E suit GaN devices?

6. Why does controlling a few harmonics capture most of class F's benefit?

# Chapter 36. Envelope Tracking, Polar, and Outphasing

Chapter 23 showed that an amplifier with a fixed supply wastes most of its power when the signal's average sits well below its peak, and that the Doherty amplifier recovers some of that efficiency. This chapter takes up the other families of efficiency enhancement. Envelope tracking and polar transmitters change the supply voltage with the signal's envelope. Outphasing transmitters keep each amplifier saturated and create amplitude by combining two constant-envelope signals at varying phase. Each trades efficiency against bandwidth, linearity, and complexity.

## 36.1 Why back-off is expensive

An OFDM signal of Book 5 has a peak-to-average power ratio, PAPR, of 8 to 10 dB even after crest-factor reduction. An amplifier must handle the peak, so on average it runs 8 to 10 dB below its saturated power. A class B stage's efficiency falls with the square root of the output power, so at 8 dB back-off its ideal efficiency is 78.5% × 10^(−8/20) = 31%. Most of the supply power is heat.

## 36.2 Envelope tracking

An envelope-tracking transmitter drives the amplifier's supply from a fast modulator that follows the signal envelope, keeping the amplifier near compression at every instant. The amplifier still receives the full RF signal, amplitude and phase, so it remains a linear amplifier, but its supply voltage, and therefore its dissipation, falls with the envelope. The overall efficiency is the product of the amplifier's efficiency near compression and the modulator's efficiency. The modulator is the hard part: it must deliver watts with high efficiency and a bandwidth several times the signal's, because the envelope is a nonlinear function of the I and Q signals and is broader than either.

ηtotal = ηPA × ηmodulator,   Benvelope ≈ 3 to 5 × Bsignal

## Worked Example 36.1

Problem. A transmitter delivers 10 W average with an 8 dB PAPR. A fixed-supply class B stage achieves its ideal back-off efficiency; an envelope-tracking design holds the amplifier at 65% with an 85% modulator. Compare the supply power.

Solution. Fixed supply: 31.25% efficiency, so 10 / 0.3125 = 32.0 W of supply power and 22 W of heat. Envelope tracking: 0.65 × 0.85 = 55.3%, so 10 / 0.5525 = 18.1 W and 8.1 W of heat. The envelope tracker cuts the supply power by 43% and the heat by almost two-thirds.

Key idea. Envelope tracking keeps the amplifier near compression by lowering its supply when the signal is small. Its gain depends on a supply modulator that is both efficient and several times wider in bandwidth than the signal.

## 36.3 Polar transmitters and EER

Kahn's envelope elimination and restoration, and the modern polar transmitter, go further. The phase is applied to a constant-envelope RF signal that drives a switching amplifier, class E or D, into saturation, and the amplitude is restored by modulating its supply. The amplifier can then be as efficient as Chapter 35's designs, but the amplitude and phase paths must be aligned in time to a small fraction of the symbol period, and the amplitude path must reach down to zero without distortion. Polar transmitters work well for narrowband signals; for 100 MHz OFDM, the envelope bandwidth of several hundred megahertz is beyond an efficient supply modulator.

## Worked Example 36.2

Problem. A 100 MHz 5G carrier is to be envelope-tracked. Estimate the bandwidth the supply modulator needs.

Solution. With the rule of thumb of three to five times the signal bandwidth, the modulator needs 300 to 500 MHz. Efficient switching supplies reach tens of megahertz, so wideband trackers combine a fast linear amplifier with a slower switcher; the linear part's lower efficiency pulls ηmodulator down. This is why handset envelope trackers are common for LTE and 5G channels up to about 100 MHz but are hard to extend to wider channels.

## 36.4 Outphasing

Chireix's outphasing transmitter splits the signal into two constant-envelope components whose phases are offset by ±φ. Each drives a saturated, efficient amplifier, and a combiner adds them; the sum's amplitude is proportional to cos φ, so the output power is Pmax cos²φ. A simple isolating combiner wastes the difference power in its load, so efficient outphasing uses a non-isolating Chireix combiner with compensating reactances, which lets each amplifier see a load that changes with φ. The efficiency then peaks at full power and at a chosen back-off, much as in a Doherty amplifier.

Pout = Pmax cos² φ,   back-off (dB) = −20 log10 cos φ

## Worked Example 36.3

Problem. Find the outphasing angle for 3, 6, and 10 dB of back-off.

Solution. cos φ = 10^(−back-off/20). For 3 dB, cos φ = 0.708 and φ = 44.9°. For 6 dB, cos φ = 0.501, φ = 59.9°, essentially 60°. For 10 dB, cos φ = 0.316 and φ = 71.6°. The phase modulators must therefore cover angles up to about 70° to 80° with precision, because small phase errors near large angles produce large amplitude errors.

## 36.5 Choosing an architecture

In 2026 the Doherty amplifier dominates base stations, where moderate bandwidth, robust linearity with predistortion, and no extra supply circuitry suit the job. Envelope tracking dominates handset power amplifiers, where the supply modulator can be integrated and the signals are narrower. Outphasing and polar transmitters appear in research and some niche products, and digital transmitters that combine switching amplifiers with digital control of amplitude and phase are a growing area. All of them rely on the digital predistortion of Chapter 23 to meet the emission masks of Book 7.

## Practice

1. Find the ideal class B efficiency at 10 dB back-off.

2. An ET system has a 70% amplifier and an 80% modulator. Find the total efficiency.

3. Find the outphasing angle for half power.

4. A 20 MHz LTE carrier is envelope-tracked. Estimate the tracker bandwidth.

5. Why does a polar transmitter struggle with wideband signals?

6. Why does a simple isolating combiner make outphasing inefficient?

# Chapter 37. Measuring a Power Amplifier

The earlier chapters designed amplifiers from load lines and device models. The amplifier on the bench rarely matches the model exactly, and the numbers that go into a datasheet, a link budget, or a certification report must be measured. This chapter describes the four measurements that characterize a power amplifier: load-pull, which finds the best load; efficiency, which counts the watts; AM-AM and AM-PM, which describe the distortion; and adjacent-channel leakage, which tells the regulator how much of the signal leaks next door.

## 37.1 Load-pull

Chapter 21 found the optimum load resistance from the supply voltage, the knee voltage, and the output power. A real transistor has parasitic capacitance and package inductance, so the best load at the device's terminals is a complex impedance that the model only approximates. A load-pull system places a tuner, mechanical or active, at the transistor's output, sweeps the load impedance across the Smith chart, and records output power, efficiency, and gain at each point. The results form contours: closed curves of constant output power and of constant efficiency, whose optima are generally at different impedances. The designer chooses a compromise load and then designs Part I's matching network to present it.

Ropt ≈ (VDD − Vknee)² / (2 Pout)

## Worked Example 37.1

Problem. A GaN transistor runs at VDD = 28 V with a knee of 3 V and must deliver 10 W. Find the load-line starting point for the load-pull sweep.

Solution. Ropt = (28 − 3)²/(2 × 10) = 625/20 = 31.25 Ω. The load-pull sweep is centered near 31 Ω plus the conjugate of the output capacitance's susceptance, and the measured contours then show how far the true optimum lies from that estimate.

## 37.2 Efficiency and power-added efficiency

Drain efficiency is the ratio of RF output to DC input. Power-added efficiency subtracts the RF input power, which matters when the gain is modest, as it is at millimeter wave or near compression. Both must be measured with the amplifier's real signal, not only a continuous wave, because modulated signals spend most of their time below peak, where efficiency is lower, as Chapter 23 showed.

ηD = Pout / PDC,   PAE = (Pout − Pin) / PDC

## Worked Example 37.2

Problem. An amplifier delivers 40 dBm with 13 dB of gain from a 16 W DC input. Find the drain efficiency and the PAE.

Solution. Pout = 10 W and Pin = 40 − 13 = 27 dBm = 0.50 W. ηD = 10/16 = 62.5%, and PAE = (10 − 0.5)/16 = 59.4%. With only 6 dB of gain, PAE would fall to (10 − 2.5)/16 = 46.9%, which is why PAE is the honest figure for low-gain stages.

## 37.3 AM-AM and AM-PM

As the input rises toward compression, the gain falls (AM-AM conversion) and the phase of the output shifts (AM-PM conversion). Both are measured by sweeping the input power and recording the gain and phase, or, with a modulated signal, by comparing every output sample with the corresponding input sample. They are the curves digital predistortion, the subject of Chapter 38, must invert. Each contributes directly to the error vector magnitude of Book 5.

## Worked Example 37.3

Problem. At the peak of a signal, the amplifier is 1 dB compressed and shifts the phase by 5°. Estimate the error vector each causes at that peak.

Solution. The amplitude error is 1 − 10^(−1/20) = 10.9% of the peak. The phase error is 5° = 0.087 rad, which displaces the vector by 8.7% of its length. Together they are far above the 1.26% that Book 5 quotes for high-order QAM, which is why such peaks must be corrected or backed off.

Key idea. Load-pull measures the load that the model only estimates; PAE counts the drive power that drain efficiency ignores; AM-AM and AM-PM curves describe the distortion that predistortion must undo.

## 37.4 Adjacent-channel leakage and intermodulation

Distortion spreads the signal's spectrum into the neighboring channels. The adjacent-channel leakage ratio is the power in the wanted channel divided by the power in an adjacent channel of the same width, measured on a spectrum analyzer with channel-power integration. 3GPP requires about 45 dB for a base station and 30 dB for a handset in NR. A two-tone test gives a simpler figure, the third-order intercept, from which the third-order products follow: each output tone's product lies below the tone by twice the difference between the intercept and the tone power.

ACLR = Pmain / Padjacent,   P_IM3 = 3Pout − 2 OIP3  (dBm)

## Worked Example 37.4

Problem. An amplifier with OIP3 = 50 dBm delivers two tones of 30 dBm each. Find the third-order products and their level relative to the tones.

Solution. P_IM3 = 3 × 30 − 2 × 50 = −10 dBm, which is 40 dB below each tone. Every 1 dB of back-off improves that ratio by 2 dB, until other mechanisms, such as memory effects, dominate.

## Practice

1. Find Ropt for VDD = 48 V, Vknee = 4 V, and 50 W.

2. Find the PAE of an amplifier delivering 20 W with 10 dB gain from 32 W DC.

3. Find the amplitude error at 2 dB compression.

4. Find the IM3 level for 25 dBm tones and OIP3 = 45 dBm.

5. A channel at 30 dBm has −16 dBm in the adjacent channel. Find the ACLR.

6. By how much does a 3 dB back-off improve the two-tone IM3 ratio?

# Chapter 38. Digital Predistortion

Chapter 23 showed that a linear amplifier must back off from compression, and that back-off costs efficiency. Digital predistortion recovers much of that efficiency. A digital block ahead of the transmitter distorts the signal in exactly the opposite way to the amplifier, so that the cascade of the two is linear. Every modern base station and many handsets use it. This chapter explains the model, the observation path that trains it, and the efficiency it buys.

## 38.1 The idea

If the amplifier compresses its peaks, the predistorter expands them; if the amplifier advances the phase at high amplitude, the predistorter retards it. In the simplest, memoryless case, the predistorter is a lookup table indexed by the input amplitude, holding the inverse of the AM-AM and AM-PM curves measured in Chapter 37. The correction cannot exceed the amplifier's saturated power: no predistorter can make an amplifier deliver more than its maximum, so the peaks of the signal must still fit below saturation, usually with help from crest-factor reduction.

## 38.2 Memory and the memory polynomial

Wideband amplifiers have memory: their output depends on recent inputs as well as the present one, because of bias-network impedances, thermal changes, and trapping in GaN. A memoryless table then leaves residual distortion that grows with bandwidth. The memory polynomial models the amplifier, or its inverse, as a sum over nonlinear orders k and delays q, with complex coefficients fitted to measured data. With K orders and M delays it has K × M coefficients, a small number that a least-squares solver updates continuously.

y(n) = Σ_k Σ_q a_kq x(n − q) |x(n − q)|^(k − 1)

## Worked Example 38.1

Problem. A memory polynomial uses orders k = 1 to 5 and memory depth M = 4. How many complex coefficients does it have, and how many if only odd orders are kept?

Solution. All orders give 5 × 4 = 20 coefficients. Odd orders only, k = 1, 3, 5, give 3 × 4 = 12. Including even orders usually improves the fit for real amplifiers, at a modest cost in computation.

## 38.3 The observation receiver

To fit the coefficients, the transmitter samples its own output through a coupler and an observation receiver, aligns the samples in time and amplitude with the input, and solves for the coefficients. Indirect learning fits the inverse directly, treating the amplifier's output as the input of a postdistorter whose coefficients are then copied to the predistorter. Because third-order distortion spreads the spectrum to three times the signal bandwidth, and fifth-order to five times, the observation path must capture several times the signal bandwidth, which sets the speed of its converters.

B_observation ≈ K × B_signal  (K = highest odd order corrected)

## Worked Example 38.2

Problem. A 100 MHz NR carrier is linearized up to fifth order. Find the observation bandwidth needed.

Solution. Fifth-order products extend over five times the signal bandwidth, so the observation receiver must capture about 500 MHz, and its ADC must sample at least that fast for complex sampling. This is one reason DPD for 400 MHz millimeter-wave carriers is difficult, and why those systems often use simpler schemes.

Key idea. Digital predistortion applies the inverse of the amplifier's nonlinearity and memory, fitted continuously from an observation receiver that captures several times the signal bandwidth. It linearizes the amplifier so that it can run closer to saturation.

## 38.4 What DPD buys

Without DPD a base-station amplifier might need 9 dB of back-off from its peak to meet ACLR; with DPD and crest-factor reduction, perhaps 6 dB. For an ideal class B amplifier, efficiency at back-off falls as π/4 times the voltage ratio, so the 3 dB change is substantial, and in a Doherty amplifier the gain is even greater because DPD corrects the Doherty's own nonlinearity, which would otherwise rule it out for linear signals.

η_classB(BO) = (π/4) × 10^(−BO/20)

## Worked Example 38.3

Problem. Compare the ideal class B efficiency at 9 dB and at 6 dB of back-off.

Solution. At 9 dB, η = 0.785 × 0.355 = 27.9%. At 6 dB, η = 0.785 × 0.501 = 39.4%. DC power for the same average output falls by a factor of 39.4/27.9 = 1.41, a 29% reduction in the power the station draws for its amplifiers, before any Doherty improvement.

## 38.5 Practical limits

DPD adds a digital block, an observation receiver, and training software, which pay for themselves in base stations but are marginal in low-power devices. It cannot correct noise or distortion that originates before the amplifier, and it must track changes in temperature, supply, and load, the antenna mismatch of Chapter 31 included. Envelope-tracking transmitters of Chapter 36 depend on it, because the supply modulation itself creates distortion that only DPD removes.

## Practice

1. How many coefficients does a memory polynomial with K = 7 and M = 3 have?

2. Find the observation bandwidth for a 20 MHz carrier linearized to fifth order.

3. Find the ideal class B efficiency at 8 dB back-off.

4. Find the ideal class B efficiency at 0 dB back-off.

5. Why can a predistorter not raise the saturated power?

6. Name two physical causes of memory in a power amplifier.

# Chapter 39. Frequency Synthesis: Fractional-N and DDS

Chapter 26 introduced the phase-locked loop and its phase noise. A real radio needs more than one frequency: it must tune to any channel in its band in fine steps, quickly, with low spurs. This chapter describes the two main ways to make such a frequency: the fractional-N PLL, which multiplies a reference by a number that need not be an integer, and direct digital synthesis, which computes the waveform sample by sample. It then compares their noise and spurs.

## 39.1 Integer-N and its limits

An integer-N PLL produces fout = N × fref, so the channel step equals the reference frequency at the phase detector. Fine steps need a low reference, and a low reference forces a narrow loop bandwidth, for stability, and a large N. Both hurt: the narrow loop settles slowly and suppresses the VCO's noise over a smaller range, and the in-band phase noise rises as 20 log N.

## 39.2 Fractional-N

A fractional-N PLL switches its divider between neighboring integers so that the average division ratio is N + F/2^k, set by a k-bit accumulator. The reference can then be high, and the step is the reference divided by 2^k, which can be a few hertz. The switching pattern creates a periodic phase error that would produce spurs, so it is shaped by a delta-sigma modulator, Book 3's technique, that pushes the error to high offsets where the loop filters it out.

fout = (N + F/2^k) × fref,   step = fref/2^k

## Worked Example 39.1

Problem. A 40 MHz reference and a 24-bit fractional accumulator must produce 2441.5 MHz. Find the step, N, and F.

Solution. The step is 40×10⁶/2²⁴ = 2.38 Hz. The ratio is 2441.5/40 = 61.0375, so N = 61 and F = 0.0375 × 2²⁴ = 629,145.6, rounded to 629,146. The rounding leaves an error of 0.4 steps, under 1 Hz.

## 39.3 In-band phase noise and the figure of merit

Inside the loop bandwidth, the output phase noise is set by the phase detector and charge pump, referred to the output by the multiplication. Datasheets express this with a normalized figure of merit, the noise of a hypothetical 1 Hz loop at a 1 Hz detector rate, from which the in-band noise follows for any detector frequency and ratio. A good modern synthesizer has a figure of merit near −226 to −230 dBc/Hz.

L_in-band ≈ FOM + 10 log₁₀ f_PFD + 20 log₁₀ N  (dBc/Hz)

## Worked Example 39.2

Problem. With FOM = −226 dBc/Hz, find the in-band phase noise at 2441.5 MHz for a 40 MHz detector frequency and for a 10 MHz detector frequency.

Solution. At 40 MHz, N = 61.04, so L = −226 + 76.0 + 35.7 = −114.3 dBc/Hz. At 10 MHz, N = 244.15, so L = −226 + 70.0 + 47.8 = −108.2 dBc/Hz. Raising the detector frequency by four improves the in-band noise by 6 dB, the 20 log N term outweighing the 10 log f term, which is the main reason fractional-N synthesizers run their detectors as fast as possible.

Key idea. Fractional-N lets the detector run fast while the step stays fine; in-band noise improves 10 log f_PFD for every 20 log N it saves. Delta-sigma control hides the fractional pattern at offsets the loop filters away.

## 39.4 Direct digital synthesis

A DDS adds a frequency tuning word to a phase accumulator at each clock, converts the top bits of the phase to an amplitude through a lookup table, and drives a DAC. The output frequency is the tuning word times the clock over 2^n, so a 32-bit accumulator with a 1 GHz clock tunes in steps of 0.23 Hz, and the frequency can change at the next clock, with continuous phase. Its limits are spurs: truncating the phase to P bits before the table gives spurs of about −6.02P dBc, and the DAC's own nonlinearity adds more. The output is useful up to about 40% of the clock, beyond which the image near the clock cannot be filtered.

fout = FTW × fclk / 2^n,   spur ≈ −6.02 P dBc

## Worked Example 39.3

Problem. A DDS uses a 32-bit accumulator, a 1 GHz clock, and 14 bits of phase into the table. Find the step and the phase-truncation spur level, and the highest practical output frequency.

Solution. The step is 10⁹/2³² = 0.233 Hz. The truncation spur is about −6.02 × 14 = −84 dBc. The highest practical output is about 400 MHz. A common architecture uses a DDS as the fine-tuning reference of a PLL, combining the DDS's agility and resolution with the PLL's reach to gigahertz.

## Practice

1. Find the step of a 50 MHz reference with a 20-bit fractional accumulator.

2. Find N and F for 5800 MHz from a 50 MHz reference with a 20-bit accumulator.

3. Find the in-band noise for FOM = −229 dBc/Hz, f_PFD = 100 MHz, N = 58.

4. Find the DDS step for a 48-bit accumulator and a 2.5 GHz clock.

5. Find the truncation spur for 12 bits of phase.

6. How much does the in-band noise improve when f_PFD doubles at fixed fout?

# Chapter 40. Power Detection and Power Control

A transmitter must know how much power it is sending. Regulators cap it, link budgets assume it, RF-exposure limits from Chapter 33 depend on it, and cellular networks command each handset to raise or lower it many times a second. This chapter describes how a transmitter measures its own output with a coupler and detector, the errors that measurement suffers, and the control loops that hold the power where it should be.

## 40.1 The coupler and the detector

A directional coupler, Chapter 9, samples a fixed fraction of the forward wave, for example 20 dB below it, and sends it to a detector. Diode detectors respond to the square of the voltage at low levels, below about −20 dBm, and to its peak at higher levels; logarithmic amplifiers give a voltage proportional to the input in decibels over 60 dB or more; RMS detectors give the true average power regardless of the waveform. An attenuator between coupler and detector places the transmitter's power range inside the detector's range.

## Worked Example 40.1

Problem. A 40 dBm transmitter uses a 20 dB coupler and a detector with a range of −30 to 0 dBm. What pad is needed, and over what output range can the power be measured?

Solution. The coupled power at full output is 40 − 20 = 20 dBm, so a 20 dB pad brings it to 0 dBm, the top of the detector's range. The bottom, −30 dBm at the detector, corresponds to 40 − 30 = 10 dBm at the output, a 30 dB control range. A log detector with 60 dB range would extend it to −20 dBm.

## 40.2 Directivity error

A real coupler also passes a little of the reflected wave to its forward port, suppressed by the coupler's directivity D in addition to the coupling. If the antenna reflects with return loss RL, the leaked reflection adds to or subtracts from the forward sample depending on phase, so the measured power is uncertain by an amount set by RL + D. Good directivity and a good antenna match keep the error small; a mismatched antenna, as from a hand on a phone, can make it large.

error = 20 log₁₀(1 ± 10^(−(RL + D)/20)) dB

## Worked Example 40.2

Problem. A coupler with 25 dB directivity is used with an antenna whose return loss is 10 dB. Find the measurement uncertainty.

Solution. RL + D = 35 dB, so the leaked wave is 10^(−35/20) = 0.018 of the forward sample in voltage. The error lies between +0.15 dB and −0.16 dB, acceptable for most control loops. With the antenna at 6 dB return loss and 20 dB directivity, the bounds widen to +0.42 and −0.45 dB.

Key idea. A coupler and detector measure the forward power; the error from the coupler's finite directivity grows as the antenna's match worsens. RMS or log detectors are needed for modulated signals with high peak-to-average ratios.

## 40.3 Detecting modulated signals

A diode detector operated in its peak-responding region reads high for signals with a high peak-to-average ratio, and the error depends on the modulation. OFDM signals, with peak-to-average ratios of 8 to 10 dB, therefore need RMS detectors, or a diode kept in its square-law region, or digital measurement in the observation receiver of Chapter 38, which computes the average power from samples exactly.

## 40.4 The control loop

Closed-loop power control compares the detected power with a target and adjusts the gain of a variable attenuator or the drive of the amplifier. In cellular systems an outer loop runs through the network: the base station measures the handset's received signal and sends transmit-power-control commands, typically steps of ±1 dB, so that every handset arrives at about the same level and the near-far problem of Book 5's interference chapter is avoided. A handset's output range is large, from about −40 dBm to its maximum of 23 dBm in the common power class, 63 dB in total, so the control must work at low power where the detector is weakest.

## Worked Example 40.3

Problem. A handset at 23 dBm receives eight consecutive −1 dB commands. Find its new power, and the factor by which its battery current for the amplifier might fall if the amplifier's efficiency stayed constant.

Solution. The power falls to 23 − 8 = 15 dBm, from 200 mW to 31.6 mW, a factor of 6.3. In practice efficiency falls at back-off, so the current falls by less unless the supply is reduced too, which is the purpose of the envelope-tracking and average-power-tracking supplies of Chapter 36.

## Practice

1. A 33 dBm transmitter uses a 30 dB coupler and a detector whose maximum is 0 dBm. What pad is needed?

2. Find the directivity error bounds for RL + D = 40 dB.

3. What is the handset output range from −40 dBm to 23 dBm?

4. Convert 15 dBm to milliwatts.

5. Why does a peak-responding detector read high for OFDM?

6. What does a log detector's output voltage track?

# Appendix A. Constants and Formulas Used in This Book

Speed of light c = 2.998 × 10⁸ m/s. Reference impedance Z0 = 50 Ω unless stated.

Reference stripline laminate: εr = 3.55, so λg = λ0/1.884; at 2.45 GHz, λ0 = 12.24 cm and λg = 6.49 cm. Reference microstrip: 0.508 mm thick, εr = 3.55, 50 Ω width 1.14 mm, εeff ≈ 2.79.

Decibels: P(dBm) = 10 log₁₀(P/1 mW); 0 dBm in 50 Ω is 0.224 V rms.

Reflection: Γ = (ZL − Z0)/(ZL + Z0); VSWR = (1 + |Γ|)/(1 − |Γ|); RL = −20 log₁₀|Γ|; ML = −10 log₁₀(1 − |Γ|²).

Line input impedance: Zin = Z0 (ZL + jZ0 tan βl)/(Z0 + jZL tan βl); quarter wave: Zin = Z0²/ZL.

L-network: Q = √(Rhigh/Rlow − 1), Xseries = Q Rlow, Xshunt = Rhigh/Q.

S-parameters: Zin = Z0(1 + S₁₁)/(1 − S₁₁); stability K = (1 − |S₁₁|² − |S₂₂|² + |Δ|²)/(2|S₁₂S₂₁|), Δ = S₁₁S₂₂ − S₁₂S₂₁.

Filter prototypes, third order: Butterworth g = 1, 2, 1; Chebyshev 0.1 dB g = 1.0316, 1.1474, 1.0316. Midband loss ≈ 4.343 Σgₖ/(FBW Qu) dB.

Rectangular waveguide TE₁₀ cutoff fc = c/2a; WR-90 a = 22.86 mm (6.56 GHz); WR-28 a = 7.112 mm (21.1 GHz).

Antennas: half-wave dipole Rrad ≈ 73 Ω, D = 1.64 (2.15 dBi); short dipole Rrad = 80π²(l/λ)²; Ae = Gλ²/4π; dish G = ηap(πD/λ)², θ₃dB ≈ 70λ/D degrees.

Link anchor: Pr/Pt = Gt Gr (λ/4πR)²; at 2.45 GHz and 100 m the path factor is 9.48 × 10⁻⁹ (−80.2 dB).

Arrays: β = −kd sin θ₀; no grating lobes for d/λ < 1/(1 + |sin θmax|); broadside beamwidth ≈ 0.886λ/(Nd) rad.

Wilkinson: lines √2 Z0, resistor 2Z0. Branch-line: Z0/√2 series arms. Coupled lines: Z0e = Z0√((1 + C)/(1 − C)), Z0o = Z0√((1 − C)/(1 + C)).

Microstrip: εeff = (εr + 1)/2 + (εr − 1)/(2√(1 + 12h/W)). 50 Ω on εr 3.55, h 0.508 mm: W = 1.14 mm. Stripline: Z0 = 30πb/[√εr(W + 0.441b)].

Bode-Fano (parallel RC): ∫ln(1/|Γ|)dω ≤ π/(RC). Chu: Qmin = 1/(ka)³ + 1/(ka). TE101: f = (c/2)√(1/a² + 1/d²).

Fresnel r1 = √(λd1d2/(d1 + d2)); two-ray breakpoint 4hthr/λ; knife edge J(0) = 6 dB; rain γ = kR^α dB/km.

Ferrite resonance f = γH, γ ≈ 28 GHz/T. Far field R = 2D²/λ. Gain comparison G = Gref + PAUT − Pref. Passive loss Te = (L − 1)T. Coupled filters Qe = g₀g₁/FBW, M = FBW/√(gᵢgᵢ₊₁). Series switch IL = 20 log|1 + Z/2Z₀|; ideal mixer loss 3.92 dB.

dBm = 10 log₁₀(P/1 mW); 1 W = 30 dBm; 1 dB of loss leaves 0.794 of the power. EIRP = Pout − Lfeed + Gant.

Load line: Ropt = (VDD − Vknee)²/(2P). 100 W: 10.13 Ω at 50 V/5 V knee; 3.13 Ω at 28 V/3 V knee.

Efficiency: ηD = Pout/PDC; PAE = (Pout − Pin)/PDC. Class A 50%; class B π/4 = 78.5%; η(α) = (α − sin α)/[4(sin(α/2) − (α/2)cos(α/2))].

Back-off: class A η ∝ Pout; class B η = (π/4)√(Pout/Pmax); ideal symmetric Doherty 78.5% at peak and at 6 dB back-off.

OIP3 = Ptone + ΔIM3/2. SFDR = (2/3)(IIP3 − Nfloor).

Thermal: Tj = Tamb + Pdiss Σθ. Ideal N-way combiner: Pout = (Σ√Pi)²/N; one of N failed leaves (N − 1)²/N².

Noise: kT0 = −174 dBm/Hz at 290 K; Te = T0(F − 1); F = F1 + (F2 − 1)/G1 + (F3 − 1)/(G1G2) + ….

PLL: fout = N fref; in-band noise = Lref + 20 log₁₀N. Leeson: L(Δf) ≈ 10 log₁₀{(2FkT/Ps)[1 + (f0/(2QLΔf))²]}. σφ = √(2∫L df); jitter = σφ/(2πf0).

Radar: Pr = PtG²λ²σ/[(4π)³R⁴]; ΔR = c/(2B); Runamb = c·PRI/2; fd = 2v/λ; FMCW fb = 2RS/c.

LNA: F = Fmin + (4Rn/Z0)|Γs − Γopt|²/[(1 − |Γs|²)|1 + Γopt|²]. Mixer image = fRF ∓ 2fIF; spurs at |m fLO ± n fRF| = fIF.

Link: Lpath = 20 log₁₀(4πdf/c); C/N0 = EIRP − Lpath + G/T + 228.6 dBHz. 12 GHz, 38,000 km: 205.6 dB.

|Γ| = (VSWR − 1)/(VSWR + 1); 10:1 gives 0.818, 66.9% reflected. Beam P = VI; perveance K = I/V^1.5.

Exposure: S = EIRP/(4πR²); FCC public 10 W/m² (1.5–100 GHz), f/1500 mW/cm² (300–1500 MHz); ground factor 2.56.

Class E (50% duty): R = 0.5768 VDD²/P, C = 0.1836/(ωR), vpk = 3.562 VDD. Outphasing: Pout = Pmax cos²φ.

PAE = (Pout − Pin)/PDC; P_IM3 = 3Pout − 2·OIP3; class B η = (π/4)10^(−BO/20). Fractional-N: fout = (N + F/2^k)fref; in-band L ≈ FOM + 10 log f_PFD + 20 log N. DDS: fout = FTW·fclk/2^n, spur ≈ −6.02P dBc. Directivity error 20 log(1 ± 10^(−(RL + D)/20)).

# Appendix B. Answers to the Practice Problems

## Chapter 1

1. λ0 = 2.998 × 10⁸/5.8 × 10⁹ = 5.17 cm; λg = 5.17/1.884 = 2.74 cm.

2. λg must be 20 cm, so f = 2.998 × 10⁸/(1.884 × 0.20) = 0.80 GHz.

3. 27 dBm = 10^2.7 mW = 501 mW. −90 dBm = 10⁻¹² W; V = √(10⁻¹² × 50) = 7.07 µV rms.

4. f = 1/(2π √(0.6 × 10⁻⁹ × 100 × 10⁻¹²)) = 650 MHz.

5. −30 + 20 − 2 − 6 = −18 dBm.

6. At series resonance the capacitive and inductive reactances cancel and only the small ESR remains, so the part is the closest thing to a short circuit available for coupling or bypassing at that frequency.

## Chapter 2

1. vp = 2.55 × 10⁸ m/s; C = 1/(75 × 2.55 × 10⁸) = 52.3 pF/m; L = 75/2.55 × 10⁸ = 294 nH/m.

2. Γ = 25/125 = 0.20; VSWR = 1.5; RL = 14.0 dB; ML = 0.18 dB.

3. Γ = (25 − 50)/75 = −1/3, so |Γ| = 0.333 and VSWR = 2.0; the negative sign says the load is below Z0.

4. Zin = 50²/200 = 12.5 Ω.

5. Zin = −jZ0 cot 45° = −j50 Ω, so C = 1/(2π × 2.45 × 10⁹ × 50) = 1.30 pF.

6. Any length of line between the load and the measurement point rotates the impedance by 2βl, so the same load reads differently at every plane; the number has meaning only at a stated plane.

## Chapter 3

1. Q = √(50/5 − 1) = 3.0. Series reactance 15 Ω, L = 15/(2π × 900 MHz) = 2.65 nH. Shunt reactance 50/3 = 16.7 Ω, C = 10.6 pF.

2. Q = √(200/50 − 1) = 1.73. The shunt element (an inductor in the high-pass form) goes across the 200 Ω side, and the series capacitor faces the 50 Ω side.

3. Rv = 50/(64 + 1) = 0.77 Ω.

4. R = 15/30 = 0.5 Ω in series with 5 Ω; 5/5.5 = 0.909, a loss of 0.41 dB.

5. 20 − 8 = 12 Ω of inductive reactance, since the load supplies j8 Ω.

6. Each section transforms by the square root of the total ratio, so each has a lower Q; the cascade has wider bandwidth and lower component loss than a single high-Q section.

## Chapter 4

1. Zq = √(100 × 50) = 70.7 Ω.

2. The formula is symmetric in Z0 and RL, so the bandwidth for 100 to 50 Ω equals that for 25 to 50 Ω, a ratio of 2 in each case: 36.7%.

3. YL = 1/(25 − j25) = 0.02 + j0.02 S, so its conductance is already 1/50 S. A stub must supply −j0.02 S, so −j cot βl = −j1 and l = λ/8.

4. The reflection coefficient's phase changes by 2βd = 2 × 360° × 0.1 = 72°, clockwise.

5. The 29.7 Ω section sits next to the 25 Ω load and the 42.0 Ω section next to the 50 Ω line.

6. The reactance of a line or stub changes with frequency in proportion to its electrical length, so shorter elements drift less from their design values as the frequency moves.

## Chapter 5

1. RL = 20 dB; IL = −20 log₁₀ 0.995 = 0.04 dB. |S₁₁|² + |S₂₁|² = 0.01 + 0.990 = 1.000, so it is lossless within the precision given.

2. Zin = 50 (1 + S₁₁)/(1 − S₁₁) = 68.3 − j31.9 Ω.

3. 20 log₁₀(1 ± 0.06) = +0.51 dB to −0.54 dB.

4. K = (1 − 0.25 − 0.25 + 0.167)/(2 × 0.2) = 1.67. With K > 1 and |Δ| < 1, it is unconditionally stable.

5. S₁₂ = S₂₁ = 0.7∠−30°.

6. Ideal opens and shorts cannot be made at microwave frequencies, and an active device terminated in them may oscillate; matched terminations are realizable and keep the device stable during measurement.

## Chapter 6

1. C₁ = C₃ = 1/(50 × 2π × 500 MHz) = 6.37 pF; L₂ = 2 × 50/(2π × 500 MHz) = 31.8 nH.

2. 10 log₁₀(1 + 3⁶) = 28.6 dB.

3. 4.343 × 3.21/(0.02 × 500) = 1.39 dB.

4. λ0 = 24.98 mm; λg = 24.98/√(1 − (6.56/12)²) = 29.8 mm.

5. fc = 2.998 × 10⁸/(2 × 0.0158) = 9.49 GHz.

6. The midband loss is inversely proportional to the resonators' Qu, and at high power that loss is heat; a cavity's Q is roughly ten times that of microstrip, so it dissipates a tenth of the power and also handles the voltage and heat.

## Chapter 7

1. λ = 5.17 cm, so 0.48λ = 2.48 cm.

2. η = 5/8 = 0.625; G = 0.625 × 1.5 = 0.94, or −0.28 dBi.

3. (0.1224/(4π × 1000))² = 9.48 × 10⁻¹¹, or −100.2 dB.

4. λ = 3.0 cm; G = 0.6 × (π × 0.3/0.03)² = 592, or 27.7 dBi; θ₃dB ≈ 70 × 0.03/0.3 = 7.0°.

5. G = 3.98; Ae = 3.98 × 0.1224²/(4π) = 4.74 × 10⁻³ m², about 47 cm².

6. A circular wave can be split into two equal linear components at right angles with a 90° phase difference; a linear antenna at any angle captures exactly one of the two equal halves of the power.

## Chapter 8

1. β = −180° × sin 45° = −127.3°.

2. 6 + 10 log₁₀ 64 = 24.1 dBi.

3. 0.886/(32 × 0.5) = 0.0554 rad = 3.2°.

4. d/λ < 1/(1 + 0.5) = 0.667.

5. Step 11.25°, σ = 3.25° = 0.0567 rad, loss = 4.343 × 0.0567² = 0.014 dB.

6. The link factor improves by 20 log₁₀(60/2.45) = 27.8 dB. At 100 m the 60 GHz oxygen absorption of about 15 dB/km takes back only 1.5 dB.

## Chapter 9

1. 106.1 Ω lines and a 150 Ω resistor.

2. 9.03 dB.

3. 45 dB.

4. 16.1 to 27.2 dB.

5. The reflections from the two identical amplifiers return through the input hybrid 180° apart at the input port and in phase at the isolated port, where the load absorbs them.

6. The S-matrix of a lossless reciprocal network must be unitary and symmetric, and for three ports those conditions cannot hold with all diagonal terms zero.

## Chapter 10

1. About 2.78.

2. 10.7/√2.78 = 6.42 mm.

3. W/b = 30π/(√3 × 50) − 0.441 = 0.647, so W = 0.52 mm.

4. About 0.074 dB/cm, since dielectric loss scales with frequency.

5. Its field lies entirely within the homogeneous dielectric between the planes.

6. Thin substrates push the cutoff of surface-wave and higher-order modes above the operating frequency and keep the traces narrow enough for the small structures at those wavelengths.

## Chapter 11

1. Q = 2.

2. 70.7 Ω and 141.4 Ω.

3. The exponent is 2.5, so Γ = 0.082, a return loss of 21.7 dB.

4. Δf = 1/(2RC ln(1/Γ)) = 4.34 GHz.

5. It allows a controlled ripple within the passband instead of requiring a maximally flat response, which spreads the zeros of reflection across the band.

6. It absorbs the device capacitances into an artificial transmission line whose characteristic impedance is resistive over a wide band, so no single capacitance has to be matched.

## Chapter 12

1. 8.89 GHz.

2. 2500.

3. 4.343 × 5/(0.005 × 10,000) = 0.43 dB.

4. Q scales as √σ, so 3000.

5. Its walls enclose the fields completely, so energy can be lost only in the walls, the dielectric, or the coupling ports.

6. They are far too large; handsets use acoustic resonators, whose wavelengths, with sound speeds of a few thousand to about 11,000 m/s, are tens of thousands of times smaller than electromagnetic ones at the same frequency.

## Chapter 13

1. 86.3 mm.

2. ka = 0.546 and Qmin = 8.0.

3. 17.6 dBi; the horn is half the size at twice the frequency, so its gain is unchanged.

4. As the fourth power of the diameter, since it goes as the square of the area.

5. The ground plane reflects all the radiation into the upper half-space, so the same power is concentrated into half the solid angle.

6. Loss lowers the Q and widens the bandwidth beyond what the size limit would allow for a lossless antenna, at the cost of efficiency.

## Chapter 14

1. 2.50 m.

2. 2.80 km.

3. 122.0 dB in free space, 129.5 dB with absorption.

4. About 6.0 dB.

5. About 10.2 dB.

6. The direct and reflected waves nearly cancel, and the residual depends on the antenna heights and distance; the λ² of the free-space term cancels against the λ dependence of the path difference.

## Chapter 15

1. 1.4 GHz.

2. |Γ| = 1/3, so 11.1% of the power.

3. √(10^(−2.5)) = 0.056.

4. The power leaves at port 3; an ideal circulator routes port 2 to port 3.

5. 21 dBm.

6. If the antenna is disconnected or shorted, all the forward power is reflected and routed to the termination.

## Chapter 16

1. 3.0 m.

2. 4.0 m.

3. 16.5 dBi.

4. −72.4 dB.

5. 10.0 dBi.

6. Half a wavelength.

## Chapter 17

1. 169.6 K.

2. 1.00 dB.

3. 288.6 K.

4. −114.0 dBm.

5. 2.3 dB.

6. 149.7 K.

## Chapter 18

1. 20.6.

2. 0.0460.

3. About 1.80 cm.

4. 0.70 dB.

5. |Ω| = 3.16.

6. 25.0 dB.

## Chapter 19

1. 0.17 dB.

2. 9.3 dB.

3. 22.6 dB.

## Chapter 20

1. 37.0 dBm, 54.0 dBm, and 63.0 dBm.

2. 43.0 − 3 + 12 = 52.0 dBm; the cable delivers 10.0 W and loses 10.0 W.

3. 10^(−0.2) = 0.631.

4. GaN-on-SiC: it operates well above 3 GHz at 28 to 50 V with high power density and good heat removal; LDMOS is near its upper frequency limit there.

5. 1 dB in total, so 40/0.794 = 50.4 W.

6. At a given power, a higher voltage swing means a smaller current and a higher optimum load resistance, closer to 50 Ω, so the matching network needs a smaller transformation ratio, a lower Q, and less loss.

## Chapter 21

1. 45²/400 = 5.06 Ω; 25²/100 = 6.25 Ω.

2. 200/45 = 4.44 A.

3. Q = √(50/3.125 − 1) = 3.87.

4. 3.1²/2 = 4.8 Ω.

5. By four.

6. A conjugate match would present the device's high output resistance as the load, so the voltage would limit at the knee and supply long before the current reached its maximum, delivering much less power than the load-line match.

## Chapter 22

1. ηD = 50/75 = 66.7%; PAE = 49/75 = 65.3%; Pdiss = 75 + 1 − 50 = 26 W.

2. 60.2%.

3. The device conducts for only part of the cycle, so for a given drive amplitude the fundamental output current is smaller; reaching full current needs a larger input swing, and the gain falls.

4. PDC = 333.3 W, so 133.3 W is dissipated.

5. Pout = 1 W, Pin = 0.158 W, PDC = 2 W; PAE = (1 − 0.158)/2 = 42.1%.

6. The switch voltage is zero, and its slope is zero, at the instant the switch closes, so no charge is dumped from the shunt capacitor.

## Chapter 23

1. 20 + 46/2 = 43 dBm.

2. 46 + 2 × 3 = 52 dBc.

3. (π/4) × 10^(−0.5) = 24.8%.

4. (π/4) × 0.316/0.5 = 49.7%.

5. Average 200 × 10^(−0.9) = 25.2 W; DC 25.2/0.55 = 45.8 W.

6. The tones fall 1 dB per decibel of input while third-order products fall 3 dB, so their ratio grows by 2 dB.

## Chapter 24

1. 45 + 30 × 2.5 = 120 °C.

2. cos²(7.5°) = 98.3%.

3. (7/8)² = 0.766, a fall of 1.16 dB.

4. |Γ| = 0.5/2.5 = 0.2, so 4.0%.

5. 2.97 dB, leaving 40 × 10^(−0.297) = 20.2 W.

6. When a device fails, the isolation loads absorb a large part of the surviving devices' power, which is far more than the small imbalance of normal operation.

## Chapter 25

1. 290 × (1.585 − 1) = 170 K.

2. F = 1.202 + (15.85 − 1)/100 = 1.351, or 1.31 dB.

3. −174 + 73.0 + 2 = −99.0 dBm.

4. (2/3)(0 + 99.0) = 66.0 dB.

5. −30 − 120 + 60 = −90 dBm.

6. A passive loss at T0 attenuates the signal but delivers the same thermal noise power kT0B, so the SNR falls by the loss, and nothing before it provides gain to hide it.

## Chapter 26

1. 20 log₁₀145 = 43.2 dB.

2. It falls by 3 dB, since the noise term is divided by Ps.

3. σφ = √(2 × 10⁻¹⁰ × 999,000) = 0.0141 rad = 0.81°; jitter = 0.0141/(2π × 10⁹) = 2.25 ps.

4. N = 145; −150 + 43.2 = −106.8 dBc/Hz.

5. Twice the 100 kHz value: 63.8 µs.

6. The divider makes the loop multiply the reference's phase deviations by N, so the reference's noise appears at the output raised by 20 log₁₀N; a smaller N multiplies it less.

## Chapter 27

1. 2⁴ = 16.

2. c/(2 × 150 MHz) = 1.0 m.

3. c × 500 µs/2 = 75 km.

4. 1 kW × 0.001 = 1 W.

5. 15.4 kHz.

6. S = 10¹⁴ Hz/s, so fb = 2 × 100 × 10¹⁴/2.998 × 10⁸ = 66.7 MHz.

## Chapter 28

1. F = 1.259 + 0.8 × 0.16/|1 + j0.4|² = 1.259 + 0.128/1.16 = 1.369, or 1.36 dB.

2. F = 1.122 × (1.259 + (10 − 1)/100) = 1.122 × 1.349 = 1.514, or 1.80 dB.

3. A real, noiseless resistive part, ωT Ls, which moves the input match toward the noise-optimum condition.

4. (15.85 − 1)/31.6 = 0.47.

5. If it has less gain, the following stages' noise is divided by a smaller number and may raise the system noise figure by more than the device's own improvement.

6. The reflections of the two identical halves reach the input through 90° hybrids with a 180° relative phase and cancel, being dissipated in the hybrid's isolated load.

## Chapter 29

1. fLO = 6.8 GHz; the image is at 7.8 GHz.

2. fLO = 830 MHz; the half-IF spur is at fRF − fIF/2 = 865 MHz.

3. 45 dB.

4. About 6.5 dB.

5. The image lies 2fIF from the wanted signal, so a higher IF moves it farther away, where a filter can attenuate it more easily.

6. The oscillator is at the carrier frequency, so any leakage appears in the middle of the transmitted channel, where it raises the EVM and cannot be filtered out.

7. 3.92 dB.

8. About +17 dBm.

## Chapter 30

1. 126.1 dB.

2. 20 + 30 − 120 + 30 = −40 dBm.

3. Path loss 209.6 dB; C/N0 = 50 − 209.6 + 10 + 228.6 = 79.0 dBHz; C/N = 79.0 − 70.0 = 9.0 dB.

4. 290(10^0.15 − 1) = 119.6 K.

5. 20 log10 3 = 9.5 dB.

6. Received C/N0 depends on antenna gain and system noise temperature only through their ratio, so G/T captures the receiving station's whole contribution in one number.

## Chapter 31

1. |Γ| = 0.333, return loss 9.5 dB, 11.1% reflected.

2. 70.7 V matched; |Γ| = 0.667, so the worst case is 117.9 V.

3. 75% of the power, or −1.25 dB.

4. With zero gate voltage the device is fully on, so powering the drain first would draw an uncontrolled current that can destroy it.

5. The smallest reflection it can measure, because forward power leaking into the reverse port masks a weak reflection.

6. The ballast resistor provides negative feedback against the falling base-emitter voltage as the device heats, preventing thermal runaway and current hogging between cells.

## Chapter 32

1. 40 kW; perveance 2/(2 × 10⁴)^1.5 = 0.71 µperv.

2. γ = 1.039, so v = 0.272c.

3. 1429 W in; 429 W of heat.

4. The klystron interacts through resonant cavities, which have narrow bandwidths; the helix is a nonresonant slow-wave structure whose phase velocity is nearly constant over a wide band.

5. It decelerates the spent electrons with a retarding voltage before they land, returning part of their energy to the supply and raising efficiency.

6. 2400 × 10^(−0.05) = 2.14 kW.

## Chapter 33

1. 10 W/m².

2. 600/1500 = 0.4 mW/cm², or 4 W/m².

3. 50 / (4π × 9) = 0.44 W/m².

4. √(1000 / (4π × 10)) = 2.82 m.

5. E = √(S × 377 Ω) = 61.4 V/m.

6. The head is in the antenna's reactive near field, where power density is not well defined and the energy couples directly into tissue, so the absorbed power per mass is what matters.

## Chapter 34

1. −170 dBm/Hz.

2. 10 log10(1 + 10^(−0.6)) = 0.97 dB.

3. Noise floor −174 + 70 + 4 = −100 dBm, so 123 dB.

4. −27 dBm.

5. The receiver is not listening while the transmitter is on, so only the switch's isolation and settling matter, not continuous isolation during transmission.

6. The noise lies at the same frequencies as the wanted signal, so any filter that removed it would also remove the signal; it has to be stopped before it reaches the receiver.

## Chapter 35

1. R = 0.5768 × 144/10 = 8.31 Ω; C = 0.1836/(2π × 13.56 × 10⁶ × 8.31) = 259 pF.

2. 3.562 × 12 = 42.7 V.

3. 1292 MHz.

4. 78.5%, the class B value.

5. Its peak voltage is about 3.6 times the supply, and GaN's high breakdown voltage tolerates that swing at a useful supply voltage.

6. Each additional harmonic adds less to the fundamental amplitude, so the first two or three do most of the waveform shaping; the device capacitance also suppresses the higher harmonics.

## Chapter 36

1. 78.5% × 10^(−0.5) = 24.8%.

2. 56%.

3. cos²φ = 0.5, so φ = 45°.

4. About 60 to 100 MHz.

5. The envelope's bandwidth is several times the signal's, and the amplitude and phase paths must be aligned precisely; efficient supply modulators cannot follow a wide envelope.

6. At back-off, the two amplifiers' outputs partially cancel, and an isolating combiner dissipates the cancelled power in its isolation resistor instead of returning it.

## Chapter 37

1. 19.4 Ω.

2. 56.3%.

3. 20.6%.

4. −15 dBm, 40 dB below each tone.

5. 46 dB.

6. 6 dB.

## Chapter 38

1. 21.

2. About 100 MHz.

3. 31.3%.

4. 78.5%.

5. The predistorter only changes the input; the amplifier's maximum output is fixed by its supply and device, so peaks beyond saturation cannot be restored.

6. Any two of: bias-network impedance at the envelope frequency, thermal changes in the device, and charge trapping in GaN.

## Chapter 39

1. 47.7 Hz.

2. N = 116, F = 0.

3. −113.7 dBc/Hz.

4. About 8.9 µHz.

5. About −72 dBc.

6. 3 dB.

## Chapter 40

1. 3 dB.

2. +0.09 dB and −0.09 dB.

3. 63 dB.

4. 31.6 mW.

5. It responds to the signal's peaks, and OFDM's peaks are 8 to 10 dB above its average power.

6. The input power in decibels, over a wide range.

# Appendix C. Sources

Chris Bowick, RF Circuit Design, 2nd edition, for the practical start of Chapters 1 to 3.

David M. Pozar, Microwave Engineering, 4th edition; Michael Steer, Microwave and RF Design, volumes 1 to 3 (open textbook), for lines, matching, S-parameters, filters, and waveguide.

Guillermo Gonzalez, Microwave Transistor Amplifiers, for stability and gain from S-parameters.

George Matthaei, Leo Young, and E. M. T. Jones, Microwave Filters, Impedance-Matching Networks, and Coupling Structures, and Jia-Sheng Hong, Microstrip Filters for RF/Microwave Applications, for Chapter 6.

Constantine Balanis, Antenna Theory, 4th edition, and John Volakis (editor), Antenna Engineering Handbook, for Chapters 7 and 8; Robert Mailloux, Phased Array Antenna Handbook, for arrays.

E. Hammerstad and Ø. Jensen, "Accurate Models for Microstrip Computer-Aided Design" (IEEE MTT-S, 1980), for the microstrip numbers. The MIT Radiation Laboratory Series is the historical record of the field.

ITU-R Recommendations P.676 (gaseous attenuation) and P.838 (rain) for millimeter-wave propagation.

E. J. Wilkinson, "An N-Way Hybrid Power Divider," IRE Transactions on Microwave Theory and Techniques, 1960, for Chapter 9.

E. O. Hammerstad, "Equations for Microstrip Circuit Design," European Microwave Conference, 1975, and H. A. Wheeler, "Transmission-Line Properties of a Strip on a Dielectric Sheet on a Plane," IEEE MTT, 1977, for Chapter 10.

R. M. Fano, "Theoretical Limitations on the Broadband Matching of Arbitrary Impedances," Journal of the Franklin Institute, 1950, for Chapter 11.

L. J. Chu, "Physical Limitations of Omni-Directional Antennas," Journal of Applied Physics, 1948, and Constantine A. Balanis, Antenna Theory, 4th edition, for Chapter 13.

ITU-R Recommendations P.526 (diffraction), P.676 (gaseous attenuation), and P.838 (rain), for Chapter 14.

Benjamin Lax and Kenneth J. Button, Microwave Ferrites and Ferrimagnetics, and Pozar, Chapter 9, for Chapter 15; IEEE Std 149, Test Procedures for Antennas, and Gary E. Evans, Antenna Measurement Techniques, for Chapter 16.

Jia-Sheng Hong, Microstrip Filters for RF/Microwave Applications, for the coupling-matrix design of Chapter 18; Stephen A. Maas, Microwave Mixers, for Chapter 19.

Steve C. Cripps, RF Power Amplifiers for Wireless Communications, 2nd edition, and Advanced Techniques in RF Power Amplifier Design, for the load line, classes, and Doherty (Chapters 21 to 23).

Andrei Grebennikov, Nathan O. Sokal, and Marc J. Franco, Switchmode RF and Microwave Power Amplifiers, for classes E and F.

Peter B. Kenington, High-Linearity RF Amplifier Design, for intermodulation and predistortion.

Kevin McClaning, Wireless Receiver Design for Digital Communications, and Behzad Razavi, RF Microelectronics, 2nd edition, for noise cascades, dynamic range, and oscillators (Chapters 25 and 26).

Floyd M. Gardner, Phaselock Techniques, 3rd edition, and Roland E. Best, Phase-Locked Loops, for the PLL; D. B. Leeson, "A Simple Model of Feedback Oscillator Noise Spectrum," Proceedings of the IEEE, 1966.

Merrill I. Skolnik, Introduction to Radar Systems, 3rd edition, and Mark A. Richards, Fundamentals of Radar Signal Processing, for Chapter 27.

W. H. Doherty, "A New High Efficiency Power Amplifier for Modulated Waves," Proceedings of the IRE, 1936.

Les Besser and Rowan Gilmore, Practical RF Circuit Design for Modern Wireless Systems, and Guillermo Gonzalez, Microwave Transistor Amplifiers, 2nd edition, for the LNA and mixers (Chapters 28 and 29).

David M. Pozar, Microwave Engineering, 4th edition, for the link equation (Chapter 30).

Jerry C. Whitaker, Power Vacuum Tubes Handbook, and A. S. Gilmour, Klystrons, Traveling Wave Tubes, Magnetrons, Crossed-Field Amplifiers, and Gyrotrons, for Chapter 32.

IEEE C95.1-2019; ICNIRP Guidelines for Limiting Exposure to Electromagnetic Fields (100 kHz to 300 GHz), 2020; 47 CFR 1.1310; and FCC OET Bulletin 65, for Chapter 33.

N. O. Sokal and A. D. Sokal, "Class E: A New Class of High-Efficiency Tuned Single-Ended Switching Power Amplifiers," IEEE JSSC, 1975; Marian K. Kazimierczuk, RF Power Amplifiers, 2nd edition; and F. H. Raab, "Maximum Efficiency and Output of Class-F Power Amplifiers," IEEE MTT, 2001, for Chapter 35.

F. H. Raab et al., "Power Amplifiers and Transmitters for RF and Microwave," IEEE MTT, 2002, and H. Chireix, "High Power Outphasing Modulation," Proceedings of the IRE, 1935, for Chapter 36.

Steve C. Cripps, RF Power Amplifiers for Wireless Communications, 2nd edition, and Fadhel M. Ghannouchi, Oualid Hammi, and Mohamed Helaoui, Behavioral Modeling and Predistortion of Wideband Wireless Transmitters, for Chapters 37 and 38; 3GPP TS 38.104 and 38.101-1 for ACLR and power-control requirements.

Dean Banerjee, PLL Performance, Simulation, and Design, and Analog Devices, A Technical Tutorial on Digital Signal Synthesis, for Chapter 39; Analog Devices and Texas Instruments application notes on RF power detectors, for Chapter 40.