Semiconductor Physics and Devices

Book 3 of the Electrical Engineering Series

Parts I–III  —  The Crystal, the Diode, and the Transistor

Lothar J. Musiol

Copyright © 2026 Lothar J. Musiol. Kindle edition. This file holds Parts I through III of a longer book. Parts IV through VI are not in it yet.

# How to Use This Part

Book 1 taught the circuit laws and the passive parts. Book 2 treated a diode, a transistor, and an op-amp as blocks you could drop into a schematic. This part opens those blocks. By the end of it you can say, with a number, how many free carriers a piece of doped silicon has, why a pn junction grows a built-in voltage with nobody applying a battery, and why the phrase "the diode turns on at 0.7 volts" is a useful sketch and a bad physical claim.

Two numbers are fixed for the whole part, so that a later chapter cannot quietly adopt a different room temperature. Thermal voltage VT is 26 mV. The intrinsic carrier concentration of silicon, ni, is 1.0×10^10 per cubic centimeter. Real tables scatter a little around both of those, because the bandgap model and the exact temperature move them. Hand work in this book uses the declared pair. When a vendor model quotes a different ni, use the vendor number for that part and keep this pair for the teaching problems.

Electron and hole mobilities depend on doping and temperature. Whenever a problem in this part does not hand you a mobility, use μn = 1000 cm²/V·s and μp = 480 cm²/V·s. Those are round working values for lightly doped silicon near room temperature, not a promise about a particular wafer.

Later parts of this book take up the bipolar transistor, the MOSFET, the inside of an op-amp, and CMOS gates. The circuit check for those parts is Tietze, Schenk, and Gamm, Halbleiter-Schaltungstechnik, 16th edition (2019), whose op-amp chapter was rewritten for that edition. High-speed boards, radio layout, switching regulators, and software-defined radio are later books in this series. They are named in the series plan so this part does not pretend to cover them.

# Chapter 1. A Crystal That Can Be Talked Into Conducting

Window glass and a processor die both contain silicon. One of them sits in a wall and refuses to carry a current you would bother to measure. The other switches billions of times a second because a small voltage was allowed to decide how many charges are free to move. The difference is not a mystery metal. It is how pure the crystal is, and which few atoms you deliberately put in.

## 1.1 Three ways a solid treats its electrons

A useful picture, good enough for every circuit in this series, sorts solids by the energy gap between the last filled band of electron states and the next empty one. In a metal the bands overlap, so a tiny electric field finds electrons already free to drift. In a good insulator the gap is several electron-volts wide, and ordinary voltages do not lift electrons across it. Silicon sits in between. Its gap is about 1.12 eV at room temperature. Germanium is narrower, about 0.66 eV, which is why a germanium junction leaks more when the board gets warm. Gallium arsenide is wider, about 1.42 eV. Silicon carbide, used where the voltage and the heat are both high, is wider still, a few electron-volts depending on the crystal form. Silicon won the ordinary chip not because its gap is magical, but because the industry learned to grow it as a nearly perfect crystal and to put an excellent insulator, silicon dioxide, on top of it.

## 1.2 Intrinsic means almost empty

In a perfect silicon crystal at absolute zero, every bond is full and no charge moves. At room temperature a few electrons have enough thermal energy to break a bond. Each broken bond leaves an electron in the conduction band and a hole, a missing bond, in the valence band. They are created in pairs, so in pure material their concentrations are equal. That common value is ni, and this book takes it as 1.0×10^10 cm⁻³.

A cubic centimeter of silicon holds about 5×10^22 atoms. Ten billion free electrons in that same volume is a very small fraction. Intrinsic silicon is a poor conductor. The whole point of doping, in the next chapter, is to raise one of those two populations by many orders of magnitude while the other falls.

## 1.3 Conductivity is charge times speed per field

Drift current density is conductivity times electric field. For a semiconductor that still has both electrons and holes,

σ = q (n μn + p μp)

where q is the elementary charge, n and p are the electron and hole concentrations, and μn and μp are the mobilities. Resistivity is 1/σ. In doped material one term usually swamps the other, which is why a resistor formula and a doped-silicon formula can look alike: both say "lots of carriers, easy current."

Key idea. Pure silicon at room temperature has very few free charges. Doping, not the word "semiconductor" by itself, is what makes a useful current.

## Worked Example 1.1

A piece of n-type silicon is doped so that n = 1.0×10^16 cm⁻³. The hole population has already fallen to the value required by the mass-action law in Chapter 2, p = ni²/n = 1.0e+04 cm⁻³. Using this book's μn, find the resistivity. The hole term is negligible next to the electron term.

σ = q n μn = (1.602×10⁻¹⁹)(1.0×10^16)(1000) = 1.602 (Ω·cm)⁻¹. ρ = 1/σ = 0.624 Ω·cm. A metal is a million times more conductive. This doped slice is already a practical semiconductor, and it is still nothing like copper.

## Practice

1. Repeat Worked Example 1.1 with n = 1.0×10^15 cm⁻³ and the same mobility. Find p and ρ.

2. Why does a wider bandgap, all else equal, mean a smaller ni and a junction that leaks less when hot?

3. Intrinsic silicon has n = p = ni. Estimate σ with both mobilities. Is the hole term ignorable here, the way it was in the worked example?

# Chapter 2. Doping, and the Partner That Gets Scarce

Soup tastes salty after a pinch of salt, not after you have replaced the water. A semiconductor is the same kind of trick. You leave the crystal almost entirely silicon, and you swap in about one foreign atom per million, or per billion. That pinch decides whether the free charges are electrons or holes, and about how many of them there are.

## 2.1 Donors and acceptors

A column-V atom such as phosphorus has one more valence electron than silicon. Sitting on a silicon site, it completes the four bonds and has one electron left, loosely held. A little thermal energy frees that electron into the conduction band. The phosphorus atom, now positive, stays put. It donated an electron. Call its concentration ND. The material is n-type, and the electrons are the majority carriers.

A column-III atom such as boron has one valence electron too few. It completes three bonds and borrows an electron from a neighboring bond, leaving a hole that can hop. The boron atom, now negative, stays put. Call its concentration NA. The material is p-type, and the holes are the majority carriers.

## 2.2 Mass action

In equilibrium, generation and recombination balance. The product of the two carrier concentrations is fixed by the crystal and the temperature, not by how you doped it:

n p = ni²

Add donors and you raise n. The product cannot rise with it, so p falls. Add acceptors and the mirror happens. This is why a heavily doped n-type region is full of electrons and starved of holes. The starved population is still real, and it will matter as soon as a junction sweeps those minorities across a boundary.

## 2.3 Charge neutrality, the short form

A lump of semiconductor on the bench is not charged up like a capacitor plate. In the bulk, away from surfaces,

n + NA = p + ND

for ionized donors and acceptors, which is the right assumption at room temperature for ordinary silicon doping. If donors dominate and ND is far above ni, then n ≈ ND and p ≈ ni²/ND. For the 10^16 cm⁻³ example, p falls to 1.0e+04 cm⁻³, six decades below the majority population. The same arithmetic with acceptors swaps the names.

Key idea. Doping sets the majority. Mass action sets the minority. You do not choose them independently.

## Worked Example 2.1

Boron is added to silicon at NA = 1.0×10^17 cm⁻³. Donors are negligible. Find the majority and minority concentrations.

Holes are the majority: p ≈ NA = 1.0×10^17 cm⁻³. Electrons are the minority: n = ni²/NA = 1.0e+03 cm⁻³. There is one free electron for every ten million holes.

## Practice

1. Phosphorus at 1.0×10^15 cm⁻³, acceptors negligible. Find n and p.

2. Someone says "n-type means there are no holes." Answer with the mass-action law and one number from this chapter.

3. Both ND = 1.0×10^16 cm⁻³ and NA = 1.0×10^15 cm⁻³ are present. Who wins, and what is the approximate majority concentration? Use n + NA = p + ND and n p = ni², and drop the small term only after you have named it.

# Chapter 3. Two Ways Charge Moves

A crowd leaves a stadium two ways. People walk downhill because the ground slopes, and people also spread from the packed gate into the empty street even if the street is flat. Electrons do both. The slope is the electric field. The packed gate is a concentration gradient. Circuits need both pictures, because a diode uses diffusion at the junction and drift in the bulk.

## 3.1 Drift

An electric field pulls electrons one way and holes the other. The drift speeds are μn E and μp E, in opposite directions, but both movements carry positive current in the direction of the field. Holes moving with the field are a positive current. Electrons moving against the field are also a positive current, because negative charge moving backward is the same as positive charge moving forward. That is why the conductivity formula adds the two terms instead of subtracting them.

## 3.2 Diffusion and the Einstein relation

Where the concentration changes with position, carriers spread from crowded toward empty. The diffusion constants Dn and Dp have units of cm²/s. In equilibrium they are not a second independent property of the crystal. Einstein's relation ties them to the mobilities and to VT:

Dn = μn VT,    Dp = μp VT

with VT in volts. This book's μn and VT give Dn = 26.0 cm²/s. Dp = μp VT = 12.48 cm²/s. The electron diffusion current density along x is q Dn dn/dx, and the sign is fixed by the fact that electrons diffusing toward +x carry negative charge that way, which is a negative current in the +x direction. Holes diffusing toward +x are a positive current. The formulas used below take magnitudes when the question is "how large," and they keep the sign when the question is "which way."

## 3.3 Drift and diffusion together

In the bulk of a uniformly doped resistor, the gradient is about zero and drift does the work. At a pn junction in equilibrium, a large diffusion current wants to pour carriers across the boundary, and a drift current from the built-in field cancels it. Net current is zero. The cancellation is exact in equilibrium and is the reason the next chapter can compute a voltage without ever connecting a battery.

Key idea. Field moves carriers as drift. A crowd moving into an empty region is diffusion. Equilibrium at a junction is those two currents canceling.

## Worked Example 3.1

Electron concentration falls from 1.0×10^16 cm⁻³ to 1.0×10^15 cm⁻³ across 1 μm (1×10⁻⁴ cm). Using this book's Dn, find the magnitude of the diffusion current density.

|dn/dx| = 9.0×10^15 / 1×10⁻⁴ = 9.0×10^19 cm⁻⁴. |Jn| = q Dn |dn/dx| = (1.602×10⁻¹⁹)(26.0)(9.0×10^19) = 374.909 A/cm². That is a serious current density for a gradient only a micron long, which is why junctions, whose gradients are even steeper, care so much about diffusion.

## Practice

1. Compute Dn and Dp from this book's mobilities and VT. Give both in cm²/s.

2. A hole gradient of 1.0×10^20 cm⁻⁴ drives hole diffusion. Find the magnitude of Jp. Which way does conventional current point, relative to the direction the holes are spreading?

3. In a long uniformly doped bar, dn/dx = 0 and E = 10 V/cm, n = 1.0×10^16 cm⁻³. Find the drift current density and compare it with Worked Example 3.1.

# Chapter 4. The Junction Sits There with a Voltage Already On It

Push a crowded room against an empty hall and leave the door open. People spill through until a jam at the doorway, people pressed shoulder to shoulder, makes further spilling as hard as the crowding that caused it. A pn junction does that with electrons and holes. Nobody has to apply a battery. The voltage is the jam.

## 4.1 What crosses, and what is left behind

On the n-side, electrons are many and holes are few. On the p-side, the reverse. Electrons diffuse into the p-side. Holes diffuse into the n-side. Each electron that leaves the n-side leaves a positive donor ion behind. Each hole that leaves the p-side leaves a negative acceptor ion behind. Those ions cannot wander off. They are stuck in the crystal. A sheet of positive charge therefore faces a sheet of negative charge, with a region between them that has been emptied of mobile carriers. That region is the depletion layer, or space-charge region.

## 4.2 Built-in voltage

The field of those two sheets points from the positive donor ions toward the negative acceptor ions, which is from n toward p. It pushes electrons back toward the n-side and holes back toward the p-side, opposing the diffusion that created it. Equilibrium is the balance. The potential difference that belongs to that balance is the built-in voltage,

Vbi = VT ln(NA ND / ni²)

for an abrupt junction with both sides doped well above ni. Notice what raises Vbi: heavier doping on either side, or a smaller ni. Notice what does not appear: the applied voltage. Vbi is a property of the two dopings and the temperature. Applying a battery changes the barrier the carriers actually face, which is Vbi minus the applied forward voltage, but it does not rewrite this formula.

## 4.3 How wide the empty region is

For a one-sided abrupt junction, almost all of the depletion width sits on the lightly doped side. If that side is n-type with doping ND and the applied voltage is V (positive V means forward),

W = sqrt( 2 εs (Vbi − V) / (q ND) )

εs is the permittivity of silicon, 11.7 times the permittivity of free space. Forward bias shrinks W. Reverse bias grows it. The square root is why doubling the reverse voltage does not double the width.

Key idea. A pn junction in a drawer already has a built-in voltage. The battery later adds to that story. It does not start it.

## Worked Example 4.1

Both sides are doped at 1.0×10^16 cm⁻³. Find Vbi. Then treat the junction as one-sided on an n-type side of that same doping, at zero applied volts, and find W. This is a teaching idealization of a symmetric junction, used so the arithmetic stays visible.

NA ND / ni² = 1.0×10^32 / 1.0×10^20 = 1.0×10^12. ln(10^12) = 27.631. Vbi = (0.026)(27.631) = 0.718 V. W = 3.048e-05 cm = 0.305 μm. A few tenths of a micron of emptied crystal is enough to hold about seven-tenths of a volt.

## Practice

1. NA = 1.0×10^18 cm⁻³ and ND = 1.0×10^16 cm⁻³. Find Vbi with this book's constants.

2. Using the W of Worked Example 4.1 as the zero-bias width, does a reverse bias of a few volts make W several times larger, or only somewhat larger? Argue from the square root before you compute.

3. Why does heating the crystal, which raises ni, lower Vbi?

# Chapter 5. The Diode Law Is a Crowd Crossing a Barrier

A turnstile at a stadium lets a few people through when the bar is high, and a great many when someone lowers it by a hand's width. The diode is that turnstile for the minority carriers waiting at the edge of the depletion layer. Lower the barrier a little and the forward current multiplies. Raise it and the forward crowd disappears, leaving only a small reverse trickle.

## 5.1 The equation

For an ideal diode the current is

I = IS ( e^(V / VT) − 1 )

V is the applied voltage, positive when the p-side is positive, which is forward bias. IS is the saturation current, set by the area, the doping, and the lifetime of the minorities. This chapter's worked diode uses IS = 1 fA = 10⁻¹⁵ A, a small-junction teaching value, not a claim about a particular part number. VT is 26 mV. The "− 1" matters near zero and in reverse. A few tenths of a volt forward, the exponential is so large that I ≈ IS e^(V/VT).

## 5.2 Sixty millivolts per decade

Ask how much extra forward voltage multiplies the current by ten, once you are well above VT. The exponential must grow by 10, so the added voltage is VT ln(10).

VT ln(10) = 0.026 × 2.3026 = 59.9 mV.

Call it 60 mV per decade of current at this book's room temperature. A real silicon diode is often closer to 60 mV only in a mid-current window. At very low current, recombination in the depletion layer makes the effective step larger. At high current, series resistance steals voltage. The ideal law is the spine. The part on the bench wears extra clothes, which is Chapter 6.

## 5.3 There is no switch at 0.7 V

A forward drop near 0.7 V is what you see when a small silicon diode is carrying a current of a good fraction of a milliamp, with an IS on the order of a femtoamp. It is an operating point, not a threshold the device crosses. Below that voltage the diode is already conducting. It is conducting less. Circuit sketches that draw an ideal switch plus a 0.7 V battery are welcome in Book 1 problems. They are the wrong picture if you are asking how leakage, a photodiode, or a low-current bias network behaves.

Key idea. Forward current is exponential in voltage. A "turn-on voltage" is a current you chose to care about, read back through that exponential.

## Worked Example 5.1

IS = 1.0×10⁻¹⁵ A and VT = 26 mV. Find I at 0.50 V, 0.60 V, and 0.70 V. Then find the forward voltage that produces 1.0 mA.

At 0.50 V, I = 2.248e-07 A = 0.225 μA. At 0.60 V, I = 1.052e-05 A = 10.52 μA. At 0.70 V, I = 4.927e-04 A = 0.493 mA. From 0.60 V to 0.70 V the current rises by a factor of 46.8, close to the decade-and-a-half that 100 mV predicts (100/60 ≈ 1.7 decades, and 10^1.67 ≈ 47). For 1.0 mA, V = VT ln(I/IS + 1) = 0.718 V. The familiar seven-tenths of a volt showed up because we asked for about a milliamp, not because the silicon changed state.

## Practice

1. Same diode. Find I at 0.65 V.

2. How much must V rise to take this diode from 0.1 mA to 1 mA, using the 60 mV/decade rule? Check it against the exact formula.

3. Reverse bias of 0.2 V. Compute I from the full equation. Why is the result about −IS, and why is a real reverse current often larger than this ideal?

# Chapter 6. The Diode You Can Buy

The part in the drawer has a glass or plastic body, two leads, a junction area someone chose, and a data sheet with a curve that only roughly matches Chapter 5. The law is still in there. Three extras decide whether a circuit works: the junction behaves as a capacitor, a large reverse voltage eventually breaks the crystal down, and a forward current does not stop the instant you reverse the voltage.

## 6.1 Junction capacitance

The depletion layer is an insulator between two conducting regions, which is a capacitor. Its value falls as reverse bias widens the layer. For an abrupt junction a serviceable model is

Cj = Cj0 / sqrt(1 − V/Vbi)

with V negative in reverse, and Cj0 the zero-bias value set by area and doping. Varactor diodes are ordinary junctions used on purpose as voltage-controlled capacitors. Rectifier diodes carry the same capacitance whether you asked for it or not. At radio frequency that capacitance is part of the matching network. At audio it is often ignorable. The frequency, not the part's name, decides.

## 6.2 Breakdown

Push reverse bias high enough and the field in the depletion layer rips bonds apart or gives carriers enough energy between collisions to create more carriers. Current then rises steeply at nearly constant voltage. A Zener diode is a junction designed so that this knee sits at a useful, specified voltage, and so that the part can survive the current you intend to run through that knee. A signal diode's breakdown voltage is a limit, not a feature. Exceed it without a current limit and the junction overheats.

Two microscopic stories share that knee. Zener tunneling dominates at low breakdown voltages. Avalanche, the carrier-multiplication story, dominates at higher ones. Data sheets still say "Zener" for both. The circuit fact is the steep reverse knee. The name on the bag is older than the distinction.

## 6.3 Reverse recovery

Forward current stores minority charge on both sides of the junction. When the circuit reverses the voltage, that stored charge has to come back out, or recombine, before the diode can block. For a interval on the order of the storage delay the diode still conducts backward. Fast-recovery and Schottky diodes exist because that interval is expensive in a switching supply. A Schottky diode is a metal-semiconductor junction. It has a lower forward drop and almost no stored minority charge, and it generally leaks more and breaks down sooner than a pn diode of similar size. Choose it when the switching edge is the problem. Choose a pn diode when the reverse voltage and the leakage are the problem.

Key idea. The exponential law is the junction. Capacitance, breakdown, and stored charge are why the data sheet is thicker than the equation.

## Worked Example 6.1

An abrupt-junction diode has Cj0 = 10 pF and the Vbi of Worked Example 4.1, 0.718 V. Find Cj at a reverse bias of 5 V.

V/Vbi = −5 / 0.718 = -6.960. 1 − V/Vbi = 7.960. The square root is 2.821. Cj = 10 pF / 2.821 = 3.54 pF. Five volts of reverse bias cut the capacitance to about a third. A tuner that expects a wide capacitance range is using this same square root, over whatever bias its varactor is allowed.

## Practice

1. Cj0 = 20 pF, same Vbi, reverse bias 2 V. Find Cj.

2. A rectifier data sheet lists a reverse-recovery time. What stored thing is that time waiting on, and why does a Schottky diode largely skip it?

3. A 5.1 V Zener is used as a reference with a series resistor from a 12 V rail. In one sentence, what does the resistor do that the junction itself does not?

# Chapter 7. GaAs, GaN, and InP Are Not One Transistor

A handset, a radar brick, and a 94 GHz receiver can all be described as "a microwave transistor on a chip." The crystals are not interchangeable. Gallium arsenide is the mature microwave compound semiconductor. Gallium nitride is the wide-bandgap material for watts, and also, on a silicon wafer, for hundreds of volts. Indium phosphide is the III-V you pick when the gate is tens of nanometers wide and the noise figure is the specification. They meet in millimeter-wave power amplifiers. Almost nowhere else.

## 7.1 Two figures of merit, not one winner

Johnson's figure of merit rewards breakdown voltage times frequency. Transit time and mobility reward a fast, light electron. GaN wins the first contest. InP wins the second. That is the whole split, and it is why a chapter that ranks them on a single "which is more advanced" scale is already wrong.

Use these working numbers, the same way Part I used a declared ni. GaAs bandgap about 1.42 eV, breakdown field about 0.4 MV/cm, bulk electron mobility about 8500 cm²/V·s. GaN bandgap about 3.4 eV, breakdown field about 3.3 MV/cm, two-dimensional-electron mobility about 1500 to 2000 cm²/V·s. InP itself is about 1.34 eV. The channel in an InP HEMT is usually InGaAs, about 0.7 to 1.3 eV, with a very high mobility and the highest saturation velocity of the usual RF III-Vs. RF supply voltage is about 3 to 12 V for GaAs, about 1 to 3 V for an InP HBT and about 1 to 5 V for an InP HEMT, and about 20 to 50 V for GaN. Power density is about 0.5 to 2 W per millimeter of gate for GaAs, about 0.3 to 1.5 W/mm for InP, and about 5 to 12 W/mm for GaN. Production GaN reaches roughly 80 to 200 GHz ft. The best published InP HEMTs are past 600 GHz ft, with fmax in the terahertz class.

## 7.2 Say HEMT or HBT before you say the crystal

GaAs comes as a MESFET, which is old, a pHEMT, and an HBT. The pHEMT is the low-noise amplifier, the switch, and some power amplifiers. The HBT is the handset power amplifier and the gain block. GaN, in any part you can buy for RF or for power conversion, is a HEMT. There is no serious GaN HBT in production. InP comes as a HEMT, or a metamorphic HEMT, with an InGaAs channel, and as an HBT. The HEMT is the low-noise amplifier, the mixer, the switch, and the low-power millimeter-wave amplifier. The HBT is high-speed analog, data converters, and optical modulator drivers, plus some power amplifiers.

Most RF HEMTs are depletion-mode. The gate wants a negative supply, and the drain must not come up before that gate is in a safe place. That is a sequencer, not the enhancement-mode MOSFET of a later chapter. Enhancement-mode GaN shows up in power conversion. Do not import that bias story into a radar PA without reading the data sheet.

Key idea. Name the device. "InP versus GaN" is not a comparison until you have said HEMT or HBT.

## 7.3 Why voltage buys watts

For a sine-wave load line, the optimum resistance is the voltage swing divided by the current swing, and the RF power is half the product of those peaks. A higher legal voltage, at the same current, is both more power and an easier match.

## Worked Example 7.1

A cell is allowed 0.20 A peak. Compare a 5 V peak swing with a 28 V peak swing. This is a GaAs-like rail against a GaN-like rail, not a promise about a foundry.

At 5 V, Ropt = 5 / 0.20 = 25 Ω, and P = (5)(0.20)/2 = 0.50 W. At 28 V, Ropt = 28 / 0.20 = 140 Ω, and P = (28)(0.20)/2 = 2.8 W. Same current, 5.6 times the voltage, 5.6 times the power, and the load the matcher has to present moved from 25 Ω toward 140 Ω, closer to a 50 Ω system and kinder to a wideband network. GaN's extra power density, several watts per millimeter rather than about one, is a second gift on top of this arithmetic. Heat still collects the bill. SiC is the reason a GaN PA can pay it. InP's substrate cannot.

## 7.4 Where each one is the right part

Use a GaAs pHEMT when the noise figure of a microwave receiver is the spec. Use a GaAs HBT for a phone or WLAN power amplifier at 3 to 5 V. Use GaAs when you want a dense MMIC whose passives sit on semi-insulating GaAs, and for many sockets above about 40 GHz where that foundry still has more gain and a better noise figure than production GaN.

Use GaN when the radio needs watts: radar, electronic warfare, a satcom uplink, a macro base station, anything past about 5 to 10 W. The rail is 28 to 50 V. The device is rugged into a bad mismatch when it is GaN on SiC. Use GaN-on-silicon for a 100 to 900 V switch that replaces a silicon MOSFET. That market contains no GaAs and no InP.

Use an InP HEMT for the lowest noise figure from about 30 GHz through W-band and into G-band, and whenever ft in the hundreds of gigahertz is the reason the gate exists. Use an InP HBT for an optical driver and for mixed-signal above about 100 Gbaud. InP is also a photonics crystal. GaN is not.

The only fair fight is millimeter-wave power. A 28 to 40 GHz base-station PA is GaN. From about 77 GHz up, at milliwatts to a few watts, InP often still has more gain and needs less combining. GaN shows up where watts and bandwidth are both required. A module may use an InP low-noise amplifier in front of a GaN power amplifier. Neither crystal is the phone PA. Below about 3 GHz at hundreds of watts, look at LDMOS or GaN, not InP. Cheap WLAN is SiGe, CMOS, or GaAs.

Key idea. A 94 GHz low-noise amplifier starts with InP. A 20 W X-band brick starts with GaN. A 650 V buck starts with GaN-on-silicon and never with InP.

## 7.5 What the circuit books still are

Cripps's load line still draws both the GaAs-era example and the GaN PA. GaN adds trapping, knee walkout, and memory, so the linearity fix is backoff or digital predistortion, not a polynomial copied from a handset HBT. InP circuit design looks like high-speed analog plus a foundry HBT or HEMT kit. A GaN PA looks like that load line, a thermal path through SiC, and a 50 V supply whose hot loop was already a subject in the regulator discussion of a later book. There is no popular textbook chapter that replaces the foundry model handbook.

## Practice

1. Repeat Worked Example 7.1 for a 40 V peak and a 0.50 A peak. Find Ropt and P.

2. A receiver at 94 GHz is 0.4 dB short of its noise-figure spec, and the input power is small. Which device family do you open first, and which do you not open?

3. Why can the same word "GaN" mean a 50 V radar transistor and a 650 V switch, while "InP" cannot follow it into the switch?

# Part II. The Bipolar Transistor

Part I stopped at the diode. This part builds the two-junction device that sits on that physics: the bipolar junction transistor, in this book's own words, checked against Tietze/Schenk 16th, Sedra/Smith, and Gray/Meyer. The constants stay the ones Part I declared: VT = 26 mV, and IS = 1 fA unless a new value is named. Book 2 uses the transistor inside circuits. This part explains why it behaves the way Book 2 assumes.

# Chapter 8. Two Junctions, One Current

Put two diodes back to back and you do not get a transistor. You get two diodes. The bipolar transistor works because its two junctions share a region so thin that carriers injected across the first junction reach the second before they have time to recombine. That one geometric fact, a base far thinner than a diffusion length, turns the diode law of Chapter 5 into a controlled current source. This chapter follows the electrons across that base and counts what arrives.

## 8.1 Three regions and four modes

An NPN transistor is an n-type emitter, a p-type base, and an n-type collector. The emitter is doped heavily, the base moderately, and the collector lightly. Each of the two junctions can be forward- or reverse-biased, which gives four modes. Forward-active, the amplifying mode, has the emitter-base junction forward-biased and the collector-base junction reverse-biased. Saturation has both forward-biased. Cutoff has both reverse-biased. Reverse-active swaps the roles of emitter and collector, works poorly because the device was not built symmetrically, and is mostly of interest when a circuit puts a transistor there by accident.

## 8.2 What crosses the base

In forward-active mode the emitter-base junction behaves like the diode of Chapter 5: it injects electrons from the emitter into the base, and the number injected rises exponentially with VBE. In a diode those minority electrons would diffuse into the p-side and recombine within a diffusion length, typically tens of micrometers. In a transistor the base width WB is a fraction of a micrometer. The electrons diffuse straight across it, reach the edge of the collector-base depletion layer, and the field there sweeps them into the collector. The collector junction's reverse bias does not stop them; it collects them, the same way Chapter 4's built-in field moves minority carriers across a junction.

Because the base is thin, the electron concentration falls almost linearly from its injected value at the emitter edge to nearly zero at the collector edge. Chapter 3's diffusion law then gives the collector current directly. The result has the diode's form, with a saturation current set by the base:

IC = IS e^(VBE / VT),    IS = q A Dn ni² / (NA,B WB)

Here A is the emitter area, Dn the electron diffusion constant in the base, and NA,B the base doping. The product NA,B WB is the base's total doping per unit area, called its Gummel number. A thinner or lighter base gives more collector current for the same VBE.

## Worked Example 8.1

Declare an emitter area A = 100 µm² (1.0×10⁻⁶ cm²), a base doping NA,B = 1.0×10¹⁷ cm⁻³, a base width WB = 0.10 µm (1.0×10⁻⁵ cm), and an electron diffusion constant in the base Dn = 10 cm²/s, lower than Appendix A's hand value because heavy doping lowers mobility. Use ni = 1.0×10¹⁰ cm⁻³. Find IS, and the base transit time WB²/(2Dn).

IS = q A Dn ni²/(NA,B WB) = (1.602×10⁻¹⁹)(1.0×10⁻⁶)(10)(1.0×10²⁰)/((1.0×10¹⁷)(1.0×10⁻⁵)) = 1.6×10⁻¹⁶ A, or 0.16 fA. That is the same order as the 1 fA this book uses for its teaching junction; real small-signal transistors land between about 0.1 fA and 10 fA. The transit time is (1.0×10⁻⁵)²/(2 × 10) = 5.0×10⁻¹² s, 5 ps. An electron spends five picoseconds crossing this base. That number returns in Chapter 9 as the floor under the transistor's speed.

## 8.3 Where the base current comes from

The base contact must supply two currents. The first is holes injected backward from the base into the emitter, because the forward-biased emitter-base junction is a diode in both directions. The second is holes consumed by the few electrons that do recombine on their way across the base. A good transistor makes both small.

The backward injection is controlled by the same physics as the forward one, with the roles exchanged: holes diffuse into the emitter, whose doping is NE and whose width is WE. If backward injection dominates, the ratio of the two currents, the current gain β = IC/IB, is approximately

β ≈ (Dn NE WE) / (Dp NA,B WB)

This is why the emitter is doped far more heavily than the base. Every factor of ten in the doping ratio is a factor of ten in gain.

## Worked Example 8.2

Keep the base of Worked Example 8.1. Declare an emitter doping NE = 1.0×10¹⁹ cm⁻³, an emitter width WE = 0.20 µm, and a hole diffusion constant in the emitter Dp = 2 cm²/s. Estimate β from the injection ratio alone.

β ≈ (10 × 1.0×10¹⁹ × 0.20×10⁻⁴)/(2 × 1.0×10¹⁷ × 0.10×10⁻⁴) = (2.0×10¹⁵)/(2.0×10¹²) = 1000. Real silicon transistors with similar dimensions show β of roughly 100 to 300. The simple ratio overestimates because heavy doping narrows the emitter's bandgap, which raises its effective ni² and therefore the backward hole injection, and because recombination in the base and at the surface adds base current. The ratio is the right guide to design direction. The data sheet is the guide to the number.

## 8.4 Kirchhoff and the declared transistor

Whatever happens inside, charge is conserved at the terminals: IE = IC + IB. The fraction of emitter current that reaches the collector is α = IC/IE = β/(β + 1).

Declared picture for this part: β = 100, IB = 10 µA. Then IC = 1.00 mA, IE = 1.01 mA, and α = 0.990. The emitter-base voltage that produces that collector current, from the same law as Chapter 5, is VBE = VT ln(IC/IS + 1) = 0.718 V with IS = 1 fA. That is the usual hand value of 0.7 V with this book's own constants underneath it.

The order of cause and effect matters. VBE sets IC through the exponential; IB follows as IC/β. Book 2 can treat the transistor as a current-controlled amplifier for convenience, but the physics is voltage control. That distinction explains why β varies widely from part to part while the relation between VBE and IC holds closely: β depends on recombination and backward injection, which are hard to control, while IC depends on the base's Gummel number, which a process controls well.

Key idea. The collector current is set by VBE through the base's Gummel number. The base current is a loss that β measures. A thin, lightly doped base and a heavily doped emitter make that loss small.

## 8.5 Saturation and stored charge

If the collector voltage falls far enough that the collector-base junction becomes forward-biased, the collector begins injecting electrons back into the base. The transistor is saturated. The collector-emitter voltage collapses to roughly 0.1 V to 0.2 V, IC no longer follows βIB, and extra base current simply piles up as stored minority charge in the base and collector. A saturated transistor is a good switch in the on state. It is a slow one to turn off, because that stored charge must be removed before the collector junction can block, exactly the reverse-recovery story of Chapter 6. Logic and switching circuits avoid deep saturation for this reason, for example with a Schottky diode from base to collector that diverts the excess base drive.

## 8.6 PNP, SiGe, and the compound HBT

A PNP transistor is the same device with every doping and every sign reversed: holes cross the base, and currents and voltages flip direction. Because holes diffuse more slowly than electrons, a PNP of the same geometry has a lower β and a longer transit time, which is why fast circuits favor NPN devices. A silicon-germanium heterojunction bipolar transistor grades germanium into the base, which lowers the base bandgap, raises IS and β, and builds in a field that speeds electrons across. It is still a bipolar transistor with the same equations. The GaAs and InP heterojunction transistors of Chapter 7 belong to the same family, for radio rather than for logic.

## Practice

1. For the transistor of Worked Example 8.1, find IS if the base width is reduced to 0.05 µm with the same doping. What happens to the transit time?

2. A transistor has β = 150. Find α, and find IE when IC = 2.0 mA.

3. With IS = 1 fA, find VBE for IC = 0.1 mA and for IC = 10 mA. How many millivolts separate them, and why?

4. Using the ratio of Section 8.3, by what factor does β change if the emitter doping is raised from 1.0×10¹⁹ to 3.0×10¹⁹ cm⁻³, all else equal? Why would a real device gain less than that?

5. In one or two sentences, explain why a saturated transistor turns off more slowly than one held in forward-active mode.

# Chapter 9. Bias, Early Effect, and the Hybrid-π

Chapter 8 gave the transistor's large-signal law: collector current exponential in VBE, base current smaller by β. A circuit designer needs two more things from the device. The first is a way to hold the operating point steady when β and VBE refuse to be constant. The second is a linear model of how the device responds to a small signal riding on that operating point. This chapter derives both from the physics, then adds the capacitances that limit how fast the device can respond.

## 9.1 Why neither β nor VBE can set the bias

β varies by a factor of two or three among transistors of the same part number, and it changes with temperature and current. A circuit that sets IB and relies on β to produce IC inherits all of that spread. VBE is no better a handle. At constant collector current, VBE falls by roughly 2 mV for every degree Celsius, because IS rises steeply with temperature. Turn that around and a fixed VBE produces a collector current that climbs as the transistor warms.

## Worked Example 9.1

A transistor is biased by a fixed VBE that gives IC = 1.0 mA at 25 °C. Using −2 mV/°C and the 60 mV per decade of Chapter 5, estimate IC at 35 °C if VBE is held fixed.

Over 10 °C, the VBE needed for 1.0 mA falls by about 20 mV. Holding VBE fixed therefore leaves the junction 20 mV more forward-biased than it needs to be, which multiplies the current by 10^(20/59.9) ≈ 2.16. The current more than doubles for a ten-degree rise. The extra current heats the junction further, which can run away in a power stage. A bias scheme must set the current by some means other than the junction voltage.

## 9.2 Emitter degeneration

The standard cure puts a resistor RE in the emitter and sets the base at a fixed voltage VB, typically from a stiff divider. Kirchhoff around the base-emitter loop gives

IE = (VB − VBE) / RE

If VB is several times VBE, a change of tens of millivolts in VBE changes the numerator only slightly, and β enters only through α = β/(β + 1), which is close to one for any reasonable β. The resistor converts an exponential device into one whose current is set by Ohm's law. This is the device-side view of the bias networks Book 2 uses.

## Worked Example 9.2

VB = 2.0 V, RE = 1.3 kΩ, and VBE = 0.718 V at the operating point. Find IE, and find IC for β = 100 and for β = 300. Then estimate the change in IE if VBE falls by 100 mV over a 50 °C rise. Ignore the divider's loading by the base current.

IE = (2.0 − 0.718)/1300 = 0.986 mA. For β = 100, IC = α IE = (100/101)(0.986) = 0.976 mA. For β = 300, IC = (300/301)(0.986) = 0.983 mA. Tripling β moved IC by under 1%. A 100 mV drop in VBE raises IE by 0.100/1300 = 77 µA, about 7.8%. Degeneration has made the bias nearly independent of β and has reduced the thermal sensitivity from a doubling per ten degrees to under 8% over fifty. A larger VB and RE would reduce the thermal term further, at the cost of supply headroom.

## 9.3 The Early effect

The collector-base depletion layer widens as VCE rises, and it widens partly into the base. The effective base width WB shrinks, the Gummel number falls, and by Chapter 8's expression IS rises. The collector current therefore increases slightly with VCE even at fixed VBE. Extrapolate the collector characteristics backward and they meet the voltage axis near a single point, −VA, where VA is the Early voltage. The large-signal law gains a factor:

IC = IS e^(VBE / VT) (1 + VCE / VA),    ro = VA / IC

The slope of that characteristic is a small-signal output resistance ro. Declared: IC = 1 mA and VA = 100 V give ro = 100 kΩ. A lightly doped base widens the depletion region's encroachment and lowers VA; a heavily doped base resists it. The same trade appears again in the MOSFET as channel-length modulation.

## 9.4 The hybrid-π model

For a small signal vbe on top of the bias, the exponential can be replaced by its tangent. Its slope is the transconductance:

gm = dIC/dVBE = IC / VT

The base current is IC/β, so the small-signal input resistance looking into the base is rπ = β/gm. The output is the current gm vbe in parallel with ro. Those three elements, rπ from base to emitter, a controlled source gm vbe from collector to emitter, and ro across it, are the hybrid-π model at low frequency.

At the declared IC = 1 mA: gm = 1 mA/26 mV = 38.5 mS. rπ = 100/0.0385 ≈ 2.60 kΩ. ro = 100 kΩ. The product gm ro = VA/VT ≈ 3850, or about 72 dB, is the largest voltage gain a single transistor can provide with an ideal current-source load, and it depends only on the Early voltage and the temperature, not on the bias current.

## Worked Example 9.3

The declared transistor drives a collector resistor RC = 5.0 kΩ to the supply, with the emitter at small-signal ground. Find the small-signal voltage gain. Then find the input amplitude at which the second harmonic reaches 1% of the fundamental, using the exponential's expansion, for which the ratio is vbe/(4VT).

The load is RC in parallel with ro: 5.0k ‖ 100k ≈ 4.76 kΩ. The gain is −gm (RC ‖ ro) = −0.0385 × 4760 ≈ −183. For 1% second harmonic, vbe/(4VT) = 0.01, so vbe = 0.04 × 26 mV ≈ 1.04 mV peak. A transistor is "small-signal" only for input swings of about a millivolt. That is the deeper reason Book 2's amplifiers wrap transistors in feedback: the loop of Book 2's Chapter 11 divides this distortion by its loop gain.

Key idea. gm = IC/VT. Raise the current and the transconductance rises in proportion. The thermal voltage is still 26 mV, and an input swing comparable to it is no longer small.

## 9.5 Capacitances and fT

Two capacitances limit the transistor's speed. Charge stored in the base in transit, the 5 ps of Worked Example 8.1, behaves like a capacitance gm τF from base to emitter, where τF is the forward transit time, base transit plus smaller delays in the junctions. Added to it is the emitter-base depletion capacitance Cje, the Chapter 6 junction capacitance. Together they form Cπ. The collector-base depletion capacitance Cμ links input and output. The transition frequency fT, where the short-circuit current gain falls to one, is

fT = gm / (2π (Cπ + Cμ)),    Cπ = gm τF + Cje

## Worked Example 9.4

Declare τF = 10 ps, Cje = 50 fF, and Cμ = 20 fF. Find fT at IC = 0.1 mA, 1 mA, and 5 mA.

At 1 mA, gm = 38.5 mS, Cπ = 0.0385 × 10 ps + 50 fF ≈ 435 fF, and fT = 0.0385/(2π × 455 fF) ≈ 13.5 GHz. At 0.1 mA, gm = 3.85 mS, Cπ ≈ 88.5 fF, and fT ≈ 5.6 GHz, because the fixed junction capacitances now dominate. At 5 mA, fT ≈ 15.4 GHz, approaching the ceiling 1/(2πτF) ≈ 15.9 GHz set by transit time alone. A data sheet's fT curve rises with current, flattens near that ceiling, and in a real device falls again at high current when the injected electrons overwhelm the base doping, an effect this simple model leaves out.

## Practice

1. Find gm and rπ at IC = 0.25 mA with β = 120.

2. A transistor with VA = 60 V is biased at IC = 0.5 mA. Find ro and the intrinsic gain gm ro.

3. In the circuit of Worked Example 9.2, VB is raised to 4.0 V and RE chosen for the same IE. Find RE and the change in IE for the same 100 mV fall in VBE.

4. For the transistor of Worked Example 9.4, find fT at IC = 2 mA.

5. Explain in one or two sentences why the intrinsic gain gm ro does not depend on the bias current.

# Part III. The MOSFET, From Square Law to Below 20 nm

The bipolar transistor controls current by injecting carriers across a junction. The MOS transistor controls it by inducing them with an electric field through an insulator, so its gate draws no steady current at all. That one difference is why every digital chip is built from MOSFETs. This part starts at the capacitor that makes the field, derives the long-channel square law, shows where it fails, and ends at the fin and the nanosheet. A note on symbols: this book reserves VT for the thermal voltage, 26 mV, so the MOSFET's threshold voltage is written Vth throughout.

# Chapter 10. The MOSFET From the Capacitor Up

A MOSFET is a capacitor with a conducting channel for one plate. Put a positive voltage on a metal gate above a thin oxide over p-type silicon, and the field first pushes holes away from the surface, then, past a threshold, pulls enough electrons to the surface to form a thin n-type layer. Connect that layer at each end to an n-type contact, a source and a drain, and the gate voltage now controls a resistor. Everything else about the device is the details of how much charge that layer holds and how fast it moves.

## 10.1 The gate capacitance

The oxide is a parallel-plate capacitor. Its capacitance per unit area is the oxide permittivity divided by its thickness:

Cox = εox / tox,    εox = 3.9 ε0 = 3.45×10⁻¹³ F/cm

## Worked Example 10.1

Find Cox for an oxide thickness of 2.0 nm, in F/cm² and in fF/µm².

Cox = 3.45×10⁻¹³ F/cm / 2.0×10⁻⁷ cm ≈ 1.73×10⁻⁶ F/cm². Since 1 µm² is 10⁻⁸ cm², that is 1.73×10⁻¹⁴ F per µm², or about 17.3 fF/µm². A transistor gate one micrometer square on such an oxide is a 17 fF capacitor. Modern processes replace silicon dioxide with high-permittivity hafnium-based dielectrics and quote an equivalent oxide thickness, the SiO₂ thickness that would give the same Cox.

## 10.2 Threshold and inversion

Below threshold, the gate voltage mostly widens a depletion layer of ionized acceptors under the oxide, just as reverse bias widens a junction in Chapter 4. At threshold Vth, the surface electron concentration reaches the bulk hole concentration, the surface is said to be strongly inverted, and further gate voltage goes almost entirely into adding electrons to the inversion layer. Above threshold, the channel charge per unit area is approximately Cox (VGS − Vth). The quantity VGS − Vth is the overdrive voltage, Vov. Vth itself is set by the doping under the gate, the oxide thickness, and the work-function difference between gate and silicon, and the process designer tunes all three.

## 10.3 The long-channel square law

With a small drain voltage, the channel is a uniform sheet of charge Cox Vov moving with mobility μn, and the device is a resistor whose conductance is μn Cox (W/L) Vov. As VDS rises, the channel charge near the drain falls, because the gate-to-channel voltage there is smaller. When VDS reaches Vov, the charge at the drain end goes to zero, the channel is pinched off, and the current stops increasing. Integrating along the channel gives the two regions:

Triode (VDS < Vov):  ID = μn Cox (W/L) [Vov VDS − VDS²/2]

Saturation (VDS ≥ Vov):  ID = ½ μn Cox (W/L) Vov²,    gm = 2 ID / Vov

The MOSFET's "saturation" is the bipolar transistor's forward-active region, the amplifying region. The bipolar transistor's "saturation" corresponds to the MOSFET's triode region. The two vocabularies grew up separately, and the collision is permanent.

## Worked Example 10.2

A long-channel NMOS has μn Cox = 200 µA/V² and W/L = 10. It is biased in saturation at Vov = 0.2 V. Find ID and gm. Compare gm with that of a bipolar transistor at the same current.

ID = ½ × 200 µA/V² × 10 × (0.2)² = 40 µA. gm = 2 × 40 µA/0.2 V = 0.40 mS. A bipolar transistor at 40 µA has gm = 40 µA/26 mV ≈ 1.54 mS, almost four times more. The comparison is clearest as transconductance efficiency, gm/ID: 2/Vov = 10 V⁻¹ for this MOSFET, 1/VT ≈ 38.5 V⁻¹ for any bipolar transistor. The MOSFET can approach the bipolar figure only by lowering Vov toward the subthreshold region of Section 10.5.

## 10.4 Channel-length modulation

Past pinch-off, the pinched region near the drain grows with VDS and the effective channel shortens, so ID rises slightly, the MOSFET's version of the Early effect. A parameter λ, roughly inversely proportional to L, models it: ID is multiplied by (1 + λ VDS), and the output resistance is ro ≈ 1/(λ ID). With λ = 0.1 V⁻¹ at 40 µA, ro = 250 kΩ. Short channels have large λ, which is one reason analog designers draw transistors longer than the minimum the process allows.

## 10.5 Below threshold

Below Vth the current does not stop. A small population of electrons still reaches the surface, they move by diffusion, and the current is exponential in VGS, like a bipolar transistor with a weaker grip:

ID ∝ e^(VGS / (n VT)),    subthreshold swing S = n VT ln(10) ≈ n × 60 mV/decade

The factor n, typically 1.2 to 1.5 for a planar device, reflects the fact that the gate controls the surface potential only through a capacitive divider between the oxide and the depletion layer. At 300 K no MOSFET of this kind can switch faster than 60 mV per decade of current, because the thermal spread of carrier energies sets that floor. This has a hard consequence for digital design: lowering Vth by 60 to 90 mV to gain speed multiplies the off-state leakage of every transistor on the chip by ten.

## 10.6 Velocity saturation

The square law assumes carrier velocity is proportional to the lateral field. In silicon, electron velocity saturates near vsat ≈ 1×10⁷ cm/s once the field reaches a few volts per micrometer. In a channel 100 nm long with a few tenths of a volt across it, the field is already there. In the fully velocity-saturated limit, the channel charge moves at vsat regardless of length, and the current becomes linear in overdrive:

ID ≈ W Cox vsat Vov,    gm ≈ W Cox vsat

## Worked Example 10.3

A short-channel NMOS has the Cox of Worked Example 10.1 and a width of 1.0 µm, and is fully velocity-saturated. Find ID at Vov = 0.3 V, using vsat = 1.0×10⁷ cm/s.

ID ≈ (1.0×10⁻⁴ cm)(1.73×10⁻⁶ F/cm²)(1.0×10⁷ cm/s)(0.3 V) ≈ 5.2×10⁻⁴ A, about 0.52 mA per micrometer of width. Real short-channel devices deliver somewhat less than this limit at the same overdrive, because the carriers are not velocity-saturated over the whole channel. Drive current per micrometer of width, not μn Cox W/L, is how short-channel processes are compared.

Key idea. A MOSFET is a field-controlled sheet of charge. Its long-channel current is quadratic in overdrive, its short-channel current approaches linear, and below threshold it is exponential with a swing no better than 60 mV per decade at 300 K.

## Practice

1. Find Cox for an equivalent oxide thickness of 1.0 nm, in fF/µm².

2. The transistor of Worked Example 10.2 is rebiased at Vov = 0.4 V. Find ID, gm, and gm/ID.

3. A MOSFET is at VDS = 0.1 V with Vov = 0.5 V. Is it in triode or saturation, and what does that mean in bipolar vocabulary?

4. A process has a subthreshold swing of 80 mV/decade. By what factor does off-state leakage rise if Vth is lowered by 120 mV?

5. Explain in one or two sentences why velocity saturation makes ID depend only weakly on channel length.

# Chapter 11. The Fin, Then the Sheet

Chapter 10 ended with two walls. Below threshold, a MOSFET cannot switch more sharply than about 60 mV per decade, so threshold cannot keep falling. And as the channel shortens, the drain's field reaches under the gate and starts to control the channel itself, which makes the swing worse, not better. By about the 22 nm generation, a planar transistor, gated from one side only, had run out of ways to keep the gate in charge. The answer was geometric: wrap the gate around the channel.

## 11.1 Why one gate is not enough

In a short planar device, the drain's electric field lowers the barrier at the source end of the channel. That effect is drain-induced barrier lowering, DIBL, quoted in millivolts of threshold shift per volt of drain voltage. It makes the off-state current depend on drain voltage and degrades the subthreshold swing well above its 60 mV floor. The cure is a channel so thin that the gate's field reaches all of it, from more than one side, before the drain's field can. Thin silicon, surrounded by gate, is the whole idea behind the fin and the nanosheet.

## 11.2 The fin

A FinFET stands the channel on its edge: a narrow vertical ridge of silicon, the fin, with the gate draped over its top and both sides. The fin's width, under 10 nm in advanced nodes, is what the gate must control, and three-sided gating controls it well. The penalty is that width is no longer a continuous design variable. Each fin carries a fixed effective width, about twice its height plus its top width, and a transistor's drive is set by how many fins it has.

W_eff = N_fin (2 H_fin + W_fin)

## Worked Example 11.1

A FinFET process has fins 50 nm tall and 7 nm wide. Find the effective width of one fin and of a four-fin transistor. Using roughly 0.5 mA/µm from Worked Example 10.3 as an order-of-magnitude drive, estimate the four-fin transistor's on-current.

One fin: 2 × 50 + 7 = 107 nm. Four fins: 428 nm. At about 0.5 mA/µm, the on-current is roughly 0.21 mA. The designer cannot ask for 300 nm or 500 nm; the choices are 107, 214, 321, 428 nm, and so on. Weste and Harris still teach how to draw the circuit, but sizing now counts fins.

## 11.3 The nanosheet and gate-all-around

A fin cannot grow taller indefinitely, because tall, thin fins are mechanically fragile and hard to etch uniformly. The next step turns the fin on its side and slices it: a stack of thin horizontal silicon sheets, each completely surrounded by gate. Gate-all-around control is the best electrostatics a channel can have, and the sheet width, unlike fin width, can be chosen within a range, which returns some continuous sizing to the designer.

W_eff = N_sheet × 2 (W_sheet + T_sheet)

Three sheets 30 nm wide and 5 nm thick give 3 × 2 × 35 = 210 nm of effective width in a footprint not much larger than a single fin's. Beyond the nanosheet, the complementary FET (CFET) stacks an nFET directly over a pFET to shrink a logic cell further. That is published research and roadmap material, IRDS and conference papers, and it should be read as such.

## 11.4 What changes for the designer

At these nodes the transistor is only part of the problem. The resistance and capacitance of the tiny copper wires above the transistors, the back end of line, often dominate delay. Random variation in a few dopant atoms or a few nanometers of line edge shifts threshold from one device to the next, so matching, the subject of analog design, gets harder even as logic density improves. Leakage is a power budget, not a footnote. Taur and Ning, 3rd edition, is the device book for this territory. Veendrick surveys what else changes: leakage, back-end RC, variability, and yield.

Above all, below about 20 nm the foundry's process design kit outranks any textbook. The PDK's device models, layout rules, and corner files describe the transistors that will actually be built; the equations of Chapter 10 are a way of understanding those models, not a substitute for them.

One material belongs to this chapter by name only. Aluminum scandium nitride, AlScN, is a piezoelectric film that CMOS-compatible lines now deposit, and it is reshaping MEMS resonators and filters. This book names it as a film. The 2026 products that use it are surveyed in Book 9.

Key idea. The fin and the nanosheet exist to keep the gate in control of a channel too short for a planar gate to hold. Width became quantized with the fin and partly continuous again with the sheet. Below 20 nm, the PDK is the authority.

## Practice

1. A fin is 42 nm tall and 6 nm wide. Find the effective width of a three-fin transistor.

2. A nanosheet transistor has four sheets, each 25 nm wide and 6 nm thick. Find its effective width.

3. A device shows a threshold shift of 50 mV when VDS rises from 0.05 V to 0.75 V. Find its DIBL in mV/V.

4. In one or two sentences, explain why a thinner channel improves the gate's control over the barrier at the source.

# Appendix A. Constants Used in This File

Thermal voltage VT: 26 mV. Declared room-temperature hand value.

Intrinsic concentration ni of silicon: 1.0×10^10 cm⁻³. Declared hand value.

Elementary charge q: 1.602×10⁻¹⁹ C.

Relative permittivity of silicon: 11.7.

Working mobilities, when a problem states none: μn = 1000 cm²/V·s, μp = 480 cm²/V·s.

Einstein, from those values: Dn = 26.0 cm²/s, Dp = 12.48 cm²/s.

Voltage per decade of ideal forward current: 59.9 mV.

Worked-diode saturation current: IS = 1.0×10⁻¹⁵ A.

Silicon bandgap cited in Chapter 1 is about 1.12 eV. Germanium about 0.66 eV. Gallium arsenide about 1.42 eV. Those three are orientation numbers, not inputs to the worked arithmetic.

Declared bipolar transistor (Part II): β = 100, IS = 1 fA, VA = 100 V, operating point IC = 1 mA, which gives VBE = 0.718 V, gm = 38.5 mS, rπ = 2.60 kΩ, ro = 100 kΩ.

VBE temperature coefficient at constant IC, hand value: −2 mV/°C.

Oxide permittivity εox = 3.9 ε0 = 3.45×10⁻¹³ F/cm (Part III).

Electron saturation velocity in silicon, hand value: vsat = 1.0×10⁷ cm/s.

Symbol note: VT is the thermal voltage; the MOSFET threshold is Vth.

# Appendix B. Answers to the Practice Problems

## Chapter 1

1. p = ni²/n = 1.0e+05 cm⁻³. σ = q n μn = 0.1602 (Ω·cm)⁻¹. ρ = 6.242 Ω·cm. Ten times fewer electrons, ten times the resistivity.

2. A wider gap means fewer bonds broken at a given temperature, so ni is smaller. Reverse current of a junction tracks that minority population, so the leakage falls.

3. σ = q ni (μn + μp) = 2.371e-06 (Ω·cm)⁻¹. The hole term is μp/(μn+μp) = 0.32 of the total, about a third. It is not ignorable. It was ignorable in the worked example only because doping had made n huge and p tiny.

## Chapter 2

1. n ≈ 1.0×10^15 cm⁻³. p = ni²/n = 1.0e+05 cm⁻³.

2. The hole concentration is ni²/ND, not zero. At ND = 10^16 cm⁻³ it is 1.0e+04 cm⁻³.

3. Donors exceed acceptors by 9.0×10^15 cm⁻³, so the material is n-type. The majority electron concentration is approximately ND − NA = 9.0×10^15 cm⁻³. The minority hole concentration is ni² divided by that majority, about 1.1×10^4 cm⁻³. The subtraction is charge neutrality with the small carrier terms dropped. Dropping them is fair here because both are far below the doping difference.

## Chapter 3

1. Dn = 26.0 cm²/s. Dp = 12.48 cm²/s.

2. |Jp| = q Dp |dp/dx| = 199.952 A/cm². Conventional current points the same way the holes are spreading.

3. Jn,drift = q n μn E = 16.022 A/cm². The diffusion example was 374.909 A/cm². A modest field of 10 V/cm in this doping already beats that particular one-micron gradient. Steeper gradients, the kind inside a junction, put diffusion back in front.

## Chapter 4

1. NA ND / ni² = 1.0×10^34 / 1.0×10^20 = 1.0×10^14. Vbi = 0.026 × ln(10^14) = 0.026 × 32.236 = 0.838 V.

2. Only somewhat larger. Width follows the square root of (Vbi − V). A few volts of reverse bias increases the term inside the root by a factor of several, and the width by the square root of that factor, not by the factor itself.

3. Vbi depends on ln(1/ni²). A larger ni shrinks the argument of the logarithm, so Vbi falls.

## Chapter 5

1. I(0.65 V) = 7.200e-05 A = 72.00 μA.

2. One decade is 59.9 mV, so the rule says 59.9 mV. Exact voltages: 0.6585 V at 0.1 mA and 0.7184 V at 1 mA, a difference of 59.9 mV. The rule is the exact ideal-diode result once I is far above IS, which it is.

3. I(−0.2 V) = -9.995e-16 A, which is −IS times (1 − e^(−0.2/VT)). e^(−0.2/VT) is about 4.56e-04, so the factor in parentheses is already indistinguishable from 1 at three significant figures, and I ≈ −IS. A real diode adds surface leakage and generation inside the depletion layer, both missing from the ideal law.

## Chapter 6

1. 1 − V/Vbi = 3.784. Cj = 20 / 1.945 = 10.28 pF.

2. The time is waiting on minority charge stored by the forward current. A Schottky junction does not store that minority population in the same way, so the recovery tail is largely absent.

3. The resistor sets the current through the knee. The junction sets the voltage. Without the resistor the supply would try to hold 12 V across a 5.1 V knee.

## Chapter 7

1. Ropt = 40 / 0.50 = 80 Ω. P = (40)(0.50)/2 = 10 W.

2. Open an InP HEMT first. Do not open a GaN power switch, and do not open a handset HBT. A survivable limiter in front of that LNA can be GaN. The noise-figure deficit of 0.4 dB is an InP problem.

3. The 650 V part is a GaN-on-silicon power HEMT. InP is a small, fragile, poorly heat-spreading substrate with a 1 to 5 V RF habit. It is not in that market.

## Chapter 8

1. IS doubles to 3.2×10⁻¹⁶ A, because IS is inversely proportional to WB. The transit time WB²/(2Dn) falls by a factor of four, to 1.25 ps.

2. α = 150/151 = 0.993. IE = IC/α = 2.0 × 151/150 = 2.013 mA.

3. VBE = 0.026 ln(10⁻⁴/10⁻¹⁵) = 0.659 V and 0.026 ln(10⁻²/10⁻¹⁵) = 0.778 V, a difference of 120 mV. Two decades of current at about 60 mV per decade.

4. β is proportional to NE, so the ratio predicts three times the gain. A real device gains less because heavier doping narrows the emitter bandgap and lowers the hole diffusion constant, both of which increase backward injection.

5. In saturation the forward-biased collector junction stores excess minority charge in the base and collector, and that charge must be removed or recombine before the collector junction can block; in forward-active mode that excess charge was never stored.

## Chapter 9

1. gm = 0.25 mA/26 mV ≈ 9.62 mS; rπ = 120/0.00962 ≈ 12.5 kΩ.

2. ro = 60 V/0.5 mA = 120 kΩ; gm ro = VA/VT = 60/0.026 ≈ 2310, about 67 dB.

3. RE = (4.0 − 0.718)/0.986 mA ≈ 3.33 kΩ. ΔIE = 0.100/3330 ≈ 30 µA, about 3.0% instead of 7.8%.

4. gm = 76.9 mS; Cπ = 0.0769 × 10 ps + 50 fF ≈ 819 fF; fT = 0.0769/(2π × 839 fF) ≈ 14.6 GHz.

5. gm is proportional to IC and ro is inversely proportional to IC, so their product VA/VT contains no current at all.

## Chapter 10

1. Cox = 3.45×10⁻¹³/1.0×10⁻⁷ ≈ 3.45×10⁻⁶ F/cm² ≈ 34.5 fF/µm².

2. ID = ½ × 200 µA/V² × 10 × 0.16 = 160 µA; gm = 2 × 160 µA/0.4 V = 0.80 mS; gm/ID = 5 V⁻¹. Doubling Vov quadrupled the current but only doubled gm.

3. VDS < Vov, so it is in triode, where it behaves as a gate-controlled resistor; in bipolar vocabulary this corresponds to saturation.

4. 120/80 = 1.5 decades, a factor of 10^1.5 ≈ 32.

5. Once carriers move at a fixed saturated velocity, the current is the channel charge per unit width times that velocity, which does not involve L; shortening the channel only raises the field in a region where velocity can no longer increase.

## Chapter 11

1. 3 × (2 × 42 + 6) = 3 × 90 = 270 nm.

2. 4 × 2 × (25 + 6) = 248 nm.

3. 50 mV/0.70 V ≈ 71 mV/V.

4. In a thin channel, every point is close to a gate electrode, so the gate's field sets the potential throughout the channel; the drain's field, which must reach in from the far end, has little channel left to influence.

# Appendix C. What This File Does Not Yet Cover

Part II, the bipolar transistor, and Part III, the MOSFET through the fin and the nanosheet, are now in this file. Still to come: Part IV is the inside of the amplifiers Book 2 treated as ideal blocks, checked against the 16th edition of Tietze, Schenk, and Gamm, whose op-amp chapter was the one rewritten for that edition. Part V is CMOS as a chip: sizing, delay, and the path from a gate to a placed design. Part VI is the handoff to the rest of the series.

Those later books already have their source shelves in the series plan. Matching and antennas belong to Book 4. Modulation and SDR belong to Book 5. Noise, PLLs, and high-power RF belong to Book 6. Emissions belong to Book 7. Regulators and magnetics belong to Book 8. Packaging and 2026 systems belong to Book 9. Bode plots as a general language belong to Book 2. Chapter 7 is the exception that belongs in this file: GaAs, GaN, and InP as device choices, with a load-line number attached. The power-amplifier design and the 28 V layout are Book 6. The 650 V switch is Book 8. The diode chapters do not depend on Chapter 7.

# Appendix D. Sources for Chapter 7

The prose above is this book's. The shelf it was checked against is short, and it is not a substitute for a foundry kit.

Cripps, RF Power Amplifiers for Wireless Communications, and Advanced Techniques in RF Power Amplifier Design. The load line is technology-agnostic. GaN adds trapping and memory.

Grebennikov, RF and Microwave Transmitter Design, and the switch-mode RF power-amplifier book with Sokal and Franco.

Quay, Gallium Nitride Electronics. Mishra and Rajan, and Morkoç, for the nitride heterostructure. Sze for the vocabulary.

Vendelin, Pavio, and Rohde for GaAs-era linear and nonlinear microwave circuits. Gonzalez and Maas remain the amplifier and mixer references.

InP devices: Rodwell's HBT and HEMT papers, and the millimeter-wave MMIC papers from JPL and Fraunhofer IAF. Foundry kits from Teledyne, WIN, Northrop Grumman, and Fraunhofer matter more than a textbook.

GaN models: Wolfspeed and Qorvo application notes. Power-conversion GaN, which Chapter 7 only points at: Erickson and Maksimović, plus the vendor power-loop note.

Tietze, Schenk, and Gamm, 16th edition (2019), remains the silicon circuit check for the bipolar and MOS chapters of Parts II and III.

# Appendix E. Sources for Parts II and III

Tietze, Schenk, and Gamm, 16th edition (2019), bipolar and amplifier chapters.

Sedra and Smith; Gray and Meyer (or Gray, Hurst, Lewis, and Meyer) for analog bipolar and IC.

Taur and Ning, Fundamentals of Modern VLSI Devices, 3rd. Veendrick, Nanometer CMOS ICs. Saha on FinFET. The PDK for anything you will actually tape out.