Communications, Wireless, and SDR

Book 5 of the Electrical Engineering Series

Lothar J. Musiol

Copyright © 2026 Lothar J. Musiol. All rights reserved. Kindle edition.

From Bits to Waveforms: Sampling, Modulation, Fading, OFDM, MIMO, and the 2026 Band Plan

# How to Use This Book

A modem turns bits into a waveform and the waveform back into bits. This book is about that waveform: how many bits it carries, how much spectrum it uses, how much noise and fading it tolerates, and how a receiver written largely in software recovers it. Chapters 1 to 4 build the link on a channel that only adds noise: Shannon's limit, sampling and complex baseband, pulse shaping, and the constellations with their error rates. Chapters 5 to 7 add the real channel and the techniques that defeat it: multipath and diversity, OFDM, coding, and multiple antennas. Chapter 8 places the result in the 2026 band plan and in a software-defined receiver.

Each chapter states its question, develops the idea in words, and works it through with declared numbers. Every number in a worked example was computed, and the answers to every practice problem are in Appendix B. Readers should be comfortable with complex numbers, the Fourier transform at the level of Book 1, and basic probability.

The series gives each subject one home. Antenna gain and the Friis path factor are Book 4's. The power amplifier, transmitter watts, the synthesizer, and the receiver noise cascade are Book 6's. Emission masks, the test chamber, and certification are Book 7's. This book uses those results where it needs them and does not recompute them. The texts in Appendix C go deeper; this book's explanations, examples, and problems are its own.

# Contents

Chapter 1. Bits Become a Waveform

Chapter 2. Sampling and Complex Baseband

Chapter 3. Pulse Shaping and the Matched Filter

Chapter 4. Constellations, Error Rates, and EVM

Chapter 5. The Real Channel: Multipath, Fading, and Diversity

Chapter 6. OFDM: Many Slow Carriers Instead of One Fast One

Chapter 7. Coding and Multiple Antennas

Chapter 8. The 2026 Band Plan and the Software-Defined Receiver

Appendix A. Constants and Formulas Used in This Book

Appendix B. Answers to the Practice Problems

Appendix C. Sources

# Chapter 1. Bits Become a Waveform

A modem has one job: turn a stream of bits into a waveform that a channel can carry, and turn the waveform that arrives back into the same bits. Everything in this book serves that job. The antenna gain is Book 4's, the transmitter's watts are Book 6's, and the test chamber is Book 7's. What belongs here is the waveform itself: how many bits it carries per second, how much spectrum it occupies, how much noise it tolerates, and how it survives a channel that echoes and fades. This chapter introduces the three quantities that tie those together, symbol rate, bandwidth, and signal-to-noise ratio, and the limit that Claude Shannon set on all of them.

## 1.1 Symbols, bits, and bandwidth

A digital transmitter sends symbols, each a choice among M possible waveforms, at a symbol rate Rs. Each symbol carries log₂M bits, so the bit rate is Rs log₂M. The bandwidth a symbol stream occupies is set by its symbol rate and by how sharply its pulses are shaped. Chapter 3 shows that with an ideal pulse the minimum bandwidth is Rs/2 at baseband, or Rs on a carrier, and that practical pulses add a roll-off factor α, typically 0.2 to 0.35.

Rb = Rs log₂M,    B ≈ Rs (1 + α)   (passband)

## Worked Example 1.1

Problem. A link sends 4 million symbols per second with 16 possible symbols each and a roll-off of 0.25. Find the bit rate, the occupied bandwidth on the carrier, and the spectral efficiency.

Solution. Each symbol carries log₂16 = 4 bits, so Rb = 4 × 4 = 16 Mbit/s. The passband bandwidth is 4 MHz × 1.25 = 5 MHz. The spectral efficiency is 16/5 = 3.2 bit/s/Hz. Doubling the bits per symbol to 8, with 256 symbols, would double the bit rate in the same bandwidth, at a cost in signal-to-noise ratio that Chapter 4 calculates: roughly 6 dB for every extra bit per symbol in each of the two quadrature dimensions.

## 1.2 Signal-to-noise ratio and Eb/N0

Noise in a receiver is, to a first approximation, white and Gaussian, with a power spectral density N0 that is set by temperature and the receiver's noise figure; the noise cascade itself is Book 6's subject. Two measures of the signal against it are used. The signal-to-noise ratio SNR = S/(N0B) compares powers in the occupied bandwidth. The energy per bit over noise density Eb/N0 compares the energy spent on each bit with N0, and lets modulations of different bandwidths be compared fairly. They are related through the spectral efficiency.

SNR = (Eb/N0) × (Rb / B)

## 1.3 Shannon's limit

Shannon proved in 1948 that a channel of bandwidth B and signal-to-noise ratio SNR can carry information without error at any rate below its capacity, and at no rate above it. The proof says that codes approaching the limit exist; it took until the 1990s for turbo and LDPC codes to come within a fraction of a decibel of it. Rewriting the capacity per unit bandwidth in terms of Eb/N0 shows that no scheme, however slow, can work below Eb/N0 = ln 2, which is −1.59 dB.

C = B log₂(1 + SNR)

## Worked Example 1.2

Problem. A 20 MHz channel has an SNR of 20 dB. Find its Shannon capacity. What SNR would be needed for a spectral efficiency of 10 bit/s/Hz?

Solution. SNR = 100, so C = 20 MHz × log₂101 = 20 × 6.66 = 133 Mbit/s, a spectral efficiency of 6.66 bit/s/Hz. For 10 bit/s/Hz, 1 + SNR = 2¹⁰, so SNR = 1023, or 30.1 dB. The last bits per hertz are expensive: from 6.66 to 10 bit/s/Hz costs 10 dB of SNR, ten times the power. Practical systems run 2 to 4 dB from the limit after coding, and they reach high efficiencies by using several antennas, which Chapter 7 shows multiplies the capacity, rather than by raising SNR alone.

Key idea. Bandwidth and signal-to-noise ratio are the two currencies of a link. Capacity grows linearly with bandwidth but only logarithmically with SNR, so each extra bit per hertz costs about 3 dB, and doubling the bandwidth is almost always cheaper than quadrupling the power.

## 1.4 What the rest of the book does

Chapter 2 moves the signal to and from complex baseband and samples it. Chapter 3 shapes the pulses so that symbols do not smear into each other. Chapter 4 arranges symbols into constellations and computes their error rates. Chapter 5 adds the real channel, with multipath and fading. Chapter 6 builds OFDM, the waveform of Wi-Fi, 4G, and 5G. Chapter 7 adds coding and multiple antennas. Chapter 8 places it all in the 2026 band plan and in the software-defined radio that can be programmed to receive it. The digital signal processing is introduced as it is needed; Lyons's Understanding Digital Signal Processing is the reference for those who want it whole.

## Practice

1. A link uses 64 possible symbols at 10 Msymbol/s with α = 0.2. Find the bit rate and occupied bandwidth.

2. Find the Shannon capacity of a 5 MHz channel at an SNR of 10 dB.

3. A system runs at 2 bit/s/Hz with Eb/N0 = 8 dB. Find its SNR in decibels.

4. What SNR is needed for 6 bit/s/Hz at the Shannon limit?

5. Find the smallest Eb/N0 at which any error-free communication is possible.

6. Explain in one or two sentences why doubling a link's bandwidth is usually a better way to double its capacity than raising its power.

# Chapter 2. Sampling and Complex Baseband

A modern radio does almost nothing in analog form. It amplifies, filters, and shifts the signal down in frequency, and then it samples it, and everything after the sampler is arithmetic. This chapter covers the three ideas that make that possible: the sampling theorem and the aliasing it warns of, the complex-baseband representation that lets a radio-frequency signal be processed at a low rate, and the two imperfections that limit what the sampled signal can show, the converter's quantization noise and the mismatch between the in-phase and quadrature paths.

## 2.1 The sampling theorem and aliasing

A signal containing no frequencies above B can be reconstructed exactly from samples taken at any rate fs greater than 2B. When the condition is broken, energy at a frequency f appears after sampling at the distance from f to the nearest multiple of fs, and nothing in the samples can tell the two apart. That folding is aliasing, and the only defense is a filter in front of the converter that removes what would fold before it is sampled. The same folding is used deliberately in bandpass sampling, where a narrow signal at a high intermediate frequency is sampled below its carrier and lands, intact, at a low frequency.

falias = | f − k fs |, with k chosen so that 0 ≤ falias ≤ fs/2

## Worked Example 2.1

Problem. A converter samples at 50 MS/s. Where does an interferer at 70 MHz appear? Where does a 140 MHz intermediate frequency appear when sampled at 61.44 MS/s?

Solution. 70 − 50 = 20 MHz, inside the first Nyquist zone of 0 to 25 MHz, so the interferer lands at 20 MHz, on top of any wanted signal there. For the intermediate frequency, 140 − 2 × 61.44 = 17.12 MHz, which is below fs/2 = 30.72 MHz, so the signal appears at 17.12 MHz. The anti-alias filter in that case must be a bandpass around 140 MHz that rejects everything in the neighboring zones, not a lowpass.

## 2.2 Complex baseband

A radio signal at carrier fc can always be written as x(t) = I(t) cos 2πfct − Q(t) sin 2πfct, where I and Q are two slowly varying real signals. Together they form the complex envelope s(t) = I(t) + jQ(t), and all the information is in it. A quadrature mixer recovers I and Q by multiplying the received signal by cosine and by minus sine at the carrier and lowpass filtering each product. The pair can then be sampled at a rate set by the signal's bandwidth, not its carrier. A 20 MHz Wi-Fi channel at 5.5 GHz needs about 20 million complex samples per second, not 11 billion real ones.

Because s is complex, its spectrum need not be symmetric, and negative frequencies mean something: they are the part of the band below the carrier. That is why a complex sampler at fs covers a band fs wide, centered on the carrier, while a real sampler at the same rate covers only fs/2. The constellation diagrams of Chapter 4 are plots of s at the symbol instants.

Key idea. Complex baseband separates the carrier from the information. Once the signal is shifted down and split into I and Q, every modulation, filter, and equalizer in this book acts on a complex number per sample, at a rate set by the bandwidth alone.

## 2.3 Converter noise and processing gain

An ideal N-bit converter driven by a full-scale sine wave has a signal-to-quantization-noise ratio of 6.02N + 1.76 dB, spread evenly from 0 to fs/2. If the wanted signal occupies only a bandwidth B, a digital filter removes the noise outside it and the ratio within B rises by the processing gain 10 log₁₀(fs/2B) for a real converter, or 10 log₁₀(fs/B) for a complex pair. Real converters fall short of the ideal because of thermal noise, nonlinearity, and clock jitter. Jitter tj on the sampling clock limits the SNR for an input at frequency f to −20 log₁₀(2πftj), independent of the number of bits.

SNRq = 6.02 N + 1.76 dB,    processing gain = 10 log₁₀(fs / B)

## Worked Example 2.2

Problem. A 12-bit complex converter samples at 122.88 MS/s, and the wanted channel is 20 MHz wide. Find the ideal quantization SNR over the full band and in the channel. What clock jitter keeps the jitter limit at 64 dB for a 100 MHz input?

Solution. Over the full band, 6.02 × 12 + 1.76 = 74.0 dB. Filtering to 20 MHz adds 10 log₁₀(122.88/20) = 7.9 dB, for 81.9 dB in the channel. For the jitter, −20 log₁₀(2π × 100 MHz × tj) = 64 dB gives tj = 1.0 ps rms. A 1 GHz input would need 100 fs for the same figure, which is why direct-RF sampling places hard demands on the clock, a subject Book 6 takes up with the synthesizer.

## 2.4 I/Q imbalance

The two arms of a quadrature mixer are never identical. If the Q arm has a gain g relative to the I arm and a phase error φ away from 90°, each frequency in the band leaks an image of itself into the mirror frequency on the other side of the carrier. The image-rejection ratio measures the leak. A 1% gain error and 1° of phase give about 40 dB, which is adequate for QPSK but not for 1024-QAM, whose decisions need errors near −36 dB or better in total. Modern receivers estimate the imbalance from the signal and correct it digitally, reaching 60 dB or more.

IRR = 10 log₁₀[(1 + 2g cos φ + g²) / (1 − 2g cos φ + g²)]

## Worked Example 2.3

Problem. Find the image-rejection ratio for g = 1.01 and φ = 1°, for g = 1.001 and φ = 0.1°, and for g = 1.05 and φ = 3°.

Solution. Substituting, the three cases give 40.0 dB, 60.0 dB, and 28.9 dB. Each tenfold reduction in both errors buys about 20 dB. The third case, typical of an uncalibrated low-cost receiver, leaves an image only 29 dB down, enough to corrupt a 64-QAM signal on the opposite side of the band.

## Practice

1. A converter samples at 100 MS/s. Where does a 130 MHz tone appear?

2. Find the ideal quantization SNR of a 14-bit converter.

3. A 16-bit converter is limited in practice to 74 dB of SNR. What is its effective number of bits?

4. A complex converter samples at 61.44 MS/s and the channel is 10 MHz. Find the processing gain.

5. Find the jitter-limited SNR for a 1 GHz input with 100 fs rms jitter.

6. Why does a complex sampler at fs cover twice the bandwidth of a real sampler at the same rate?

# Chapter 3. Pulse Shaping and the Matched Filter

A symbol is not a point; it is a pulse with a duration and a spectrum. If pulses are too narrow, they occupy too much bandwidth. If they are too wide, they overlap, and each symbol's decision is disturbed by its neighbors, which is intersymbol interference. This chapter shows how the raised-cosine family of pulses avoids both, why the filtering is split equally between transmitter and receiver, and why the receive filter that best fights noise is the one matched to the pulse.

## 3.1 Nyquist's condition for zero interference

Symbols sent every T seconds can be separated perfectly if the overall pulse, from transmitter through receiver, is one at its own sampling instant and zero at every other multiple of T. The narrowest spectrum that does this is the sinc pulse, whose spectrum is a rectangle of width 1/(2T) at baseband. The sinc is impractical: its tails decay only as 1/t, so it must be very long, and a small timing error lets the tails of many symbols add up. The raised-cosine pulse keeps the zero crossings of the sinc but rounds the edges of its spectrum, spending an extra fraction α of bandwidth, the roll-off, to make the tails decay as 1/t³.

h(t) = sinc(t/T) × cos(παt/T) / [1 − (2αt/T)²],   bandwidth = (1 + α)/(2T) at baseband

## Worked Example 3.1

Problem. A satellite link sends 30 Msymbol/s. Find the occupied radio-frequency bandwidth with α = 0.35, 0.20, and 0.05. How fast could it signal in a 36 MHz transponder at each of the first two roll-offs?

Solution. On a carrier the occupied bandwidth is Rs(1 + α): 30 × 1.35 = 40.5 MHz, 30 × 1.20 = 36.0 MHz, and 30 × 1.05 = 31.5 MHz. In a 36 MHz transponder the symbol rate can be 36/1.35 = 26.7 Msymbol/s at α = 0.35 or 36/1.20 = 30.0 Msymbol/s at α = 0.20, a 12.5% gain in throughput from the filter alone. The DVB-S2 standard offered roll-offs of 0.35, 0.25, and 0.20, and its S2X extension added 0.15, 0.10, and 0.05 for exactly this reason.

## 3.2 The price of a small roll-off

A sharp roll-off saves bandwidth but makes the pulse longer and the link more sensitive to timing error. The sensitivity can be computed directly: sample the raised-cosine pulse a tenth of a symbol late and add the magnitudes of all the neighboring pulses at that instant, which is the worst case of their interference. For α = 1.0 the sum is 0.079, for α = 0.5 it is 0.197, for α = 0.25 it is 0.331, and for α = 0.1 it is 0.510, against a main tap of about 0.98. A system that uses a small roll-off therefore needs good timing recovery and an equalizer, and the filters need more taps.

## 3.3 The matched filter

In white noise, the receive filter that maximizes the signal-to-noise ratio at the sampling instant is the time-reversed copy of the received pulse, the matched filter. Its peak output SNR depends only on the pulse energy E and the noise density, not on the pulse's shape: 2E/N0 for a real pulse, or E/N0 per complex dimension in the convention of this book. That is why error rates are written in terms of Eb/N0.

hMF(t) = p(T0 − t),   SNRmax = 2E / N0

The two requirements fit together. The raised-cosine response is split into two root-raised-cosine filters, one in the transmitter and one in the receiver. The receiver's filter is then matched to the transmitted pulse, and the product of the two is the raised cosine, which meets Nyquist's condition. Every modern single-carrier standard, from satellite to the cable modem, uses this arrangement.

Key idea. Split the pulse shaping into two equal root-raised-cosine halves. The transmit half limits the spectrum, the receive half is the matched filter that maximizes SNR, and together they make a raised cosine with no interference at the sampling instants.

## 3.4 Eye diagrams and timing

An eye diagram overlays the received waveform symbol period by symbol period. The opening of the eye at the sampling instant is the margin against noise; its width is the margin against timing error. Intersymbol interference closes the eye vertically, and jitter closes it horizontally. The receiver's timing loop steers the sampling point to the widest opening. With complex baseband, an eye is drawn for I and for Q separately, and the constellation of Chapter 4 is the same information seen at one instant.

## Worked Example 3.2

Problem. A rectangular pulse of amplitude 0.2 V and duration 1 µs is received in white noise with N0 = 10⁻¹⁰ V²/Hz (one-sided, for a 1 Ω reference). Find the matched-filter output SNR at its peak.

Solution. The pulse energy is E = A²T = 0.04 × 10⁻⁶ = 4 × 10⁻⁸ V²·s. The peak SNR is 2E/N0 = 8 × 10⁻⁸/10⁻¹⁰ = 800, or 29.0 dB. Any other filter with the same noise gives a lower peak SNR; an integrate-and-dump circuit over exactly the pulse duration is the matched filter in this case.

## Practice

1. Find the occupied bandwidth of 5 Msymbol/s with a roll-off of 0.22.

2. A 6 MHz channel uses α = 0.15. What is the highest symbol rate it can carry?

3. Why are the transmit and receive filters each made root-raised-cosine rather than putting the whole raised cosine in the transmitter?

4. A pulse has energy 10⁻⁹ V²·s and N0 = 2 × 10⁻¹² V²/Hz. Find the matched-filter peak SNR in dB.

5. Which closes the eye horizontally, noise or timing jitter?

6. From Section 3.2, by what factor does the worst-case interference grow when α is reduced from 0.5 to 0.1 at a timing error of a tenth of a symbol?

# Chapter 4. Constellations, Error Rates, and EVM

At the sampling instant, each received symbol is a single complex number. The transmitter chose it from a set of points, the constellation, and the receiver decides which point was sent by choosing the nearest one. Errors happen when noise pushes the received point across a decision boundary. This chapter computes how often that happens for the common constellations, shows why each extra bit per symbol costs about 6 dB in a square QAM, and connects the error rate to the error-vector magnitude that a test instrument reports.

## 4.1 BPSK and QPSK

Binary phase-shift keying sends +A or −A. With matched filtering in white Gaussian noise, the bit error rate is Q(√(2Eb/N0)), where Q is the tail probability of a unit normal distribution. Quadrature PSK sends an independent BPSK stream on I and on Q. It carries two bits per symbol in the same bandwidth with the same bit error rate for the same Eb/N0, which is why it is the workhorse of every link that has little SNR to spare.

BER(BPSK, QPSK) = Q(√(2 Eb/N0))

## Worked Example 4.1

Problem. Find the BPSK bit error rate at Eb/N0 = 6.8 dB and at 9.6 dB.

Solution. At 6.8 dB, Eb/N0 = 4.79 and Q(√9.57) = Q(3.09) = 9.9 × 10⁻⁴, about 10⁻³. At 9.6 dB, Eb/N0 = 9.12 and Q(4.27) = 9.7 × 10⁻⁶. Three decibels more SNR lowers the error rate by a factor of about a hundred near these values, which is why uncoded links have a sharp "waterfall" and why the error rate alone is a poor guide to how much margin a link has.

## 4.2 Square QAM

Quadrature amplitude modulation places M points on a square grid, √M levels on each axis. The symbol error rate, written in terms of the energy per symbol Es, is well approximated by the formula below. Because each axis is an independent √M-level signal and doubling the number of levels on an axis halves the spacing for the same average power, each extra bit per axis costs about 6 dB. With Gray coding, in which neighboring points differ in one bit, the bit error rate is approximately the symbol error rate divided by log₂M.

SER ≈ 4 (1 − 1/√M) Q(√(3 Es / ((M − 1) N0)))

Solving for a symbol error rate of 10⁻³ gives the Es/N0 each constellation needs, uncoded: 4-QAM 10.35 dB, 16-QAM 17.63 dB, 64-QAM 23.97 dB, 256-QAM 30.08 dB, 1024-QAM 36.14 dB, and 4096-QAM 42.17 dB. The steps are close to 6 dB, as predicted. Phase-shift keying with eight or more points uses only a circle and is less efficient: 8-PSK needs 15.7 dB for three bits, against 17.6 dB for 16-QAM's four, and by Eb/N0 the two are 10.9 and 11.6 dB. PSK survives in links whose amplifiers must run in saturation, because it has a constant envelope; 16-QAM has a peak-to-average power ratio of 1.8, or 2.6 dB, before any filtering.

## Worked Example 4.2

Problem. A 16-QAM link has Es/N0 = 18 dB. Find its approximate symbol and bit error rates.

Solution. Es/N0 = 63.1, so the argument is √(3 × 63.1/15) = 3.55 and the symbol error rate is 4 × 0.75 × Q(3.55) = 5.7 × 10⁻⁴. With Gray coding the bit error rate is about 5.7 × 10⁻⁴/4 = 1.4 × 10⁻⁴. Compared with the 17.63 dB needed for 10⁻³, 0.4 dB more SNR has cut the error rate by almost half.

Key idea. In a square QAM, every extra bit per symbol costs about 3 dB, and every extra bit per axis about 6 dB. Going from 1024-QAM to 4096-QAM gains 20% in throughput and costs 6 dB, four times the signal-to-noise power ratio.

## 4.3 Error-vector magnitude

A receiver measures the quality of a modulated signal by comparing each received symbol with the ideal point it was decided to be. The error-vector magnitude is the rms length of the difference, relative to the rms length of the ideal points. When the impairment is noise-like, EVM in decibels is the negative of the SNR in decibels. A transmitter's EVM therefore sets a ceiling on the SNR any receiver can see from it, however strong the signal. Standards specify the transmit EVM each modulation requires. For example, IEEE 802.11ax requires −35 dB for 1024-QAM, and 802.11be requires −38 dB for its 4096-QAM rates, which also use rate-5/6 or 3/4 coding.

EVM(%) = 100 × 10^(EVM(dB)/20),   EVM(dB) ≈ −SNR(dB)

## Worked Example 4.3

Problem. Express −38 dB, −35 dB, and −25 dB of EVM as percentages. Why can 4096-QAM run at −38 dB EVM when the uncoded table above asks for 42 dB?

Solution. 10^(−38/20) = 1.26%, 10^(−35/20) = 1.78%, and 10^(−25/20) = 5.62%. The 42 dB figure is for an uncoded symbol error rate of 10⁻³. Wi-Fi 7 uses 4096-QAM only with a strong LDPC code of rate 3/4 or 5/6, which lowers the required SNR by several decibels (Chapter 7). The transmit EVM is only one contribution: the receiver's noise and its own phase noise add to it, so the link needs a few decibels more than the EVM figure at the antenna.

## Practice

1. Find the BPSK bit error rate at Eb/N0 = 6.8 dB, to two significant figures.

2. From the table in Section 4.2, how much more Es/N0 does 256-QAM need than 64-QAM for the same symbol error rate?

3. A 64-QAM link runs at 26 dB of Es/N0. Is its symbol error rate above or below 10⁻³?

4. Convert 2% EVM to decibels.

5. What is the peak-to-average power ratio of the 16-QAM constellation, with points at ±1 and ±3 on each axis?

6. Why does QPSK have the same bit error rate as BPSK at the same Eb/N0?

# Chapter 5. The Real Channel: Multipath, Fading, and Diversity

Chapter 4's error rates assumed a channel that only adds noise. A real radio channel does more. The signal arrives by several paths, reflected from walls, vehicles, and the ground, each with its own delay, strength, and phase. When the paths add in phase the signal is strong; when they cancel it can fall twenty or thirty decibels in a fraction of a wavelength. When the transmitter or receiver moves, the pattern sweeps past and the signal fades in time. This chapter describes the channel by two numbers, its delay spread and its Doppler spread, computes what fading costs, and shows how diversity wins most of it back. Path loss and antenna gain are Book 4's; here the concern is what multipath does to the waveform.

## 5.1 Delay spread and coherence bandwidth

The spread of arrival times is summarized by the rms delay spread στ. Indoors it is typically 10 to 100 ns; in a city it may be a few microseconds. Its reciprocal sets the coherence bandwidth, the frequency span over which the channel's response is roughly constant. A common working estimate is Bc ≈ 1/(5στ). A signal narrower than Bc sees flat fading: the whole band rises and falls together. A signal wider than Bc sees frequency-selective fading, a response with notches in it, and needs an equalizer or the OFDM of Chapter 6.

Bc ≈ 1 / (5 στ)

## Worked Example 5.1

Problem. Find the coherence bandwidth for delay spreads of 100 ns and 50 ns. Is a 20 MHz Wi-Fi channel flat or selective in either case?

Solution. 1/(5 × 100 ns) = 2 MHz and 1/(5 × 50 ns) = 4 MHz. Both are far narrower than 20 MHz, so the channel is frequency-selective across the Wi-Fi band in both rooms. Each 312.5 kHz OFDM subcarrier of Chapter 6, though, is narrower than either coherence bandwidth, so each subcarrier sees a flat channel, which is exactly what OFDM is designed to exploit.

## 5.2 Doppler and coherence time

A receiver moving at speed v through the interference pattern sees each path shifted in frequency by up to the maximum Doppler shift fd = vfc/c. The channel then changes on a time scale set by the coherence time, which for the classical model of scattering from all directions is about Tc ≈ 0.423/fd. A symbol much shorter than Tc sees a channel that is nearly fixed during the symbol; the receiver must still track it from one packet or slot to the next.

fd = v fc / c,    Tc ≈ 0.423 / fd

## Worked Example 5.2

Problem. Find the maximum Doppler shift and coherence time at 30 km/h and 2.45 GHz, and at 120 km/h and 3.5 GHz.

Solution. 30 km/h is 8.33 m/s, so fd = 8.33 × 2.45 × 10⁹/2.998 × 10⁸ = 68.1 Hz and Tc = 0.423/68.1 = 6.2 ms. 120 km/h is 33.3 m/s, so fd = 389 Hz and Tc = 1.09 ms. A 5G slot at 30 kHz subcarrier spacing lasts 0.5 ms, comfortably inside the faster case, which is one reason the pilot signals in a slot are enough to track the channel.

## 5.3 The cost of Rayleigh fading

With many paths and no dominant one, the received amplitude follows a Rayleigh distribution and the power an exponential one. A link at a given average SNR then spends part of its time in deep fades, and those fades dominate its error rate. For BPSK with average Eb/N0 = γ, the bit error rate becomes ½[1 − √(γ/(1 + γ))], which for large γ is about 1/(4γ). It falls only tenfold for every 10 dB of SNR, instead of with the waterfall of Chapter 4.

BER(Rayleigh, BPSK) = ½ [1 − √(γ/(1 + γ))] ≈ 1/(4γ)

## Worked Example 5.3

Problem. What average Eb/N0 does BPSK need for a bit error rate of 10⁻³ in Rayleigh fading? Compare it with the AWGN channel.

Solution. Solving the exact expression gives γ = 24.0 dB; the approximation 1/(4γ) = 10⁻³ gives γ = 250, also 24.0 dB. On the AWGN channel the same error rate needed 6.8 dB, so fading has cost 17 dB, fifty times the power. No amount of extra transmitter power is a sensible answer to that; diversity is.

## 5.4 Diversity

Diversity gives the receiver several copies of the signal that fade independently. Antennas spaced half a wavelength or more apart, different frequencies, different times, and coding across them all provide it. With L independent branches combined by maximal-ratio combining, which weights each branch by its own amplitude and phase, the error rate at high SNR falls as the L-th power of SNR. For BPSK at 10⁻³, the average SNR needed per branch is 24.0 dB with one branch, 11.1 dB with two, and 4.0 dB with four. The second antenna is worth about 13 dB. Coding and interleaving across subcarriers in OFDM, and space-time coding across transmit antennas, deliver the same kind of gain.

Key idea. In a fading channel, the error rate is set by how often the signal is in a deep fade, not by its average strength. Two independently fading copies are rarely both in a fade at once, so diversity buys more than ten decibels, far more than power can.

## Practice

1. Find the coherence bandwidth for an outdoor delay spread of 1 µs.

2. Find the maximum Doppler shift at 60 km/h and 5.9 GHz.

3. Find the coherence time for the Doppler shift of the previous problem.

4. Using the approximation 1/(4γ), what average Eb/N0 gives a Rayleigh BPSK bit error rate of 10⁻⁴?

5. Two receive antennas are combined by maximal-ratio combining. From Section 5.4, how much less SNR per branch is needed than with one antenna for 10⁻³?

6. Why does a 20 MHz signal suffer frequency-selective fading in a room where a 300 kHz signal does not?

# Chapter 6. OFDM: Many Slow Carriers Instead of One Fast One

Chapter 5 showed that a wideband signal in a multipath channel sees a frequency response full of notches, and that a single fast carrier would need a long and power-hungry equalizer to undo them. Orthogonal frequency-division multiplexing takes the opposite approach. It divides the band into many narrow subcarriers, each slow enough to see a flat channel, and sends a separate QAM symbol on each. A fast Fourier transform builds and separates them, a short guard interval absorbs the echoes, and equalization becomes one complex division per subcarrier. Wi-Fi, 4G, 5G, digital television, and DSL all use it.

## 6.1 Orthogonal subcarriers

N subcarriers spaced Δf apart are orthogonal over a symbol of duration T = 1/Δf: over that interval each completes a whole number of cycles more than its neighbor, so their product integrates to zero. An inverse FFT of N QAM values produces exactly such a sum, sampled at N Δf. The receiver's FFT separates them again without any analog filter. Each subcarrier's spectrum is a sinc that overlaps its neighbors, but at the center of each subcarrier all the others pass through zero, so there is no interference.

Δf = fs / N,    T = 1 / Δf

## Worked Example 6.1

Problem. The original 802.11a/g Wi-Fi uses a 64-point FFT at 20 MS/s with a 0.8 µs guard interval, and 48 data subcarriers plus 4 pilots. Find the subcarrier spacing, the useful symbol time, the total symbol time, the guard overhead, and the top rate with 64-QAM at code rate 3/4.

Solution. Δf = 20 MHz/64 = 312.5 kHz, so T = 3.2 µs, and with the guard the symbol is 4.0 µs. The guard takes 0.8/4.0 = 20% of the time, a loss of 10 log₁₀(1/0.8) = 0.97 dB. The 52 used subcarriers occupy 52 × 312.5 kHz = 16.25 MHz of the 20 MHz channel. The rate is 48 × 6 bits × 3/4 / 4.0 µs = 54 Mbit/s, the familiar top figure of 802.11a/g.

## 6.2 The cyclic prefix

The guard interval is filled with a copy of the end of the symbol, a cyclic prefix. As long as every echo arrives within the prefix, the receiver's FFT window contains a whole number of cycles of every delayed copy, and the channel's effect on each subcarrier is a single complex gain, H(k). The receiver measures H(k) from known pilots and divides it out: one complex division per subcarrier replaces the long equalizer. The 0.8 µs Wi-Fi prefix covers path differences up to 0.8 µs × c = 240 m, ample indoors. The cost is the prefix overhead and the fact that subcarriers in a notch carry little; coding across subcarriers, Chapter 7, recovers that loss as frequency diversity.

Key idea. OFDM turns a frequency-selective channel into N flat ones. The cyclic prefix makes the channel's convolution circular, so the FFT diagonalizes it, and each subcarrier needs only one complex gain to equalize.

## 6.3 Wi-Fi 6 and 7, and 5G numerology

Later Wi-Fi generations kept the idea and changed the numbers. 802.11ax and 802.11be use a spacing of 78.125 kHz, a quarter of the original, so the useful symbol is 12.8 µs, and guard intervals of 0.8, 1.6, or 3.2 µs; with the 0.8 µs guard the symbol is 13.6 µs. The longer symbol makes the fixed guard a smaller fraction of the time. 5G NR instead defines a family of numerologies with spacing Δf = 15 × 2^µ kHz for µ = 0 to 4 and beyond, keeping 14 symbols per slot, so a slot lasts 1/2^µ ms.

The useful symbol times are 66.7, 33.3, 16.7, 8.33, and 4.17 µs for spacings of 15, 30, 60, 120, and 240 kHz. The normal cyclic prefix at 15 kHz is 144 samples at 30.72 MS/s, 4.69 µs, and it shrinks in the same proportion as the symbol. Wide spacings tolerate Doppler and phase noise, which matter at millimeter wave; narrow spacings tolerate long delay spreads, which matter in large outdoor cells. A 100 MHz carrier at 30 kHz uses 273 resource blocks of 12 subcarriers, 3276 in all, occupying 98.28 MHz, and a 4096-point FFT at 122.88 MS/s.

## Worked Example 6.2

Problem. Find the slot length and useful symbol time for 5G numerology µ = 1 and µ = 3, and say which would be used in a 28 GHz cell.

Solution. µ = 1 gives 30 kHz spacing, a 33.3 µs useful symbol, and a 0.5 ms slot. µ = 3 gives 120 kHz, 8.33 µs, and a 0.125 ms slot. At 28 GHz the Doppler shift at a given speed is eight times that at 3.5 GHz and oscillator phase noise is worse, so the wider 120 kHz spacing is used; the shorter prefix is acceptable because the narrow beams of millimeter-wave arrays cut the delay spread.

## 6.4 Peak-to-average power

An OFDM signal is the sum of many independent subcarriers, so by the central limit theorem its samples are nearly Gaussian, with occasional large peaks. The peak-to-average power ratio is the price OFDM pays, and the transmitter's power amplifier pays it in back-off and efficiency, a subject Book 6 owns. The theoretical maximum is 10 log₁₀N, 18.1 dB for 64 subcarriers, but it almost never occurs. Under the Gaussian approximation, the probability that a symbol's PAPR exceeds z is about 1 − (1 − e^(−z))^N. The level exceeded by one symbol in a thousand is 10.4 dB for N = 64, 11.0 dB for 256, and 11.6 dB for 2048. The figure grows only slowly with N. Clipping and filtering, tone reservation, and the DFT-spread OFDM of the 4G and 5G uplink reduce it.

P(PAPR > z) ≈ 1 − (1 − e^(−z))^N

## Practice

1. An OFDM system uses a 1024-point FFT at 15.36 MS/s. Find the subcarrier spacing and useful symbol time.

2. Find the guard overhead in percent for a 12.8 µs symbol with a 3.2 µs guard.

3. What is the longest path difference a 4.69 µs cyclic prefix can absorb?

4. Find the subcarrier spacing, useful symbol time, and slot length of 5G numerology µ = 2.

5. Find the theoretical maximum PAPR for 256 subcarriers.

6. Explain why a receiver needs only one complex multiplication per subcarrier to equalize an OFDM symbol.

# Chapter 7. Coding and Multiple Antennas

The modulations of Chapter 4 are several decibels from Shannon's limit, and the fading channel of Chapter 5 is far worse. Two techniques close most of the gap. Error-correcting codes add structured redundancy so the receiver can repair errors, and they recover most of the distance to the limit. Multiple antennas at both ends open several parallel paths through the same bandwidth, and they raise the limit itself. This chapter shows what each buys and finishes with the calculation behind Wi-Fi 7's headline figure.

## 7.1 What a code does

A code of rate R = k/n maps k information bits to n coded bits. The extra n − k bits let the decoder detect and correct errors, at the cost of a factor R in throughput or a factor 1/R in bandwidth. The simplest example is the Hamming (7,4) code: four data bits and three parity bits, any single error in the seven corrected. With a raw bit error rate of 10⁻², the probability that a 7-bit block contains two or more errors, and so is not corrected, is 2.0 × 10⁻³. Codes are judged by their coding gain, the reduction in the Eb/N0 needed for a target error rate, with the rate loss already counted because Eb is energy per information bit.

## 7.2 Convolutional, turbo, and LDPC codes

For decades the standard was the rate-1/2 convolutional code with constraint length 7, decoded by the Viterbi algorithm. It gives a coding gain of about 5 dB at a bit error rate of 10⁻⁵ and is still in the 802.11 standard. Turbo codes, announced in 1993, and low-density parity-check codes, invented by Gallager in 1962 and rediscovered in the 1990s, decode iteratively and come within about a decibel of the limit for long blocks. LDPC codes are used in Wi-Fi from 802.11n onward, in DVB-S2, and for the 5G data channels; 5G uses polar codes for its control channels. The limit itself, for a spectral efficiency η, requires Eb/N0 of at least (2^η − 1)/η.

Eb/N0 ≥ (2^η − 1) / η

## Worked Example 7.1

Problem. Find the minimum Eb/N0 for spectral efficiencies of 1, 2, 6, and 10 bit/s/Hz.

Solution. For η = 1, (2 − 1)/1 = 1, or 0 dB. For η = 2, 3/2 gives 1.76 dB. For η = 6, 63/6 gives 10.2 dB. For η = 10, 1023/10 gives 20.1 dB. As η falls toward zero the bound tends to ln 2, the −1.59 dB of Chapter 1. Uncoded BPSK needs 9.6 dB for a bit error rate of 10⁻⁵ at η = 1, so a good code on such a link has up to about 9.6 dB to recover; practical LDPC codes recover about 8.

Key idea. Coding moves a link toward Shannon's limit, and multiple antennas move the limit. Modern links use both: an LDPC code within a decibel or two of capacity, on top of several spatial streams.

## 7.3 MIMO

With Nt transmit and Nr receive antennas in a rich scattering environment, the channel is a matrix H, and the link can carry up to min(Nt, Nr) independent spatial streams through the same bandwidth. The capacity is the sum over the channel's eigenmodes. In the ideal case of equal, independent eigenmodes and the transmit power split equally, an N × N link at total SNR ρ has a capacity of N log₂(1 + ρ/N) bit/s/Hz. At ρ = 20 dB this is 6.66 bit/s/Hz for one antenna at each end, 11.3 for 2 × 2, and 18.8 for 4 × 4. One transmit antenna with four receive antennas gives only log₂(1 + 4ρ) = 8.6: receive combining raises SNR, but only spatial multiplexing multiplies the number of streams.

C ≈ N log₂(1 + ρ/N)  bit/s/Hz  (N × N, ideal)

Real channels have correlated antennas and unequal eigenmodes, and line-of-sight links with closely spaced antennas may support only one useful stream. Multiple antennas can also be used for diversity, as in the Alamouti space-time code, or for beamforming toward one user or several users at once, which is multiuser MIMO; the arrays themselves are Book 4's subject.

## Worked Example 7.2

Problem. An 802.11ax (Wi-Fi 6) link uses 80 MHz, which has 980 data subcarriers, 1024-QAM with rate-5/6 coding, and a 13.6 µs symbol. Find its rate per spatial stream. Find the 160 MHz, eight-stream peak.

Solution. Per stream, 980 × 10 bits × 5/6 / 13.6 µs = 600.5 Mbit/s, the figure printed on most Wi-Fi 6 adapters as 600 Mbit/s for one stream. At 160 MHz there are 1960 data subcarriers, and with eight streams the rate is 1960 × 10 × 5/6 / 13.6 µs × 8 = 9.61 Gbit/s, Wi-Fi 6's quoted peak of 9.6 Gbit/s.

## Worked Example 7.3

Problem. Wi-Fi 7 (802.11be) uses 320 MHz channels with 3920 data subcarriers, 4096-QAM at rate 5/6, a 13.6 µs symbol, and up to eight spatial streams. Find its paper peak rate.

Solution. 4096-QAM carries 12 bits, so per stream the rate is 3920 × 12 × 5/6 / 13.6 µs = 2.88 Gbit/s. Eight streams give 23.06 Gbit/s, the "about 23 Gbit/s" of the specifications. It is a paper figure. It needs a clean 320 MHz channel, which exists only in the 6 GHz band of Chapter 8, an SNR above about 38 dB on every stream, and eight antennas at each end; a phone with two antennas reaches a quarter of it at best.

## Practice

1. A rate-3/4 code is used on a 40 Mbit/s coded stream. What is the information rate?

2. Find the minimum Eb/N0 for 4 bit/s/Hz.

3. Find the ideal capacity of a 2 × 2 link at 30 dB total SNR and compare it with a 1 × 1 link at the same SNR.

4. Find the Wi-Fi 6 rate for one spatial stream in 20 MHz, which has 234 data subcarriers, with 1024-QAM at rate 5/6.

5. How many spatial streams can a 4 × 2 link carry?

6. Why does a 1 × 4 link gain less capacity at high SNR than a 4 × 4 link?

# Chapter 8. The 2026 Band Plan and the Software-Defined Receiver

The previous chapters built a waveform without asking where in the spectrum it would go. This chapter answers that question for the systems most readers will build or test in 2026, Wi-Fi and cellular, and then follows the signal into the receiver that turns it back into bits. The receiver is described as a software-defined radio, the architecture in which a fixed analog front end feeds a converter and everything else is code, because that is both how commercial chips are now built and how a student can experiment with every idea in this book. Emission masks and the test chamber belong to Book 7, and the power amplifier and synthesizer to Book 6.

## 8.1 Unlicensed and licensed spectrum

Wi-Fi uses unlicensed spectrum: anyone may transmit, within power and etiquette rules, and the cost is that nobody is protected from anyone else. Cellular uses licensed spectrum, bought at auction for exclusive use in a region, plus some unlicensed operation called NR-U. The two share physics and do not share regulators. The 2.4 GHz band, 2400 to 2483.5 MHz, is 83.5 MHz wide and holds only three non-overlapping 20 MHz channels, 1, 6, and 11, shared with Bluetooth, microwave ovens, and much else. It is crowded. The 5 GHz band, with several hundred megahertz depending on the country, is the workhorse of Wi-Fi 5 and 6, with 80 and 160 MHz channels.

## 8.2 The 6 GHz band

Where it is fully open, as in the United States, the 6 GHz band runs from 5925 to 7125 MHz, 1200 MHz in all. A 320 MHz channel fits three times, with 240 MHz left over for narrower channels. Many countries, including those of the European Union, opened only the lower part, about 500 MHz; a slice of that size holds one 320 MHz channel. Indoor low-power operation needs no coordination. Standard-power operation, including outdoors, is permitted under an automated frequency coordination system that checks a database of incumbent microwave links before allowing a device to transmit. The band is the reason Wi-Fi 7's 320 MHz channels, and so the 23 Gbit/s of Chapter 7, are possible at all. Wi-Fi 8, the 802.11bn project expected around 2028, aims at reliability and latency rather than another doubling of the constellation or the channel.

## Worked Example 8.1

Problem. How many 320 MHz and 160 MHz channels fit in the full 6 GHz band, and how many 320 MHz channels in a 480 MHz national allocation?

Solution. 1200/320 = 3.75, so three 320 MHz channels fit with 1200 − 960 = 240 MHz left, and the remainder holds one 160 MHz channel and one 80 MHz channel. Of 160 MHz channels alone, 1200/160 = 7.5, so seven fit. A 480 MHz allocation holds one 320 MHz channel with 160 MHz left. In practice the standard fixes the channel centers, and the 320 MHz channels may overlap in two staggered sets, but the counts of non-overlapping channels are as calculated.

## 8.3 The cellular ranges

5G NR defines two frequency ranges: FR1, from 410 MHz to 7.125 GHz, and FR2, from 24.25 to 71 GHz, the millimeter-wave range, itself split at 52.6 GHz. 5G-Advanced, the 3GPP releases from 18 onward, extracts more from those same bands rather than adding new ones. The research toward 6G around 2030 targets the gap between them, 7.125 to 24.25 GHz, sometimes called FR3, where wider channels than FR1 are possible with better coverage than FR2; candidate bands in that range are on the agenda of the 2027 World Radiocommunication Conference. Sub-terahertz frequencies of roughly 90 to 300 GHz remain a study item, not a 2026 phone band. Wi-Fi at 6 GHz and licensed cellular in the upper 6 GHz range sit next to each other in some countries, and power limits and coordination are how they share.

Key idea. The waveform is this book; the license is the band table. A design starts by asking which band, how wide a channel, and under what rules, and the answers fix the symbol rate and constellation before a line of code is written.

## 8.4 Receiver architectures

Four architectures cover nearly all receivers. The superheterodyne mixes to a fixed intermediate frequency, filters there sharply, and then converts to baseband; it has the best selectivity and the most parts. The zero-IF, or direct-conversion, receiver mixes straight to complex baseband with a quadrature mixer. It is cheap and integrates well, and it suffers from DC offset, flicker noise, and the I/Q imbalance of Chapter 2. The low-IF receiver mixes to a small offset to avoid the DC problems and relies on digital image rejection. Direct-RF sampling digitizes the antenna signal, after filtering and amplification, with a multi-gigasample converter; the clock jitter limit of Chapter 2 is its constraint. Most phones and Wi-Fi chips use zero-IF or low-IF; laboratory SDRs such as the ADALM-Pluto use a zero-IF transceiver chip with 12-bit converters.

## 8.5 Sensitivity

A receiver's sensitivity is the weakest signal it can demodulate at a target error rate. It is set by the thermal noise floor, −174 dBm/Hz at room temperature, plus 10 log₁₀B for the bandwidth, plus the receiver's noise figure, plus the SNR the modulation and code require. The noise figure itself, and how it builds up through the chain of stages, is Book 6's subject.

Psens (dBm) = −174 + 10 log₁₀B + NF + SNRreq

## Worked Example 8.2

Problem. A 20 MHz receiver has a 7 dB noise figure. Find its noise floor, and its sensitivity for a mode that needs 25 dB of SNR.

Solution. 10 log₁₀(20 × 10⁶) = 73.0 dB, so the floor is −174 + 73.0 + 7 = −94.0 dBm. With 25 dB of required SNR the sensitivity is −69 dBm. The 802.11a standard requires −65 dBm for its 54 Mbit/s mode, so this receiver would pass with 4 dB of margin. The same receiver needs only about −90 dBm for BPSK at rate 1/2, which is why the rate falls step by step as a user walks away from an access point.

## 8.6 SDR as a laboratory

A software-defined radio lets every chapter of this book be tried on real signals. The PySDR online textbook and the GNU Radio framework are good laboratories, though they are not substitutes for this book's theory. A useful sequence of exercises is: capture a broadcast FM station and display its spectrum; decimate the capture and plot the eye of a known digital signal; build a raised-cosine transmitter and matched-filter receiver in loopback; measure EVM against transmit gain to find where the amplifier starts to compress; and decode the beacon frames of a Wi-Fi access point, which use the 312.5 kHz OFDM of Chapter 6. Transmitting requires care: outside licensed amateur bands or a shielded enclosure, the laboratory should be receive-only.

## Practice

1. How many non-overlapping 20 MHz channels fit in the 2.4 GHz band?

2. How many 80 MHz channels fit in 1200 MHz?

3. Name the upper and lower edges of 5G FR1 and FR2.

4. Find the noise floor of a 160 MHz receiver with a 5 dB noise figure.

5. The receiver in the previous problem needs 30 dB of SNR for 1024-QAM. Find its sensitivity.

6. Which receiver architecture is most affected by DC offset and I/Q imbalance, and why?

# Appendix A. Constants and Formulas Used in This Book

Shannon capacity: C = B log₂(1 + SNR); minimum Eb/N0 = ln 2 = −1.59 dB; for spectral efficiency η, Eb/N0 ≥ (2^η − 1)/η.

SNR = (Eb/N0)(Rb/B); Rb = Rs log₂M; passband bandwidth ≈ Rs(1 + α).

Ideal converter: SNR = 6.02N + 1.76 dB; processing gain 10 log₁₀(fs/B) for complex sampling; jitter limit −20 log₁₀(2πf tj).

Q(x) = ½ erfc(x/√2). BPSK/QPSK: BER = Q(√(2Eb/N0)). Square M-QAM: SER ≈ 4(1 − 1/√M) Q(√(3Es/((M − 1)N0))).

Es/N0 for an uncoded symbol error rate of 10⁻³: 4-QAM 10.35 dB, 16-QAM 17.63, 64-QAM 23.97, 256-QAM 30.08, 1024-QAM 36.14, 4096-QAM 42.17 dB.

EVM(%) = 100 × 10^(EVM(dB)/20); −38 dB = 1.26%.

Coherence bandwidth Bc ≈ 1/(5στ); Doppler fd = vfc/c; coherence time Tc ≈ 0.423/fd. Rayleigh BPSK BER = ½[1 − √(γ/(1 + γ))] ≈ 1/(4γ).

Wi-Fi (802.11a/g): 64-point FFT at 20 MS/s, 312.5 kHz, 3.2 µs + 0.8 µs guard. Wi-Fi 6/7: 78.125 kHz, 12.8 µs + 0.8 µs = 13.6 µs. 5G NR: Δf = 15 × 2^µ kHz, 14 symbols per slot, slot = 1/2^µ ms.

Wi-Fi 7 paper peak: 3920 × 12 × 5/6 / 13.6 µs = 2.88 Gbit/s per stream; × 8 streams = 23.06 Gbit/s.

Bands: 2.4 GHz ISM 2400–2483.5 MHz; 6 GHz 5925–7125 MHz (1200 MHz: three 320 MHz channels, 240 MHz left); 5G FR1 410 MHz–7.125 GHz, FR2 24.25–71 GHz; 6G research 7.125–24.25 GHz.

Thermal noise −174 dBm/Hz at 290 K; sensitivity = −174 + 10 log₁₀B + NF + SNRreq.

# Appendix B. Answers to the Practice Problems

## Chapter 1

1. Rb = 10 × 6 = 60 Mbit/s; B = 10 MHz × 1.2 = 12 MHz.

2. C = 5 MHz × log₂11 = 5 × 3.46 = 17.3 Mbit/s.

3. SNR = 8 dB + 10 log₁₀2 = 8 + 3.0 = 11.0 dB.

4. SNR = 2⁶ − 1 = 63, or 18.0 dB.

5. Eb/N0 = ln 2 = 0.693, or −1.59 dB.

6. Capacity grows in proportion to bandwidth but only with the logarithm of SNR, so at high SNR doubling the capacity by power alone requires roughly squaring the SNR, far more than doubling it.

## Chapter 2

1. 130 − 100 = 30 MHz, which is below 50 MHz, so the tone appears at 30 MHz.

2. 6.02 × 14 + 1.76 = 86.0 dB.

3. ENOB = (74 − 1.76)/6.02 = 12.0 bits.

4. 10 log₁₀(61.44/10) = 7.9 dB.

5. −20 log₁₀(2π × 10⁹ × 10⁻¹³) = 64.0 dB.

6. Each complex sample carries two independent real numbers, so the band from −fs/2 to +fs/2 is unambiguous; a real signal's spectrum is symmetric, so only 0 to fs/2 carries independent information.

## Chapter 3

1. 5 × 1.22 = 6.1 MHz.

2. 6/1.15 = 5.22 Msymbol/s.

3. The receive filter must be matched to the pulse to maximize SNR, and the overall response must be a Nyquist pulse; splitting the raised cosine equally satisfies both, while a full raised cosine at the transmitter would leave the receiver either unmatched or adding further filtering.

4. 2E/N0 = 2 × 10⁻⁹/2 × 10⁻¹² = 1000, or 30.0 dB.

5. Timing jitter; noise closes it vertically.

6. 0.510/0.197 = 2.6 times.

## Chapter 4

1. 9.9 × 10⁻⁴.

2. 30.08 − 23.97 = 6.1 dB.

3. Below, since 26 dB exceeds the 23.97 dB required.

4. 20 log₁₀(0.02) = −34.0 dB.

5. Peak power 18, average 10, so 1.8, or 2.6 dB.

6. QPSK is two independent BPSK signals in quadrature; each carries half the symbol energy and one bit, so Eb, noise per dimension, and the decision are the same as for BPSK.

## Chapter 5

1. 1/(5 × 1 µs) = 200 kHz.

2. 16.7 m/s × 5.9 × 10⁹/2.998 × 10⁸ = 328 Hz.

3. 0.423/328 = 1.29 ms.

4. γ = 2500, or 34.0 dB.

5. 24.0 − 11.1 = 12.9 dB.

6. The coherence bandwidth indoors is a few megahertz; the 20 MHz signal spans several coherence bandwidths and sees the notches, while the 300 kHz signal fits inside one and fades as a whole.

## Chapter 6

1. 15.36 MHz/1024 = 15 kHz; T = 66.7 µs.

2. 3.2/16.0 = 20%.

3. 4.69 µs × 2.998 × 10⁸ m/s = 1.41 km.

4. 60 kHz, 16.7 µs, and 0.25 ms.

5. 10 log₁₀256 = 24.1 dB.

6. The cyclic prefix makes the channel act as a circular convolution over the FFT window, so the FFT turns it into a separate complex gain on each subcarrier, which a single multiplication by its inverse removes.

## Chapter 7

1. 40 × 3/4 = 30 Mbit/s.

2. 15/4 = 3.75, or 5.74 dB.

3. 2 log₂(1 + 500) = 17.9 bit/s/Hz, against log₂1001 = 9.97 bit/s/Hz.

4. 234 × 10 × 5/6 / 13.6 µs = 143.4 Mbit/s.

5. min(4, 2) = 2.

6. Receive combining in a 1 × 4 link raises the SNR of a single stream by up to 6 dB, which adds about two bits per second per hertz, while a 4 × 4 link carries four streams and multiplies the capacity by about four.

## Chapter 8

1. Three: channels 1, 6, and 11.

2. 1200/80 = 15.

3. FR1 runs from 410 MHz to 7.125 GHz; FR2 from 24.25 to 71 GHz.

4. −174 + 82.0 + 5 = −87.0 dBm.

5. −87.0 + 30 = −57.0 dBm.

6. The zero-IF receiver, because it mixes the wanted signal directly to DC, where offsets and flicker noise sit, and depends on the quadrature mixer's two arms being matched to reject the image at the mirror frequency.

# Appendix C. Sources

Bernard Sklar, Digital Communications, 2nd edition, for modulation, error rates, and coding (Chapters 1, 3, 4, and 7).

Michael Rice, Digital Communications: A Discrete-Time Approach, for sampling, pulse shaping, and synchronization (Chapters 2 and 3).

Richard G. Lyons, Understanding Digital Signal Processing, 3rd edition, for the FFT, decimation, and complex signals.

C. Richard Johnson, William Sethares, and Andrew Klein, Software Receiver Design, and Travis Collins, Robin Getz, et al., Software-Defined Radio for Engineers, for Chapter 8; Steven Ellingson, Radio Systems Engineering, for receiver architecture and sensitivity.

Andrea Goldsmith, Wireless Communications, and David Tse and Pramod Viswanath, Fundamentals of Wireless Communication, for fading, diversity, OFDM, and MIMO (Chapters 5 to 7).

Qizheng Gu, RF System Design of Transceivers for Wireless Communications, and Behzad Razavi, RF Microelectronics, for zero-IF and I/Q imbalance.

C. E. Shannon, "A Mathematical Theory of Communication," Bell System Technical Journal, 1948. IEEE 802.11-2020 and the 802.11ax and 802.11be amendments; 3GPP TS 38.101 and 38.211 for 5G NR numerology and frequency ranges.

PySDR (Marc Lichtman, online) and the GNU Radio project, as laboratories for Chapter 8.