RF, Microwave, and Antennas

Book 4 of 9

Lothar J. Musiol

Copyright © 2026 Lothar J. Musiol. Nine-book series. Worked numbers are computed here. Book 2 is a separate merged manuscript. Do not paste Cripps, Pozar, Ott, or Sklar.

Book 4 of 9. Old 4, 8, 9, and 10. Millimeter wave is this book at a shorter wavelength.

# Chapter 1. One Microwave Book

Bowick is the on-ramp: matching, the Smith chart, a first filter, a first amplifier. Pozar, or Steer volumes 1 to 3, is the microwave text. Book 1 already taught the Smith chart. This book uses it.

Millimeter wave uses Maxwell's equations. What changes is the loss per millimeter, the waveguide size, the beamwidth, and how much the etch and the bond-wire count. There is no second mmWave volume.

Key idea. Shorter wavelength is not a new subject. It is the same subject with less forgiveness.

# Chapter 2. Lines and Matching

A TEM line is characterized by Z0 and delay. A 50 Ω system is a convention, not a law. A quarter-wave transformer matches R to Z0 when its own impedance is sqrt(R Z0).

Worked: match 25 Ω to 50 Ω. Zq = √(25×50) = 35.36 Ω. At 2.45 GHz, λ = 12.24 cm in free space. On a board with √er = √3.55 = 1.884, the guided wavelength is 6.49 cm, so a quarter-wave is 1.62 cm. Laminate numbers at 10 GHz are Book 9. This chapter only needs that RO4000-class material exists and FR-4 is not it at this frequency.

L-section matching and stubs are the rest of Bowick. Cavities and waveguide filters appear when microstrip dies from voltage and heat. Those high-power filters sit after a PA; the PA is Book 6.

# Chapter 3. The Antenna

Gain, beam, polarization, sidelobes, scan. An array's element spacing is about λ/2 if you do not want grating lobes in visible space. An active electronically scanned array puts a small amplifier at each element. That amplifier is still Book 6.

Friis: Pr/Pt = Gt Gr (λ/4πR)². At 2.45 GHz, λ = 0.1224 m. At R = 100 m, isotropic, the space factor is 9.482e-09, which is -80.2 dB. Antenna gains multiply that factor. Book 6 owns transmitter power and the statement that 1 dB of feed loss is 1 dB of EIRP.

Key idea. This book supplies Gt and Gr. Book 6 supplies Pt. Book 7 supplies the legal mask on what left the antenna.

# Chapter 4. Arrays and mmWave

A 28 GHz wavelength is 10.7 mm. A 77 GHz automotive radar wavelength is 3.9 mm. Patch antennas become millimeters. Bond wires become inductors you cannot ignore. PTFE or RO3003-class boards, thin copper, tight etch: specified in Book 9, used here.

Practice. 1. Repeat the quarter-wave example at 10 GHz on the same er. 2. A 10 dBi antenna at each end of the 100 m link: by how many decibels does received power rise? 3. Why is mmWave not its own book in this series?